3GPP TSG RAN WG1 Meeting #85	R1-164041

Nanjing, China, 23 - 27 May 2016

Agenda Item:	7.1.2

Source:	Huawei, HiSilicon

Title:	Phase noise model for above 6 GHz

Document for:	Discussion and decision

# Introduction

At carrier frequency above 6 GHz, phase noise becomes significant and could result in performance degradations if not considered properly. Careful attention to the system design is needed to mitigate this effect. In the Chairman’s Notes of the RAN1 meeting #84bis [1], it was agreed that:

- All companies are requested to analyze/evaluate following aspects

  - Realistic phase noise

In this contribution, we review some RF circuit configurations, discuss the RF material properties and outline a phase noise model to be considered and utilized for the design and evaluation of the new radio in the bands above 6 GHz.

# General Considerations on Phase Noise Model

In several previous 3GPP submissions, different phase noise models have been considered, including: one-pole-one-zero model [2], one-pole model [3], phase-locked-loop (PLL) based model [4] and others [5]. To better understand the modelling of phase noise, the configurations of RF circuits and the fabrication methods and materials need to be taken into consideration.

## Configuration of RF Circuits

For the new radio in bands above 6 GHz, a practical design approach is to utilize a large antenna array, with significant beam-forming gain to overcome the path loss. Such an antenna array driven by multiple RF chains has the following advantages:

- To form multiple beams and transmit towards different users with spatial division;

- To transmit multiple streams towards a single user using MIMO transmission;

- To overcome the limitation on the maximum number of antenna elements supported by a single chip.

The structure of the RF circuitry used to drive the antenna array becomes important when considering the effects of phase noise. Figure 1 illustrates a practical antenna array and driver configuration with a common oscillator driving separate multiplier units for each antenna grouping.

Figure 1. Multiple partially coherent RF-chains sharing a common oscillator but different multipliers

This is a partially coherent RF structure with a shared common oscillator but with different multipliers for each group of antenna elements. In theory, a multiplier with a factor of N degrades the phase noise by 20 x log10(N) (dB).  Overall, the output phase noise level depends on the performance of the common oscillator and the multiplier factor (N). This configuration has been discussed also in the IEEE 802.11ay task group [6]. In the arrangement depicted in Figure 1, the major component of the phase noise from the oscillator will be common to different RF chains with the addition of independent additive noise introduced by the multipliers. The following advantages are achieved by such a RF structure:

- Reduced circuit cost and feeder loss [7] with multiple RF chains sharing one low-frequency oscillator but different multipliers;

- Maintaining some correlation among the phase noise of the different RF chains which may facilitate some phase noise suppression at the receiver side.

With these advantages, we suggest the following observations:

Observation 1: RF structures with partially coherent phase noise (such as the one depicted in Figure 1) should be considered, at least for the BS side.

Observation 2: A partially coherent phase noise model should be considered in the design and evaluation of the new radio in the bands above 6 GHz.

## Fabrication Methods and Materials

While there are many different fabrication methods, the most common fabrication materials are CMOS, GaAs, SiGe and GaN. With a review of the state of the art [8]-[25], a summary on the phase noise level achieved by different fabrication methods and materials is given in Figure 2.  From this it can be seen that:

Observation 3: For 30GHz band, the typical phase noise level measured at 1 MHz offset is from -114 to -93 dBc/Hz, while that for 70 GHz band is from -108 to -81 dBc/Hz.

While GaAs-based devices can provide a lower phase noise level, it is still expensive and power-consuming. The CMOS-based devices are available at lower cost and have less power consumption. Taking the cost and power constraint at the UE side into consideration, it appears reasonable to assume CMOS-based design for the UE side. On the other hand, for the BS side, GaAs-based design may be assumed.

Figure 2. A brief summary of the phase noise level achieved by different fabrication methods and materials

Observation 4: The phase noise model should consider constraints on the cost and energy consumption, especially for the UEs, when using various technologies.

# Proposed Model and Parameters

As discussed in Section 2, it is beneficial to use a RF structure with partially coherent phase noise and practical materials for the UE and BS. The proposed detailed phase noise modelling and parameters are outlined further in this section.

## Proposed Model

In the time domain, the phase noise of the i-th RF chain in Figure 1 can be expressed as

Yit=At+nit+Ci

where nit is the independent additive noise at the i-th RF chain brought by the multiplier. Ci is a randomly generated, but constant, phase offset of the i-th RF chain. At is the coherent component, which can be interpreted as the output of the oscillator with a perfect multiplier, the PSD of which will be described in the next subsection.

Here nit is employed to model the white and flicker noise in the multipliers [26]. As shown in Figure 3, the PSD of nit can be described by two parameters, the corner frequency f0 and the noise floor at infinity PSDinf, where a typical value of f0 is 10kHz and PSDinf will be described in the next subsection.

Figure 3. Typical PSD of additive noise from multipliers

As for how many partially coherent phase noise should be modeled, given the numbers of transmit and receive antenna elements (up to 256*32 @ 30 GHz and 1024*64 @ 70 GHz), {4, 8} and {2, 4} would be reasonable working assumptions for the BS and UE side, respectively.

## Suggested Parameters

In this contribution, we utilize the PLL-based phase noise model defined in [4] to express the phase noise experienced at each RF chain, i.e., Yit in eq. ⑴. To be specific, the PSD of the phase noise is characterized by:

STotalf=SReff+SPLLf,   when f≤loop BWSVCO_ v2f+SVCO_v3(f),    when f>loop BW

where

SRef / PLL / VCO_v2 / VCO_v3= PSD0∙1+ffzk1+fk (dB)

PSD0=FOM+20logfc-10logP1 mW (dB)

FOM is the figure of merit, fc is the carrier frequency and P is the consumed power. After reviewing the recent literature [27]-[35], based our expectation for the phase noise level achievable with reasonable cost and power consumption, we propose to consider the following parameters for the phase noise model at the UE (CMOS-based) and BS (GaAs-based) side, respectively (see Table 1):

Table 1. Parameters for proposed phase noise models

|  | Model 1, UE, Loop BW = 187kHz |  |  |  | Model 2, BS, Loop BW = 112kHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | REF clk | PLL | VCO V2 | VCO V3 | REF clk | PLL | VCO V2 | VCO V3 |
| FOM | -215 | -240 | -175 | -130 | -240 | -245 | -187 | -130 |
| fz | Inf | 1.00E+04 | 50.30E+06 | Inf | Inf | 1.00E+04 | 8.00E+06 | Inf |
| P (mW) | 10 | 20 | 20 |  | 10 | 20 | 50 |  |
| k | 2 | 1 | 2 | 3 | 2 | 1 | 2 | 3 |

For a single RF chain, the PSD of the proposed phase noise models at both UE and BS side for 30 GHz and 70 GHz are depicted in Figure 4. Other companies’ proposals (scaled to the target frequency) [2]-[5] are also shown in these diagrams for reference. As discussed in Section 2.2, for the BS side, the GaAs-based model could be applied to approximate the relatively lower level of phase noise. Similarly, to support low-cost and energy-efficient devices, the CMOS-based model appears as a more appropriate assumption for the UE side. For instance, as can be seen in Figure 4 and Table 1, while a low phase noise level can be achieved by GaAs, the power consumption is also higher.

To facilitate the generation of multiple partially coherent phase noise, we point out that the noise floor at infinity PSDinf in Figure 3 and Figure 4 is frequency-dependent and can be read from the scaled PSD. Furthermore, the coherent component At in Eq. ⑴ can be generated by removing the noise floor in Figure 4 and extending the curve to infinity, maintaining the same slope.

Figure 4. PSD of proposed phase noise model at both UE and BS side (fc=30, 70 GHz)

In line with the above observation on the variability of devices performance it seems that any phase noise model falling in the range between the models proposed for UE and BS in Table 1 would be a reasonable working assumption.

Proposal 1: The phase noise models described in Table 1 are adopted for the design and evaluation of the new radio for above 6GHz.

# Conclusion

Based on the discussions in this paper, for the phase noise model above 6 GHz, we have the following observations and proposal:

Observation 1: RF structures with partially coherent phase noise (such as the one depicted in Figure 1) should be considered, at least for the BS side.

Observation 2: A partially coherent phase noise model should be considered in the design and evaluation of the new radio in the bands above 6 GHz.

Observation 3: For 30GHz band, the typical phase noise level measured at 1 MHz offset is from -114 to -93 dBc/Hz, while that for 70 GHz band is from -108 to -81 dBc/Hz.

Observation 4: The phase noise model should consider constraints on the cost and energy consumption, especially for the UEs, when using various technologies.

Proposal 1: The phase noise models described in Table 1 are adopted for the design and evaluation of the new radio for above 6GHz.

# References

3GPP, TSG RAN WG1 Meeting #84bis, “RAN1 Chairman’s Notes”, Apr. 2016.

3GPP, R1-163113, NTT DOCOMO, “Initial views on numerology for new radio access technology”, Apr. 2016.

3GPP, R1-162386, Intel, “Numerology for new radio interface”, Apr. 2016.

3GPP, R1-162885, Nokia, Alcatel-Lucent Shanghai Bell, “On the phase noise model for 5G New Radio evaluation”, Apr. 2016.

3GPP, R1-163227, Ericsson, “Numerology for NR”, Apr. 2016.

K. Zeng et. al., “Considerations on phase noise model for 802.11ay”, IEEE Document 802.11-16-0390r0, Mar. 2016.

Rogers Corporation, “RO4000® series high frequency circuit materials”.

N. Pohl et. al., “A low-power 80 GHz FMCW radar transmitter with integrated 23 GHz downconverter VCO”, IEEE Bipolar/BiCMOS Circuits Tech. Meeting, Oct. 2011.

J. Lee et. al., “A dual-band VCO using inductor splitting for automotive radar system at W-band”, Int. Symp. Integrated Circuits, Dec. 2014.

C. Ameziane et. al., “79GHz push-push oscillators in 0.13μm SiGe BiCMOS technology”, IEEE Int. Conf. Elec., Circuits Systems, Aug. 2008.

C. Ameziane et. al., “An 80GHz range synchronized push-push oscillator for automotive radar application”, IEEE Radio Freq. Integrated Circuits Symp., May 2010.

Z. Liu et. al., “A 70GHz VCO with 8GHz tuning range in 0.13um CMOS technology”, European Microwave Integrated Circuit Conf., Oct. 2008.

D. D. Kim et. al., “A 70GHz manufacturable complementary LC-VCO with 6.14GHz tuning range in 65nm SOI CMOS”, IEEE Int. Solid-State Circuits Conf., Feb. 2007.

W. Fei et. al., “Design and analysis of wide frequency-tuning-range CMOS 60 GHz VCO by switching inductor loaded transformer”, IEEE Trans. Circuits Systems I, Oct. 2013.

A. Chakraborty et. al., “A low-phase-noise monolithically integrated 60 GHz push push VCO for 122 GHz applications in a SiGe bipolar technology”, IEEEE Bipolar/BiCMOS Circuits Tech. Meeting, Sep. 2013.

T. Y. Lu et. al., “Wide tunning range 60 GHz VCO and 40 GHz DCO using single variable inductor”, IEEE Trans. Circuits Systems I, Oct. 2012.

T. Nakamura et. al., “A push-push VCO with 13.9-GHz wide tuning range using loop-ground transmission line for full-band 60-GHz transceiver”, IEEE J. Solid-State, Apr. 2012.

Y. C. Chiang et. al., “60 GHz CMOS VCO using a fourth-order resonator”, IEEE Microwave Wireless Components Lett., Jul. 2015.

S. P. Sah et. al., “An 8th sub-harmonic injection locked V-band VCO for low power LO routing in mm-Wave beamformers”, IEEE Radio Freq. Integrated Circuits Symp., Jun. 2014.

X. Lan et. al., “A V-Band monolithic AlGaN/GaN VCO”, IEEE Microwave Wireless Components Lett., Jun. 2008

Y. H. Chang et. al., “A V-Band push-push VCO with wide tuning range using 0.18UM CMOS process”, IEEE Microwave Wireless Components Lett., Dec. 2014.

R. Shu et. al., “A 40 GHz CMOS VCO with resonated negative-conductance cell”, IEEE Int. Symp. Radio-Frequency Integration Tech., Nov. 2012.

J. C. Chien et. al., “A 40-GHz wide-tuning-range VCO in 0.18-μm CMOS”, Symp. VLSI Circuits, 2006.

H. D. Lee et. al., “A low-power 50-GHz LC-VCO in a 65-nm CMOS technology”, Asia-Pacific Microwave Conf., Dec. 2015.

T. Xi et. al., “Low-phase-noise 54GHz quadrature VCO and 76GHz/90GHz VCOs in 65nm CMOS process”, IEEE Radio Freq. Integrated Circuits Symp., Jun. 2014.

J. Breitbarth et. al., “Additive (residual) phase noise measurement of amplifiers, frequency dividers and frequency multipliers”, Microwave J., 2008.

W. Fei et. al., “A 96-GHz oscillator by high-Q differential transmission line loaded with complementary split-ring resonator in 65-nm CMOS”, IEEE Trans. Circuits Systems II, Mar. 2013.

F. M. De Paola et. al., “A 71–73 GHz voltage-controlled standing-wave oscillator in 90 nm CMOS technology”, European Solid-State Circuits Conf., Sep. 2008.

H. Li et. al., “Wide-band VCOs in SiGe production technology operating up to about 70GHz”, IEEE Microwave Wireless Comp. Lett., Oct. 2003.

R. Weber et. al., “A novel tuning concept for wideband VCOs based on a shunt-FET”, Microwave Integrated Circuits Conf. Sep. 2009.

T. Nakamura et. al., “A 59GHz push-push VCO with 13.9GHz tuning range using loop-ground transmission line for a full-band 60GHz transceiver”, Int. Solid-State Circuits Conf., Feb. 2009.

H. L. Kao et. al., “Low phase noise V-band push –push voltage controlled oscillator using 0.15 um GaAs pseudomorphic high electron-mobility transistor technology”, IET Microwaves, Antennas & Propagation, Apr. 2012.

H. L. Kao et. al., “A low phase noise Ka-band voltage controlled oscillator using 0.15 μm GaAs pHEMT technology”, Int. Symp. Design Diagnostics of Elec. Circuits & Systems, Apr. 2012.

D. D. Kim et. al, “A 70GHz manufacturable complementary LC-VCO with 6.14GHz tuning range in 65nm SOI CMOS”, IEEE Int. Solid-State Circuits Conf., Feb. 2007.

L. Li et. al., “Design and analysis of a 90 nm mm-wave oscillator using inductive-division LC tank”, IEEE J. Solid-State Circuits, Jul. 2009.