3GPP TSG RAN WG1 Meeting #86						R1-168415

Gothenburg, Sweden, 22 - 26 Aug 2016

Source: 			Ad-Hoc chair (Intel)

Title:		Chairman's notes of AI 7.2.14 on Study on Further enhancements to Coordinated Multi-Point Operation

Agenda item:	7.2.14

Document for:		Endorsement

### Study on Further enhancements to Coordinated Multi-Point Operation

SID in RP-160954

R1-166335	TP on TR 36.741 Study on FeCoMP	ZTE Corporation, Intel Corporation

Agreement:

- Endorse TP for TR36.741 in R1-166335

R1-167622	TP for TR36.741 on Section 5	ZTE Corporation, Intel Corporation

R1-167648	feCoMP Motivations and Techniques	Ericsson

#### Remaining details of the deployment scenarios and system-level evaluation assumptions

R1-166331	Remaining details of evaluation assumptions for FeCoMP	ZTE Corporation, ZTE Microelectronics

R1-167647	feCoMP evaluation assumption omissions and errata	Ericsson

R1-166544	Remaining details of evaluation assumptions	Intel Corporation

R1-166709	Remaining details for simulation assumptions for feCoMP	Samsung

R1-167088	Simulation assumptions related to new measurements and LTE architecture	IAESI, Thales, Fairspectrum

R1-166316	Remaining details of evaluation assumptions	Qualcomm Incorporated

R1-168253	WF on further details of simulation assumptions for FeCoMP, ZTE Corporation, ZTE Microelectronics, Intel, ASTRI, Qualcomm, Ericsson, Nokia, Alcatel-Lucent Shanghai Bell, Samsung

Agreement:

- Add the following details into the simulation assumptions:

  - Performance Metric / Parameter

    - Mean, 5%, 50%, 95% user throughput

    - Served cell throughput

    - Resource utilization (RU)

    - Packet arrival rate λ

  - Overhead

    - 3 symbols for DL CCHs, 2 CRS ports and DM-RS overhead according to number of scheduled layers

  - Antenna Polarization Model

    - Model -2 from 36.873

  - Feedback

    - PUSCH 3-2 for non-reciprocity operation (PUSCH 3-0 for reciprocity based operation)

    - CQI, PMI and RI reporting triggered per 5ms

    - Feedback delay is 5 ms

    - Other parameters can be also used

- Baseline schemes are at least below:

  - Rel-13 FD-MIMO without coordination for CS/CB

  - DPS/DPB for NCJT

  - Other option can be also used

- Support M=1 in antenna configuration only for small cell layer in evaluation of Scenario D

- Fix the reference errors proposed in R1-167647

#### Potential enhancements for non-coherent joint transmission

Include initial evaluation results

R1-166865	Discussion on enhancements for non-coherent JT	LG Electronics

R1-167142	Discussion on non-coherent JT schemes for FeCoMP	Huawei, HiSilicon

R1-166317	Enhamcements for non-coherent JT	Qualcomm Incorporated

R1-166332	Initial evaluation results of non-coherent JT	ZTE Corporation, ZTE Microelectronics

R1-166333	Potential enhancements for non-coherent JT	ZTE Corporation, ZTE Microelectronics

R1-166334	QCL Issues of Joint Transmission	ZTE Corporation, ASTRI

R1-166462	Enhancement for non-coherent joint transmission	CATT

R1-166545	CSI feedback enhancements for non coherent JT	Intel Corporation

R1-166546	Control signalling enhancements for non coherent JT	Intel Corporation

R1-166547	Evaluation results for non coherent JT	Intel Corporation

R1-166710	Discussion on potential enhancements for non-coherent JT	Samsung

R1-166711	Preliminary evaluation results for non-coherent JT for FeCoMP	Samsung

R1-167143	Potential enhancements for non-coherent JT	Huawei, HiSilicon

R1-167144	Initial simulation results of non-coherent JT	Huawei, HiSilicon

R1-168267	WF on non-coherent JT schemes for FeCoMP, Huawei, HiSilicon, Intel, SoftBank Corp, ZTE, CATT, Qualcomm

Agreement:

- Capture the following non-coherent JT definition and classification in the TR

  - Definition: Non-coherent JT scheme corresponds to the transmission scheme where transmission of the MIMO layer(s) is performed from two or more transmission points (TPs) without adaptive precoding across the TPs.

  - Non coherent JT schemes can be classified based on the CWs transmitted from TPs:

    - Case 1: Different CWs are transmitted from different TPs. Each TP perform adaptive precoding independently

    - Case 2a: The same CW is transmitted from different TPs with spatial diversity (e.g. SFBC) coding/ spatial multiplexing.

    - Case 2b: The same CW is transmitted from different TPs with SFN.

  - Non coherent JT schemes can be classified based on overlapping of resource allocations from different TPs:

    - Scheme1: Fully overlapping scheme

      - In this scheme the resource allocations from different TPs for a UE are fully overlapped

    - Scheme 2: Partly overlapping scheme

      - In this scheme the resource allocations from different TPs for a UE are partly overlapped

    - Scheme 3: Non overlapping scheme,

      - In this scheme the resource  allocations from different TPs for a UE are not overlapped

#### Potential enhancements for coordinated scheduling/beamforming with FD-MIMO

Include initial evaluation results

R1-166318	Enhancements for coordinated scheduling/beamforming with FD-MIMO	Qualcomm Incorporated

R1-166336	Potential enhancements for CS/CB with FD-MIMO	ZTE Corporation, ZTE Microelectronics

R1-166463	Coordinated beamforming with FD-MIMO	CATT

R1-166548	Enhancements and evaluation of CS/CB CoMP	Intel Corporation

R1-166712	Discussion on potential enhancements for CS/CB with FD-MIMO	Samsung

R1-166866	Discussion on enhancements for CS/CB with FD-MIMO	LG Electronics

R1-167145	Potential enhancements for CS/CB with FD-MIMO	Huawei, HiSilicon

R1-167146	Initial simulation results of CS/CB with FD-MIMO	Huawei, HiSilicon

R1-167649	Preliminary FeCoMP Results for Indoor Hotspot Scenario	Ericsson

      - Others