3GPP TSG RAN WG1 Meeting #86b                                 	           R1-1609013

# Lisbon, Portugal 10th - 14th October 2016

# Source:        Samsung

Title:           Linear combination W2 codebook

Agenda Item:   7.2.2.1.1

Document for:  Discussion and Decision

- Introduction

In RAN1#86, the following agreement about advanced CSI was made [1].

Agreement:

- Specify CSI feedback enhancement with the following advanced CSI feedback framework:

  - Reduced space (eigenvectors)/W1 is constructed based on one of the following alternatives (TBD RAN1#86bis):

    - Alt1. Orthogonal basis (e.g. orthogonal DFT matrix)

    - Alt2. Non-orthogonal basis (e.g. Rel.13 Class A W1 for rank-1 and/or 2)

  - Reduced space representation/W2 is to further combine selected beams

  - Granularity of weighting(phase and/or amplitude) can be either wideband only or wideband/subband, and is constructed based on one of the following alternatives (TBD RAN1#86bis):

    - Alt1. Phase and amplitude

    - Alt2. Phase-only weighting

  - How the enhanced framework can be applicable for Class A and/or Class B eMIMO-Types is FFS

  - FFS: How to handle the relationship between advanced CSI feedback and legacy CSI feedback framework

- Companies are encouraged to provide results comparing the above alternatives, considering a mix of smaller and larger numbers of ports within the following antenna port configurations

  - {4,8,12,16,20,24,28,32} ports

  - Focus on rank<=2 scenario MU-MIMO for evaluation

  - Feedback overhead needs to be taken into account

  - For {4,8,12,16, 20,24,28,32}-port scenario, companies are encouraged to compare their proposals to dual-stage codebook enhancement with increased number of beams in W1

This contribution discusses the two alternatives for W2 codebook (highlighted) in the agreement. The discussion about the two alternatives for W1 codebook and rank 2 linear combination codebook are provided in companion contributions [2, 3] The analysis of per SB CSI reporting overhead for different LC codebook alternatives are also provided. Finally, based on the simulation results and overhead analysis, a proposal is made for the linear combination W2 codebook.

- Linear Combination W2 Codebook

Figure 1: Linear combination pre-coder

In advanced CSI reporting based on a linear combination codebook, a UE is configured with a double-codebook: W = W1W2, where

- W1 is for WB and long-term first PMI i1 or PMI pair (i1,1,i1,2) feedback, which indicates a 2N1N2 × 2L basis matrix B=b0b1…bL-10   0  0  00  0   0  0 b0b1…bL-1 whose columns correspond to L DFT beams for the two polarizations, and

- W2 is for SB and short-term second PMI i2 feedback, which indicates a coefficient vector c for linear combination of columns of B.

An illustration of the proposed LC codebook for L = 4 is shown in Figure 1. The LC pre-coder for layer r = 0, 1, …, R-1 of the rank-R LC codebook is given by

p(r)=1√rBc(r)Bc(r),

where

c(r)=e0ϕ0(r)e0e1ϕ1(r)e1⋯eL-1⋯ϕL-1(r)eL-11c1(r)⋮cL-1(r)=1c1(r)⋯cL-1(r)ϕ0(r)ϕ1(r)c1(r)⋯ϕL-1(r)cL-1(r)T.

In this contribution, we focus on the design of LC W2 codebook. In particular, we propose the following three components in the W2 codebook

- Beam selection: per SB, L out of the 4 beams indicated by (i1,1,i1,2) are selected for LC, where L = 2,4;

- Co-phase vector: ϕ(r)=ϕ0(r)ϕ1(r)⋯ϕL-1(r) corresponds to the co-phase vector for layer r = 0,1,..,R-1 of the rank-R W2 codebook; and

- Coefficient vector: c(r)=1c1(r)⋯cL-1(r) corresponds to the coefficient vector for layer r = 0,1,..,R-1 of the rank-R W2 codebook.

Note that the coefficient for the 1st beam can be assumed to be 1, i.e., c0(r)=1, for all layers r.

Rank 1 LC W2 codebook

For rank 1, each co-phase ϕl(r), where l=0,1,…,L-1 and each coefficient cl(r), where l=1,2,…,L-1 belongs to QPSK alphabet {1, j, -1, -j}. The i2 reporting overhead for two L values is shown in Table 1. In this table, the overhead of SB beam selection of L = 2 out of 4 beams (6 such selection possibilities) is taken into consideration.

Table 1: i2 reporting overhead for rank 1 LC codebook

|  | Co-phase (2nd antenna polarization) | Coefficient | L=2 (includes 3 bits for SB beam selection) | L = 4 |
| --- | --- | --- | --- | --- |
| i2 reporting overhead (beam selection, co-phase, coefficient) | ϕl0∈1,j,-1,-j for all l∈{0,1,…,L-1} | cl0∈{1,j,-1,-j} for all l∈{1,…,L-1} | 9 | 14 |

Rank 2 LC W2 codebook

For rank 2, the LC codebook alternatives as shown in Table 2 are proposed in the companion contribution [3], where Coph0 – Coph2 and Coef0 – Coef2 respectively are three alternatives for each of rank 2 co-phase and coefficient vectors, the summary of which are provided in Table 3 and Table 4. The rank-2 i2 reporting overhead for two L values is shown in Table 2. The overhead of SB beam selection of L = 2 out of 4 beams (6 such selection possibilities) is also taken into consideration. The codebook alternatives CB0 – CB7 assume the above-mentioned structure of the LC pre-coders (i.e. separate L co-phase values and L - 1 coefficients). The alternative CB8 assumes that co-phase is not separated from coefficients. In other words, L co-phase values are lumped together with L - 1 coefficients, and the total 2L – 1 coefficients are reported. Note that the rank 2 i2 reporting overhead for CB8 is approximately 2 times of rank 1 i2. On the other hand, the rank i2 overhead of CB0 – CB7 is at the most equal to rank 1 i2.

Table 2: Rank 2 LC codebook alternatives and i2 reporting overhead

| Rank 2 LC codebook | Alternative | Co-phase | Coefficient | #bits for co-phase | #bits for coefficient | #bits in total | L=2 (includes 3 bits for SB beam selection) | L=4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB0 | Figure 1 (separate co-phase) | Coph0 | Coef1 | 2L | L-1 | 3L-1 | 8 | 11 |
| CB1 |  |  | Coef2 |  | 2(L-1) | 4L-2 | 9 | 14 |
| CB2 |  | Coph1 | Coef0 | L | 2(L-1) | 3L-2 | 7 | 10 |
| CB3 |  |  | Coef1 |  | L-1 | 2L-1 | 6 | 7 |
| CB4 |  |  | Coef2 |  | 2(L-1) | 3L-2 | 7 | 10 |
| CB5 |  | Coph2 | Coef0 | 2L | 2(L-1) | 4L-2 | 9 | 14 |
| CB6 |  |  | Coef1 |  | L-1 | 3L-1 | 8 | 11 |
| CB7 |  |  | Coef2 |  | 2(L-1) | 4L-2 | 9 | 14 |
| CB8 | No separate co-phase | - | - | - | 8L-4 | 8L-4 | 15 | 28 |

Table 3: Alternatives for Rank 2 co-phase vectors

| Alternative | Rank-2 co-phase for two layers ϕl0ϕl1 (2nd antenna polarization) | Number of co-phase pair combinations for L beams | #bits |
| --- | --- | --- | --- |
| Coph0 | ϕl0=ϕl1∈{1,j,-1,-j} | 4L | 2L |
| Coph1 | ϕl0ϕl1∈1-1,j-j | 2L | L |
| Coph2 | ϕl0ϕl1∈1-1,j-j,-11,-jj | 4L | 2L |

Table 4: Alternatives for Rank 2 coefficient vectors

| Alternative | Rank-2 coefficient for two layers cl0cl1 | Number of coefficient pair combinations for L beams | #bits |
| --- | --- | --- | --- |
| Coef0 | cl0=cl1∈{1,j,-1,-j} | 4L-1 | 2(L-1) |
| Coef1 | cl0cl1∈1-1,j-j | 2L-1 | L-1 |
| Coef2 | cl0cl1∈1-1,j-j,-11,-jj | 4L-1 | 2(L-1) |

Rank > 2 LC W2 codebook

For rank > 2, the LC codebook can be the same as Class A codebook. This is because of following reasons:

- The main advantage of LC codebook is enhanced MU-MIMO performance through better channel quantization, which can be realized with max per-UE rank = 2.

- Rank > 2 codebook requires orthogonal beams, which makes rank > 2 LC codebook design complicated.

Proposal 1: For rank-1 and 2 advanced CSI reporting, support linear combination codebook W = W1W2, where

- W1 is Class A W1 codebook based as proposed in [2];

- W2 codebook has three components:

  - Beam selection: L = 2 or 4 beams are selected for LC per SB;

  - Coefficients to linearly combine L beams; and

  - Co-phase for L beams.

- The rank-1 LC W2 codebook is as shown in Table 1; and

- The rank-2 LC W2 codebook is one of CB0 – CB7 in Table 2.

Proposal 2: For rank >2, advanced CSI reporting is based on Rel. 13 or its extension in Rel. 14 Class A codebook.

The W2 component of the proposed LC codebook can be applied to enhance Class B CSI reporting. For instance, for K = 1 and P ports, the UE can report a PMI, which indicates a linear combination of L = P/2 ports. Similarly, the idea is applicable to enhance hybrid CSI feedback scheme in which two types of CSI-RS resources, for example non-precoded and beamformed CSI-RS, are associated with two eMIMO-Types. The PMIs reported in one or both of the two CSIs can be the linear combination codebook based PMI.

Proposal 3: The W2 component of the proposed LC codebook is used to enhance Class B and hybrid CSI reporting.

- Simulation Results

To evaluate the performance of the proposed rank 1 and rank 2 LC W2 codebook (Table 1 and Table 2), system-level simulation results are provided for the following codebooks:

- Reference: Rel. 13 Class A codebook (and its extension to more than 16 ports) with UE-specific configuration of Codebook-Config = 2, 3, and 4

- LC codebooks CB0 – CB8 and UE-specific configuration of Codebook-Config = 2, 3, and 4

- Upper bound: unquantized LC coefficients (L = 4) based on LS solution (please see [2] for details).

As proposed in the companion contribution [2], the Rel. 13 Class A rank 1 W1 codebook for Codebook-Config = 2, 3, and 4 is assumed as the W1 basis for LC codebook in this evaluation. To study the impact of WB beam power (W2 codebook Alt 1 in the agreement mentioned in Section 1), results are provided for both of the following two cases:

- Equal beam power: L beams are assumed to have the same magnitude or beam power; and

- Unequal beam power: Each of L beams can have magnitude or power level from a 2-bit uniform codebook in (0,1]. Note that the first beam can be assumed to have power one. So, 2(L-1) WB bits are needed to report beam powers of L beams.

The non-full-buffer system-level evaluation is carried out for UMi-2GHz channel model in medium (50% target RU) traffic loading scenario. The detailed results can be found in Table 6 in the Appendix. The results are provided for 32 antenna ports with (N1,N2) = (4,4). Here, we assume that the first dimension is horizontal and the second dimension is vertical. The downtilt angles in the elevation domain are chosen according to [4]. In these simulations, full-port non-precoded CSI-RS is used for CSI estimation, and the corresponding CSI-RS overhead is taken into account in the final throughput calculation. Cell association antenna pattern is approximated by one-TXRU pattern, and proportional fair scheduling (max 4 layers per time-frequency resource) have been used. For MU-MIMO, SLNR precoding is considered. The relevant simulation parameters are enlisted in Table 5. The rest of the simulation assumption is according to [4]. The performance gains of the above-mentioned codebooks are summarized in Figure 2 – Figure 5. The rank 2 i2 reporting overhead for these codebooks are shown in Figure 6. Based on these results, we can make the following observations:

Observation:

- The performance gain with 2-bit WB beam power is marginal.

  - Compare Figure 2 and Figure 3 for L = 2, and Figure 4 and Figure 5 for L = 4.

- While the performance of L = 2 (SB beam selection) is good, L = 4 offers significantly large 5% UPT gain (~10% additional gain) at the cost of more overhead.

- The rank 2 CB0 and CB1 (same co-phase for two layers) show good performance in 5% UPT, but poor performance in avg. UPT.

- The rank 2 CB5, CB6, and CB7 (four pairs of different co-phase for two layers) show good performance in both 5% and avg. UPT.

- Expectedly, CB8 shows the best performance but at the cost of approximately 2 times rank 2 i2 reporting overhead when compared with CB0 – CB7 which is not desirable considering periodic reporting of CSI on PUCCH.

Based on these observations and the tradeoffs between performance gain and rank i2 reporting overhead, we make the following proposal.

Proposal 4: For LC codebook:

- WB beam power is not supported; and

- The rank 2 W2 LC codebook is according to one of CB5, CB6, and CB7 in Table 2.

Figure 2: L = 2, equal beam power

Figure 3: L = 2, WB beam power

Figure 4: L = 4, equal beam power

Figure 5: L = 4, WB beam power

Figure 6: Rank 2 i2 reporting overhead comparison

- Conclusion

This document proposes an enhanced CSI reporting based on linear combination codebook. The proposals and observations made can be summarized as follows.

Proposal 1: For rank-1 and 2 advanced CSI reporting, support linear combination codebook W = W1W2, where

- W1 is Class A W1 codebook based as proposed in [2];

- W2 codebook has three components:

  - Beam selection: L = 2 or 4 beams are selected for LC per SB;

  - Coefficients to linearly combine L beams; and

  - Co-phase for L beams.

- The rank-1 LC W2 codebook is as shown in Table 1; and

- The rank-2 LC W2 codebook is one of CB0 – CB7 in Table 2.

Proposal 2: For rank >2, advanced CSI reporting is based on Rel. 13 or its extension in Rel. 14 Class A codebook.

Proposal 3: The W2 component of the proposed LC codebook is used to enhance Class B and hybrid CSI reporting.

Observation:

- The performance gain with 2-bit WB beam power is marginal.

  - Compare Figure 2 and Figure 3 for L = 2, and Figure 4 and Figure 5 for L = 4.

- While the performance of L = 2 (SB beam selection) is good, L = 4 offers significantly large 5% UPT gain (~10% additional gain) at the cost of more overhead.

- The rank 2 CB0 and CB1 (same co-phase for two layers) show good performance in 5% UPT, but poor performance in avg. UPT.

- The rank 2 CB5, CB6, and CB7 (four pairs of different co-phase for two layers) show good performance in both 5% and avg. UPT.

- Expectedly, CB8 shows the best performance but at the cost of approximately 2 times rank 2 i2 reporting overhead when compared with CB0 – CB7 which is not desirable considering periodic reporting of CSI on PUCCH.

Proposal 4: For LC codebook:

- WB beam power is not supported; and

- The rank 2 W2 LC codebook is according to one of CB5, CB6, and CB7 in Table 2.

References

[1] RAN1#86, Chairman’s notes

[2] R1-1609012, “Linear combination W1 codebook” Samsung.

[3] R1-1609014, “Rank 2 linear combination codebook” Samsung.

[4] 3GPP, TR36.897v1.0.1, “Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE”

Appendix: Simulation Assumptions and Results

Table 5: Simulation Parameters

| Parameters | Values |
| --- | --- |
| Simulation Type | Non-full-buffer (Medium load 70% Target RU, Lambda = 4) |
| Channel model | UMi-2GHz |
| Number of BS (H,V) antenna elements | (8,8), x-polarized, subarray partition |
| (N1,N2, P) | 32 ports: (4,4,2) |
| BS (H,V) antenna spacing | (0.5, 0.8)λ |
| BS and MS antenna polarizations | BS: (+45°,-45°); MS: (0°, 90°) |
| Number of UE antennas | 2 |
| SU/MU pre-coding | SLNR |
| Scheduling | MU, Proportional fair, up to 4 layers |
| Channel estimation | Ideal |
| Transmission rank | 1,2 |
| Receiver | MMSE-IRC |
| Codebook | Reference: Class A, (O1,O2) = (4,4), UE-specific Codebook-Config = 2,3,4 LC codebook: W1: Rel. 13 Class rank 1 W1, W2 as proposed Upper bound: Unquantized W2 for L = 4 Spacing between two adjacent W1 beam group (s1,s2) = (1,1) |

Table 6: Non-full buffer simulation results for 32 ports: UMi-2GHz

| L | Beam power | W2 Codebook | RU | Avg. UPT | 50% UPT | 5% UPT | Avg. UPT gain | 50% UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Class A | 55.5% | 21.49 | 17.89 | 7.30 | 100.0% | 100.0% | 100.0% |
| 2 | Equal beam power | CB0 | 52.4% | 23.20 | 20.72 | 8.61 | 108.0% | 115.8% | 118.0% |
|  |  | CB1 | 52.4% | 23.38 | 20.78 | 8.68 | 108.8% | 116.1% | 118.9% |
|  |  | CB2 | 53.0% | 22.89 | 20.20 | 8.42 | 106.5% | 112.9% | 115.4% |
|  |  | CB3 | 53.1% | 22.76 | 20.01 | 8.33 | 105.9% | 111.8% | 114.1% |
|  |  | CB4 | 52.8% | 23.14 | 20.41 | 8.48 | 107.7% | 114.0% | 116.2% |
|  |  | CB5 | 52.3% | 23.29 | 20.73 | 8.51 | 108.4% | 115.8% | 116.6% |
|  |  | CB6 | 52.4% | 23.30 | 20.94 | 8.52 | 108.5% | 117.0% | 116.8% |
|  |  | CB7 | 52.3% | 23.47 | 20.76 | 8.68 | 109.2% | 116.0% | 119.0% |
|  |  | CB8 | 51.3% | 24.16 | 21.57 | 8.95 | 112.4% | 120.5% | 122.7% |
| 2 | WB beam power | CB0 | 52.6% | 22.90 | 20.31 | 8.65 | 106.6% | 113.5% | 118.6% |
|  |  | CB1 | 52.3% | 23.26 | 20.62 | 8.52 | 108.3% | 115.2% | 116.8% |
|  |  | CB2 | 53.0% | 22.88 | 20.41 | 8.34 | 106.5% | 114.0% | 114.4% |
|  |  | CB3 | 52.9% | 22.95 | 20.20 | 8.40 | 106.8% | 112.9% | 115.1% |
|  |  | CB4 | 52.7% | 23.13 | 20.51 | 8.47 | 107.7% | 114.6% | 116.1% |
|  |  | CB5 | 52.2% | 23.39 | 20.83 | 8.67 | 108.9% | 116.4% | 118.9% |
|  |  | CB6 | 52.3% | 23.42 | 20.83 | 8.59 | 109.0% | 116.4% | 117.8% |
|  |  | CB7 | 51.9% | 23.67 | 21.16 | 8.64 | 110.2% | 118.3% | 118.4% |
|  |  | CB8 | 51.4% | 24.07 | 21.62 | 8.92 | 112.0% | 120.8% | 122.3% |
| 4 | Equal beam power | CB0 | 52.0% | 22.44 | 21.07 | 9.65 | 104.4% | 117.8% | 132.3% |
|  |  | CB1 | 52.2% | 23.10 | 20.91 | 9.23 | 107.5% | 116.9% | 126.6% |
|  |  | CB2 | 52.5% | 22.99 | 20.78 | 8.96 | 107.0% | 116.1% | 122.8% |
|  |  | CB3 | 52.4% | 22.52 | 20.83 | 9.06 | 104.8% | 116.4% | 124.1% |
|  |  | CB4 | 52.3% | 23.02 | 20.94 | 8.89 | 107.1% | 117.0% | 121.8% |
|  |  | CB5 | 51.8% | 23.62 | 21.40 | 8.93 | 109.9% | 119.6% | 122.5% |
|  |  | CB6 | 52.2% | 22.89 | 20.83 | 9.03 | 106.5% | 116.4% | 123.8% |
|  |  | CB7 | 51.8% | 23.51 | 21.28 | 9.06 | 109.4% | 118.9% | 124.2% |
|  |  | CB8 | 50.2% | 25.20 | 22.87 | 9.31 | 117.3% | 127.8% | 127.6% |
| 4 | WB beam power | CB0 | 51.7% | 22.92 | 21.22 | 9.48 | 106.7% | 118.6% | 129.9% |
|  |  | CB1 | 51.9% | 23.40 | 21.19 | 9.10 | 108.9% | 118.4% | 124.7% |
|  |  | CB2 | 52.0% | 23.25 | 21.04 | 9.01 | 108.2% | 117.6% | 123.5% |
|  |  | CB3 | 52.3% | 22.58 | 20.62 | 9.09 | 105.1% | 115.2% | 124.6% |
|  |  | CB4 | 52.0% | 23.22 | 21.02 | 9.01 | 108.1% | 117.5% | 123.5% |
|  |  | CB5 | 51.8% | 23.66 | 21.39 | 8.98 | 110.1% | 119.5% | 123.2% |
|  |  | CB6 | 51.9% | 23.10 | 21.05 | 9.21 | 107.5% | 117.7% | 126.2% |
|  |  | CB7 | 51.5% | 23.70 | 21.39 | 9.19 | 110.3% | 119.5% | 126.0% |
|  |  | CB8 | 49.8% | 25.36 | 23.24 | 9.41 | 118.0% | 129.9% | 129.0% |
| 4 | - | Unquantized | 46.5% | 28.61 | 26.27 | 10.33 | 133.2% | 146.8% | 141.5% |