TSG-RAN WG1 #86bis	R1-1609775

Lisbon, Portugal, October 10 – 14, 2016

Source:	Ericsson

Title:	Path loss model for evaluation of mMTC

Agenda Item:	8.1.8

Document for:	Discussion and Decision

# Introduction

The scenario “Urban coverage for massive connection” for NR evaluations is described in section 6.1.7 of TR 38.913 [1]. Relevant KPIs for this scenario include latency for infrequent small packets (section 7.6 of [1]) and coverage (section 7.10 of [1]), both of which are specified to be evaluated against a maximum coupling loss (MCL) of 164 dB. The evaluation assumptions for this scenario are specified in Table A.2.1-3 of 38.802 [2], with relevant parameters copied below for convenience. During the RAN1#86 meeting some concern was raised whether the agreed evaluation assumption could result in 164 dB maximum coupling loss [3], and as a result an email discussion [86-25] on calibrating the path loss model was initiated. The aim of this email discussion was to compare the companies’ implementations of the agreed evaluation model and compare it with the 164 dB MCL tail required for the KPI evaluation. The results of the calibration were circulated in an excel file on the RAN1 reflector. The 95th percentile of the coupling loss is around 150 dB and the 99th percentile is around 154 dB. From this it could be concluded that the existing evaluation assumptions including the path loss model falls short of generating the 164 dB MCL.

In this contribution we discuss how the evaluation assumptions can be modified for a 164 dB MCL target to meet the conditions required for the KPI evaluations.

Extract from [Table A.2.1-3: System level evaluation assumptions for urban coverage for massive connection and extreme long coverage] in TR 38.802 [2]

| Parameters | Urban coverage for massive connection (Uplink) |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 1732m |
| Carrier frequency | 700MHz |
| Channel model | 3D UMa Take 5GCM output into account if applicable |
| BS antenna configurations | Rx: 2 and 4 ports (8 as optional) |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi, including 3dB cable loss |
| UE antenna gain | -4dBi |

# Discussion

As mentioned in the introduction, the agreed evaluation assumptions for the “Urban coverage for massive connection” scenario are not able to produce the 164 dB MCL needed for the KPI evaluations.

Observation: The agreed evaluations assumptions in 38.802 for Urban coverage for massive connection are not compatible with the 164 dB MCL needed for KPI evaluation per 38.913

From this observation it is obvious that some modification of the evaluation assumptions is needed. Based on the extract from Table A.2.1-3 in [2] above it can be suggested to address this discrepancy by either modifying the channel model or by modifying the antenna gain. We will address both options below.

## Path loss model

The agreed channel model “3D UMa” refers to the model for the urban macro scenario in [4] which has a path loss model and a building penetration loss model. The latter is a rather simple model consisting of a fixed 20 dB penetration loss plus an additional 0.5 dB/m loss for devices located deeper into the building. The indoor depth is randomly generated within the range 0-25 m, resulting in a total outdoor-to-indoor loss between 20 dB and 32.5 dB. This can be compared with the simulation assumptions used in the NB-IoT study [5] where significantly higher building penetration loss was used to model the margins needed for e.g. devices in deep indoor conditions.

A simple remedy would then be to increase the building penetration loss for a certain fraction of the indoor devices to represent those located in e.g. basements or other deep indoor locations. This could have the form of an additional loss from a uniform distribution between Y1 and Y2 dB applied to X% of the indoor users on a per UT basis (i.e. all links to a certain UT are affected by the same additional loss). By adjusting X, Y1, and Y2, the tail of the coupling loss distribution can be adjusted to meet the 164 dB target.

## Antenna model

The agreed model assumes an 8 dBi BS antenna element gain and 2 or 4 Rx ports. For comparison, the vast majority of existing sites at low bands such as 700 MHz are equipped with higher gain sector antennas typically equipped with cross-polarized antennas. To make the evaluations relevant for operators with existing or planned deployments at low bands it can therefore be argued that the antenna model for the evaluations should be adjusted. A useful model would be to use (M,N,P,Mg,Ng) = (6,1,2,Nrx,1,2) with each TXRU mapped to a single panel rather than a single element. The antenna gain of each panel (column) including the element and array gain then becomes 16 dBi which represents a quite typical BS antenna at 700 MHz.

## Evaluation

The path loss calibrations were performed with the 8 dBi element model and the 3D UMa channel model. Here we experiment with combinations of the added penetration loss discussed in section 2.1 and the revised antenna model described in section 2.2. By trial and error, it was found that the model settings in Table 1 resulted in the desired MCL on the 99th percentile. This is confirmed by the corresponding CDFs shown in Figure 1. Note that while the same 99th CL percentile is reached by both models there is a large difference in the average coupling loss.

Table 1 Model parameters and resulting coupling loss percentiles for different model alternatives

|  | Antenna model | X | Y1 [dB] | Y2 [dB] | 95th percentile CL [dB] | 99th percentile CL [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Existing | Existing (8 dBi) | N/A | N/A | N/A | 141 | 145 |
| Alt 1 | Existing (8 dBi) | 20% | 10 | 19 | 156 | 164 |
| Alt 2 | (M,N,P,Mg,Ng) = (1,Nrx,2,6,1) with one TXRU per panel (16 dBi) | 20% | 10 | 30 | 153 | 164 |

Figure 1 Coupling loss distributions for the existing mMTC evaluation assumptions compared with two alternatives for modifying the evaluation assumptions

Based on this investigation we have the following proposal:

Proposal: Adjust the evaluation assumptions for the scenario “Urban coverage for massive connection” according to either of these two alternatives:

- Alt 1: Add an additional building penetration loss of U(10,19) dB for 20% of the indoor users

- Alt 2: Add an additional building penetration loss of U(10,30) dB for 20% of the indoor users, and modify the BS antenna configuration to (M,N,P,Mg,Ng) = (1,Nrx,2,6,1) with each TXRU mapped to a single panel

# Conclusions

The following proposal is made:

Proposal: Adjust the evaluation assumptions for the scenario “Urban coverage for massive connection” according to either of these two alternatives:

- Alt 1: Add an additional building penetration loss of U(10,19) dB for 20% of the indoor users

- Alt 2: Add an additional building penetration loss of U(10,30) dB for 20% of the indoor users, and modify the BS antenna configuration to (M,N,P,Mg,Ng) = (1,Nrx,2,6,1) with each TXRU mapped to a single panel

# References

3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”, v0.4.0, Aug 2016.

3GPP TR 38.802, “Study on New Radio (NR) Access Technology”, v0.1.0, Aug 2016.

3GPP R1-168543, “WF on Path Loss Calibration for mMTC”, Qualcomm Incorporated, Mediatek, Gothenburg, Sweden, 22nd- 26th August 2016.

3GPP TR 36.873, “Study on 3D channel model for LTE”.

3GPP TR xx.yyy, “NB-IoT”