3GPP TSG-RAN WG1 #86bis 	R1-1610141

10th – 14th October 2016

Lisbon, Portugal

Agenda item:	8.1.3.1

Source: 	Qualcomm Incorporated

Title: 	Short block-length design

Document for:	Discussion/Decision

# Introduction

The purpose of this document is provide more detailed discussion on channel coding performance, computational and implementation complexity, and latency tradeoffs of TBCC and Polar codes in the short block-length regime. This is a continuation of the analysis first provided in [2][3][4][8], with further details to address both URLLC/mMTC where there is CRC-aided list decoding, and Control Channel where there is no CRC available for pruning candidates from a list decoder.

# Code Description

Figure 1. Transmit chain diagram.
 For URLLC/mMTC simulation setting, 16-bit-CRC is assumed. 
For control channel simulation setting, no CRC is appended.

## TBCC

To support native rate of 1/3, 1/6 and 1/12, we adopt the nested polynomial structure introduced in [5], which we restate here in Table 1 (same as Table 1 in [5]). The first three polynomials are used to get native rate 1/3, the first six polynomials are used to for rate-1/6 evaluation, and the first 12 polynomials are used for rate-1/12 evaluation. Puncturing is done using LTE’s row-column interleaver and cyclic buffer design. The trellis construction in the decoder follows standard list Viterbi algorithm as illustrated in Figure 2. Note that each state keeps L strongest survivors (L*64 survivors per-stage), where L denotes the list size.

Table 1 Nested polynomials for low rate TBCC

| n | Polynomial | df | Adf |
| --- | --- | --- | --- |
| 1 | 133 | - | - |
| 2 | 171 | 10 | 11 |
| 3 | 165 | 15 | 3 |
| 4 | 117 | 20 | 2 |
| 5 | 135 | 25 | 1 |
| 6 | 157 | 30 | 1 |
| 7 | 135 | 36 | 4 |
| 8 | 123 | 40 | 1 |
| 9 | 173 | 46 | 3 |
| 10 | 135 | 51 | 2 |
| 11 | 171 | 56 | 2 |
| 12 | 135 | 61 | 1 |
| 13 | 173 | 66 | 1 |
| 14 | 135 | 72 | 4 |
| 15 | 145 | 76 | 1 |
| 16 | 157 | 82 | 3 |
| 17 | 135 | 87 | 2 |
| 18 | 117 | 92 | 2 |
| 19 | 135 | 97 | 1 |
| 20 | 157 | 102 | 1 |

State0

State1

State2

State3

Stage n

Stage n+1

ListSize=6

Figure 2 List Viterbi Algorithm

## Polar Codes

In RAN1#85, the code construction of Polar codes is discussed based on Gaussian Approximation (GA) with given SNR and the corresponding performance was evaluated [6][7].  For the following sections, we consider both Polar codes with CRC to assist decoding, as well as Polar codes without CRC attachment but still optimized for minimum distance. More construction details are provided in the next sections.

The construction SNRs listed in Table 2 are used to obtain the performance in the next section.

Table 2 The construction SNR (dB) in GA for URLCC/mMTC

| Information bits |  | 20 | 40 | 200 | 600 | 1000 |
| --- | --- | --- | --- | --- | --- | --- |
| Code rate | 1/12 | -5.0 | -5.0 | -5.0 | -5.0 | -5.0 |
|  | 1/6 | -3.0 | -3.0 | -3.0 | -3.0 | -3.0 |
|  | 1/3 | 0 | 0 | 0 | 0 | 0 |

# Performance without CRC-List Decoder (Control channel)

## Simulation Assumptions

In this section, for the control channel evaluation, we consider constructions and decoding algorithms which do not append a separate CRC used for pruning candidates from list decoders. For small payloads, such constructions can reduce overhead and lead to improved performance. This is particularly useful for smaller control channel payloads. For this case, we polar codes constructed according the SNRs listed below.

The BLER comparison of Polar and TBCC shown here conform to the simulation assumptions list in Table 3, where the parameters covered in this document is highlighted in bold text.

Table 3 Control channel simulation assumptions

| Channel | AWGN |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | QPSK |  |  |  |  |  |  |
| Coding Scheme | Repetition | Simplex | TBCC | Turbo | LDPC | Reed-Muller | Polar |
| Code rate (for evaluation purposes) | 1/24*, 1/12, 1/6, 1/3, 1/2, 2/3 |  |  |  |  |  |  |
| Decoding algorithm** | ML | ML | List-Viterbi | Scaled max log MAP | Adjusted min-sum | FHT | SC list |
| Info. block length (bits w/o CRC) (for evaluation purposes) *** | 1, 2, 4, 8, 16, 32, 48, 64, 80, 120, 200 |  |  |  |  |  |  |
| * Code rate 1/24 is valid for info block length of 1-2 bits ** Other variants of agreed algorithms can be used for encoding and decoding (Complexity details should be illustrated) *** Each of these info. block lengths shall be evaluated at at least one of the code rates. Other info. block lengths and code rates are not precluded. Similar info. and encoded block lengths should be used for the evaluation. Total coded bits = info. Block length/code rate. Note: these info. block length and code rate are only for initial performance evaluations. They are not interpreted as design targets or assumptions for complexity analysis. Companies are encouraged to provide information on complexity of their decoders, and on decoding latency |  |  |  |  |  |  |  |

## Simulation Results

Below we plotted the 10-3 BLER achieving SNR as a function of the information block size in Figure 3. The construction SNRs are -3dB, 1dB, 2dB and 4dB for rate of 1/6, 1/3, ½, and 2/3, respectively.

Observation 1:

Under control channel simulation assumption with no CRC:

- At K=16,32,48,64,80, across all code rate (2/3,1/2,1/3,1/6)

  - TBCC(L=1) outperforms (or performs similarly as) Polar(L=32)

- At K=120, 200

  - TBCC(L=1) performs similarly as Polar(L=32) at rate-2/3

  - Polar(L=32) outperforms TBCC(L=1) at rate-1/2, rate-1/3  and rate-1/6

Figure 3 TBCC/Polar performance comparison (1e-3 PER achieving SNR) (no CRC)

# Performance with CRC-List Decoder (URLLC/mMTC)

## Simulation Assumptions

List decoding improves the error-correction performance of polar codes. With a sufficiently large list size, the decoder can achieve the maximum likelihood (ML) performance of polar codes. Using a cyclic-redundancy check as an outer code, results in a concatenated polar-CRC code with higher minimum Hamming distance than the polar code alone, which, in conjunction with list decoding, leads to improved error-correction performance.

In this section, we show the BLER and false-positive rate comparison of Polar and TBCC/CC conforming to the simulation assumptions list in Table 4, where the parameters covered in this document is highlighted in bold text.

The complexity and latency analysis will be provided in the next section.

Table 4 URLLC and mMTC simulation assumptions

| Channel | AWGN |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | QPSK, 16 QAM*** |  |  |  |
| Coding Scheme | CC & TBCC | LDPC | Polar | Turbo |
| Code rate | 1/12, 1/6, 1/3 |  |  |  |
| Decoding algorithm* | List-X Viterbi | min-sum | List-Y | Max-log-MAP |
| Info. block length** (bits w/o CRC) | 20, 40, 200, 600, 1000 |  |  |  |
| CRC length | 16 |  |  |  |
| - * These algorithms are starting points for further study. Other variants of agreed algorithms can be used for encoding and decoding (Complexity details should be illustrated) - ** At least these info. block length and code rate shall be evaluated. Other info. block lengths and code rates are not precluded. Similar info and encoded block lengths should be used for the evaluation. Total coded bits = info. Block length/code rate. Note: these info. block length and code rate are only for initial performance evaluations. They are not interpreted as design targets or assumptions for complexity analysis. - ***16 QAM results are not provided here. |  |  |  |  |

## Simulation Results

In Figure 4 to Figure 10, we plot the 10-3 BLER achieving SNR, as a function of the list size. Each figure shows the results under a single block length. As mentioned in the previous section, we adopt the nested polynomial structure as introduced in [5] to construct rate 1/3, 1/6 and 1/12 convolutional code.  From the figures, the following observations can be made.

Observation 2:

Under URLLC/mMTC simulation assumption with 16-bit CRC:

- At K=20 and K=40, across all code rate (1/3, 1/6, 1/12)

  - TBCC performs similarly as Polar when ListSize>16.

  - TBCC outperfoms Polar when ListSize<16

- At K=100

  - TBCC performs similarly as Polar at code rate 1/3

  - Polar outperforms TBCC by <0.5dB at code rate 1/6

  - Polar outperforms TBCC by <1dB at code rate 1/12

- At K=200,600,1000

  - Polar outperfoms TBCC

Figure 4. Achievable SNR versus Information Blocklength

Figure 5 TBCC/CC/Polar BLER at K=20

Figure 6 TBCC/CC/Polar BLER at K=40

Figure 7 TBCC/CC/Polar BLER at K=100

Figure 8 TBCC/CC/Polar BLER at K=200

Figure 9 TBCC/CC/Polar BLER at K=600

Figure 10 TBCC/CC/Polar BLER at K=1000

In the next set of figures, we show the false-positive rate of Polar and TBCC as a function of the list size. The false-positive rates are obtained by injecting the decoder with the following two types of LLR inputs and counting the number of times the decoder claims a CRC-passing output.

- Gaussian noise: This input emulates the case when no codeword is transmitted. (null hypothesis)

- Noise-free Random BPSK: This input emulates the case when some codeword is transmitted but it is intended for a different target receiver. (random is as a result of scrambling)

In the case when the 16-bit CRC is not used to assist decoding, the false positive rate with either of the above two types of random LLR inputs is Pf=2-16. For a random list decoder where L randomly chosen codewords are checked against the 16-bit CRC, it’s false positive rate can be expressed as

PfL=1-1-PfL≈L×2-16

→log2Pf(L)≈-16+log2L

In Figure 11 and Figure 12, we plot the false positive rate of TBCC and Polar with a 95% confidence interval under different K=20 and K=40, and the following observations are made

Observation 3:

- TBCC and Polar have similar false positive rate

- Radom Gaussian LLR and Random BPSK LLR yield similar false positive rate

- The false positive rate of both TBCC and Polar match with the equation log2Pf(ListSize)≈-16+log2ListSize

Figure 11 False-positive rate of TBCC/Polar at K=20

Figure 12 False-positive rate of TBCC/Polar at K=40

# Implementation considerations

In this section, we compare TBCC and Polar from three different angles: (1) computational complexity, which is closely related to the decoder power consumption (2) implementation complexity, which reflects the chip area requirement, and (3) decoding latency, which impacts the UE processing timeline.

## Computational complexity

### TBCC decoder

| TBCC Decoder Component | Complexity |
| --- | --- |
| Branch metric calculation | min2CX+1-2, CX×S2X×2K/X |
| Trellis development for state determination | S2X ×K/X+S(2X-1)×K/X |
| Trellis development for list decoding | S(L+2X-1) ×K/X +S(2X-1)L×K/X |
| Handling of the final stage and backtrace | LlogL+KL |
| Parameters: - Number of info bits (containing CRC): K - Number of coded bits per info bit: C - Number of states: S - Number of input states per state: 2X (Radix 2X) - List Size: L |  |

### Polar SC-List Decoder

| Polar SC-List Decoder Component | Complexity |
| --- | --- |
| Single Parity Check | abs(): N*log(N)*L sign():N*log(N)*L min(): 0.5N*log(N)*L |
| Repetition | add()/sub(): 0.5N*log(N)*L |
| Path Metric Calculation | abs(): LN add():LN |
| Sorting | comp(): 2LK(log22L) |
| Parameters: - Number of info bits (containing CRC): K - Number of coded bits: N - List Size: L |  |

### Computational complexity comparison

In this subsection, we provide a complexity study for the K=40, N=120 scenario, which best represents control channel use case. In Figure 13, we plot the computation complexity of TBCC and Polar as a function of the list size, from which the following observation can be made

Observation 4:

- Under the same list size, Polar SC-List decoder has less computation complexity compared with TBCC. The ratio of TBCC computation complexity over that of Polar SC-List is a decreasing function of the list size: at ListSize=1, the ratio is ~7.5; at ListSize=16, the ratio is ~2.

Figure 13 computational complexity of TBCC and Polar SC-List as a function of list size

In Figure 14, we incorporate the BLER performance result from Figure 6 and plot the 10-3 BLER achieving SNR as a function of calculation complexity, with the corresponding list size marked in the figure.  The figure shows that

Observation 5:

- TBCC(L=16) and Polar(L=32) have the similar complexity count and similar 10-3 BLER achieving SNR, whereas false positive rate of Polar(L=32) is twice that of TBCC(L=16).

Figure 14 Computational complexity vs Performance

## Implementation complexity

### TBCC Viterbi Decoder Implementation Complexity

To reduce decoding latency, we use a radix-4 Viterbi decoder, where two consecutive trellis steps are merged and processed simultaneously. In this section, we only discuss the implementation complexity of an L=1 decoder. The implemented decoder can be used to perform the state-determination pass in addition to the Viterbi decoding.

#### Branch Metrics

Each of the 4.2M radix-4 branch metrics is the sum of two radix-2 branch metrics:

γlSi'', S= γl-1Si'', Sj'+ γl(Sj', S)

There are 2.2.2M radix-2 branch metrics in the merged radix-4 trellis stage, each of which is calculated according to

γlSi', S=j=13yl(j)xl(j)

xl ∈{-1, 1}, so multiplying by it can only change the sign of the other operand and can be implemented using a subtractor. Therefore, a single radix-2 branch metric calculator requires three subtractors and two adder.

Calculating the radix-4 branch metrics requires 4.2M adders and calculating the component radix-2 branch metrics requires 5.2.2.2Madders for a total of 24.2M adders.

#### State Metrics (Forward Recursion)

The state metric, αl, for an even-indexed trellis stage, Sl, which corresponds to two bits l and l-1, is calculated using an add-compare-select (ACS) unit:

αlS=max0 ≤ j<4(αl-2Si''+ γlSi'', S)

The ACS unit comprises four adders and three maximum value calculations (comparators). To prevent numerical saturation of the state metrics, a constant offset is subtracted from all metrics in a trellis stage when any of the metric exceeds a threshold. The total implementation complexity of the ACS unit and the offsets is 5.2M adders and 3.2M comparators.

The backtracking step is excluded from the complexity logic implementation analysis since it only comprises look-ups from memory. It will be included in the memory complexity analysis.

#### Memory Requirements

The TBCC requires four different memories to be implemented. The first is the input LLR memory, which stores two sets of N channel LLRs of Qc bits each. Double buffering the input LLRs enables the decoder to decode a codeword while receiving another. The state-metric memory stores 2M metrics of Qi bits each. The survivor memory stores 2M M-bit indices.

To facilitate backtracking, the two input bits associated with the winning branch at each state are stored for the entire trellis, requiring K2M bits.

The total memory (in bits) is

2NQc+2M(Qi+M+K)

### Polar List Decoder Implementation Complexity

A polar code of length N is the concatenation of two polar sub-codes of length N/2. This concatenation process is applied recursively until sub-codes of length 1 are reached. These length-1 sub-codes either carry an information bit, or a frozen bit. Successive-cancellation (SC) decoding calculates the input to each sub-code recursively until length-1 sub-codes are reached. At which point, the single bit is estimated to be 0 if it is frozen, or using the sign of the LLR (threshold detection) if it is an information bit. Recursively decoding every sub-code leads to SC decoders having high latency.

Simplified SC (SSC)-based decoding reduces latency by directly decoding sub-codes when implementation and computational complexity constraints permit. Examples of these sub-decoders are:

- Decoders for rate-0 sub-codes whose output is known a priori to be the all-zero vector.

- Decoders for rate-1 sub-codes whose output is the element-wise hard-decision decoding of the input LLRs.

- Exhaustive-search maximum-likelihood (ML) decoders, the maximum length and dimension of the sub-codes on which they operate is limited by available computational elements.

List decoding can also benefit from SSC latency reduction techniques. One major difference between SC-List and SSC-List decoding is that, in the latter, directly-decoded sub-codes can generate multiple candidates per list item. Whereas SC-List decoding, because it arrives at single-bit sub-codes, only generates two candidates per list item. We base the proposed polar decoder implementation on [9].

#### Memory Requirements

The decoder requires 2NQc bits to store two sets of channel LLRs. In addition, each list element stores i=1log2(N2)N2i≈N LLRs for a total of LNQi bits of internal LLR memory.

Similarly, the list decoder stores a 2N-bit estimated codeword and NL internal bit estimates.

While the list decoder also stores L path metrics (of Qm bits each), the size of this memory is negligible compared to the rest and we ignore it in this analysis. Therefore, the total memory required by the list decoder is approximately 2N(Qc + 1) + N*L(Qi + 1) bits.

In [10], recalculating LLR values corresponding to the larger stages in the decoder instead of storing them was proposed as method to reduce the decoder memory requirements. The output LLRS for stages log2N2, log2N4, log2N8, and log2N16 are not stored, reducing the internal LLR memory to i=1log2(N32)2i≈N16 LLRs per list element.

The total memory for the polar list decoder becomes

2NQc+1+N16L(Qi+1) bits. The impact of the LLR recalculation on implementation complexity is discussed in a later section.

Memory width is LP LLRs, where P is the level of parallelism and affects latency as will be discussed later.

To maintain clock frequency, pipeline registers are inserted at the output of the stage log2N4 recalculation. This adds PL LLR registers to the memory for a total of

2NQc+1+N16LQi+1+PLQi bits.

#### F and G blocks:

The decoder implements L*P/2 f and g blocks. This number is limited by the available memory width per list item (P).

Each f block performs signasignbmin⁡(a, b) and consists of two 2s-complement-to-sign-magnitude converters, a minimum-value calculator, and one sign-magnitude-to-2s-complement converter. The converters can be implemented using one adder each and the minimum value calculator can be implemented using a comparator.

Each g block performs b+a or b-a depending on a combination of estimated bits us and consists of an adder and a subtractor.

The total complexity of the f and g logic is L*P/2 * (3 + 2) = 5/2LP adders and L*P/2 comparators.

When the LLR recalculation method is used to reduce memory [10], an additional LP4+P8+P16=716LP f and g blocks are required, increasing the total complexity to 7516LP adders and 1516LP comparators.

#### Sub-code Decoders

Rate-1 decoders need to find both the first and second minimum magnitudes (min-1 and min-2, respectively). The required resources to implement these operations are P absolute value calculators (subtractors) and 2P – 3 comparators per list item [11].

The ML decoder requires 1824L adders and 4L Min-1 and Min-2 calculators with 8 inputs that require 4L*(2*8 – 3) = 52L comparators.

The repetition code decoder requires P adders per list item to accumulate its input that is segmented into blocks of P LLRs.

#### Path-metric Sorting

The list of path metrics is sorted using a bitonic sorting network with 2L inputs. The basic building block in a sorting network is a 2-input sorter composed of a comparator and two 2x1 multiplexers. A sorting network with M inputs has i=1log2MiM/2=M4log2M(log2M+1) 2-input sorters, leading to an implementation complexity of M4log2M(log2M+1) comparators.

Due to the presence of ML decoders, the decoder requires L + 3 sorting networks that have 2L inputs Since the rate-1 decoder generates 4 candidates for each path, it requires a 4L-input sorting network. Therefore, the decoder requires L+2 2L-input and one 4L-input sorters. The total number of comparators required is L+2L2log22Llog22L+1+Llog24L(log24L+1) comparators.

### Summary

|  | TBCC (L=1) | Polar |
| --- | --- | --- |
| Adders | 29.2M | L(10716P+1824) |
| Comparators | 3.2M | L4716P+49+L+2L2log22L(log22L+1)+Llog24L(log24L+1) |
| Memory (bits) | 2NQc+2M(Qi+M+K) | 2NQc+1+N16L(Qi+1)+PLQi |

#### Example

To numerically compare the two decoders, we use the following parameters:

- Code: N = 120, K = 56 (40 information bits + 16 CRC bits).

- TBCC: M = 6, Qc = 6, Qi = 10.

- Polar: P = 32, Qc  = 6, Qi = 7.

|  | TBCC | Polar (L = 4) | Polar (L = 8) |
| --- | --- | --- | --- |
| Adders | 1,856 | 8,152 | 16,304 |
| Comparators | 192 | 796 | 2,184 |
| Memory (bits) | 6,048 | 2,816 | 3,952 |

Observation 6:

- Based on the number of computational resources and memory requirements, we expect the TBCC decoder to have smaller implementation area than a polar list decoder with comparable error-correction performance and the same code parameters.

## Latency

### TBCC decoding latency

In the radix-4 decoder, the state-determination pass requires K/2 clock cycles. Similarly, the forward recursion and the backtracking require K/2 clock cycles each. The total latency will be 3K/2 cycles.

### Polar SC-List Decoder Latency

Using a sorting network, one can sort 2L path metrics in (log2(2L)+1) log2 (2L)/2 cycles assuming each stage in the sorting network is performed in a cycle. A less conservative estimate assumes that 4 stages can be performed in a clock cycle, reducing the sorting latency to ⌈(log2(2L)+1) log2 2L/(2*4)⌉ cycles. Since sorting occurs only after an information bit is estimated, the per-codeword latency due to sorting in SC-list decoding is
K(log2(2L)+1) log2 2L2*4

cycles. Therefore, the total latency of an SC-list decoder is
2s=2log2N2sPN2s+K(log2(2L)+1) log2 2L2*4

clock cycles.

SSC-list decoding has lower latency, but it is dependent on the location of frozen bits (the code construction parameters) in addition to the code length and rate. The latency of an SSC-list in clock cycles is the sum of the latency of all sub-codes in the pruned code graph:

i2NiP+ Si

where Ni is the length of sub-code i, 2Ni/P is the number of cycles required to calculate the sub-decoder’s input, and Si is the latency due to sorting and pipelining in the sub-code decoder.

Si depends on the type of sub-code:

- It is 0 if the sub-code will not be directly decoder, i.e. if the decoder will perform an F or a G function.

- It is the latency of a 2L-input sorting network for rate-1 sub-codes.

- It varies based on sub-code length and rate for ML decoding.

For N= 120, K = 56, P = 32, and a code constructed for the AWGN with BPSK modulation and noise variance σ2=0.25, the latency (in clock cycles) is

|  | SC-List (L = 4) | SC-List (L=8) | SSC-List (L = 4) | SSC-List (L = 8) |
| --- | --- | --- | --- | --- |
| Latency (cycles) | 359 | 415 | 75 | 83 |

### Summary

|  | TBCC | Polar SSC-List |
| --- | --- | --- |
| Latency (cycles) | 3K2 | i2NiP+ Si |

#### Example

To numerically compare the two decoders, we use the following parameters:

- Code: N = 120, K = 56 (40 information bits + 16 CRC bits).

- TBCC: M = 6.

- Polar: P = 32.

|  | TBCC | Polar (L = 4) | Polar (L = 8) |
| --- | --- | --- | --- |
| Latency (cycles) | 84 | 75 | 83 |

Observation 7:

- TBCC (L=1) and the polar SSC-List decoder (L = 4, L = 8) have comparable latency for the (120, 40+16) code example.

# Conclusions

Observation 1:

Under control channel simulation assumption with no CRC:

- At K=16,32,48,64,80, across all code rate (2/3,1/2,1/3,1/6)

  - TBCC(L=1) outperforms (or performs similarly as) Polar(L=32)

- At K=120, 200

  - TBCC(L=1) performs similarly as Polar(L=32) at rate-2/3

  - Polar(L=32) outperforms TBCC(L=1) at rate-1/2, rate-1/3  and rate-1/6

Observation 2:

Under URLLC/mMTC simulation assumption with 16-bit CRC:

- At K=20 and K=40, across all code rate (1/3, 1/6, 1/12)

  - TBCC performs similarly as Polar when ListSize>16.

  - TBCC outperfoms Polar when ListSize<16

- At K=100

  - TBCC performs similarly as Polar at code rate 1/3

  - Polar outperforms TBCC by <0.5dB at code rate 1/6

  - Polar outperforms TBCC by <1dB at code rate 1/12

- At K=200,600,1000

  - Polar outperfoms TBCC

Observation 3:

- With the same list size, TBCC and Polar have similar false positive rate

- Radom Gaussian LLR and Random BPSK LLR yield similar false positive rate

- The false positive rate of both TBCC and Polar match with the equation log2Pf(ListSize)≈-16+log2ListSize

Observation 4: For the scenario of K=40 N=120, which best represents control channel use-case,

- Under the same list size, Polar SC-List decoder has less computational complexity compared with TBCC. The ratio of TBCC computational complexity over that of Polar SC-List is a decreasing function of the list size: at ListSize=1, the ratio is ~7.5; at ListSize=16, the ratio is ~2.

Observation 5: For the scenario of K=40 N=120, which best represents control channel use-case,

- TBCC(L=16) and Polar(L=32) have the similar computational complexity count and similar 10-3 BLER achieving SNR, whereas false positive rate of Polar(L=32) is twice that of TBCC(L=16).

Observation 6:

- Based on the number of computational resources and memory requirements, we expect the TBCC decoder to have smaller implementation area than a polar list decoder with comparable error-correction performance and the same code parameters.

Observation 7:

- TBCC (L=1) and the polar SSC-List decoder (L = 4, L = 8) have comparable latency for the (120, 40+16) code example.

# References

- RP-160671, New SID Proposal: Study on New Radio Access Technology

- “TBCC design overview”, R1-164701, Qualcomm Incorporated, RAN1#85, Nanjing, China.

- “TBCC – Performance Evaluation”, R1-164702, Qualcomm Incorporated, RAN1#85, Nanjing, China.

- “Channel coding evaluation assumptions - performance and complexity”, R1-164704, Qualcomm Incorporated, RAN1#85, Nanjing, China.

- “Performance Evaluation of TBCC and Polar Codes”, Ericsson, RAN1#85, Nanjing, China.

- “Polar code design overview”, R1-164699, Qualcomm Incorporated, RAN1#85, Nanjing, China.

- “Polar – Performance Evaluation”, R1-164700, Qualcomm Incorporated, RAN1#85, Nanjing, China.

- “Short block-length design”, R1-166373, Qualcomm Incorporated, RAN1#86, Gothenburg, Sweden.

- “A High Throughput List Decoder Architecture for Polar Codes”, Lin et al, T-VLSI, Vol 24, No. 6, Jun. 2016.

- “Computational and implementation complexity of channel coding schemes,” R1-167213, Huawei, HiSIlicon, RAN1 86, Gothenburg, Sweden.

- “Algorithms of Finding the First Two Minimum Values and Their Hardware Implementation,” Wei et al, IEEE T-CAS-I, Vol 55, No. 11, Dec. 2008.