3GPP TSG RAN WG1 #88																					R1-1702191

Athens, Greece, 13th - 17th February 2017

Source: 	Intel Corporation

Title:	On transmission scheme 2 for DL

Agenda item:	    8.1.2.1.2

Document for:	Discussion and Decision

# Introduction

In RAN1 NR Adhoc, Jan. 2017, the following agreement was made regarding DL transmission scheme 2 for NR [1]:

Agreements: - For Transmission scheme 2, down selection(s) on DMRS based transmission schemes will be done in RAN1#88 at least for rank 1 - For rank 1, - Precoder cycling with transparent DMRS - Precoder cycling with non-transparent DMRS - Small-delay CDD with transparent DMRS - DMRS based SFBC - For rank>1, - Precoder cycling with transparent DMRS - Precoder cycling with non-transparent DMRS - Layer shifting - Precoder cycling with transparent DMRS and layer shifting - Small-delay CDD with transparent DMRS - Large-delay CDD with non-transparent DMRS

In this contribution, rank-1 downlink transmission scheme 2 without spatial information and with partial spatial information are discussed.  Performance comparison using link-level and system-level simulation results are provided.

# Discussion

## Precoder Selection for Precoder Cycling

### Without spatial information

In some cases spatial channel state information may be not reliable or available at the gNB transmitter. In this case gNB should apply open-loop transmission scheme that should achieve full spatial coverage. The simplest transmission scheme with full spatial coverage is single antenna transmission. However, in case of relatively large number of antenna elements at the gNB, it may be challenging to gNB to allocate all transmit power in a single antenna element. Another transmission scheme with full spatial coverage relies on the precoder cycling, where the precoding is applied to all antennas of gNB and changes in frequency and/or time. Comparing to single antenna transmission, the precoding cycling scheme can achieve full power utilization without any implementation limitation, and therefore, it should be considered as baseline for NR.

One of the important aspect of the precoding cycling transmission scheme is selection of the precoder set for the cycling. In this paper, we compared two different precoder sets, one with DFT vectors and the other with non DFT vectors.

Non DFT precoding vectors were obtained from DFT vectors W by using beam broadening operation, where each DFT vector is multiplied by beam broadening function as follows:

,

where Wb is resulting precoding corresponding to broaden beam, N1 or N2 is number of antennas in horizontal and vertical domain and {p,c} are parameters that determine the desired beam width of the precoding. The examples of beam patterns for different values of c are illustrated in Figure 1.

Figure 1: Examples of beam broadening with 4 antennas for different c values

To compare the performance of open-loop scheme with different precoding vectors sets, link-level simulations were carried out. For simulations, eight transmit antennas at the gNB were assumed. The channel with medium and high correlations were considered. LTE 8Tx rank-1 codebook with and without beam broadening function were used for precoding cycling, where the precoder was randomly chosen in every PRB. Other details of simulation assumptions can be found in Appendix B (Scenario 1).

Figure 2: Link level simulation results for precoder cycling with different c values and different MCS

From the simulation results in Figure 2, it can be seen that precoding cycling based on the non-DFT vectors with the beam broadening function provides noticeable gains over DFT precoding cycling. The performance gains are increasing for the channel with high correlation.  Based on the result the following observation and proposal can be made:

Observation: Beam broadening function provides noticeable gains over DFT precoding for open-loop transmission scheme with precoding cycling.

Proposal: Consider beam broadening function for open-loop schemes in NR.

### With partial spatial information

In case of high speed scenarios, some of CSI information provided by the UE to gNB may become outdated due to feedback delay and channel aging. In this case, it may be beneficial to rely only on the subset of the reported CSI information. For example, in LTE, semi-open loop transmission scheme were introduced, where gNB only relies on W1 feedback for downlink transmission by applying precoding cycling only for the precoding vectors indicated by i1 report.

To identify the CSI parameters that can be used for transmission in high speed scenarios, link-level evaluations were carried out with eight transmit antennas at the gNB. Channel model with high and medium spatial correlation were considered. The performance of the transmission schemes with polarization cycling, polarization and beam cycling were compared to each other. The performance results for closed-loop scheme is also provided as a reference. In case of polarization cycling, the beamformer was selected according to the best reported beam in CSI report, while polarization was chosen randomly on each PRB. In case of polarization and beam cycling, the beamformer cycling was performed across all beams and polarizations within W1 report.

Figure 3: Link-level simulation results for semi-open loop and closed loop schemes

From the simulation results presented in Figure 3, it can be seen that the transmission scheme with polarization cycling provides better performance comparing to other evaluated schemes.

Observation: For high-speed scenario, transmission scheme with polarization cycling provides better performance than transmission scheme with polarization and beamforming cycling.

Proposal: Consider polarization cycling in semi-open loop schemes in NR.

## Transmission Scheme 2

### Large Resource Allocation

In case of relatively large resource allocation, or data is allocated across multiple PRBs, the number of precoder applied can be sufficient to achieve diversity. In this case, it is desired to have a single port transmission so that it does not create more interference to neighboring cell.

For example, SFBC precoding can also provide polarization diversity gain. However, since the number of transmission streams per subcarrier is two, compared to polarization cycling, SFBC reduces interference rejection capabilities of the interference aware receivers. To evaluate the performance impact, a system-level simulations were carried out for dense urban scenario with all outdoor UEs and 120 km/h speed. For evaluations FTP traffic model 1 was used. The system level performance for transmission scheme with polarization cycling and SFBC precoding were compared assuming MMSE-IRC receiver. LTE codebook was used for the precoding vectors.

Table 1: System level simulation results for different multi antenna schemes with MMSE-IRC receiver

| Traffic load (λ) |  | low (2.5) |  | medium (4.7) |  |
| --- | --- | --- | --- | --- | --- |
| MIMO scheme |  | Polarization cycling | Polarization SFBC | Polarization cycling | Polarization SFBC |
| UE average packet throughput, Mbps | Average | 46.80 | 44.30 (-5%) | 37.50 | 32.63 (-13%) |
|  | 5% of CDF | 22.72 | 19.36 (-15%) | 14.25 | 10.04 (-29%) |
|  | 50% of CDF | 55.25 | 53.34 (-4%) | 36.37 | 30.11 (-17%) |
|  | 95% of CDF | 55.85 | 55.84 (0%) | 55.78 | 55.73 (-1%) |
| Resource utilization |  | 18.1% | 19.8% | 37.5% | 44.4% |

From the simulation results presented in Table 1, it can be seen that SFBC precoding provides worse performance comparing to polarization cycling across PRBs.

Observation: For interference limited scenarios, transmission scheme with polarization cycling across PRBs provides better performance comparing to SFBC transmission scheme.

Proposal: Consider PRB level precoder (polarization) cycling as a baseline transmission scheme 2 in NR.

### Small Resource Allocation

In case of relatively small resource allocation, the number of precoder applied may not be sufficient to achieve diversity. For example, only one precoder can be applied in case of PRB level precoder cycling when only one PRB is allocated to a UE. To overcome this problem, SFBC, RE level cycling or partial slot level cycling transmission schemes can be introduced. As pointed out in [2], RE level cycling has problem of interference estimation. In RE level cycling, two ports DMRS are transmitted, and applying different precoder for different data REs. It is possible to estimate interference from data REs, but it requires non-colliding DMRS design between different TRPs. On the other hand, partial slot level precoder cycling does not have this issue since exactly same precoder is applied in DMRS REs and data REs.

The example of partial precoding cycling is illustrated in Figure 4, where one DL slot is shown.

Figure 4: Illustration of the precoding assignment in the partial slot precoding cycling

It can be seen that the DL slot is partitioned in two parts with two different precoders assigned for the DL slot transmission. If the interference averaging is not applied across the different parts of slot, the interference rejection capabilities of MMSE-IRC receiver should remain the same.

To demonstrate that this scheme can provide the performance similar to RE level cycling in noise limited scenario, a link level evaluations were carried out. The performance of partial slot cycling was compared to the performance of SFBC, RE-level and PRB-level cycling in the noise limited scenario. Simulation results for two MCS are represented in Figure 5. The details of simulation assumptions can be found in Appendix B (Scenario 2).  It can be seen from the Figure 5 that SFBC provides best performance in noise limited scenario since it has full diversity order regardless of code rate. The performance of partial slot-level cycling is comparable to RE-level cycling.

Figure 5: Performance comparison of different precoding cycling approaches in noise limited scenario

To compare performance of different transmission schemes in interference limited scenario, link level simulations were carried out with same transmission scheme on serving and interfering BS. The details of simulation assumptions can be found in Appendix B (Scenario 2). It can be observed from the Figure 6 that the performance of RE level precoding cycling and SFBC transmission schemes with MMSE-IRC receiver is worse compared to the MMSE-IRC receiver with partial slot-level precoding cycling.

Figure 6: Performance comparison of different precoding cycling approaches for serving and interfering BS

Observation: For small resource allocation, partial slot precoder cycling provides additional diversity gain over PRB level precoder cycling, and it shows similar link performance as RE level cycling in noise limited scenario while it does provide better interference rejection capabilities to the neighboring cells.

Based on the observations above the following proposal can be made.

Proposal: Adopt partial slot precoding cycling as transmission scheme 2.

# Summary

In this contribution transmission schemes without spatial information and with partial spatial information are discussed.

The following observations were made:

- Beam broadening function provides noticeable gains over DFT precoding for open-loop transmission scheme with precoding cycling.

- For high-speed scenario, transmission scheme with polarization cycling provides better performance than transmission scheme with polarization and beamforming cycling.

- For interference limited scenarios, transmission scheme with polarization cycling across PRBs provides better performance comparing to SFBC transmission scheme.

- For small resource allocation, partial slot precoder cycling provides additional diversity gain over PRB level precoder cycling, and it shows similar link performance as RE level cycling in noise limited scenario while it does provide better interference rejection capabilities to the neighboring cells.

Based on the observations the following proposals was made:

- Consider beam broadening function for open-loop schemes in NR.

- Consider PRB level precoder (polarization) cycling as a baseline transmission scheme 2 in NR.

- Adopt partial slot precoding cycling as transmission scheme 2.

# References

- Chairman’s notes, 3GPP TSG RAN WG1 Meeting NR Ad-Hoc, Spokane, USA, 16th – 20th January 2017

- R1-1700335, On DMRS based DL (open-loop and semi-open loop) MIMO, Intel Corporation

# Appendix A

## Noise limited Scenario

## Interference limited Scenario

# Appendix B

| Parameter for SLS | Value |
| --- | --- |
| Scenario | Dense urban |
| Layout | Single layer: Macro layer: Hex. Grid |
| Channel model | 3D UMa, ISD = 200 m |
| Carrier frequency | 4 GHz |
| Aggregated system bandwidth | 200 MHz (UL + DL) |
| Simulation bandwidth | 20 MHz per CC |
| Tx power | BS: 44 dBm PA scaled down with simulation BW when system BW is higher than simulation BW. |
| UE distribution | Uniform 100% outdoor (120 kmph) |
| BS antenna configuration | 8x8x2 URA X-pol, slants -45/+45 degree, 1x8x2 TXRU layout 0.5-wavelength horizontal spacing 0.8-wavelength vertical spacing |
| UE antenna configuration | 2 Rx X-pol, slant 0/90 degrees |
| Traffic model | FTP 1 |
| TRP association | RSRP based Handover margin = 3dB |
| Channel estimation | Ideal |
| Interference covariance estimation | Ideal |
| CSI feedback | LTE codebook; (N1,N2) = (8,1); (O1,O2) = (4,1); Codebook Config. 1. i1 feedback only |
| Transmission mode | DM-RS based SU-MIMO Maximum rank of transmission is 1 |
| Scheduling | Proportional Fair |
| OLLA | 10% BLER target |
| Elevation beamforming | One vertical beam per TXRU electrically down-tilted to 100 degrees |
| UE receiver | MMSE-IRC |
| Max HARQ transmissions | 4 |

| Parameter for LLS | Value |  |
| --- | --- | --- |
| Scenario | Scenario 1 | Scenario 2 |
| Channel model | EVA; 300 Hz maximum Doppler frequency | TDL-B; DS = 100 ns; UE speed = 3 km/h |
| Allocation size | 50 PRB | 1 PRB |
| BS antenna configuration | 8 Tx X-pol with high (0.9) and medium (0.3) spatial correlation and medium (0.3) polarization correlation. | 2 Tx X-pol |
| UE antenna configuration | 2 Rx with no correlation between elements | 2 Rx X-pol |
| Channel estimation | Ideal | DMRS-based |
| Interference estimation | No interference | DMRS-based |
| Noise estimation | Ideal | Ideal |
| Interference | No interference | Noise limited scenario: no interference. Interference limited scenario: 1 interfering BS (INR = 10 dB). |
| CSI feedback periodicity | 5 subframes | No CSI feedback |
| CSI feedback delay | 8 subframes | No CSI feedback |
| UE receiver | MMSE | MMSE-IRC |
| MCS | QPSK 1/2; 16-QAM 1/2. | QPSK 1/5; QPSK 1/2; 16-QAM 1/2; 16-QAM 3/4; 64-QAM 2/3; 64-QAM 5/6; |
| Max HARQ transmissions | 0 | 0 |