3GPP TSG RAN WG1 Meeting #89                                                                R1-1707463

Hangzhou, P.R. China 15th – 19th May 2017

Source:	CATT

Title:	On NR RACH Preamble Design

Agenda Item:	7.1.1.4.1

Document for:	Discussion and Decision

Introduction

In RAN1# 88bis [1], some details regarding the NR RA preamble design were agreed as follows.

Agreements:

- NR RACH capacity shall be at least as high as in LTE

  - Such capacity is achieved by time/code/frequency multiplexing for a given total amount of time/frequency resources

- Zadoff-Chu sequence is adopted in NR

  - FFS other sequence type and / or other methods in addition to Zadoff-Chu sequence for the scenario, e.g., high speed and large cells

    - FFS definition of large cell and high speed

  - FFS other sequence type and / or other methods for capacity enhancements, e.g.:

    - At least in multi-beam and low speed scenario, regarding multiple/repeated PRACH preamble formats, option 2 with OCC across preambles

      - FFS: Option 2 with OCC across multiple/repeated preambles in high speed scenarios

    - PRACH preamble design composed with multiple different ZC sequences

    - Sinusoidal modulation on top of option 1

Agreements:

- For Zadoff-Chu sequence type, the RAN1 specifications will support two NR-PRACH sequence lengths (L)

  - L = 839: SCS = {1.25, 2.5, 5} KHz

  - Select one of

    - L = 63/71: SCS = {15, 30, 60, 120, 240} KHz

    - L = 127/139: SCS = {7.5, 15, 30, 60, 120} KHz

  - FFS: Supported sub-carrier spacings for each sequence length

- FFS for other sequence types

Agreements:

- Waveform for RACH message 3 can be DFT-S-OFDM or CP-OFDM. Network signals directly or indirectly RACH message 3 waveform to UE:

  - The network signals the waveform for RACH message 3 in the remaining minimum SI as one bit

Agreements:

- In NR, the RACH configuration provides at least:

  - RACH time/freq. information

  - RACH preamble format

In this contribution we further investigate some aspects of the NR RACH preamble sequence design, focusing on the consecutive multiple/repeated RACH preambles option 4 and ZC sequence, and performance comparison of NR-PRACH option 4 and NR-PRACH option 2. In our companion contribution [2], we study the NR 4-step RA procedures.

RACH Preamble Sequence Design in NR

Several aspects need to be considered for the NR preamble including the transmission duration, numerology and resource provisioning. In this contribution we touch upon some of these considerations.

In NR frame structure design, the self-contained structure is the primary frame structure to support fast transmission and short processing time with HARQ-ACK feedback within a slot as shown in Figure 1. As shown, a slot may contain a DL transmission part, gap period and UL transmission part. The duration of each DL/UL transmission part is expected to be short to support high speed data transmission with low latency for eMBB applications. Thus, the RACH preamble sequence should be short enough to fit the dynamic TDD and self contained structure.

As agreed in RAN1#88bis, Zadoff-Chu sequence is adopted in NR and the RAN1 specifications will support two NR-PRACH sequence lengths (L):

- L = 839: SCS = {1.25, 2.5, 5} KHz

- Select one of

  - L = 63/71: SCS = {15, 30, 60, 120, 240} KHz

  - L = 127/139: SCS = {7.5, 15, 30, 60, 120} KHz

For the length of short PRACH sequence, L =71 with SCS = {15, 120, 240} KHz is preferred.

For above 6 GHz, SCS =15 KHz is the same as the data channel in order to reuse the FFT module in the receiver for the data channel.

For below 6 GHz, SCS = 120 or 240 KHz can be reused as the same SCS has been approved in NR-PSS and NR-SSS.

Proposal 1: For the length of short PRACH sequence, L =71 with SCS = {15, 120, 240} KHz should be supported.

Figure 1: NR frame structure – slot and subframe

As earlier mentioned, a short preamble sequence has the advantage of low detection complexity, albeit with lower coverage it supports RTT estimation for smaller cell sizes. It also implies that more root sequences are required to meet the target collision probability. In other words, the PRACH capacity is reduced for shorter preamble sequences all other things being equal. The lower coverage implies that higher SINR is required for a target detection performance. In order to improve the preamble detection performance and reduce the collision probability of a short preamble sequence, a multi-stage preamble sequence could be considered. Multi-stage preamble sequence is designed to have a set of multiple short preamble sequences in combination as a single RA Msg1 transmission. The UE would select a set of short preamble sequences transmitted in a single time frequency resources as configured by the network. The network (gNB/TRP) would detect the entire set of short preamble sequences to declare a successful RACH Msg1 detection. A multi-stage short preamble sequence could eliminate the ambiguity of RACH preamble peak detection, which relaxes the SINR requirement for preamble detection. If the short preamble sequence is selected independently at each stage, the collision probability will be reduced exponentially.

Proposal 2: Multi-stage short preamble sequence should be considered for RA Msg1 for better detection performance and low collision probability.

The multi-stage short preamble sequence with short CP and GT is more suitable for small cells, but not for large cells in NR. As agreed in RAN1 #86, NR should support both short and long preamble lengths. So the configurability of both short and long preamble lengths should be considered. One solution is to reuse the same scheme of LTE preamble, i.e., define different RACH preamble formats with different preamble lengths.

Proposal 3: Multi-stage short preamble and long preamble should both be supported and different RACH preamble format can be defined with different preamble lengths.

For LTE, the maximum number of preambles supported in a cell is 64. For NR, the maximum number of preambles supported in a cell should be no less than that as LTE, e.g., 64.

Sequence Design

For NR RACH preamble option 4 with multiple-stage sequence as shown in Figure 2 where two short sequences in combination as one RA Msg1. Two different RACH sequences with CP are used and GT is reserved at the end of the two consecutive preamble sequences. Each preamble is selected independently by UE and detected independently by gNB. As gNB would detect the both short preamble sequences before declaring a successful RA Msg1 detection, which means that the RA Msg1 is successfully detected if and only if the preamble-1 and preamble-2 are both successfully detected.

The  root Zadoff-Chu sequence is also adopted as the preamble sequence, which is defined by

xun=e-jπunn+1NZC, 0≤n≤NZC-1													(1)

where  is a prime number.

Figure 2: NR RACH preamble with two-stage preamble sequence

For NR PRACH option 4, the proposed preamble sequence below 6GHz and above 6GHz are given in Table I and Table II, respectively.

Table I. Proposed preamble sequence of NR-PRACH option 4 below 6GHz

|  | Sequence Type | Sequence Length | Subcarrier spacing [KHz] | Transmission BW [MHz] | N_OS | N_RP | Ts (ms) | CP(Ts) | GT(Ts) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type1 | Zadoff-Chu | 421 | 2.5 | 1.08 | 2 | 1 | 1/30720 | 1584 |  |
| Type 3 | Zadoff-Chu | 71 | 15 | 1.08 | 2 | 1 | 1/30720 | 448 | 288 |

Table II. Proposed preamble sequence of NR-PRACH option 4 above 6GHz

|  | Sequence Type | Sequence Length | Subcarrier spacing [KHz] | Transmission BW [MHz] | N_OS | N_RP | Ts (ms) | CP(Ts) | GT(Ts) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type 4 | Zadoff-Chu | 71 | 120 | 8.64 | 2 | 1 | 1/(4*30720) | 448 |  |
| Type 5 | Zadoff-Chu | 71 | 240 | ## 17.28 | 2 | 1 | 1/(8*30720) | 448 |  |

Sequence Type3 of NR PRACH option 4 is derived from LTE format 4. Table III gives an example format of the two-stage preamble in time domain, based on similar structure in Figure 3.

Table III. Detail of NR-option4 preamble sequence Type3

| TCP1 | TSEQ1 | TCP2 | TSEQ2 | TGT |
| --- | --- | --- | --- | --- |
| 14.6us (448×Ts) | 66.7us (2048×Ts) | 14.6us (448×Ts) | 66.7us (2048×Ts) | 7.4us (288×Ts) |

For comparison, the LTE format 0/ 4 and NR-option 1 preamble sequence are given in Table IV.

Table IV. Preamble sequence of LTE format 0 / 4 and NR-option 1

|  | Sequence Type | Sequence Length | Subcarrier spacing [KHz] | Transmission BW [MHz] | N_OS | N_RP | Ts (ms) | CP(Ts) | GT(Ts) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline 1, LTE format 0 | Zadoff-Chu | 839 | 1.25 | 1.08 | 1 | 1 | 1/30720 | 3168 | x |
| Baseline 2, LTE format 4 | Zadoff-Chu | 139 | 7.5 | 1.08 | 1 | 1 | 1/30720 | 448 | x |
| NR-option 1–Type 1 in [3] | Zadoff-Chu | 71 | 15 | 1.08 | 14 | 1 | 1/30720 | 0 | 2048 |

Performance Analysis

Auto-correlation and cross-correlation

Property 1: The ZC sequences of any length have ‘ideal’ normalized cyclic autocorrelation (i.e. the correlation with the circularly shifted version of itself is a delta function) [4].

Property 2: The absolute value of the cyclic cross-correlation function between any two ZC sequences is constant and equal to , if  (where  and  are the sequence root indices) is relatively prime with respect to  [4]. As  is a prime number for both LTE and NR RACH preambles, the condition is guaranteed if  and .

Therefore, the auto-correlation and cross-correlation performance are better with a larger for a single ZC sequence. Obviously, the detection performance of LTE RACH preamble with one-stage sequence is better than that of each individual stage of NR RACH preamble option 4 with multi-stage sequence under the same false detection rate, because the ZC sequence length for each individual stage is shorter. However, this does not automatically lead to the detection performance of NR RACH preamble option 4 with multi-stage sequence will be worse than the detection performance of LTE RACH preamble with one-stage sequence. For example, with our two step detection algorithm, which first detects the preamble indexes of the short sequences and then uses the whole long sequences for the confirmation of the detected preambles, will obtain the similar performance as that of RACH preamble design with one-stage sequence in absence of frequency offset. In addition, NR RACH preamble option 4 with multi-stage sequence will have superior detection performance than RACH preamble with one-stage sequence with high frequency offset as discussed later.

Comparison of total number of preambles

For fair comparison, only a PRACH sub-band and a PRACH time slot are assumed in the following analysis.

For LTE RACH preamble format 4 as shown in (1), the total sequence number is

,                                                                     (2)

where  denotes the round down operation.

For NR RACH preamble option 4 as shown in (1) and figure 2, the total sequence number is

.                                                                  (3)

Take , and  and 71 for LTE RACH preamble format 4 and NR RACH preamble separately for example, the total sequence number can be calculated as , and .

The total number of multi-stage sequence is much larger than that of one-stage sequence (), which will result in a much lower collision probability.

In order to ensure the number of one-stage sequence and multi-stage sequence are the same, a different solution is to configure multiple PRACH sub-bands in the same PRACH time slot, and / or multiple PRACH time slots within the same PRACH sub-bands. However, the solution will be at the expense of PRACH resources.

Collision probability

A collision is assumed to occur only if multiple UEs make a random access in the same random-access slot and frequency band with the same preamble sequence for one-stage sequence and the same whole sequences for multi-stage sequence. If there are a larger number of potential UEs, the UE arrival is a Poisson distribution and the arrival rate is λ, the random access collision probability has been analyzed in [5] and given by

(4)

where  is the total number of random-access opportunities per second per cell and  is the random-access intensity, i.e., there are   random-access attempts per second and cell.

For fair comparison, only a PRACH sub-band and a PRACH time slot are assumed in the following analysis. Table V shows the collision probability of two types of RACH preamble sequence with different arrival rate . The number of cells in the network is 19.

Table V. Collision probability of two types of RACH preamble sequence

| \ \ | LTE RACH preamble format 4 | NR RACH preamble option 4 |
| --- | --- | --- |
| =1.0 | 0.013 | 0.00016 |
| =5.0 | 0.061 | 0.00078 |
| =10.0 | 0.12 | 0.0015 |
| =15.0 | 0.17 | 0.0023 |
| =20.0 | 0.22 | 0.0031 |
| =50.0 | 0.47 | 0.0077 |

As shown in Table V, the collision probability of multi-stage sequence is much lower than that of one-stage sequence.

The impact of frequency offset on the time offset

As well known, the correlation of two ZC sequences with the same root index is a function of both the frequency offset and the time offset between them, rather than the time offset alone (or so called the detection ambiguity between frequency offset and time offset). During PRACH detection, if there is a large frequency offset between the received preamble and local one, the peak of correlation may not provide the accurate PRACH timing offset estimate. Since NR is going to support much higher carrier frequency than LTE with much shorter CP length, accurate timing estimation will become even more critical in NR PRACH detection performance than in LTE.

Consider that the impact of the frequency offset on the time offset estimation is ZC root index dependent, or more specifically, the maximum correlation of two ZC sequences with the same root index u occurs when Δλ=u ⋅Δk mod(NZC) [7], where  Δλ is the frequency offset normalized to the PRACH subcarrier and Δk is the shift offset between the two ZC sequences [7], using the correlation results from more than one ZC sequence with different root indexes provide an effective way to mitigate the issue.

Let us assume we have two-stage short preamble scheme where preamble-1 has root index u1 and preamble-2 has root index u2 and the frequency offset is not changed for both preambles. The assumption is reasonable before the very short time duration of the frequency offset. Now, we have two independent detections for the preamble-1 and preamble-2. Then, the time offset estimates for both preamble detections will be different if both detection are successful, due to fact that the two preambles have different root indexes. It is clear that the two timing detections will provide more information to improve the timing accuracy than when the same root index is used in both preambles, and how to combine the two time estimates to come with an optimal timing estimation may need further study. As an extreme case, if the ZC-sequence for the preamble-2 is a conjugate of the ZC-sequence for the preamble-1, the impact of frequency offset on the timing error to the detection of both preambles will have the same magnitude but in the exact opposite directions. In this case, the impact of the frequency offset on the timing error can be eliminated at the cost of increasing the collision probability. As a more general case, one may estimate the impact of the frequency offset on each of the timing offset estimation, and then use the relationship of Δλ=u ⋅Δk mod(NZC)  to further estimates and improve the timing offset estimation accuracy. The effectiveness of using two-stage short preambles to improve the timing offset estimation performance are illustrated later in Figure 24 and Figure 25.

Observation 1: NR RACH preamble option 4 with multi-stage sequence can provide more robust timing offset estimation under the impact of frequency offset than RACH preamble with one-stage sequence.

Solution of potential detection ambiguity

If Preamble-1 and Preamble-2 are ZC-sequences for two-stage short preamble scheme, the detection ambiguity may take place when two UEs transmit the preamble sequences with the same PRACH resources. For example, if UE1 transmits the preamble sequences with the preamble index pair {a, b}; and UE2 with the preamble index pair {c, d}, the BS may detect preamble indexes {a, c} for the first preamble as well as preamble indexes {b, d} for the second preamble. It results in 4 possible preamble sequences from the combinations of the detected preamble indexes, namely, {a, b}, {a, d}, {c, b}, and {c, d}.

In the following, we present two methods for the solution of the detection ambiguity.

Method1: Based on (relative) timing estimation

Figure 3 shows the CASE when UE1 sends {a, b} and UE2 sends {c, d}. At stage-one, the gNB receiver detects preamble index a at T2 and preamble index c at T5. At stage-two, the gNB receiver detects preamble index b at T2 and preamble index d at T5. Since the short-preambles transmitted from the same UE should be detected at BS with the same TA, the gNB will be able to determine the preambles {a, b} from one UE; and {c, d} from another UE.  So the gNB will broadcast the preamble sequence {a, b} and {c, d} to all UE in Msg 2.  Thus, based on the relative timing estimation, the detection ambiguity problem can be solved. Furthermore, we can define a ‘Judgement Win’ to define the timing location of a preamble sequence.

Figure 3: Illustration of method1 based on (relative) timing estimation

Method2:A Two-Stage PRACH Design without Ambiguity

The following is a modified two-stage design that will avoid aforementioned detection ambiguity.

The first preamble sequence Preamble-1, p1n=xun is still the Zadoff-Chu sequence with root index u as shown in (1).

The second preamble sequence p2n is defined as

p2n=xvns(n),   0≤n≤NZC-1                                                               (5)

where xvn is the Zadoff-Chu sequence with root index v, and s(n) is the cyclic shift of an m-sequence according to

sn=sn+n0mod63,  0≤n≤NZC-1                                                     (6)

where n0 is the cyclic shift determined by the first preamble sequence p1n. For example, n0 can be the preamble index u of p1(n). , , is defined by

(7)

with initial conditions.

During PRACH detection, once the BS detects successfully a first preamble p1n, the BS determines the corresponding scramble sequence for the second preamble, and perform the de-scramble operation before detecting the second preamble. In this way, the detected second preamble p2n will be associated asscoated with the first preamble p1n, but not other first preambles.

The design will avoid the BS detection ambiguity discussed before. Using previous example, if there are two UEs transmitting the same preamble sequences with the same PRACH resources, UE1 with preamble index pair {a, b}; and UE2 with the preamble index pair {c, d}. When BS detects successfully the first preamble indexes a and c, the BS will conduct the scramble operation and preamble detection for the second preamble by the use of the indexes a and c separately. When index a is used for the de-scramble operation, the BS will detect the second preamble with index b. Similarly, when index c is used for the de-scramble operation, the BS will detect second preamble with index d.

Performance Evaluation

The simulation assumptions are given in Table VII of Appendix A.

Note: For all simulation results from section 2.3.1 to 2.3.5, the legend ‘LTE format 4’ means that LTE PRACH format 4 with detection method in Appendix B; ‘NR option4-seq1’ and ‘NR option4-seq2’ mean that preamble sequence 1 and sequence 2 of NR option4 are detected independently with the same detection method as LTE format 4; ‘NR option4-seq-comb’ indicates that preamble sequence 1 and preamble sequence 2 of NR option4 are detected together to get the optimum detection results. ‘NR-option 1-type 1-one seg’ means NR-PRACH option 1-type 1 with one segment correlation algorithm. ‘NR-option 1-type 1-two seg’ means NR-PRACH option 1-type 1 with two segment correlation algorithm in [3].

PAPR / CM

The PAPR/CM values in dB [6] are given in Table V.

Table V. PAPR / CM values of some LTE formats and NR-RACH-options

|  | PAPR [dB] | CM [dB] |
| --- | --- | --- |
| LTE format 0 (Ncs=167) | 3.03 | -0.25 |
| LTE format 4 (Ncs=12) | 3.36 | 0.23 |
| NR-Option4-Type1 (Ncs=167) | 3.49 | 0.34 |
| NR-Option4-Type3 (Ncs=12) | 3.35 | 0.11 |
| NR-Option1-Type1 (Ncs=54) | 2.88 | -0.03 |
| NR-Option1-Type1 (Ncs=12) | 4.29 | 1.21 |

Miss-detection probability performance

The miss detection performances in five CASEs are evaluated in this section.

CASE 1: Without initial timing offset for NR option 4-type 3 and LTE format 4;

CASE 2: initial timing offset uniform in [0, 10us] for NR option 4-type 3 and LTE format 4.

CASE 3: initial timing offset uniform in [0, 50us] for NR option 4-type 1; uniform in [0, 100us] for LTE format 0.

CASE 4: initial timing offset uniform in [0, 50us] for NR option 4-type 1 and NR-option1-type1; uniform in [0, 100us] for LTE format 0.

CASE 5: initial timing offset uniform in [0, 50us] for NR option4-type1 and NR option2-type1.

CASE1: Without initial timing offset 【LTE format 4 vs NR-option4-type3】

The simulation results of miss detection without timing error for NR-option 4-type3 and LTE format 4 are given from Figure 4 to Figure 9, where Ncs=12 is used. As shown in Figure 4 and Figure 5 with frequency offset of 0 Hz, the performance of ‘NR option4-seq-comb’ is better than ‘NR option4-seq1’ and ‘NR option4-seq2’, and that of ‘LTE format 4’ is the best. Compared with ‘NR option4-seq1’ and ‘NR option4-seq2’ @ miss detection probability of 1%, ‘NR option4-seq-comb’ achieves gain of 1.6 dB and 1.9 dB for EVA and ETU, respectively, ‘LTE format 4’ achieves gain of 3.5 dB and 3.6 dB for EVA and ETU, respectively.  As shown in Figure 6 and Figure 7 with frequency offset of 3.75KHz, the miss detection performance of ‘NR option4-seq1’, ‘NR option4-seq2’ and ‘LTE format 4’ are nearly the same, while ‘NR option4-seq-comb’ is better, with gain of 1.3 dB and 2 dB @ miss detection probability of 1% for EVA and ETU, respectively. Figure 8 and Figure 9 compare the miss detection performance with frequency offset of 5 KHz. The performance of ‘NR option4-seq1’ and ‘NR option4-seq2’ are the same, and better than ‘LTE format 4’. The performance of ‘NR option4-seq-comb’ is best, with gain of 4.0 dB and 4.1 dB @ miss detection probability of 10% for EVA and ETU, respectively.

CASE1.1: frequency offset =0 Hz

Figure 4: CASE1.1 (a): EVA-0 Hz                                         Figure 5: CASE1.1 (b): ETU-0 Hz

CASE1.2: frequency offset =3.75 KHz

Figure 6: CASE1.2 (a): EVA-3.75KHz                   Figure 7: CASE1.2 (b): ETU-3.75KHz

CASE1.3: frequency offset = 5 KHz

Figure 8: CASE1.3 (a): EVA-5KHz                         Figure 9: CASE1.3 (b): ETU-5KHz

CASE2: With initial timing offset 【LTE format 4 vs NR-option4-type3】

The simulation results of miss detection for NR-option 4-type3 and LTE format 4 are given from Figure 10 to Figure 11. As shown in Table VII, the timing error is uniform in [0, 10us] for NR option-4-type3 and LTE format 4, and Ncs=12 is used.

CASE2.1: frequency offset =600 Hz (0.15ppm@4GHz), TDL-C 3km

Figure 10: Miss detection performance under frequency offset =600 Hz (0.15ppm@4GHz), TDL-C 3km

CASE2.2: frequency offset =600 Hz (0.15ppm@4GHz), TDL-C 120km

Figure 11: Miss detection under frequency offset =600 Hz (0.15ppm@4GHz), TDL-C 120km

CASE3: With initial timing offset 【LTE format 0 vs NR-option4-type1】

The simulation results of miss detection for NR-option 4-type1 and LTE format 0 are given from Figure 12 to Figure 14. As shown in Table VII, the timing error is uniform in [0, 50us] for NR option-4-type 1 and uniform in [0, 100us] for LTE format 0, and Ncs=167 is used.

CASE3.1: frequency offset =600 Hz (0.15ppm@4GHz), AWGN

Figure 12: Miss detection performance under frequency offset =600 Hz (0.15ppm@4GHz), AWGN

CASE3.2: frequency offset =600 Hz (0.15ppm@4GHz), TDL-C 3km

Figure 13: Miss detection performance under frequency offset =600 Hz (0.15ppm@4GHz), TDL-C 3km

CASE3.3: frequency offset =600 Hz (0.15ppm@4GHz), TDL-C 120km

Figure 14: Miss detection performance under frequency offset =600 Hz (0.15ppm@4GHz), TDL-C 120km

CASE4: With initial timing offset 【LTE format 0 vs NR-option4-type1 vs NR-option1-type1】

The simulation results of miss detection for NR-option 4-type1, NR-option-type1 and LTE format 0 are given from Figure 15 to Figure 16. As shown in Table VII of Appendix A, the timing error is uniform in [0, 50us] for NR option-4-type 1 and uniform in [0, 100us] for LTE format 0.

In CASE 4 of initial timing offset, Ncs =167 is used for both of NR PRACH option 4-type 1and LTE PRACH format 0, Ncs =54 is used for NR PRACH option 1-type 1.

CASE 4.1: frequency offset =600 Hz (0.15ppm@4GHz), AWGN

Figure 15: Miss detection performance under frequency offset =600 Hz (0.15ppm@4GHz), AWGN

CASE 4.2: frequency offset =600 Hz (0.15ppm@4GHz), TDL-C 3km

Figure 16: Miss detection performance under frequency offset =600 Hz (0.15ppm@4GHz), TDL-C 3km

CASE5: With initial timing offset 【NR-option2 vs NR-option4】

In this section, the performance of NR-option2 and NR-option4 with the frequency offset =600 Hz (0.15ppm@4GHz) are compared. The simulation assumptions are given in Table VII of Appendix A, where the timing error is uniform in [0, 50us] for NR option4-type1 and NR option2-type1. Ncs =167 is used for both of NR option 2-type 1 and option 4-type 1.

Figure 17 and Figure 18 show the miss detection performance of NR option 2 and NR option 4 with type1-Ncs=167 under AWGN and TDL-C channel, respectively. The SNR of NR option 4 is 0.6 dB better than that of NR option 2 at the miss detection probability of 1%. The reason lies in that the detection method of NR option 2 in [8] is based on correlation combining of two OCC sequences. In this case, Tseq =1/2.5K = 400us, and the normalized phase offset is Foffset* Tseq = 600*400e-6 = 0.24. Since the two sequences are not highly correlated, the correlation combining in NR option 2 causes performance degradation.

CASE 5.1: Type1-Ncs=167-AWGN

Figure 17: Miss detection performance with type1-Ncs=167-AWGN

CASE 5.2: Type1-Ncs=167-TDL-C 3km/h

Figure 18: Miss detection performance with type1-Ncs=167-TDL-C 3km/h

False alarm rate performance

The false alarm rates are given in Figure 19 and Figure 20.

1) Ncs= 12 for LTE format 4 and NR-option 4-type 3

Figure 19: False alarm performance of NR-PRACH option 4-type 3 and LTE format 4

2) Ncs = 167 for LTE format 0 and NR-option 4-type 1, Ncs= 54 for NR-option 1-type 1

Figure 20: False alarm performance of NR-PRACH option 4-type 1, option 1-type 1 and LTE format 0

Timing estimation accuracy

The timing estimation results from CASE 2 to CASE 5 are given from Figure 21 to Figure 27, where SNR = 0 dB.

CASE2: With initial timing offset 【LTE format 4 vs NR-option4-type3】

CASE2.1: frequency offset =600 Hz (0.15ppm@4GHz), AWGN

Figure 21: Timing estimation error under frequency offset =600 Hz (0.15ppm@4GHz), AWGN

CASE2.2: frequency offset =600 Hz (0.15ppm@4GHz), TDL-C 3km

Figure 22: Timing estimation error under frequency offset =600 Hz (0.15ppm@4GHz), TDL-C 3km

CASE2.3: frequency offset =600 Hz (0.15ppm@4GHz), TDL-C 120km

Figure 23: Timing estimation error under frequency offset =600 Hz (0.15ppm@4GHz), TDL-C 120km

CASE4: With initial timing offset 【LTE format 0 vs NR-option4-type1 vs NR-option1-type1】

CASE 4.1: frequency offset =600 Hz (0.15ppm@4GHz), AWGN

Figure 24: Timing estimation error under frequency offset =600 Hz (0.15ppm@4GHz), AWGN

CASE 4.2: frequency offset =600 Hz (0.15ppm@4GHz), TDL-C 3km

Figure 25: Timing estimation error under frequency offset =600 Hz (0.15ppm@4GHz), AWGN 3km

CASE5: With initial timing offset 【NR-option2 vs NR-option4】

CASE 5.1: Type1-Ncs=167-AWGN

Figure 26: Timing estimation performance with type1-Ncs=167, AWGN

CASE 5.2: Type1-Ncs=167-TDL-C 3km/h

Figure 27: Timing estimation performance with type1-Ncs=167, TDL-C 3km/h

MCL

Table VI gives an example of MCL value for each design. For CASE2.1 with frequency offset =600 Hz (0.15ppm@4GHz), TDL-C 3km, SCS is 15 KHz. For CASE 3.3 with frequency offset =600 Hz (0.15ppm@4GHz), TDL-C 3km, SCS is 2.5 KHz.

Table VI. MCL value

| Subcarrier spacing (kHz) | 2.5 |  | 15 |
| --- | --- | --- | --- |
| Transmitter |  |  |  |
| (0) Max Tx power (dBm) | 23.0 |  | 23.0 |
| (1) Actual Tx power (dBm) | 23.0 |  | 23.0 |
| Receiver |  |  |  |
| (2) Thermal noise density (dBm/Hz) | -174.0 |  | -174.0 |
| (3) Receiver noise figure (dB) | 5.0 |  | 5.0 |
| (4) Interference margin (dB) | 0.0 |  | 0.0 |
| (5) Occupied channel bandwidth (MHz) | 1.08 |  | 1.08 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log((5)) (dBm) | -108.7 |  | -108.7 |
| (7) Required SINR (dB) | 2 |  | -5 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -106.7 |  | -102.42 |
| (9) MCL = (1) - (8) (dB) | 129.7 |  | 136.7 |

Results analysis

In above five subsections from 2.3.1 to 2.3.5, we compare the performance of LTE format 0 / 4 and NR-PRACH-option 1 / 4 in terms of five metrics: PAPR / CM, miss detection, false alarm rate, timing estimation accuracy and MCL.

The PAPR / CM values of NR-PRACH-option 1/4 are the same as LTE format 0/4, except that the case of NR-Option1-Type1 (Ncs=12) is a little larger.

For miss detection at false alarm rate 0.1%, we can observe that with low frequency offset (0 Hz and 600Hz), the miss detection performance of NR option 4 is worse than LTE format 4 and format 0 as the auto-correlation of short ZC sequence for NR option 4 is small than that of long ZC sequence for LTE format 4 and format 0. With high frequency offset (3.75 KHz and 5 KHz), the miss detection performance of NR option 4 is better than LTE format 4 as NR option 4 has larger SCS and is more insensitive to frequency offset. When the maximum frequency offset (Foffset) is 3.75 KHz. For LTE PRACH format 4 where Tseq=4096 Ts=133.3us, the normalized phase offset is Foffset* Tseq = 0.5. For NR PRACH option 4 where Tseq =2048 Ts=66.67us, the normalized phase offset is Foffset* Tseq = 0.25. When the maximum frequency offset (Foffset) is 5 KHz. For LTE PRACH format 4 where Tseq=4096 Ts=133.3us, the normalized phase offset is Foffset* Tseq = 0.67. For NR PRACH option 4 where Tseq =2048 Ts=66.67us, the normalized phase offset is Foffset* Tseq = 0.33.

For timing estimation accuracy, NR-PRACH-option 1 and option 4 have almost the same accuracy as LTE format 0 and format 4, respectively.

Observation 2: NR RACH preamble option 4 with two-stage sequence can achieve a better performance than LTE RACH preamble with one-stage sequence with large frequency offset.

Conclusion

This paper discusses the NR RACH preamble sequence design with the consideration of self-contained frame structure and dynamic TDD configuration, and analyzes the performance of multi-stage sequence and one-stage sequence. We have the following observations:

- Observation 1: NR RACH preamble option 4 with multi-stage sequence can provide more robust timing offset estimation under the impact of frequency offset than RACH preamble with one-stage sequence.

- Observation 2: NR RACH preamble option 4 with two-stage sequence can achieve a better performance than LTE RACH preamble with one-stage sequence with high frequency offset.

Based on the analysis, we propose:

- Proposal 1: For the length of short PRACH sequence, L =71 with SCS = {15, 120, 240} KHz should be supported.

- Proposal 2: Multi-stage short preamble sequence should be considered for RA Msg1 for better detection performance and low collision probability.

- Proposal 3: Multi-stage short preamble and long preamble should both be supported and different RACH preamble format can be defined with different preamble lengths.

Reference

- Chairman's Notes RAN1_88bis

- R1-1707464, “Further details on NR 4-step RA Procedure”, CATT

- R1-1611906, “NR PRACH preamble evaluations”, Ericsson

- “LTE – The UMTS Long Term Evolution: From Theory to Practice”, Edited by S. Sesia, I. Toufik and M. Baker, Second Edition

- R1-061369, “LTE random-access capacity and collision probability,” Ericsson, RAN1#45, Shanghai, China, May 08-12, 2006

- R1-060023, “Cubic metric in 3GPP-LTE,” Motorola

- Min Hua, et al., “Analysis of the Frequency Offset Effect on Random Access Signals”, IEEE TRANSACTIONS ON COMMUNICATIONS, VOL. 61, NO. 11, NOVEMBER 2013.

- R1-1706171, “PRACH design consideration”, Qualcomm Incorporated.

Appendix A: Simulation assumptions

The simulation assumptions are shown in Table VII.

Table VII. Simulation assumptions

|  | Below 6GHz |
| --- | --- |
| Carrier Frequency | 4 GHz |
| Channel Model | TDL-C, AWGN EVA, ETU |
| Delay Scaling | 100 ns for TDL-C |
| Antenna Configuration at BS | (M,N,P) = (1,1,2) with omni-directional antenna element |
| Antenna Configuration at the UE | (1,1,2) with omni-directional antenna element |
| Frequency Offset | CASE 1: 0 Hz, 3.75 KHz, 5 KHz; CASE 2, CASE 3, CASE 4, CASE 5: 0.05 ppm at TRP , 0.1 ppm at UE; |
| UE speed | 3 km/h, 120 km/h |
| Initial timing Offset | CASE 1: Without for NR option-4-type 3 and LTE format 4; CASE 2: Uniform in [0, 10us] for NR option 4-type 3 and LTE format 4; CASE 3: Uniform in [0, 50us] for NR option 4-type 1 and uniform in [0, 100us] for LTE format 0; CASE 4: Uniform in [0, 50us] for NR option 4-type 1 and NR-option 1-type 1; uniform in [0, 100us] for LTE format 0; CASE 5: Uniform in [0, 50us] for NR option4-type1 and NR option2-type1. |
| Preamble Detector | For LTE and NR option 4, refer to Appendix B. For NR option 1, refer to [3]. For NR option 2, refer to [8]. |
| FFT size | 2048 |
| Sampling Frequency | 30.72 MHz |
| LTE PRACH format | For LTE PRACH format 4, Ncs =10 and Ncs =12 are used in CASE1 and CASE2 of initial timing offset, respectively. For LTE PRACH format 0, Ncs =167 is used in CASE 3 and CASE 4 of initial timing offset. A UE is selected, which uses preamble sequence index 1 from a given preamble sequence set with the dimensions of 64. |
| NR PRACH Option | Type1 and Type3 of NR-PRACH option4 in Table I are used. For NR PRACH option 4-type3, Ncs =10 and Ncs =12 are used in CASE1 and CASE2 of initial timing offset, respectively. For NR PRACH option 4-type1, Ncs =167 is used in CASE 3 and CASE 4 of initial timing offset. For NR PRACH option 1-type1, Ncs =54 is used in CASE 4 of initial timing offset. A UE has two short preamble sequences. The short preamble sequences are selected from the same preamble sequences set with the dimensions of 64. The first preamble uses sequence index 1 and second preamble uses sequence index 2. |

- Fixed false alarm rate: 0.1% when input at receiver is noise only (Definition of false alarm rate as in 36.104, section 8.4.1)

- Single cell scenario

- Single mobile scenario

Preamble detection metrics

The following three error cases which the threshold is set such that the false alarm probability does not exceed 0.1%, when the input is noise only, can result in a false alarm or a missed detection:

- Detecting different preamble than the one that was sent

- Not detecting a preamble at all

- Correct preamble detection but with the wrong timing estimation

False Alarm Probability is given by the ratio of total number detected but not transmitted preambles and the total number of possible detection occurrences.

The missed detection probability is the ratio between the total number of transmitted but not detected preambles with the correct timing estimate, and the total number of transmitted preambles within an observation interval. The observation interval should be sufficiently large.

The timing estimation error is given by:

ϵ=Testimate- Tinitial_timing_offset– DelaySpread

The estimated timing is the outcome of the timing estimator. The initial timing offset is a uniform random values are given in Table 5-1.

For 4 GHz, assume that the data SCS is 30 KHz and CP is 2.3us, then timing estimation error should be less than 1.2 us. For 30 GHz, assume that the data SCS is 120 KHz and CP is 0.6us, then timing estimation error should be less than 0.3 us

Appendix B: Detection Method for PRACH Preamble

LTE PRACH

The detection method for LTE PRACH format 0 / format4 is shown in Figure 28. First, we calculate the correlation power and noise power of the received preambles with 64 candidate preamble sequences at the receiver. Then, we calculate the detection variable (ratio of correlation power and noise power), and compare detection variable with a given threshold.

Figure 28: Illustration of detection method for LTE PRACH format 0 / format 4

NR PRACH Option 4

There are two short-preamble sequences for NR PRACH option 4. In the simulation, the two short-preambles are first detected separately as described in Section 2.2.5 to solve the ambiguity issue. More optimal detection results are then obtained based on the combination of the detection results of the short-preambles, especially to improve PRACH timing offset detection performance under large frequency offset. One of the possible detection implementations for NR PRACH option4 is shown in Figure 29.

Figure 29: Illustration of detection method for NR PRACH option 4