3GPP TSG RAN WG1 Meeting #89			R1-1707805

Hangzhou, P. R. China, 15th – 19th May 2017

Source:		SoftBank, Ericsson, NTT DOCOMO, Sharp, ZTE

Title:	Further clarification of IoT

Agenda Item:	6.2.8.2

Document for:	Decision

Background

As for the performance metrics for aerial UE evaluations, the following agreement was made in RAN1#88bis.

Agreement:

- The following performance metrics should be considered for aerial study:

  - Packet throughput

    - UL and DL packet throughput statistics for aerial UEs Data traffic

    - UL and DL packet throughput statistics for all UEs Data traffic

    - UL and DL packet throughput statistics of terrestrial UEs Data traffic

  - Interference

    - UL IoT (interference over thermal) and DL wideband SINR statistics for reference

    - FFS other DL statistics such as RSRP and RSRQ

  - Other metrics are not precluded

Though RAN1 agreed to use IoT as a reference metric to show the uplink interference situation, we would propose to modify the agreement to “effective IoT” due to the following reasons:

We already have a clear definition of effective IoT in section A.2.1.8 of TR36.814, which can help to avoid the companies’ misunderstanding.

Effective IoT can represent the interference rejection effect by MMSE-IRC receiver, which RAN1 has already agreed to employ as a baseline receiver

Effective IoT has been widely used for RAN1 evaluations e.g. simulator calibration.

Proposal

Add the following note to further clarify the agreement in RAN1#88bis:

Interference

  - UL IoT (interference over thermal) and DL wideband SINR statistics for reference

Note: UL IoT means effective IoT defined in section A.2.1.8 of TR36.814