3GPP TSG RAN WG1 Meeting #89	       	R1-1708991

Hangzhou, China, 15 - 19 May 2017

Source: 	Ericsson

Title:	Corrections to 36.214

Agenda Item:	6.1.3

Document for:	Discussion and Decision

# Introduction

In this contribution we describe a problem with the definition of S-RSSI in 36.214 [1].

# Discussion

Section 5.1.28 in 36.214 describes the S-RSSI measurement in the following way:

| Definition | Sidelink RSSI (S-RSSI) is defined as the linear average of the total received power (in [W]) per SC-FDMA symbol observed by the UE only in the configured sub-channel in SC-FDMA symbols 1, 2, …, 6 of the first slot and SC-FDMA symbols 0,1,…, 5 of the second slot of a subframe The reference point for the S-RSSI shall be the antenna connector of the UE. If receiver diversity is in use by the UE, the reported value shall not be lower than the corresponding S-RSSI of any of the individual diversity branches |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

The S-RSSI measuremetnt is defined as a linear average over SC-FDMA symbols 1, 2, 3, 4, 5, and 6 in the first slot and SC-FDMA symbols 0, 1, 2, 3, 4, and 5 in the second slot (i.e., excluding the first symbol -AGC- and the last symbol -GP-). However, some of the averaged symbols are DM-RS symbols thus strictly speaking not SC-FDMA symbols.

Proposal:

- The S-RSSI measurement is defined as a linear average over OFDM symbols 1, 2, 3, 4, 5, and 6 in the first slot and OFDM symbols 0, 1, 2, 3, 4, and 5 in the second slot.

We present a corresponding CR in [2].

# Conclusion

In this contribution, we have discussed an issue related to the definition of S-RSSI measurement.  We have proposed the following.

Proposal:

- The S-RSSI measurement is defined as a linear average over OFDM symbols 1, 2, 3, 4, 5, and 6 in the first slot and OFDM symbols 0, 1, 2, 3, 4, and 5 in the second slot.

# Reference

- TS 36.214, 3GPP

- R1-1708993, “[DRAFT CR] Correction to the Definition of S-RSSI measurement”, Ericsson