3GPP TSG-RAN WG1 #89 	R1-1709782

May 15th – 19th, 2017

Hangzhou, P. R. China

Agenda item:	7.1.4.2.1.1

Source: 	Qualcomm Incorporated

Title: 	Early termination for Polar codes

Document for:	Discussion/Decision

# Introduction

In RAN1#87, Polar codes were adopted as the channel coding scheme for uplink control information and downlink control information (working assumption) for eMBB except for very small block length [1]. A detailed design of Polar codes is proposed in [3] for the control channel in eMBB systems. A single CRC for joint dectection and CRC-aided SCL decoding (CA-SCL) is propsed in that contribution. The additional CRC bits are related to the list sizes used in the SCL decoder and are used to maintain false alarm and performance requirements with low overhead.

Polar codes with a distributed CRC are discussed in [4][5]. The two scheme have been compared in [6]. The main conclusions are that distributed CRC has very high implementation complexity unsuitable for NR control channel and that CA-Polar outperforms distributed CRC in terms of performance and complexity.

Another early termination scheme that uses distributed parity was propose in [7]. In this contribution, we will evalute the gain of this early termination scheme for Polar codes based on the following evaluation agreement from RAN1#88b [2]:

Agreement:

- J CRC bits are provided (which may be used for error detection and may also be used to assist decoding and potentially for early termination)

  - J may be different in DL and UL

  - J may depend on the payload size in the UL (0 not precluded)

- In addition, J’ assistance bits are provided in reliable locations (which may be used to assist decoding and potentially for early termination)

- J + J’ <= the number of bits required to satisfy the FAR target (nFAR) + 6

  - Working assumption:

    - For DL, nFAR = 16 (at least for eMBB-related DCI)

    - For UL, nFAR = 8 or 16 (at least for eMBB-related UCI; note that this applies for UL cases with CRC)

- J’>0

- Working assumption: J”<=2 additional assistance bits are provided in unreliable locations (which may be used to assist decoding and potentially for early termination)

  - Can be revisited in RAN1#89 if significant benefit is shown from a larger value of J” without undue complexity – companies are encouraged to additionally evaluate J”=8

- The J’ (and J” if any) bits may be CRC and/or PC and/or hash bits (downscope if possible)

- Placement of the J, J’ (and J” if any) assistance bits is FFS after the study of early termination techniques

  - Appended?

  - Distributed?

    - evenly?

    - unevenly?

Conclusion:

- Study until RAN1#89 polar code construction techniques to facilitate early termination (i.e. before decoding all the information bits) without degrading BLER performance or latency (especially considering the time for deinterleaving the information and assistance bits) compared to purely implementation based methods such as path-metric based pruning

  - e.g. assistance bits distributed in the codeword in such a way that error detection can be performed after partial decoding

  - Investigate performance, complexity and FAR impacts

  - Study of use of data-independent scrambling to facilitate early termination is also not precluded

We will discuss different schemes of early termination in this contribution.

# Discussion on implementation challenges of distributed CRC based on bit permutation for polar code

Polar codes with distributed CRC are discussed in [4][5]. The two schemes have been compared in [6]. From a UE implementation point of view, distributed CRC, though quite an interesting idea, is practically unimplementable due to unstructured interleaving of both info and CRC bits depending on each individual K value.

The additional operations of distributed CRC for early termination is depicted in Figure A. It is seen that the transform of generator matrix to up triangle matrix must be done for each (N,K). It is very complicated for the encoding and decoding. In the decoding, the positions of the information bits related to given CRC bit must be obtained and there is no structure of such association. The unsturcutred parity checking must be done for all the candidate paths, which will further increase the latency and complexity and may block implementation of fast decoding schemes.

Figure A. Additional operations of Distributed CRC for early termination

The latest design for distributed CRC is discussed in [11]. A single interleaver pattern but with large interleaver size according to the maximum K is designed for any information size less than K. However, the complexity and benefits of this scheme are still not clear. First, the early termination gain is reduced very much because the location of the CRC bits is close to the end without triangularization of the CRC generation matrix. Because the original early termination gain is not large, it becomes even less for this design. Second, the complexity of deriving the interleaver index for given K and CRC generate matrix is very high. Thirdly, the decoding latency will be increased because of the additional interleaver, which adds up to the overall timeline. The saving of latency in some cases at the expense of worst case latency (which drives the entire UE timeline). The memory size inceases because interleaver index, CRC bits index and CRC generator matrix are needed to be stored. The early termiantion gain of this scheme is small with price of large memory and latency. Finally, we should all remember that, one main reason CRC is popular is its simplicity in implementation that, the encoding/decoding can be done via simple shift register operation instead of going to high complexity binary matrix multiplication. However, the proposed design in [11] relies on the manipulation of binary generator matrix of CRC and is highly undesirable in high speed control channel decoding.

In comparison, CA-Polar which has lower encoding and decoding complexity and latency outperforms distributed CRC in terms of better performance and lower complexity.

Observation 0: polar code with distributed CRC based on bit permutation is requires high complexity and increases worse-case latency for a limited gain in early termination.

# Modifications to distributed CRC based Polar

As discussed in previous chapter, distributed CRC is practically unimplementable. [7] proposes a modified distributed CRC scheme which fixes interleaving structure and parity check functions so that they are no longer dependent on (N, K). But such modification is also breaching the original CRC parity check functions, so this scheme has risk of higher false alarm rate and BLER performance.

In order to quantify the violation of original CRC parity check function, we evaluated the hamming distance spectrum of original 11 bit CRC (polynomial is 0x532) and [7]’s scheme (8 bit CRC + 3 distributed CRC). Assuming K = 22, then hamming distance spectrum is

It can be observed that min hamming distance is 6 for original 11 bit CRC, but only 4 with [7]’s modification. Increase of min hamming distance could lead to deterioration of False alarm rate and BLER performance. Further evaluation of the potential issues of distributed CRC/parity schemes need to be very carefully done to avoid the undesirable side effect.

Next, we will also evaluate the early termination savings for [7]’s scheme.

# Decoding complexity and latency for SCL decoding

In this section, we investigate the decoding complexity and latency for successive-cancellation list (SCL) decoding. Because the complexity and latency for SCL decoding is based on SC decoding, we discuss the decoding complexity and latency for SC decoding first.

We develop a simple, approximate relation to provide an intuitive understanding of decoding latency and complexity. We then compare it with more accurate latency estimates, showing that the simple approximation provides a good indicator for successive-cancellation polar list decoder latency reduction due to early termination.

Average decoding latency is an important indicator of expected polar decoder power consumption in PDCCH. There will be a number of hypotheses to test, 44 in LTE, and the sooner the decoder can finish testing these hypotheses, the sooner the decoder memory can be powered down. Memory requires significant amount of energy to maintain its state, contributing greatly to the power consumption of a design with large memory such as a polar list decoder.

## Decoding complexity and latency for SC decoding

We start with a very simple example of an SC decoder with N=8. The detailed complexity and latency are depicted in Figure 1. It is seen that the total clocks are 2N-2 and the total sum of number of f functions and g functions are Nlog2N with half for f function and half for g function. Therefore, the total complexity is Nlog2N*average complexity of f function and g function.

Figure 1. The decoding complexity and latency for SC decoder with N = 8

We are interested in the complexity and clock cycles used after the i-th bit is decoded in the SC decoder. The number of decoding clock cycles used after the i-th bit is decoded for N=64, 128, 256, 512 is depicted in Figure 2. It is seen that the number of decoding clock cycles used after the ith bit is decoded is approximately 2*i-2 for variable N. The decoding latency is linearly increased with the number of decoded bits in an SC decoder.

The decoding  complexity after the ith bit is decoded for N=64, 128, 256, 512 are depicted in Figure 3. It is seen that the decoding complexity after the ith bit is proportional to the number of decoded bits. For larger N, the decoding complexity becomes larger.

Figure 2. The decoding latency for SC decoder for variable N

Figure 3. The decoding complexity for SC decoder for variable N

## Decoding complexity and latency for SCL decoding

After obtaining the decoding complexity and latency of SC decoding, we use a similar proceedure to estimate the complexity and latency of SCL decoding.

- Decoding latency

Latency of SCL decoding is approximated by the sum of 2N-2 cycles and the decoding latency of sorting for non-frozen bits. It is assumed that all the L candidate paths are decoded in parallel. The sorting latency is assumed to take α clock cycles (where α is a number depending on implementation). Therefore, the total decodeing latency will be 2N-2+K+J with K+J clocks for the sorting operation of K+J information bits.

- Decoding complexity

The decoding complexity is the sum of LNlog2N and the decoding complexity of sorting for non-frozen bits. According to the analysis in contribution [8], the total complexity of one f function and one g function is 5 additions and one comparison. Therefore, average complexity of a single f or g function is Q=3 additions, where a comparison is assumed to have the same complexity as addition. The complexity for  sorting is LS=L2log22L(log22L+1) comparisons since sorting networks will be used to reduce decoding latency. Therefore, the total decoding complexity is L(QNlog2N+(K+J)S) comparators by adding K+J sorting complexity of LS with S=12log22L(log22L+1).

With the calculation of the decoding complexity and latency for SCL decoding, we calculate the gain for early termination in terms of complexity and latency reduction in the following section.

# Definition of gains for early termination

Define the total number of simulations as T.

For the t-th simulation run,

Define the number of u domain bits the decoder terminates at as P_t.

The early termination gain for the i-th run can be calculated as (2N-2) – (2P_t – 2).

The overall early termination latency gain is to average over a total of T’s simulation runs.

- Latency gain

The total number of clock cycles used for decoding up to be the P_t-th bit is very close to 2*P_t-2. Therefore, the number of cycles is approximated as 2*P_t-2 when early termination happens at index P_t. Assuming we perform T experiments for early termination, the latency gain of early termination for SC will be

(SC)

Where T(2N-2) is the total clocks used for T experiments and 2N-2 is the clocks for one test. The saving is 2N-2 – (2P_t-2) where 2P_t-2 is the used clocks before early termination. Based on this, the latency gain of early termination for SCL is obtained by assuming  α clocks for sorting candidate paths for non-frozen bits. It will be

(SCL)

Where J is the number of parity check bits and It is the number of information bits bits that are not decoding after early termination. Base on this definition, the latency gain in [7] is obtained in the following section.

- Complexity gain

Suppose the C(Pt) stands for the decoding complexity after the P_t-th bit is decoded and the early termination happens at index Pi. Assume we test T times for early termination, the complexity gain of early termination for SC will be

(SC)

Where TNlog2N is the total decoding complexity for T test times in terms of average complexity of f function and g function.  The saving is Nlog2N-C(Pt) where C(Pt) is the decoding complexity before early termination. Based on this, the complexity gain of early termination for SCL is obtained by assuming LS operators for sorting candidate paths for information bits. It will be

(SCL)

Where J is the number of parity check bits and It is the number of information bits that are not decoding after early termination. Base on this definition, the complexity gain in [7] is obtained in the following sections.

# Evaluation on Early Termination Gains

One example implementation of early termination is as described in [7], where 3 distributed single parity check bits are roughly evenly spaced among information bits. The decoding latency gain and complexity gain of the scheme are obtained by jointly checking the three parity check bits.

The overall early termination probability/ratio is shown in Figure 8. The probably looks high. However, it should be noted that some of the early termination occurs at the very stage of the decoding (as shown in Figure 9), hence the right metric to look at is the early termination latency gain and computational complexity gain.

The latency gains of early termination are depicted in Figure 10-12 for L=8, L=16 and L=32, respectively. The complexity gains of early termination are depicted in Figure 13-15 for L=8, L=16 and L=32 respectively.

It is seen that for L=8, all the complexity gains are around less than 10% except the case of 2/3 rate in the DCI region of interest (40~80 bit payload sizes). Note that in PDCCH blind hypotheses, the latency/complexity due to high aggregation level (equivalently low coding rate) is of more relevance. In such a case, the gain from early termination is on average less than 10%. Note that, this is assuming no grant case. In the presence of a grant transmitted to the user, there will be no expected saving. The complexity gains reduce to less than 2% and 1% for L=16 and 32, respectively. this is the case for both α=1 and α=3, with α=3 having slightly more gain. However, this does not change the overall trend and observation of limited early termination gain.

It is seen that majority of the latency gain and complexity gain is less than 10% although the probability of the early termination gain is around 50%. It is noted that three single parity check bits are jointly used to do early termination.

Considering the results above, the benefit of having distributed bits for early termination design is quite limited while the risk of having such an immature design is high. Single CRC based CA-Polar is a good choice for the control information of both DL and UL.

Observation 1: The complexity gains of the design in [7] at rate less than 2/3 is less than 5% for L=8.

Observation 2: The complexity gains of the design in [7] at rate less than 2/3 is less than 2% and 1% for L=16 and L=32, respectively.

Figure 8. Early termination ratio for variable K with rates of 1/6, 1/3, 1/2 and 2/3

Figure 9. The distrution of early termination poistions for K=80 with rate of 1/3 and L=8

Figure 10. Latency gain of early termination for L=8 (left: α=1; right: alpha = 3)

Figure 11. Latency gain of early termination for L=16 (left: α=1; right: alpha = 3)

Figure 12. Latency gain of early termination for L=32 (left: α=1; right: alpha = 3)

Figure 13. Complexity gain of early termination for L=8

Figure 14. Complexity gain of early termination for L=16

Figure 15. Complexity gain of early termination for L=32

## Cycle Count Results

Figure 16 shows the latency gains of early termination calculated by counting decoding cycles according to the polar decoder controller schedule. It can be observed that these results are closely aligned with results in Figure 10. Therefore, the approximate metric used in this contribution is a good indicator of gains due to early termination.

Figure 16 Latency gains of early termination using cycle count

# Reduced-Complexity Decoding Algorithms

So far, the analysis in this contribution used a basic SCL decoder. The SCL decoder treats all information bits equally, i.e. the cost in latency and complexity of estimating a reliable information bit is the same as that of estimating an unreliable information bit. Significant reductions in latency and computational complexity can be achieved by exploiting information-bit reliability and distribution.

An example of a method utilizing bit reliability to reduce complexity is selective path extension [9], also referred to as decision-aided SCL decoding in [10]. In this method, new candidates and list pruning only take place at unreliable information bits. Therefore, the sorting latency and complexity is eliminated from reliable bit position, greatly reducing decoding latency and computational complexity. In polar codes, reliable bits are more likely to be encountered later in the decoding process, i.e. reliable bits are concentrated in large decoding-index locations. This reduces the gains of early termination since the cost of decoding the later information bits is significantly lower than decoding the earlier information bits.

Simplified successive cancellation (SSC) list decoding is another method to reduce decoding complexity and computational complexity [8]. In this method, contiguous group of information bits are decoded simultaneously and only one sorting operation is performed per group instead one sorting operation per bit. Since reliable bits in polar codes are concentrated in large-index (later) locations, large groups of contiguous information bits are more likely to be encountered later in the decoding process. The reduces the complexity and latency of decoding the later parts of the code and reduces the gains of early termination. For example, the latency savings from early termination for the N = 480, K = 64+16+3, L = 8 using the single parity-bit ET scheme is 5.5% for SCL decoding, but decreases to 4.5% for SSCL.

Observation 4: Early termination gains decrease when reduced-complexity decoding algorithms are used.

# Early termination of single CRC CA-polar

It is also worth mentioning that, CA polar with CRC attached at the end of decoding order is also capable of early termination after all the information bits are decoded successfully. As per agreement from RAN1-88b,

Conclusion:

- Study until RAN1#89 polar code construction techniques to facilitate early termination (i.e. before decoding all the information bits) without degrading BLER performance or latency (especially considering the time for deinterleaving the information and assistance bits) compared to purely implementation based methods such as path-metric based pruning

  - e.g. assistance bits distributed in the codeword in such a way that error detection can be performed after partial decoding

  - Investigate performance, complexity and FAR impacts

  - Study of use of data-independent scrambling to facilitate early termination is also not precluded

The current simple formula calculation does not yet take that into account early termination from checking the CRC attached at the end. When that is accounted for, early termination gain will further be reduced especially for payload sizes in DCI region of interest. Hence, the gain will be even smaller. On the other hand, additional CRC/parity checking in the process of decoding will add additional latency/complexity to the overall process. It is not clear whether such a design gives any positive return in early termination, while it could potentially lead to degradation in performance.

Especially it should be emphasized that, the interleaving of information bit will increase the decoding latency as well as increase additional HW cost and it is highly undesirable.

# Path-Metric Based Early Termination for CA Polar

Early termination can also be done for CA Polar where a continuous, non-distributed CRC (or parity) is used. In SCL decoding, a path metric is maintained for each individual path to reflect reliability. Such path metric values can be used to detect failure in early stages as well. One possible implementation is: check each path’s metric, if all of them are very high (big penalty), then we can early terminate. There’re also other possible implementations: compare path metrics of different paths. If they are very similar, then decoding is terminated. One example of implementation based scheme is shown in [12]. Here, we provide a different scheme that shows decent early termination performance compared with distributed CRC based schemes.

In order to derive a robust metric, we use the following metric:

Pruning metric = (max_path_metric - min_path_metric)/min_path_metric

Compare Pruning metric to a threshold to determine early termination

This is a ratio metric, so robust to SNR variations. Main motivation is that correct path’s metric should be well distinguishable from other wrong paths. The value of the threshold can be chosen to increase early termination probability, maintain performance, and have low implementation complexity. For example, setting the threshold to 1/32 achieves good early termination probability without degrading the performance for the following examples and can be implemented using simple bit shifts without having to implement division.

As shown in Figure 17, by selecting an appropriate threshold to path metric based early termination, it can be observed that no BLER degradation occur for L = 8 and R = 1/3 for the following examples.

Figure 17 BLER when using the path-metric implementation-based scheme for early termination.

Since BLER is not degraded, then we can safely add early termination to decoder. From below figure, the early termination based on path metric only can achieve ~40% early-termination probability independent of the SNR in the case there is no signal. This method is purely based on CA Polar without modification to the code construction. Note that, this is achieve via utilizing simple metrics. Further improvements of metric based early pruning schemes is possible and can be further evaluated.

Note that different metrics can also be combined to achieve different tradeoffs in performance [12].

# Conclusions

Observation 0: polar code with distributed CRC based on bit permutation is requires high complexity and increases worse-case latency for a limited gain in early termination.

Observation 1: The complexity gains of the design in [7] at rate less than 2/3 is less than 5% for L=8.

Observation 2: The complexity gains of the design in [7] at rate less than 2/3 is less than 2% and 1% for L=16 and L=32, respectively.

Observation 3: The gain of early termination is very small and it is not necessary to do early termination in price of introducing additional complexity.

Proposal 1: Adopt CA-SCL (with one long CRC) solution of Polar codes for control channel for better performance and lower complexity and latency.

# References

- Chairman’s notes RAN1_87.

- Chairman’s notes RAN1#88bis_final.

- R1-1700832, “Detailed design of Polar codes for control channel,” Qualcomm Incorporated, RAN1 adhoc, Spokane, USA

- R1-1701630, “Design of CRC-assisted Polar code, Ericsson,” RAN1#88, Athens, Greece

- R1-1703497, “Details of CRC distribution of Polar design,” Nokia, Alcatel-Lucent Shanghai Bell, RAN1#88, Athens, Greece

- R1-1705631, “Evaluation of CA with distributed CRC bits,” Qualcomm Incorporated, RAN1#88b, Spokane, USA

- R1-1705757, “Distributed simple parity check Polar codes,” DoCoMo, RAN1#88b, Spokane, USA

- R1-1613561, “Control channel complexity considerations,” Qualcomm Incorporated, RAN1#87, Spokane, USA

- Y. Fan et al., “A Low-Latency List Successive-Cancellation Decoding Implementation for Polar Codes,” in IEEE Journal on Selected Areas in Communications, vol. 34, no. 2, pp. 303-317, Feb. 2016.

- R1-1700090, “On latency, power consumption and implementation complexity for polar codes,” Huawei, HiSilicon, RAN1 #87a, Spokane, USA.

- R1-1708833, Design details of distributed CRC, Nokia, Alcatel-Lucent Shanghai Bell, RAN1#89, Hangzhou, China

- R1-1708047, “Early termination of polar decoding”, Samsung