3GPP TSG RAN WG1 #89	R1-1709820

May 15th to 19th 2017

Hangzhou, China

Title:	[DRAFT] LS response on NR minimum carrier bandwidth

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	AT&T, CMCC [RAN1]

To:	RAN4

Contact Person:

#### Name:	Tom Novlan, Wu Dan

E-mail Address:	tnovlan@labs.att.com, wudan@chinamobile.com

Attachments:

1. Overall Description:

RAN1 would like to thank RAN4 for informing them of their agreements on the minimum supported carrier bandwidth for NR:

- The set of supported NR bandwidths for each frequency band will be band specific

- For frequency range up to 6 GHz, the minimum possible carrier bandwidth for NR is 5 MHz For frequency range from 6 GHz to 52.6 GHz, the minimum possible carrier bandwidth for NR is 50 MHz

- RAN4 will further determine mapping between frequency band and the set of support carrier bandwidth values in consideration with above

RAN1 would like to inform RAN4 that because of the inability of the 50MHz minimum carrier bandwidth to support SS Blocks with 240kHz SCS, RAN1 agreed that a minimum carrier bandwidth value of 100MHz should be supported in addition to 50MHz in order to accommodate SS Blocks with 240kHz SCS for bands, where appropriate in the frequency range from 24 GHz to 52.6 GHz.

[RAN1 would also like to inform RAN4 that because of the inability of the 5MHz minimum carrier bandwidth to support SS Blocks with 30kHz SCS, RAN1 agreed that a minimum carrier bandwidth value of 10MHz should be supported in addition to 5MHz in order to accommodate SS Blocks with 30kHz SCS for bands, where appropriate in the frequency range below 6 GHz.]

2. Actions:

To RAN4

RAN1 would like to kindly request RAN4 to support a minimum carrier bandwidth value of 100MHz in addition to 50MHz in order to accommodate SS Blocks with 240kHz SCS for bands, where appropriate, in the frequency range from 24 GHz to 52.6 GHz.

[RAN1 would like to kindly request RAN4 to support a minimum carrier bandwidth value of 10MHz in addition to 5MHz in order to accommodate SS Blocks with 30kHz SCS for bands, where appropriate, in the frequency range below 6 GHz.]

3. Date of Next TSG-RAN WG1 Meetings:

RAN1 NR adhoc	27th – 30th June 2017		 Qingdao, China

RAN1 #90	21st – 26th August 2017	Prague, Czech Republic