3GPP TSG RAN WG1 Meeting #90						R1-1715129

Prague, P.R. Czech 21th – 25th August 2017

Source: 			Ad-Hoc chair (Intel)

Title:	Chairman's notes of AI 5.2.4 on Further enhancements to Coordinated Multi-Point Operation for LTE

Agenda item:	5.2.4

Document for:		Endorsement

### Further enhancements to CoMP Operation for LTE

WID in RP-171031

Email approval on the LS to RAN2/4 capturing all the agreements

Email approval on the LS to RAN2 including RRC parameters

#### Remaining details on QCL assumptions and PDSCH RE mapping

Limit to 1 contribution per 1 company/organization/university

R1-1712123	Remaining details on QCL assumptions and PDSCH RE mapping	Huawei, HiSilicon

R1-1712314	Remaining details on QCL assumptions and PDSCH RE mapping	ZTE

R1-1712492	Remaining details on QCL and PDSCH RE maping	Intel Corporation

R1-1712620	Discussion on enhancement to resource allocation for NC-JT	ASTRI

R1-1713036	Remaining details on QCL assumptions and PDSCH RE mapping for NCJT	Qualcomm Incorporated

R1-1713538	Remaining details on QCL assumptions and PDSCH RE mapping	Samsung

R1-1714351	Remaining details on QCL assumptions and PDSCH RE mapping	Ericsson

R1-1714730	WF on QCL and PDSCH RE Mapping for NC-JT	ZTE

Agreements:

- PDSCH starting symbol is maximum value across PDSCH starting symbols for 1st and 2nd set

Possible agreements:

- Non-overlapping resource allocation is supported

  - RRC signalling is supported to configure independent indication of RA corresponding to 2 CWs is supported within one RA field.  When it is configured,

    - The scheduling granularity is twice coarser, i.e., new RBG size is twice of the original RBG size for code points indicating NC-JT (two sets are indicated by DCI)

    - RA payload is the same

    - RA type 0 is assumed

    - QCL and PDSCH RE mapping in allocated resources corresponding to CW k is done only according to the PQI parameter set of CW k, where k=1,2

    - UE doesn’t expect RAs indicating the same sets RBs

Possible agreements:

- 3 bits are supported for PQI in addition 2 bits for PQI

- 2 bits or 3 bits are RRC configured

#### Enhancements to CSI feedback for NC-JT

Limit to 1 contribution per 1 company/organization/university

R1-1712095	CSI measurement and reporting for non-coherent JT	Huawei, HiSilicon

R1-1712315	Enhancements to CSI feedback	ZTE

R1-1712493	CSI for NC-JT	Intel Corporation

R1-1713037	Enhancement to CSI feedback for NCJT	Qualcomm Incorporated

R1-1713539	Enhancements to CSI feedback for NC-JT	Samsung

R1-1714352	On CSI feedback enhancements supporting non-coherent JT	Ericsson

Agreements:

  - UE can be configured with K = 2 NZP CSI-RS resources and one CSI-IM resource

    - Two NZP CSI-RS resources are not QCL-ed with each other according to QCL Type B assumptions

    - The number of antenna ports that can be configured for NZP CSI-RS resource are {1,2,4,8}

  - CRI = 0 – CSI reporting is performed for the 1st NZP CSI-RS resource following legacy behavior of Class B FD-MIMO

  - CRI = 1 – CSI reporting is performed for the 2nd NZP CSI-RS resource following legacy behavior of Class B FD-MIMO

  - CRI = 2 CSI reporting is performed for both NZP CSI-RS resources

    - Two sets of CSIs (PMI/CQI/RI) are calculated within single CSI process based on K = 2 NZP CSI-RS resources. Each set of CSI corresponds to each CW

      - Combinations of reported RIs are restricted to the following sets {RI1, RI2} = {1,1},{1,2},{2,1},{2,2},{2,3},{3,2}{3,3},{3,4},{4,3},{4,4}

    - The PDSCH layers (υ1 layers for CW1 and υ2 layers for CW2) to antenna port mapping (15,…,14+P1 and 15,…,14+P2 ) for corresponding CW is given by:

y1(15)(i)⋮y1(14+P1)(i)=W1(i)x1(0)(i)⋮x1(υ1-1)(i)    for CW1

y2(15)(i)⋮y2(14+P2)(i)=W2(i)x2(0)(i)⋮x2(υ2-1)(i)    for CW2

  - where P1 and  P2 are the number of NZP CSI-RS ports for the two configured CSI-RS resources.  W1 and W2 are precoding matrices from legacy codebooks for CW1 and CW2.

  - CSI of one CW is calculated based on interference assumption that inter-CW interference is derived from the CSI-RS resource corresponding to other CW. Inter-CW interference is derived from channel measurement obtained from other NZP CSI-RS resource and precoding matrix according to equation above.

  - Codebook subset restrcition is configured per NZP CSI-RS resource

  - Pc is configured per NZP CSI-RS resource

  - For aperiodic CSI reporting, UE reports CSI for single CRI equal to 0, 1 or 2

  - For periodic CSI reporting, UE reports CSI for single CRI value equal to 0 or 1 following legacy Rel-14 behavior

  - Note: it is up to spec editor how to capture the above agreement

Possible agreements:

- Alt. 1

  - UE configured with new QCL assumption for DM-RS antenna ports supports CSI process configuration that may optionally include configuration of other (reference) CSI process identity (ID)

    - If the ID of the reference CSI process is configured for a given CSI process, UE should use channel measurements obtained from NZP CSI-RS resource of the reference CSI process and reported CSI (i.e., CRI, RI, PMI and CQI) for the reference CSI process to calculate interference for a given CSI process together with interference measurements obtained on the configured CSI-IM resource

    - Only one CSI process can be configured as reference CSI process for a given serving cell

  - For aperiodic CSI reporting, UE should not expect aperiodic CSI request for CSI process configured with ID of the reference CSI process without CSI request for the reference CSI process

  - UE for CSI calculation for a given CSI process configured with ID of the reference CSI process should use CSI components of the fully reported CSI of the reference CSI process to calculate the interference for a given CSI process

- Alt. 2

  - UE configured with new QCL assumption for DM-RS antenna ports supports configuration of up to two NZP CSI-RS resources per CSI process

    - The NZP CSI-RS resources are not QCL-ed with each according to QCL Type B assumptions

    - The supported number of antenna ports per NZP CSI-RS resource is two

    - For CSI feedback that antenna ports from two NZP CSI-RS are indexed by using all antenna ports from 1st NZP CSI-RS resource followed by all antenna ports from the 2nd NZP CSI-RS resource. The antenna port order within NZP CSI-RS resource remains the same.

  - The codebook is constructed by block concatenation of all possible combinations of rank 1 and rank 2 PMIs from two port codebook for Wa and Wb as follows

  - i1 is wideband and indicates the structure of the precoding matrix

  - i2 is wideband or subband (depending on the reporting modes) and indicates precoding within the selected matrix structure

  - Codebook subset restriction is bitmap of all states of PMI and RI

  - Aperiodic CSI reporting with above codebook is supported

  - Periodic CSI reporting with above codebook is supported using Rel-13 periodic CSI reporting

- Alt. 3

  - Support configuration of at least one CSI-RS resource that may optionally include configuration of a second CSI-RS resource

    - Each DM-RS antenna port group can be QCL’d with a different CSI-RS resource

    - If the second CSI-RS resource is configured, UE should use channel measurements obtained from the two CSI-RS resources  to calculate CSI together with interference measurements obtained on the configured CSI-IM resource

  - A PMI and an RI are reported for each CSI-RS resource, where RI=0 can be reported for one CSI-RS resource

    - One CQI report is produced for up to two codewords

  - Aperiodic CSI reporting contains 2 RIs and up to 2 PMIs if the second CSI-RS resource is configured

    - Otherwise, Rel-14 CSI is reported

  - Rel-14 wideband periodic CSI-reporting is supported

- Alt. 4

  - UE can be configured with K = 2 NZP CSI-RS resources and one CSI-IM resource

    - Two NZP CSI-RS resources are not QCL-ed with each other according to QCL Type B assumptions

    - The number of antenna ports that can be configured for NZP CSI-RS resource are {1,2,4,8}

  - CRI = 0 – CSI reporting is performed for the 1st NZP CSI-RS resource following legacy behavior of Class B FD-MIMO

  - CRI = 1 – CSI reporting is performed for the 2nd NZP CSI-RS resource following legacy behavior of Class B FD-MIMO

  - CRI = 2 CSI reporting is performed for both NZP CSI-RS resources

    - Two sets of CSIs (PMI/CQI/RI) are calculated within single CSI process based on K = 2 NZP CSI-RS resources. Each set of CSI corresponds to each CW

      - Combinations of reported RIs are restricted to the following sets {RI1, RI2} = {1,1},{1,2},{2,1},{2,2},{2,3},{3,2}{3,3},{3,4},{4,3},{4,4}

    - The PDSCH layers (υ1 layers for CW1 and υ2 layers for CW2) to antenna port mapping (15,…,14+P1 and 15,…,14+P2 ) for corresponding CW is given by:

y1(15)(i)⋮y1(14+P1)(i)=W1(i)x1(0)(i)⋮x1(υ1-1)(i)    for CW1

y2(15)(i)⋮y2(14+P2)(i)=W2(i)x2(0)(i)⋮x2(υ2-1)(i)    for CW2

  - where P1 and  P2 are the number of NZP CSI-RS ports for the two configured CSI-RS resources.  W1 and W2 are precoding matrices from legacy codebooks for CW1 and CW2.

  - CSI of one CW is calculated based on interference assumption that inter-CW interference is derived from the CSI-RS resource corresponding to another CW. Inter-CW interference for CRI = 2 is derived from channel measurement obtained from other NZP CSI-RS resource and precoding matrix according to equation above.

  - Codebook subset restrcition is configured per NZP CSI-RS resource

  - Pc is configured per NZP CSI-RS resource

- For aperiodic CSI reporting, UE reports CSI for CRI = 0, 1 or 2

- For periodic CSI reporting, UE reports CSI for CRI = 0 or CRI = 1 following legacy Rel-14 behavior

- Note: it is up to spec editor how to capture the above agreement

#### Other enhancements for NC-JT

Including resource allocation, DMRS port to layer mapping, support of TM9, etc.

Limit to 2 contributions per 1 company/organization/university

R1-1712093	Discussion on TM9 for non-coherent JT	Huawei, HiSilicon

R1-1712094	DMRS port to layer mapping and resource allocation for non-coherent JT	Huawei, HiSilicon

R1-1712316	Other enhancements for NC-JT	ZTE

R1-1712494	On other enhancements for NC-JT	Intel Corporation

R1-1713038	Other enhancements for NCJT	Qualcomm Incorporated

R1-1713540	On the support of NC-JT for TM9	Samsung

R1-1714353	On DMRS port to layer mapping for NC-JT	Ericsson

R1-1714354	On feCoMP support for TM9	Ericsson

R1-1714679	WF on DMRS port to layer mapping for NC-JT	Ericsson, Intel Corporation

Also supported by Huawei, HiSilicon

Agreements:

- When UE is indicated with PQI containing two sets of QCL and RE mapping parameters and multiple CW transmission, the following DMRS port to layer mapping is used

  - Note: This implies that layers corresponding to different CWs are mapped to FDMed DMRS ports for rank 3-8. Rank-2 port mapping is the same as legacy to maintain DMRS overhead.

- Note: For Rank 1 DM-RS ports 7 is used

R1-1714742	WF on NC-JT support for TM9	Intel Corporation

Also supported by Softbank

Possible agreements:

- New QCL assumption for DM-RS antenna ports and association with CWs is also supported in TM9

    - Two NZP CSI-RS resources can be configured for the UE

      - NZP CSI-RS resources are not QCL-ed with each other

      - 1st NZP CSI-RS resource is QCL-ed with serving cell CRS wrt to all parameters

      - 2nd NZP CSI-RS resource is QCL-ed with serving cell CRS wrt to Doppler shift and Doppler spread

    - Two PDSCH RE mapping sets

      - 1st PDSCH RE mapping set around serving cell CRS

      - 2nd PDSCH RE mapping set

    - REs corresponding to 1st CW around serving cell CRS

    - RE corresponding to 2nd CW (if present) around CRS

    - 1 bit field is introduced in DCI Format 2C in TM9 to switch between two QCL assumptions for DM-RS antenna ports and two PDSCH RE mapping sets

      - All DM-RS antenna ports are QCL-ed with each other and antenna port of NZP CSI-RS resource QCL-ed with CRS of the serving cell wrt to all parameters. PDSCH RE around according to 1st PDSCH RE mapping set.

      - DM-RS antenna ports associated with 1st CW are QCL-ed with antenna ports of the 1st NZP CSI-RS resource and DM-RS antenna ports associated with 2nd CW (if present) are QCL-ed with antenna ports of the 2nd NZP CSI-RS resource. PDSCH RE mapping according to 2nd PDSCH RE mapping set

#### Other