3GPP TSG RAN WG1 Meeting #92			R1-1801304

Athens, Greece, February 26th – March 2nd, 2018

3GPP TSG RAN WG2 Meeting #100                                                    R2-1714239

- Reno, USA, Nov 27th to Dec 1st  2017

Title:	LS about LTE CA SCell New State agreements

Release:	Rel-15

Work Item:	LTE_euCA

Source:	RAN WG2

To:	RAN WG1, RAN WG4

CC:	RAN

Contact Person:

#### Name:	Prasad Kadiri

E-mail Address:	pkadiri@qti.qualcomm.com

1. Overall Description:

During RAN2#100 meeting, RAN2 has agreed  to introduce a new SCell state in RRC_Connected mode (tentatively called a “fast SCell activation state”) that allows to reduce latency for SCell activation compared to activation latency from (legacy) SCell deactivated state to (legacy) SCell activated state.

The following RAN2 agreements characterize the new ‘fast SCell activation state’:

- Transition between the existing SCell states and the new SCell state is done via MAC CE

- UE can be configured to provide periodic CRS-based CQI reporting in the new SCell state

- UE is not required to perform PDCCH monitoring in the new SCell state

The CQI/PMI/RI reports in the new ‘fast SCell activation state’ are to be usable by the eNB to start scheduling on SCell after transitioning into SCell activated state.

In RAN2 understanding, this will have some impacts to RAN1 and RAN4 specifications. Therefore, RAN2 would respectfully request the RAN1/4 to work on the following aspects:

- RAN1 to specify CRS-based CQI/PMI/RI reporting details (e.g. procedures and reporting periodicities) for the new SCell state in CRS-based transmission modes

- RAN4 to specify, if needed, delay and interruption requirements for state transitions between the new SCell state and legacy SCell states

- RAN4 to specify, if needed, accuracy requirements for the CRS-based CQI reporting in the new SCell state

2. Actions:

To RAN WG1:

ACTION:      RAN2 respectfully requests RAN1 to specify CRS-based CQI/PMI/RI reporting details (e.g. procedures and reporting periodicities) for the new SCell state in CRS-based transmission modes

To RAN WG4:

ACTION:      RAN2 respectfully requests RAN4 to specify, if needed,

- Delay and interruption requirements for state transitions between the new SCell state and legacy SCell states

- accuracy requirements for the CRS-based CQI reporting in the new SCell state

3. Date of Next TSG-RAN2 Meetings:

TSG RAN2 Meeting #101		Feb 26 – March 2, 2018		Athens, Greece

TSG RAN2 Meeting # 101bis                 April 16th – April 20th 2018            China