3GPP TSG RAN WG1 Meeting #93	R1-1805913

Busan, Korea, May 21st – May 25th, 2018

Agenda Item:	7.4.5

Source:	Huawei, HiSilicon

Title:	LLS evaluation results for eMBB scenario

Document for:	Discussion

Introduction

RAN1#92 agreed the link-level evaluation parameters for NoMA (Non-Orthogonal Multiple Access) as summarized in Appendix. In this contribution, we will present the link-level evaluation results for eMBB scenario.

Evaluation method

In the simulation, we assume a carrier frequency of 4GHz. 2-symbol DMRS with NR Type-2 configuration is used, which supports at most 12 orthogonal DMRS ports. Given this assumption, the maximum number of UEs in the simulation is 12, and each UE is allocated with the whole 12 PRBs. The number of transmit antenna is 1, and the number of receive antennas is 4. For link-level calibration purpose, the OMA single UE performance for eMBB scenario is provided in the Appendix. Both ideal channel estimation (ICE) and realistic (RCE) are considered, and MMSE algorithm is applied for channel estimation.

Among the NoMA schemes proposed up to RAN1#92, we evaluate SCMA [2], MUSA [3], NCMA [4], and WSMA [5], and present their performance for eMBB scenario. The overall evaluation methodology for eMBB is similar to that for mMTC as described in [6], including the transmitter and receiver (section 2.1 of [6]), signature allocation (section 2.2 of [6]), and results presentation method (section 2.3 of [6]). And for each case, the optimal MCS, layers, and SF (spreading factor) selection for each scheme could be different from that of the mMTC case.

Result Comparison and Analysis

As expected, in all the figures under equal SNR setup (Figure 1) or unequal SNR setup (Figure 2), the required per UE SNR with all compared schemes (SCMA, MUSA, WSMA, and NCMA) increases when # of multiplexed UEs goes up. In the following, we will discuss for each case how the required SNR changes in terms of TBS, # of Rx antennas, and how it varies between different simulated schemes.

Equal SNR Setup

In Figure 1 (ICE), all the curves of SCMA, MUSA, WSMA, and NCMA are ascendant with # of UEs.

In term of TBS (transport block size)

The curves of the smaller TBSs are flatter than those of the larger TBSs.

- Less than 0.2 dB required-SNR increases for # of UE from 2 to 12 in case of TBS=20 bytes.

- More than 2.0 dB required-SNR increases for # of UE from 2 to 12 in case of TBS=150 bytes.

In term of NoMA schemes

The curves of sequence spreading based NoMA schemes are very close to each other, and always above the curves of SCMA for given TBS and # of UE. The gap between the required SNR of SCMA and sequence spreading based NoMA schemes increases with the TBS. The gap is 0.4 dB ~ 0.6 dB when TBS is 80 bytes, 0.8 dB ~ 1.1 dB when TBS is 120 bytes, and 1.3 dB ~ 2.3 dB when TBS is 150 bytes. These gaps indicate that SCMA requires lower SNR to serve the same # of UEs with the same TBS requirements compared with the sequence spreading based NoMA schemes.

In term of channel estimation

The required SNR in terms of # of simultaneous UEs increases faster with RCE than with ICE. This is because the channel estimation error will propagate during MUD and interference cancellation, and the performance degradation increases with the number of UEs. For 1T2R with RCE, less than 0.2dB required-SNR increases for # of UE from 2 to 12 in case of TBS=10 bytes, and more than 2.0dB required-SNR increases for # of UE from 2 to 12 in case of TBS=75 bytes.

However, with RCE, the relative performance difference between the NoMA schemes do not change much compared with ICE. The curves of sequence spreading based NoMA schemes are close to each other, and their required SNRs for different TBS values are higher than that of SCMA.

Observation 1: SCMA can provide large SNR gain over sequence spreading based NoMA schemes in eMBB scenario with equal-SNR and different number of multiplexed UEs for both ICE and RCE cases.

| (a). TDLA-30ns, 1T4R, ICE | (b). TDLC-300ns, 1T4R, ICE |
| --- | --- |
| (c). TDLA-30ns, 1T4R, RCE | (d). TDLC-300ns, 1T4R, RCE |

Figure 1. Required SNR vs # of UEs for equal SNR

Unequal SNR Setup

In term of TBS (transport block size)

The curves of the smaller TBSs are flatter than those of the larger TBSs.

- Less than 0.2 dB required-SNR increases for # of UE from 2 to 12 in case of TBS=20 bytes.

- More than 1.5 dB required-SNR increases for # of UE from 2 to 8 in case of TBS=150 bytes.

In term of NoMA schemes

The curves of sequence spreading based NoMA schemes are very close to each other, and always on top of the curves of SCMA for given TBS and # of UE. The gap between the required SNR of SCMA and sequence spreading based NoMA schemes increases with the TBS. The gap is 0.4 dB ~ 0.5 dB when TBS is 80 bytes, 0.8 dB ~ 1.0 dB when TBS is 120 bytes, and 1.3 dB ~ 1.7 dB when TBS is 150 bytes. These gaps indicate that SCMA requires lower SNR to serve the same # of UEs with the same TBS requirements compared with the sequence spreading based NoMA schemes.

In term of Channel Estimation

The required SNR in terms of # of simultaneous UEs increases faster with RCE than with ICE. This is because the channel estimation error will propagate during MUD and interference cancellation, and the performance degradation increases with the number of UEs. For RCE, less than 0.2dB required-SNR increases for # of UE from 2 to 12 in case of TBS=10 bytes, and more than 1.8dB required-SNR increases for # of UE from 2 to 12 in case of TBS=75 bytes.

However, with RCE, the relative performance difference between the NoMA schemes do not change much compared with ICE. The curves of sequence spreading based NoMA schemes are close to each other, and their required SNRs for different TBS values are higher than that of SCMA.

Observation 2: SCMA can provide large SNR gain over sequence spreading based NoMA schemes in eMBB scenario with unequal-SNR and different number of multiplexed UEs for both ICE and RCE cases.

| (a). TDLA-30ns, 1T4R, ICE | (b). TDLC-300ns, 1T4R, ICE |
| --- | --- |
| (c). TDLA-30ns, 1T4R, RCE | (d). TDLC-300ns, 1T4R, RCE |

Figure 2. Required SNR vs # of UEs for unequal SNR

Conclusions

In this contribution, we present the link-level evaluation results for eMBB scenario. Based on the discussion, we have following observations and proposal:

Observation 1: SCMA can provide large SNR gain over sequence spreading based NoMA schemes in eMBB scenario with equal-SNR and different number of multiplexed UEs for both ICE and RCE cases.

Observation 2: SCMA can provide large SNR gain over sequence spreading based NoMA schemes in eMBB scenario with unequal-SNR and different number of multiplexed UEs for both ICE and RCE cases.

Proposal 1: SCMA should be considered as candidate NoMA scheme for eMBB scenario.

- References

RAN1 Chairman’s notes, RAN1#92, Athens, Greece, February 26-March 2, 2018.

R1-1803663, “Discussion on the design of SCMA”, Huawei, HiSilicon, RAN1#92bis, Sanya, China, April 16th – April 20th, 2018.

R1-1803615, “Preliminary link-level simulation results of NOMA”, ZTE, RAN1#92bis, Sanya, China, April 16th – April 20th, 2018.

R1-1802229, “Transmitter side signal processing schemes for NCMA”, LGE, RAN1#92, Athens, Greece, February 26th – March 2nd, 2018.

R1-1802767, “Signature design for NoMA”, Ericsson, RAN1#92, Athens, Greece, February 26th – March 2nd, 2018.

R1-1805911, “LLS evaluation results for mMTC scenario”, Huawei, HiSilicon, RAN1#93, Busan, Korea, May 21st – May 25th, 2018.

- Appendix

- A.1 Link-level evaluation parameters

Table 1 Link-level evaluation assumptions

| Parameters | mMTC | URLLC |  | eMBB |  | Further specified values |
| --- | --- | --- | --- | --- | --- | --- |
| Carrier Frequency | 700 MHz | 700 MHz or 4 GHz |  | 4 GHz, 700 MHz as optional |  |  |
| Waveform (data part) | CP-OFDM and DFT-s-OFDM | CP-OFDM as starting point |  | CP-OFDM as starting point |  |  |
| Channel coding | URLLC: NR LDPC eMBB: NR LDPC mMTC: NR LDPC |  |  |  |  | The choice of channel coding here is only for the performance evaluation purpose for NoMA study |
| Numerology (data part) | SCS = 15 kHz, #OS = 14 | Case 1: SCS = 60 kHz, #OS = 7 (normal CP), optionally 6 (ECP) Case 2: SCS = 30 kHz, #OS = 4 |  | SCS = 15 kHz #OS = 14 |  |  |
| Allocated bandwidth | 6 as the starting point | 12 as the starting point |  | 12 as the starting point |  | For high payload such as 75 bytes, larger number of RBs can be considered. |
| TBS per UE | At least five TBS that are [10, 20, 40, 60, 75] bytes. Other values higher than 10 bytes are not precluded. Lower than 0.1 bits/RE is optional | At least five TBS that are [10, 20, 40, 60, 75] bytes. Other values higher than 10 bytes are not precluded. |  | At least five TBS that are [20, 40, 80, 120, 150] bytes. Other values higher than 20 bytes are not precluded. |  | #bits per RE calculation does not include DMRS overhead (e.g., REs of one every 7 symbols for DMRS would not be used to carry the data) |
| Target BLER for one transmission | 10% | 0.1% |  | 10% |  |  |
| Number of UEs multiplexed in the same allocated bandwidth | To be reported by companies. |  |  |  |  | Companies are encouraged to perform evaluations with various number of UEs Note: refined set of numbers of UEs should be further discussed in the next meeting. |
| BS antenna configuration | 2 Rx or 4 Rx for 700MHz, 4Rx or 8 Rx for 4 GHz 8Rx as optional |  |  |  |  | CDL model in 38.901 should be considered for 8Rx |
| UE antenna configuration | 1Tx |  |  |  |  |  |
| Propagation channel & UE velocity | TDL-A 30ns and TDL-C 300ns in TR38.901, 3km/h, CDL optional |  |  |  |  |  |
| Max number of HARQ transmission | 1 as starting point. |  | 1 as starting point. More values, 2 for URLLC can be used. |  | 1 as starting point. |  |
| Channel estimation | Ideal channel estimation results should be reported for calibration Realistic channel estimation |  |  |  |  |  |
| MA signature allocation (for data and DMRS) | Fixed/Random |  |  |  |  | Proponents report the details of random MA signature allocation (whether without or with collision) |
| Distribution of avg. SNR | Both equal and unequal | Equal |  | Both equal and unequal |  | Uniform discrete values for unequal case,, range [x - a, x + a] (dB) with 1 dB step, where x is the average SNR among UEs, and the deviation [a=3] |
| Timing offset | 0 as starting point. For grant-free without perfect TA, value is TBD |  |  |  |  |  |
| Frequency error | 0 as starting point. The value(s) is TBD. |  |  |  |  |  |
| Traffic model for link level | Full buffer as starting point. Non-full-buffer model (like Poisson arrival of fixed packet size) is optional. |  |  |  |  |  |
| For link level calibration purpose only | OMA single user whose spectral efficiency is the same as per UE SE in NoMA. AWGN curves can be provided also. |  |  |  |  |  |

Note: for the case when a parameter has a “OR” condition, companies are encouraged to evaluate all the corresponding values

- A.2 OMA single UE performance

| (a). TDLA-30ns, 1T4R | (b). TDLC-300ns, 1T4R |
| --- | --- |

Figure 3. BLER Performance of OMA Single UE

- A.3 BLER Curves

| (a). TBS=80, TDLA-30ns, 1T4R | (b). TBS=120, TDLA-30ns, 1T4R | (c). TBS=150, TDLA-30ns, 1T4R |
| --- | --- | --- |
| (d). TBS=80, TDLC-300ns, 1T4R | (e). TBS=120, TDLC-300ns, 1T4R | (f). TBS=150, TDLC-300ns, 1T4R |

Figure 4. BLER Performance of SCMA (Equal SNR)

| (a). TBS=80, TDLA-30ns, 1T4R | (b). TBS=120, TDLA-30ns, 1T4R | (c). TBS=150, TDLA-30ns, 1T4R |
| --- | --- | --- |
| (d). TBS=80, TDLC-300ns, 1T4R | (e). TBS=120, TDLC-300ns, 1T4R | (f). TBS=150, TDLC-300ns, 1T4R |

Figure 5. BLER Performance of MUSA (Equal SNR)

| (a). TBS=80, TDLA-30ns, 1T4R | (b). TBS=120, TDLA-30ns, 1T4R | (c). TBS=150, TDLA-30ns, 1T4R |
| --- | --- | --- |
| (d). TBS=80, TDLC-300ns, 1T4R | (e). TBS=120, TDLC-300ns, 1T4R | (f). TBS=150, TDLC-300ns, 1T4R |

Figure 6. BLER Performance of NCMA (Equal SNR)

| (a). TBS=80, TDLA-30ns, 1T4R | (b). TBS=120, TDLA-30ns, 1T4R | (c). TBS=150, TDLA-30ns, 1T4R |
| --- | --- | --- |
| (d). TBS=80, TDLC-300ns, 1T4R | (d). TBS=120, TDLC-300ns, 1T4R | (f). TBS=150, TDLC-300ns, 1T4R |

Figure 7. BLER Performance of WSMA (Equal SNR)