3GPP TSG RAN WG1 Meeting #93                                                                                        R1-1806453

Busan, Korea, May 21 – May 25, 2018

Agenda Item:	6.2.9

Source:	Huawei, HiSilicon

Title:	Discussion on PRG size for TM9 and TM10

Document for:	Discussion and decision

Introduction

This contribution discusses the PRG size defined in current LTE specification. We observe these PRG size definition impact the DMRS channel estimation performance, especially when UE is localized in low SNR area. A simple solution is proposed to address this issue.

Discussion

In 3GPP TS 36.213, the size of PRG is dependent on system bandwidth as

| System Bandwidth () | PRG Size () (PRBs) |
| --- | --- |
| ≤10 | 1 |
| 11 – 26 | 2 |
| 27 – 63 | 3 |
| 64 – 110 | 2 |

When UE is in TM9/10 and RI/PMI reporting is configured, UE shall assume the precoding granularity is P’ resource blocks in the frequency domain. Such definition has the merit of allowing flexible MU operation on PRG  level, e.g., one UE can be paired with different UE on different PRG. As a result, UE has to estimate the channel in frequency domain using DMRS within each P’ resource blocks. Compared with channel estimation on CRS, DMRS based channel estimation suffers from worse performance due to less number of DMRS symbols in frequency for filtering. This issue is essential when UE is localized in low SINR area. In this scenario, the merit of allowing flexible MU operation will disappear since SU is the typical use case. And, the channel estimation accuracy may dominate the receiver performance.

A link level simulation is performed to investigate the performance. In this simulation, the system bandwidth is 20Mhz, and UE is scheduled to transmit PDSCH on the entire bandwidth. The channel model is 3GPP Urban Macro. PRG size of 2 RBs is as the baseline. The performance gain is shown in Figure1 when the PRG size is set to the entire system bandwidth. One wideband precoder is applied on scheduled PDSCH for both 2-RB PRG size and system-bandwidth PRG size.

Figure1: Performance gain of PRG with size of system bandwidth over 2PRBs PRG size when wideband precoder is applied

From the simulation results, we observed when UE is in the SNR area of below 10dB, PRG size of entire system bandwidth has better performance than current PRG size definition. Particularly, the performance gain is around 8%~20% when SNR range is -5dB ~2dB, which is the typical working SNR area of cell edge UEs.

One argument is that because wideband preocoder is applied, certainly PRG size of the entire system bandwidth has better performance, and when subband precoder is used, the situation may be different. However, we believe that wideband precoder and large PRG size is the typical configuration for cell edge UEs. To confirm it, we also run the simulations, where the performance is compared between PRG size of system bandwidth when applying wideband precoder and 2PRBs PRG size when applying subband precoder. The simulation result is shown in Figure2.

Figure2: Performance gain of PRG size of system bandwidth when applying wideband precoder over 2PRBs PRG size when applying subband precoder

From Figure2 we observed that when subband precoder is applied on PDSCH, small PRG size has better performance than large PRG size with wideband precoder in relative high SNR area. However when SNR range is -5dB ~5dB, PRG size of system bandwidth shows better performance.

Based on the above discussion and evaluations, we believe introducing larger PRG size is necessary and would benefit TM9 and TM10.

To address this issue, one simple way is to introduce a higher layer parameter to indicate another PRG size that is different from legacy defined values. We observe the PRG size of whole scheduled bandwidth should be supported and would be sufficient at this stage. Hence we propose

Proposal: introduce higher layer parameter SizeOfSchedResourceBlocks to indicate whether the PRG size is whole scheduled bandwidth.

Conclusions

This contribution evaluate the impact of PRG size on the TM9/10 performance. We found the PRG size of whole scheduled bandwidth has significant performance gain over legacy PRG size in low SNR area. Hence we have the following proposal

Proposal: introduce higher layer parameter SizeOfSchedResourceBlocks to indicate whether the PRG size is whole scheduled bandwidth.