3GPP TSG RAN WG1 Meeting #93                                                                        R1-1807328

Busan, Korea, May 21th – 25th, 2018

Agenda item:	7.1.1.1

Source: 	Qualcomm Incorporated

Title: 	Remaining details on synchronization signal design

Document for:	Discussion/Decision

# Introduction

This contribution discusses remaining aspects on synchronization signal design.

# On k0 Signalling

During RAN1 #92b, it was discussed whether k0μ values using in OFDM symbol generation formula should be RRC-signalled as done in the existing RRC or be derived from the existing RRC parameters such as offset from point A to the first usable PRB (offsetToCarrier Ngrid,xstart,μ) and the carrier BW (carrierBandwidth Ngrid,xsize,μ) i.e., the value of k0μ  for any numerology µ is obtained using

k0μ=Ngrid,xstart,μNscRB+Ngrid,xsize,μ2∙NscRB-Ngrid,xstart,μ0NscRB+Ngrid,xsize,μ02∙NscRB-k0μ0×2μ0-μ

Here, the question is how we should select the value of μ0 if k0μ is derived from the existing RRC parameters. From our perspectives, μ0 should be selected such that the UE can align FFT and f0 placement across multiple gNBs operating on the same channel even if the full sets of numerologies used by the gNBs are not identical. Furthermore, k0μ0 values should be RRC-signaled. Hence, we made the following proposal:

Proposal 1: The value of k0μ  for any numerology µ is obtained by

k0μ=Ngrid,xstart,μNscRB+Ngrid,xsize,μ2∙NscRB-Ngrid,xstart,μ0NscRB+Ngrid,xsize,μ02∙NscRB-k0μ0×2μ0-μ

where μ0 is the numerology with highest SCS configured in the carrier, and the value of k0μ0∈{-6,0,+6} is obtained by RRC signaling.

# Power Offset between SSB and PDCCH

RAN1 #92b made the following agreements:

Agreements: - In initial cell selection and idle mode, the UE may assume that the EPRE offset between PDCCH DMRS and SSS in the QCL’ed SSB is in the range [X, Y]dB when PDDCH with CRC scrambled by SI-RNTI, P-RNTI and RA-RNTI - FFS: value of X and Y and dependence on numerology - ¦X¦ & Y are no more than 12

It could be shown that 6.02dB represents 1 bit of ADC dynamic range. Hence, by limiting the power uncertainty less than 6dB, 1 bit of the ADC budget is sacrificed. Also, it should be noted that AGC also needs to accommodate the uncertainty due to other factors (e.g., power variations due to fading, waveform PAPR, beamforming mismatch etc.).

Proposal 2: The ratio of PDCCH DMRS EPRE to SSS EPRE is in the range [X, Y] = [-6, 6]dB for any combination of SSB and PDCCH numerologies.

# Conclusion

This contribution has made the following proposals:

Proposal 1: The value of k0μ  for any numerology µ is obtained by

k0μ=Ngrid,xstart,μNscRB+Ngrid,xsize,μ2∙NscRB-Ngrid,xstart,μ0NscRB+Ngrid,xsize,μ02∙NscRB-k0μ0×2μ0-μ

where μ0 is the numerology with highest SCS configured in the carrier, and the value of k0μ0∈{-6,0,+6} is obtained by RRC signaling.

Proposal 2: The ratio of PDCCH DMRS EPRE to SSS EPRE is in the range [X, Y] = [-6, 6]dB for any combination of SSB and PDCCH numerologies.