3GPP TSG RAN WG1 Meeting #93 		R1-1807970

Busan, Korea, May 21st – 25th, 2018

Title:	LS on CSI measurement and reporting in DRX active time

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

#### Name:	Mihai Enescu

E-mail Address:	mihai.enescu@nokia.com

1. Overall Description:

RAN1 has agreed below UE procedures for CSI measurement and reporting in DRX active time

Agreement

In DRX mode operation, PUSCH-based SP-CSI is not reported when UE is not in Active Time.

The following wording could be used in section 5.2.3 for your reference:

Agreement

The most recent CSI measurement occasion occurs in DRX Active time for CSI to be reported

- If a UE is configured with higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig  the UE shall derive the interference measurements for computing the CSI value reported in uplink slot n based on the most recent, no later than the CSI reference resource, occasion of CSI-IM and/or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) associated with the CSI resource setting.

2. Actions:

To RAN2:

ACTION: RAN1 respectfully asks RAN2 to take the above information into consideration.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting 94	20 - 24 August 2018		Gothenburg, SE

TSG RAN WG1 Meeting 94-Bis                           	8 - 12 October 2018 		Chengdu, CN