3GPP TSG RAN WG1 Meeting #94	R1-1808290

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda Item:	7.2.1.3

Source: 	Fujitsu

Title:	Considerations on procedures related to NOMA

Document for:	Discussion/Decision

Introduction

This contribution is revised from R1-1806122.

According to the Rel-15 NOMA SID, the following items will be further studied [1].

- Transmitter side signal processing schemes for non-orthogonal multiple access

- Receivers for non-orthogonal multiple access

- Procedures related to the non-orthogonal multiple access

- Link and system level performance evaluation or analysis for non-orthogonal multiple access

During the last few RAN1 meetings, major link and system level simulation assumptions have been discussed and agreed. In this contribution, some general considerations on item 3 (procedures related to the non-orthogonal multiple access) are provided for discussion.

Discussion

According to the Rel-15 NOMA SI, the following will be studied for the procedures related to the non-orthogonal multiple access.

Procedures related to the non-orthogonal multiple access [RAN1]

- UL transmission detection

- HARQ, including transmission scheme, feedback scheme, and combining scheme

- Link adaptation MA signature allocation/selection

- Synchronous and asynchronous operation

- Adaptation between orthogonal and non-orthogonal multiple access

From the perspective of PUSCH related procedures, a UE will perform an initial transmission based on a dynamic grant (grant-based) or a configured grant (grant-free). For NOMA, reference signal and signature for data transmission (such as sequence, interleaver, codeword, etc.) are crucial NOMA signatures for enabling multi-user multiplexing. To perform a transmission, a UE needs to at least know which reference signal and sequence/interleaver/codeword will be used. Therefore, NOMA signature determination should be specified for the UE. For transmission with dynamic grant, dynamic signaling can be used to indicate the aforementioned NOMA signatures to the UE. For transmission with configured grant, the NOMA signatures can be indicated to the UE by using semi-static signaling, or alternatively the UE can perform a random selection from a set of (pre)configured NOMA signatures. One use case for the random selection could be supporting autonomous transmission from INACTIVE UEs since signaling may not be possible. Another use case could be letting a group of sporadic-traffic UEs (e.g. sensors), the number of which is larger than that of NOMA signatures, share the same physical resources to improve spectral efficiency. For the latter use case, even if some UEs are triggered to transmit by the same event, they may still have a degree of freedom to avoid NOMA signature collisions by perform a random selection. If using a random selection approach, it would be beneficial to define a one-to-one mapping between the reference signal and signature for data transmission. Hence, once a gNB detects a reference signal, it could immediately use the associated signature for data decoding instead of performing blind decoding for multiple times.

Proposal 1: For NOMA with configured grant, NOMA signature determination for the UE should be further studied. Configuring the NOMA signature by the gNB and/or randomly selecting the NOMA signature by the UE can be used as a starting point.

NOMA signature collision could happen especially for transmissions with configured grant. Collison could occur when more than one UE use the same NOMA signature for transmission, where the NOMA signature could be reference signal and/or signature for data transmission. This could happen if the number of UEs is larger than the number of available NOMA signatures. Therefore, suitable methods to avoid such collisions can be considered. These methods can be considered from the outset of NOMA signature design, e.g., by guaranteeing a reasonably large size pool of NOMA signatures, or leveraged by introducing hopping schemes to reduce the possibilities of continuous collisions. For example, NOMA signature hopping can be performed on a slot or symbol basis. Also the aforementioned random selection of NOMA signatures can be considered.

Proposal 2: Potential NOMA signature collision avoidance methods, such as enlarging the NOMA signature pool size, introducing NOMA signature hopping, randomly selecting the NOMA signature and so on, can be further studied for configured grant in NOMA.

Following PUSCH procedures, a UE needs to know the utilized MCS level and TB size. In NR, the TB size is calculated by the UE based on the indicated/configured MCS level and allocated resources. In NOMA, however, a same TB size could spread over more frequency and/or time resources than what would have been divided among UEs in OMA. Hence, the resource redundancy introduced by NOMA spreading may need to be taken into account for determining allocated resources and then the TB size. For example, TB size scaling may be considered to compensate the spreading effect.

Link adaptation is important for improving efficiency by adapting to channel variations. For OMA, link adaptation can generally refer to adaptation of modulation order, code rate, resource allocation, redundancy version, etc. Signature for data transmission is the most unique characteristic of NOMA compared with OMA, and thus signature adaptation can be considered as an additional degree of freedom for link adaptation. Taking the spreading sequence based NOMA scheme as an example, signature adaptation could refer to switching among spreading sequences with different spreading factors. Similar principles could also apply to other NOMA schemes. Whether and how NOMA signature adaptation is supported for dynamic grant and configured grant should be further studied.

Proposal 3: Potential NOMA spreading impacts on TB size determination (e.g. whether TB size scaling is needed), and whether and how to achieve link adaptation, also including potential signature adaptation, should be further studied for NOMA.

For transmission with dynamic grant, a UE initiates a transmission under control of the gNB; however for transmission with configured grant, a gNB needs to identify which UE has performed a transmission. In the latter, the gNB will be able to discover a transmission by blind detecting a certain NOMA signature. Previously, the demodulation reference signal is used. It may be further studied whether other signatures can also be leveraged. Furthermore, to feedback HARQ-ACK to a particular UE, the gNB has to associate the transmission with a certain UE ID, such as C-RNTI, CS-RNTI, etc. However, there may not always be a one-to-one mapping between a demodulation reference signal and a particular UE ID. Also, if the number of UEs is larger than that of available NOMA signatures, it would be beneficial to further study the methodologies on how a gNB could acquire the required UE ID. A possible case for this scenario is considering mMTC UEs with sporadic traffic that could possibly share the same resources to improve efficiency.

Proposal 4: For configured grant in NOMA, UE detection needs further study on:

- How to identify an uplink transmission from a certain UE (e.g., based on UE-specific reference signals or also based on other signatures)

- How to acquire the UE ID (e.g., a UE ID is always associated with a UE-specific reference signal or can also be carried in the user data payload)

After initiating a transmission, a UE will wait within a certain time duration for HARQ-ACK from the gNB. The HARQ-ACK indication can be implicit, e.g., a UL grant scheduling a retransmission and a new transmission can imply NACK and ACK, respectively. If there is neither retransmission nor new transmission, it could be a subject of further study to establish whether an explicit HARQ-ACK is always required, and how to transmit this HARQ-ACK when necessary. An advantage of NOMA is that more UEs can transmit simultaneously by using the same resources. In the case of configured grant, miss-detection at the gNB side is more likely to happen and thus no HARQ-ACK feedback will be provided. If a UE does not receive any such HARQ-ACK information, the consequential related UE behavior may need further considerations. For example, if the UE can autonomously transmit the TB later to have another try, it is still possible for the gNB to successfully decode the previously missed TB.

Proposal 5: NOMA related HARQ-ACK feedback mechanisms and relevant UE behaviors, if any, should be further studied. For example, it may be considered to enable a UE to autonomously transmit the previous TB if neither ACK nor NACK is received.

Conclusions

In this contribution, we share some views on procedures related to NOMA. Some design principles and major design aspects are highlighted. Above all, the proposals are summarized as follows.

Proposal 1: For NOMA with configured grant, NOMA signature determination for the UE should be further studied. Configuring the NOMA signature by the gNB and/or randomly selecting the NOMA signature by the UE can be used as a starting point.

Proposal 2: Potential NOMA signature collision avoidance methods, such as enlarging the NOMA signature pool size, introducing NOMA signature hopping, randomly selecting the NOMA signature and so on, can be further studied for configured grant in NOMA.

Proposal 3: Potential NOMA spreading impacts on TB size determination (e.g. whether TB size scaling is needed), and whether and how to achieve link adaptation, also including potential signature adaptation, should be further studied for NOMA.

Proposal 4: For configured grant in NOMA, UE detection needs further study on:

- How to identify an uplink transmission from a certain UE (e.g., based on UE-specific reference signals or also based on other signatures)

- How to acquire the UE ID (e.g., a UE ID is always associated with a UE-specific reference signal or can also be carried in the user data payload)

Proposal 5: NOMA related HARQ-ACK feedback mechanisms and relevant UE behaviors, if any, should be further studied. For example, it may be considered to enable a UE to autonomously transmit the previous TB if neither ACK nor NACK is received.

- References

[1] RP-171043, Revision of study on 5G non-orthogonal multiple access, RAN#76, June 2017.