3GPP TSG RAN WG1 Meeting #94 									R1-1808613

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda Item:	7.2.2.3.2

Source:	Apple Inc.

Title:	On UL Signals and Channels for NR-U

Document for:	Discussion/Decision

# Introduction

LTE based unlicensed operations, i.e. LAA/eLAA, were introduced starting from Rel-13. A Rel-15 study item [1] on NR-based unlicensed access has been approved to study the solutions applicable to potential spectrum candidates both below and above 6Ghz, as well as various deployment scenarios, including CA or DC based LAA and standalone based unlicensed operations.

In RAN1 #93, it was further agreed that [2]:

Agreement:

- An interlaced waveform can have benefits in some scenarios including

  - Link budget limited cases with given PSD constraint

  - As one option to efficiently meet the occupied channel bandwidth requirement.

- A waveform contiguous in frequency may be adequate in some scenarios

  - To inherit legacy contiguous allocation designs.

Note: It is RAN1’s understanding that the temporal allowance of not meeting occupied channel bandwidth by regulation can be exploited if the minimum bandwidth requirement, e.g., 2 MHz, is satisfied.

Agreement:

- Support for Rel-15 NR PRACH formats can be considered. Exclusion of the support of certain formats is to be identified.

  - Note: It is RAN1’s understanding that certain formats do not meet the minimum bandwidth requirement by regulation.

- It is identified that interlaced based PRACH can be beneficial.

- The following aspects can be considered for Interlace waveform based PRACH design for 4-step random access:

  - Interlacing based on PRB or REs

  - Targeted cell sizes

  - Targeted PRACH capacity

  - Targeted false alarm and detection rates

  - Targeted timing estimation accuracy

  - Number of formats

  - Multiplexing with other channels such as block interlaced PUCCH and PUSCH

In this contribution, we briefly discuss potential challenges and solutions to PRACH channel design in unlicensed spectrum access. In our companion papers [5][6], we also briefly discuss other potential enhancements to physical layer procedures and DL signals design for unlicensed access.

# PRACH Waveform

NR PRACH preamble formats inherit similar design as LTE. The preambles are based on ZC sequence with constant envelope property to maximize PA efficiency and coverage. It consists of two categories: the long sequence formats with 839 tones and short sequence formats with 139 tones, respectively, as shown in Figure 3-1.

Figure 3-1 NR PRACH preamble formats

Each category also supports different sets of numerologies, leading to different BW, transmission length and coverage.

PRACH support is also critical for unlicensed spectrum access, including derivation of uplink timing or for stand-alone deployment. However, direct application of existing NR PRACH formats to unlicensed spectrum is not straightforward, due to certain regulation requirements. Specifically, the transmission in the unlicensed spectrum is subject to the following requirements on occupied channel bandwidth (COB) and power spectrum density (PSD) [2][3]

- COB requirements:

  - The bandwidth containing 99% of the power of the signal, shall be between 80% and 100% of the Nominal Channel Bandwidth (NCB)

  - The OCB in 60GHz bands shall be more than 70% of the NCB

- PSD requirements:

  - For 5GHz spectrum: maximal EIRP is 23dBm and PSD shall not exceed 10dBm/MHz.

  - For 60GHz spectrum: maximal EIRP is 40dBm and PSD shall not exceed 13dBm/MHz.

As shown in Figure 3-1, the maximal PRACH pre-amble bandwidth using a long sequence format is 839 × 5kHz≈4.2mHz. When using short sequence format, the maximal pre-amble bandwidth is also 139×60kHz≈8.3mHz, assuming up to 60kHz numerology is adopted for PRACH preamble in sub-6GHz spectrum.

We notice that in either case, the PRACH preamble will not satisfy the OCB requirements above, given the channel allocation is in unit of 20Mhz for the 5GHz unlicensed spectrum. Further, due to the PSD requirements, the maximal transmit power is also limited. For example, with 4.2mHz BW using the long sequence format, the maximal transmit power is limited to ~16dBm for 5GHz spectrum. This is much lower than the 23dBm allowed by the regulation and imposes unnecessary constraint on the UL coverage. Therefore, the PRACH preamble waveform needs to be modified for the unlicensed spectrum in order to meet the regulation requirements and allow the UE to reach the corresponding maximum transmit power.

It should be emphasized that a desirable solution should introduce minimal changes to the existing design for licensed spectrum and should also keep the benefits of the existing PRACH preamble waveform as much as possible (e.g. the inherent constant envelope property from the ZC sequence).

One solution is to keep the existing PRACH preamble design, but simply apply “tone-interlacing” by factor M, to expand the PRACH preamble BW such that both COB requirements and target transmit power are met. Figure 3-2 illustrates an example with M=3, where PRACH preamble tones are mapped to 1 out every 3 subcarriers.

Figure 3-2 PRACH preamble with tone interlacing

By using the tone interlacing, the PRACH preamble BW can be flexibly configured by the network, through factor M, based on deployment as well as the regulation requirement. In addition, the tone-interlacing still preserves the time-domain characteristics of the original PRACH waveform, which allows maximal PA efficiency. Different “comb” of the interlace can be treated as different PRACH preamble groups, and preambles within each group could still be separated through cyclic shifts, subject to delay spread constraints.

Proposal 1: PRACH preamble design for NR unlicensed should be based on existing design with minimal changes to meet the regulation and maximal transmit power requirements.

Proposal 2: For NR unlicensed, adopt tone-interlacing on top of the existing NR PRACH preamble design.

# Conclusions

In this contribution, we briefly discuss a few existing Rel-15 NR features that could be leveraged for NR based unlicensed access. We have the following observation and proposals:

Proposal 1: PRACH preamble design for NR unlicensed should be based on existing design with minimal changes to meet the regulation and maximal transmit power requirements.

Proposal 2: For NR unlicensed, adopt tone-interlacing on top of the existing NR PRACH preamble design.

# References

- RP-172021, “Revised SID on NR-based Access to Unlicensed Spectrum”, 3GPP RAN #77, Sapporo, Japan, 11th – 14th September 2017.

- “Chairman’s notes RAN1 93 final”, 3GPP RAN1 #93, Busan, Korea, 21st – 25th, May, 2018.

- ETSI EN 301 893, “5GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”

- ETSI EN 302 567, “Multiple-Gigabit radio equipment operating in the 60GHz band”

- R1-1808611, “Channel access procedures for NR-U”, Apple Inc., 3GPP RAN1 #94, Gothenburg, Sweden, 20th – 24th, August 2018.

- R1-1808612, “On DL signals and channels for NR-U”, Apple Inc., 3GPP RAN1 #94, Gothenburg, Sweden, 20th – 24th, August 2018.