3GPP TSG RAN WG1 Meeting #94		R1-1809003

Gothenburg, Sweden, August 20th – 24th, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | - | rev | - | Current version: | 14.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for interference randomization for NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh-Core |  |  |  |  |  |  | Date: |  | 2018-08-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current description for interference randomziaiton in NB-IoT, it is unclear if the scrambling sequence has length of <Number of RE in a subframe> or <Number of RE in a slot> and same sequence is applied to first and second slot. Clarify that it is the former |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the scrambling sequence has a length of <Number of RE in a subframe> |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.3.4, 10.2.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 10.2.3.4	Mapping to resource elements

NPDSCH can be mapped to one or more than one subframes, , as given by clause 16.4.1.3 of 3GPP TS 36.213 [4], each of which shall be transmitted times.

For each of the antenna ports used for transmission of the physical channel, the block of complex-valued symbols  shall be mapped to resource elements  which meet all of the following criteria in the current subframe:

-	the subframe is not used for transmission of NPBCH, NPSS, or NSSS, and

-	they are assumed by the UE not to be used for NRS, and

-	they are not overlapping with resource elements used for CRS as defined in clause 6 (if any), and

-	the index  in the first slot in a subframe fulfils  where is given by clause 16.4.1.4 of 3GPP TS 36.213 [4].

The mapping of  in sequence starting with  to resource elements  on antenna port  meeting the criteria above shall be in increasing order of first the index  and then the index, starting with the first slot and ending with the second slot in a subframe. For NPDSCH not carrying BCCH, after mapping to a subframe, the subframe shall be repeated for  additional subframes, before continuing the mapping of  to the following subframe. For NPDSCH associated with C-RNTI when interferenceRandomisationConfig is configured, or NPDSCH associated with RA-RNTI, TC-RNTI or P-RNTI and transmitted in an NB-IoT carrier configured by SystemInformationBlockType22-NB, or NPDSCH associated with G-RNTI or SC-RNTI, define as the block of complex-valued symbols mapped to subframe number  ns/2and radio frame number . Each complex-valued symbol  shall be multiplied with before its transmission, with

where the scrambling sequence is given by clause 7.2, and shall be initialized at the start of each subframe with .

The mapping of  is then repeated until  subframes have been transmitted. For NPDSCH carrying BCCH, the is mapped to  subframes in sequence and then repeated until  subframes have been transmitted.

The NPDSCH transmission can be configured by higher layers with transmission gaps where the NPSDCH transmission is postponed. There are no gaps in the NPDSCH transmission if where  is given by the higher layer parameter dl-GapThreshold and  is given by [4]. The gap starting frame and subframe is given by  where the gap periodicity,, is given by the higher layer parameter dl-GapPeriodicity. The gap duration in number of subframes is given by , where  is given by the higher layer parameter dl-GapDurationCoeff. For NPDSCH carrying the BCCH there are no gaps in the transmission.

The UE shall not expect NPDSCH in subframe  if it is not a NB-IoT downlink subframe, except for transmissions of NPDSCH carrying SystemInformationBlockType1-NB in subframe 4. In case of NPDSCH transmissions, in subframes that are not NB-IoT downlink subframes, the NPDSCH transmission is postponed until the next NB-IoT downlink subframe.

#### 10.2.5.5	Mapping to resource elements

The block of complex-valued symbols  shall be mapped in sequence starting with  to resource elements  on the associated antenna port which meet all of the following criteria:

-	they are part of the NCCE(s) assigned for the NPDCCH transmission, and

-	they are not used for transmission of NPBCH, NPSS, or NSSS , and

-	they are assumed by the UE not to be used for NRS, and

-	they are not overlapping with resource elements used for CRS as defined in clause 6 (if any), and

-	the index  in the first slot in a subframe fulfils  where is given by clause 16.6.1 of 3GPP TS 36.213 [4].

The mapping to resource elements  on antenna port  meeting the criteria above shall be in increasing order of first the index  and then the index, starting with the first slot and ending with the second slot in a subframe. Denote  as the complex-valued symbols that are mapped to resource elements meeting the criteria above in subframe ns/2, with the insertion of <NIL> elements in the locations of resource elements which are not part of the NCCE(s) assigned for the NPDCCH transmission.

For NPDCCH associated with RA-RNTI, TC-RNTI or P-RNTI and transmitted in an NB-IoT carrier configured by SystemInformationBlockType22-NB, or NPDCCH associated with G-RNTI or SC-RNTI, or for NPDCCH associated with C-RNTI when interferenceRandomisationConfig is used according to [11], each complex-valued symbol , shall be multiplied with ,where

where the scrambling sequence  is given by clause 7.2, and shall be initialized at the start of each subframe with .

The NPDCCH transmission can be configured by higher layers with transmissions gaps where the NPDCCH transmission is postponed. The configuration is the same as described for NPDSCH in clause 10.2.3.4.

The UE shall not expect NPDCCH in subframe  if it is not a NB-IoT downlink subframe. In case of NPDCCH transmissions, in subframes that are not NB-IoT downlink subframes, the NPDCCH transmission is postponed until the next NB-IoT downlink subframe.