3GPP TSG RAN WG1 Meeting #94		     R1-1809073

Gothenburg, Sweden, 20th - 24th August 2018

Agenda Item:	7.2.3.2

Source:	AT&T

Title:	Evaluation methodology and simulation results for NR IAB

Document for:	Discussion/Approval

# Introduction

As part of the Study Item on Integrated Access and Backhaul for NR [1], 3GPP has agreed to identify and evaluate potential solutions for following requirements and aspects associated with the efficient operation of integrated access and wireless backhaul for NR:

- Efficient and flexible operation for both inband and outband relaying in indoor and outdoor scenarios

- Multi-hop and redundant connectivity

- End-to-end route selection and optimization

- Support of backhaul links with high spectral efficiency

- Support of legacy NR UEs

In this paper, we provide initial system simulation results for key IAB deployment scenarios.

# System level simulation results for IAB

IAB may be used for both coverage and capacity improvement for 5G networks without needing to deploy fiber to every node. In this section we provide system level results based on a heterogeneous network deployment with the following parameters:

- Network dropping: Hexagonal grid dropping as donor gNBs (Macro layer) with 25m height, then randomly or planned dropping Micro layer as relay node (Micro layer) with 10m height and UEs dropped randomly in the area.

- Frequency: 39GHz

- Bandwidth: 200MHz

- SCS: 120kHz

- Tx power: 44dBm (macro), 30dBm (small cell)

- Antenna element gain: 8 dB

- Channel model according to 38.802.

- Max number of hops: 2

- IAB node association: Max RSRP

Based on this scenario, Figure 1 illustrates the RSRP distribution for different backhaul links between the donor (macro) nodes and associated IAB nodes of hop order 1 and between IAB nodes of hop order 1 and associated IAB hop order nodes of hop order 2.

Figure 1. RSRP distribution for access and multi-hop backhaul links

Figure 1 clearly illustrates the improved link quality of backhaul links compared to the access links due to the height advantage and larger antenna array size. For example, the 10% and 50% values of the backhaul link distributions experience roughly 20dB gains over the access links in the case of the first hop (macro to small cell) and 10dB gains in the case of the second hop (small cell to small cell). While these gains may vary depending on the deployment (e.g. homogeneous vs. HetNet and FR1 vs. FR2), the general trend is expected to remain and motivates spectral efficiency and MIMO transmission enhancements for IAB links compared to regular access links taking into account the increased capability of IAB nodes as well [2].

Observation 1: Multi-hop backhaul links experience significant gains in RSRP compared to access links.

Comparing the signal-to-interference ratio (SIR) of an IAB topology may also give insights into the system design. Figure 2 illustrates two cases: 1) all IAB nodes are assumed to transmit simultaneously in the DL without restriction and 2) hop orders utilize orthogonal resources for their transmissions.

Figure 2. SIR distribution for multi-hop backhaul links

Figure 2 illustrates the benefits of optimizing the resource allocation in a multi-hop network both from a half-duplex constraint consideration as well as for mitigating cross link interference [2]. However further considerations should be given to how to configure the resource partitions considering load metrics and latency benefits of dynamic TDD.

Observation 2: Multi-hop backhaul networks benefit from resource allocation partitioning across backhaul links.

Now we provide results for a Homogeneous IAB mmWave deployment and compare the DL transport block throughput for different numbers of donor (fiber capable) nodes.

Figure 3. DL TB throughput distributions for different numbers of donor nodes

Figure 3 illustrates the ability of IAB to significantly improve system performance in scenarios with coverage holes created by sparse deployment of fiber-capable base stations. Despite the fact that the IAB and donor nodes in the IAB scenario operate with a half-duplex constraint, a significant improvement in 10%, 50%, and 90% throughput is observed since coverage is improved for a significant fraction of users (note users in outage are not considered these plots) and additional nodes provide a system capacity boost.

Observation 3: IAB provides coverage and system performance improvements even when nodes are subject to a half-duplex constraint.

# Conclusion

In this contribution, we provide system-level simulation results for multi-hop IAB networks in a heterogeneous deployment of macro and small cells in FR2. The following observations are made:

Observation 1: Multi-hop backhaul links experience significant gains in RSRP compared to access links.

Observation 2: Multi-hop backhaul networks benefit from resource allocation partitioning across backhaul links.

Observation 3: IAB provides coverage and system performance improvements even when nodes are subject to a half-duplex constraint.

Based on these observations the following proposal is made:

Proposal 1: Capture the observations in this contribution in TR 38.874. RAN1 should further study potential gains and techniques for resource allocation and interference mitigation across multiple backhaul hops.

# Reference

[1] RP-170831, Study on Integrated Access and Backhaul for NR, AT&T, Qualcomm, Samsung.

[2] R1-1809073, Overview of physical layer enhancements for IAB, AT&T.