3GPP TSG RAN WG1 Meeting #94			R1-1809167

Gothenburg, Sweden, August 20th – 24th, 2018

Source:	NTT DOCOMO, INC.

Title:	Updated evaluation results of dense urban-eMBB for IMT-2020 self-evaluation

Agenda Item:	7.2.7.2

Document for: 	Discussion

- Introduction

At the RAN1#93 meeting, the initial evaluation results for Dense Urban–eMBB Config A and Config B  have been presented [1], from which the DL 5th percentile spectral efficiency can’t meet the ITU requirements. For further enhancmenet, we implement the multi-pannel combination at UE side for Dense Urban-eMBB config B. Based on the enhancement, we present the updated evaluation results of Dense Urban-eMBB config B.

- ITU Requirements

The minimum requirement for the 5th percentile user spectral efficiency and average spectral efficiency for Dense Urban-eMBB sceanrio is provided as follow [2].

TABLE 1

5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Dense Urban – eMBB (NOTE 1) | 0.225 | 0.15 |
| NOTE 1: This requirement will be evaluated under Macro TRxP layer of Dense Urban – eMBB test environment as described in [3]. |  |  |

TABLE 2

Average spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Dense Urban – eMBB (NOTE 1) | 7.8 | 5.4 |
| NOTE 1: This requirement applies to Macro TRxP layer of the Dense Urban – eMBB test environment as described in [3]. |  |  |

The defination of user experienced data is also defined in [2]. User experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time.

In case of one frequency band and one layer of transmission reception points (TRxP), the user experienced data rate could be derived from the 5th percentile user spectral efficiency through equation (1). Let W denote the channel bandwidth and SEuser denote the 5th percentile user spectral efficiency. Then the user experienced data rate, Ruser is given by:

Ruser = W × SEuser 	(1)

The minimum requirement of user experienced data rate is listed as follow.

TABLE 3

User Experienced Data Rate

| Test environment | Downlink (Mbit/s) | Uplink (Mbit/s) |
| --- | --- | --- |
| Dense Urban – eMBB | 100 | 50 |

- Evaluation Results of DL Spectral Efficiency

In this section, we provide the downlink results of Dense Urban–eMBB Config A/B. The detailed evaluation assumptions can be found in Annex.

The initial evaluation results of DL average spectral efficiency and DL 5th percentile spectral efficiency for Config A/B are summarized in Table 4 and Table 5. The results of user experienced data rate are provided based on initial results of the 5th percentile spectral efficiency.

TABLE 4

Average spectral efficiency and 5th percentile user spectral efficiency for Dense Urban – eMBB Config A

| Antenna configuration | SEavg (bit/s/Hz/TRxP) | ITU Requirement (bit/s/Hz/TRxP) | SEuser (bit/s/Hz) | ITU Requirement (bit/s/Hz) |
| --- | --- | --- | --- | --- |
| 16 TXRU of TRxP | 12.06 | 7.8 | 0.38 | 0.225 |
| 32 TXRU of TRxP | 15.67 | 7.8 | 0.5459 | 0.225 |

TABLE 5

Average spectral efficiency and 5th percentile user spectral efficiency for Dense Urban – eMBB Config B

| Antenna configuration | SEavg (bit/s/Hz/TRxP) | ITU Requirement (bit/s/Hz/TRxP) | SEuser (bit/s/Hz) | ITU Requirement (bit/s/Hz) |
| --- | --- | --- | --- | --- |
| 32 TXRU of TRxP, w/o combination at UE side | 13.84 | 7.8 | 0.2194 | 0.225 |
| 32 TXRU of TRxP, w/ combination at UE side | 17.48 | 7.8 | 0.2441 | 0.225 |

Based on above evaluation results, we have following observation.

Observation 1: The minimum requirement for DL 5th percentile user spectral efficiency and DL average spectral efficiency can be met for Dense Urban-eMBB configuration A and B.

- Evaluation Results of UL Spectral Efficiency

In this section, we provide the preliminary uplink results of Dense Urban–eMBB Config A. The detailed evaluation assumptions can be found in Annex.

The initial evaluation results of UL average spectral efficiency and UL 5th percentile spectral efficiency are summarized in Table 6. The results of user experienced data rate are provided based on initial results of the 5th percentile spectral efficiency

TABLE 6

Average spectral efficiency and 5th percentile user spectral efficiency for Dense Urban – eMBB Config A

| Test environment | SEavg (bit/s/Hz/TRxP) | ITU Requirement (bit/s/Hz/TRxP) | SEuser (bit/s/Hz) | ITU Requirement (bit/s/Hz) |
| --- | --- | --- | --- | --- |
| Dense Urban – eMBB Config A (4 layer) | 5.76 | 5.4 | 0.24 | 0.15 |
| Dense Urban – eMBB Config A (8 layer) | 6.11 | 5.4 | 0.21 | 0.15 |

Based on above evaluation results, we have following observation.

Observation 2: The minimum requirement for UL 5th percentile user spectral efficiency and UL average spectral efficiency can be met for Dense Urban-eMBB configuration A.

- Evaluation Results of DL User Experienced Data Rate

In this section, we will analyze the minimum bandwidth to meet the ITU requirement on user experienced data rate based on initial results of the 5th percentile spectral efficiency in Table 7.

TABLE 7

DL user experienced data rate

| Test environment | Bandwidth, W (MHz) | DL user experienced data rate (Mbit/s) | ITU requirement (Mbit/s) |
| --- | --- | --- | --- |
| Dense Urban – eMBB Config A (32 TXRU of TRxP) | 100 | 54.59 | 100 |
|  | 200 | 109.18 | 100 |
|  | 300 | 163.77 | 100 |
| Dense Urban – eMBB Config B (w/ combination at UE side) | 400 | 97.64 | 100 |
|  | 500 | 122.05 | 100 |
|  | 600 | 146.46 | 100 |

Based on above results, we have following observation.

Observation 3: The requirement on DL user experienced data rate can be met for Dense Urban configuration A, when the bandwidth is larger than 200MHz

Observation 4: The requirement on DL user experienced data rate can be met for Dense Urban configuration B, when the bandwidth is larger than 500MHz.

- Evaluation Results of UL User Experienced Data Rate

In this section, we will analyze the minimum bandwidth to meet the ITU requirement on user experienced data rate based on initial results of the 5th percentile spectral efficiency in Table 8

TABLE 8

UL user experienced data rate

| Test environment | Bandwidth, W (MHz) | DL user experienced data rate (Mbit/s) | ITU requirement (Mbit/s) |
| --- | --- | --- | --- |
| Dense Urban – eMBB Config A | 150 | 36 | 50 |
|  | 200 | 48 | 50 |
|  | 250 | 60 | 50 |

Observation 5: The requirement on UL user experienced data rate can be met for Dense Urban configuration A, when the bandwidth is larger than 250MHz

- Summary

In this contribution, we provide the results of Dense Urban-eMBB Config A and Config B, including the DL/UL average spectral efficiency, the DL/UL 5th percentile user spectral efficiency, the DL/UL user experienced data rate. The observations are summarized as follow:

Observation 1: The minimum requirement for DL 5th percentile user spectral efficiency and DL average spectral efficiency can be met for Dense Urban-eMBB configuration A and B.

Observation 2: The minimum requirement for UL 5th percentile user spectral efficiency and UL average spectral efficiency can be met for Dense Urban-eMBB configuration A.

Observation 3: The requirement on DL user experienced data rate can be met for Dense Urban configuration A, when the bandwidth is larger than 200MHz

Observation 4: The requirement on DL user experienced data rate can be met for Dense Urban configuration B, when the bandwidth is larger than 500MHz.

Observation 5: The requirement on UL user experienced data rate can be met for Dense Urban configuration A, when the bandwidth is larger than 250MHz

References

- R1-1807085, “Evaluation results of dense urban-eMBB for IMT-2020 self-evaluation”, NTT DOCOMO, INC, May 2018.

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, June 2017.

Annex: Evaluation assumptions

| Dense Urban – eMBB |  | Config A | Config B |
| --- | --- | --- | --- |
| Carrier frequency for evaluation |  | 1 layer (Macro) with 4 GHz | 1 layer (Macro) with 30 GHz |
| Total transmit power per TRxP |  | 41 dBm for 10 MHz bandwidth | 37 dBm for 40 MHz bandwidth |
| Percentage of high loss and low loss building type |  | 20% high loss, 80% low loss (applies to Channel model B) | 20% high loss, 80% low loss |
| Inter-site distance |  | 200 m | 200 m |
| Number of antenna elements per TRxP |  | 128Tx/Rx, (M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 256Tx/Rx = (M,N,P,Mg,Ng) = (4,8,2,2,2), (dH,dV) = (0.5, 0.5)λ. (dg,H,dg,V) = (4.0, 2.0)λ +45°, -45° polarization |
| Number of TXRU per TRxP |  | 32TXRU, (Mp,Np,P,Mg,Ng) = (2,8,2,1,1) | 32TXRU, (Mp,Np,P,Mg,Ng) = (2,8,2,1,1) |
| Number of UE antenna elements |  | DL: 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ UL: 16Tx/Rx, (M,N,P,Mg,Ng) = (1,8,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization | DL:32Tx/Rx = (M,N,P,Mg,Ng) = (2,4 ,2,1,2), (dH,dV) = (0.5, 0.5)λ UL: 16Tx/Rx, (M,N,P,Mg,Ng) = (1,8,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization |
| Number of TXRU per UE |  | DL: 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) UL: 16TXRU, (M,N,P,Mg,Ng) = (1,8,2,1,1) (1-to-1 mapping) | DL: 32TXRU, (Mp,Np,P,Mg,Ng) = (2,8,2,1,1) UL: 16TXRU, (M,N,P,Mg,Ng) = (1,8,2,1,1) (1-to-1 mapping) |
| Device deployment |  | 80% indoor, 20% outdoor (in car) Randomly and uniformly distributed over the area under Macro layer | 80% indoor, 20% outdoor (in car) Randomly and uniformly distributed over the area under Macro layer |
| UE speeds of interest |  | Indoor users: 3km/h Outdoor users (in-car): 30 km/h | Indoor users: 3km/h Outdoor users (in-car): 30 km/h |
| Traffic model |  | Full buffer |  |
| UE density |  | 10 UEs per TRxP |  |
| UE antenna height |  | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |  |
| Electronic tilt |  | 108°in LCS | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = [-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16] Zenith angle θj = [7*pi/12, 2*pi/3, 3*pi/4, 5*pi/6] NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix (2D DFT beam) |
| UT attachment |  | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula) from port 0 The UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. |
| Wrapping around method |  | Geographical distance based wrapping |  |
| Polarized antenna model |  | Model-2 in TR36.873 |  |
| Scheduler |  | DL: MU PF (Max 12 layer) UL: SU PF | DL: SU PF (Max 8 layer) UL: SU PF |
| Subband number |  | 10 | 5 |
| Downlink Overhead | SS/PBCH block | 1 SS/PBCH block per 20ms | 24 SS/PBCH block per 20ms |
|  | PDCCH | 2 OFDM symbols per slot | 0.5 OFDM symbols per slot |
|  | DMRS (Type II) | 4, 8, 12 ports |  |
|  | CSI-RS | 4, 8, 16, 24, 32 ports with periodicity of 5ms |  |
|  | TRS | 4 CSI-RS resource in 2 consecutive slots per 20ms, 50PRB |  |
|  | PT-RS | N.A. | time desity is 4OS, frequency density is 4PRB |
| Uplink Overhead | PUCCH | 14 OS, 4PRB for FDD |  |
|  | DMRS | 6, 12 ports |  |
|  | SRS | 2 OFDM symbols per 5 slots |  |
|  | PT-RS | N.A. | time desity is 4OS, frequency density is 4PRB |
| Receiver |  | Ideal-IRC |  |
| CSI feedback |  | Ideal |  |
| Channel estimation |  | Ideal |  |