3GPP TSG RAN WG1 Meeting #94-bis	R1-1810655

Chengdu, China, 8th – 12th October 2018

Agenda Item:	6.2.1.9

Source: 	Sony

Title:	Considerations on using RSS for measurements

Document for:	Discussion and Decision

# Introduction

A new work item (WI) on Rel-16 MTC enhancements for LTE was approved in RAN Plenary Meeting #80 [1]. The objective is to specify the set of improvements for machine-type communications for BL/CE UEs.

One of the objectives are related to mobility enhancement:

Mobility Enhancement:

- Consider improving the DL RSRP and, if needed, RSRQ measurement accuracy, through use of RSS [RAN1, RAN4, RAN2]

- Specify relaxation of RRM measurements for serving cell for UEs using WUS for at least low mobility UEs [RAN4, RAN2]

An agreement on RSS was made in RAN1 meeting #94 in Göteborg [2]:

Agreement

From RAN1 perspective, it is feasible to use RSS for measuring RSRP for cells at least for IDLE mode mobility

- RAN1 to identify the related parameters in the next RAN1 meeting

In this contribution, we discuss some aspects of using RSS for measurement for MTC, particularly on required parameters in order to facilitate RSS measurements.

# Discussion

Re-synchronization Signal (RSS) was originally introduced in rel.15. The main purpose of RSS was to assist UE to perform synchronization and thus, it reduces system acquisition time. The RSS has at least the following characteristics / properties [3]:

- Resynchronization signal (RSS) configuration information is provided in SIB.

- The periodicity of the first subframe of RSS is configurable to 160, 320, 640 and 1280 ms (including ON/OFF).

- The RSS bandwidth is 2 PRBs.

- The RSS duration is configurable among 8, 16, 32, 40 ms. The RSS duration configuration is signaled in SI.

- RSS TX boosting information is signalled in SI among {0dB, 3dB, 4.8dB, 6dB} relative to CRS.

Assuming the RSS is configured in a cell, a UE can use RSS for re-synchronization purpose, that is densely allocated within a subframe, rather than PSS/SSS for synchronization purpose. Moreover, the UE can also skip in a certain time period on using CRS for synchronization purpose. CRS is allocated sparsely which can take a long duration to estimate time/frequency offset, particularly in extended coverage mode. RSS with a power boosting brings extra benefit for the UE.

In legacy LTE, the UE performs measurement by utilizing the received CRS. Thus, based on similar thought above RSS can be beneficial for measurements, particularly to improve the accuracy and to reduce UE power consumption. The RSS configuration is known by the UE. During IDLE mode, the UE knows when the RSS will be available. In extreme case, RSS can be configured with 1280 ms periodicity. In such configuration, the usage of RSS for measurement can be very limited. In this case, the UE can perform measurement using CRS only. In case the RSS is available when the UE needs to perform measurement, the UE can use either RSS or both CRS and RSS. Thus, we consider it is up to UE implementation when to use RSS for measurement and it is within RAN4 scope to specify the performance requirements when RSS is used for measurements.

Proposal 1: It is up to the UE implementation when to use RSS for measurement.

In order to facilitate RSS for measurement, we consider the UE needs to acquire and utilize RSS related parameters:

- ce-rss-freqPos-config	Freq. position (lowest physical resource block number) of RSS for BL/CE UEs

- ce-rss-timeOffset-config	Time offset (in frames) of RSS for BL/CE UEs

- ce-rss-duration-config	Duration (in subframes) of RSS for BL/CE UEs

- ce-rss-periodicity-config	Periodicity (in ms) of RSS for BL/CE UEs

- ce-rss-powerBoost-config	Power offset of RSS for BL/CE UEs relative to LTE CRS (in dB)

Other parameters may be needed to increase the usability of RSS for measurements.

Proposal 2: RSS for measurement can be facilitated when the UE has acquired at least RSS parameters in rel.15, such as RSS frequency location, RSS time offset, RSS duration, and RSS periodicity. FFS on the need of other parameters.

The neighbor cell measurements using RSS is feasible since RSS also provides PCID. Here, the RSS can be used in addition to CRS, PSS and SSS of the neighbor cell to improve the accuracy of the neighbor cell measurements.  However, to achieve this, the UE needs to know the RSS configuration of its neighbor.  This can be provided in the SIBs of the serving cell.  Providing the neighbor cells’ RSS configuration is also useful for PSM operations since it may be possible for a UE to wake up from PSM in another cell.  In addition to the RSS related parameters listed above, it is also beneficial for the UE to know the SFN difference between the neighbor and serving cells. RAN1 should inform RAN2 on these lists of parameters required to facilitate RSS measurement for the serving and neighbor cells.

Proposal 3: The RSS configurations (i.e. list of parameters) of the neighbor cells are broadcasted in the SIB.

Proposal 4: RAN1 to send LS to RAN2 and RAN4 informing RAN2 the following RSS parameters required to facilitate the use of RSS for UE measurements for serving and neighbor cells:

- ce-rss-freqPos-config: Freq. position (lowest physical resource block number) of RSS for BL/CE UEs

- ce-rss-timeOffset-config: Time offset (in frames) of RSS for BL/CE UEs

- ce-rss-duration-config: Duration (in subframes) of RSS for BL/CE UEs

- ce-rss-periodicity-config: Periodicity (in ms) of RSS for BL/CE UEs

- ce-rss-powerBoost-config: Power offset of RSS for BL/CE UEs relative to LTE CRS (in dB)

- SFN offset between neighbour and serving cells

and inform RAN4 to specify any performance requirements when UE is configured to use RSS for its measurements.

# 3	Conclusions

In this contribution, we propose the following:

Proposal 1: It is up to the UE implementation when to use RSS for measurement.

Proposal 2: RSS for measurement can be facilitated when the UE has acquired at least RSS parameters in rel.15, such as RSS frequency location, RSS time offset, RSS duration, and RSS periodicity. FFS on the need of other parameters.

Proposal 3: The RSS configurations (i.e. list of parameters) of the neighbor cells are broadcasted in the SIB.

Proposal 4: RAN1 to send LS to RAN2 and RAN4 informing RAN2 the following RSS parameters required to facilitate the use of RSS for UE measurements for serving and neighbor cells:

- ce-rss-freqPos-config: Freq. position (lowest physical resource block number) of RSS for BL/CE UEs

- ce-rss-timeOffset-config: Time offset (in frames) of RSS for BL/CE UEs

- ce-rss-duration-config: Duration (in subframes) of RSS for BL/CE UEs

- ce-rss-periodicity-config: Periodicity (in ms) of RSS for BL/CE UEs

- ce-rss-powerBoost-config: Power offset of RSS for BL/CE UEs relative to LTE CRS (in dB)

- SFN offset between neighbour and serving cells

and inform RAN4 to specify any performance requirements when UE is configured to use RSS for its measurements.

# References

- RP181450,” New WID on Rel-16 MTC enhancements for LTE,” Ericsson

- RAN1#94 Chairman notes, Göteborg, August 2018.

- R1-1807971, “Agreement Summary for Rel.15 efeMTC for LTE,” WI rapporteur (Ericsson)