3GPP TSG RAN WG1 Meeting #94bis								    R1-1811202

Chengdu, China, October 8th – 12th , 2018

Agenda Item:	7.2.2.3.1

Source:	InterDigital Inc.

Title:	On design of downlink signals and channels for NR-U

Document for:	Discussion and Decision

# Introduction

In the RAN1 #92bis [1], agreements were reached to study the design changes needed to support different NR channels and signals in NR-U including the PDCCH/PDSCH, PUCCH/PUSCH, and UL reference signals applicable to the operational frequency range.

In the RAN1#93 meeting [2], further agreements were reached on the need to support DL to UL and UL to DL switching within a shared gNB COT and LBT requirements were defined. This is summarized below:

Agreement:

- Single and multiple DL to UL and UL to DL switching within a shared gNB COT is identified to be beneficial and can be supported

  - LBT requirements to support single or multiple switching points, include

    - For gap of less than 16us: no-LBT can be used

      - Restrictions/conditions on when no-LBT option can be used will be further identified, in consideration of fair coexistence.

    - For gap of above 16us but does not exceed 25us: one-shot LBT can be used

      - Restrictions/conditions on when one-shot LBT option can be used will be further identified, in consideration of fair coexistence.

    - For single switching point, for the gap from DL transmission to UL transmission exceeds 25us: one-shot LBT is used

      - Further study needed on how many one-shot LBT attempts is allowed for granted UL transmission

    - FFS: For multiple switching points, for the gap from DL transmission to UL transmission exceeds 25us, one-shot LBT is used. Regulations for this option.

Agreement:

- Scheduling multiple TTIs for PUSCH each using a separate UL grant in the same PDCCH monitoring occasion is identified as beneficial

- Scheduling multiple TTIs for PUSCH using a single UL grant is identified as beneficial and should be supported in NR-U

In the RAN1#94 meeting, a summary of the discussions was captured in [5] and led to agreements on an NR-U-DRS signal [4]. The agreements are summarized below:

Agreement:

- Inclusion of the CSI-RS and RMSI-CORESET(s)+PDSCH(s) (carrying RMSI) associated with SS/PBCH block(s) in addition to the SS/PBCH burst set in one contiguous burst (tentatively referred to as the NR-U DRS) can be beneficial for

  - Meeting OCB requirement

  - Compacting signals in time domain to limit the required number of channel access and for short channel occupancy

  - Support of stand-alone NR-U deployments

  - Support of automatic neighbour relations (ANR) functionality in an NR-U deployment

  - Resolution of PCI confusion in an NR-U deployment

  - Note: The NR-U DRS (it can be called something else in the future) can include signals and channels that are required for cell acquisition etc. and is not limited only to reference signals

- The transmission of additional signals such as OSI and paging within the NR-U DRS is allowed and can be beneficial

- Note: This does not imply that RMSI-CORESET+PDSCH and CSI-RS can only be transmitted as part of the NR-U DRS, and does not imply that these are necessarily part of all NR-U DRS transmissions.

In this contribution, we address some further aspects related to downlink channels in NR-U based on the agreements made.

# NR-U DRS

An NR-U DRS signal has been agreed on that includes an SS/PBCH burst set that may include the CSI-RS and RMSI-CORESET(s)+PDSCH(s) (carrying the RMSI) associated with SS/PBCH block(s) [4].

NR-U-DRS Multiplexing: To multiplex the signals, any of the three patterns that NR R15 supports for multiplexing the SS/PBCH block and the CORESET/RMSI may be used. However, some restrictions may be needed based on the transmission bandwidth, the subcarrier spacing used and the frequency band. NR R15 supports three possible multiplexing patterns. Pattern 1 would allow for transmission of the NR-U-DRS with any sub-carrier spacing (SCS) at any bandwidth. However, Pattern 1 may have a problem with 15 kHz or 30 kHz SCS due to the gap between the SS/PBCH block and CORESET/RMSI. The gap should be set to less than 25 usec to allow for CAT 2 LBT on transmission of the CORESET/RMSI and prevent the need an elaborate CAT 4 LBT procedure.

Patterns 2 and 3 may compact the signals in the time domain, reducing the channel occupancy and assist in meeting the OCB requirement. The shorter channel occupancy may help in ensuring that a limited LBT, e.g. CAT 2 LBT, may be used for the NR-U-DRS signals. However, with a sub-carrier spacing of 60 kHz, a 20 MHz transmission BW may not be sufficiently wide to carry both the SS/PBCH block and the CORESET/RMSI and pattern 1 may have to be used. This may not be an issue for transmission in the FR2 band due to the availability of larger bandwidths.

Proposal 1: Multiplexing Pattern 1 should consider the gap between the SS/PBCH block and the CORESET/RMSI to take into account the need for LBT.

Proposal 2: Multiplexing Pattern 2/3 may be considered for 30kHz SCS and below in the case of 20MHz minimum bandwidth for NR-U.

Structure based on LBT: Due to LBT, the transmission of NR-U DRS may fail. As such, the structure of the NR-U-DRS and the way it is transmitted should factor in the possibility of non-transmission due to LBT failure. In LTE-LAA, a DRS consisting of synchronization and reference signals can be transmitted within a periodically occurring time window called the DRS measurement timing configuration (DMTC) occasion which has a duration of 6 msec and a configurable period of 40/80/160 msec. The transmission of DRS is also subject to LBT. In NR, the concept of an NR-U-DRS window may also need to be introduced, but may need to be modified to account for the addition of multiple blocks. This modification may be by one of the following:

- Alternative 1: Time Shifting in which every occasion where LBT was successful may be used as offset for all the contiguous SSB/DRS following it.

- Alternative 2: Cyclic Extension in which we may truncate the start of the SS/PBCH block burst set upon LBT failure and cyclically wrap the dropped indices around to the end of the burst set transmission.

- Alternative 3: Creating SSB Tx opportunities with multiple SSB indices in which a mechanism may allow network to flexibly position SSB index and indicate the timing information.

A more detailed discussion of the three alternatives described above may be found in [6].

On the configurable period supported by the NR-U-DRS window, as NR-U is mandated to support stand-alone deployments, a minimum SS/PBCH periodicity of 40 msec may not be sufficient given that there is no assisting licenced carrier. As such higher periodicities should be allowed. Note that the NR-U-DRS window should support transmission of any additional channels, including paging, with the flexibility of adding or removing any signal to allow DRS window technique to be used for stand-alone signals if periodicities are different.

Proposal 3: The design of the NR-U-DRS window should take into consideration the multiple SS/PBCH blocks transmitted and the fact that NR-U may be a standalone system.

# DL RS

DL RS may be either the Demodulation Reference Symbol (DMRS), the Channel State Information Reference Symbol (CSI-RS) or the Phase Tracking Reference Symbol (PTRS).

DMRS and PTRS: The DMRS and the PTRS are embedded in the DL data allocation and are only transmitted when a downlink burst is transmitted. As this will most likely be on a successful LBT process there is no effect of unlicensed access on either reference signal.

CSI-RS: NR Release 15 supports aperiodic CSI-RS, semi-persistent CSI-RS and periodic CSI-RS and may be transmitted independently of a DL data transmission.

For aperiodic CSI-RS, in the case that there is no need for LBT (e.g. < 16 us), the existing NR aperiodic CSI-RS mechanism can be used in NR-U. However, in the case that the value of the aperiodic triggering offset in the DCI requires that there is a need for LBT before the CSI-RS is sent, NR-U should consider enabling multiple CSI-RS transmission opportunities for the transmission of the aperiodic CSI-RS.

This may be implemented by allocating separate CSI-RS resources for each transmission opportunity or creating a CSI-RS resource window within which the UE may monitor for the transmission of the CSI-RS as illustrated in Figure 2.

Figure 2: Aperiodic CSI-RS for NR-U with LBT

For periodic and semi-persistent CSI-RS, the CSI-RS may not be transmitted periodically because of LBT failure. As such, in LTE LAA, only CSI-RSs that overlap with a transmit burst are transmitted [3]. In addition, LTE LAA allows a CSI-RS to be configured with the Discovery Reference Signal (DRS) in which synchronization signals and reference signals are transmitted in a periodically occurring time window that has a fixed duration and a configurable period. The DRS is transmitted within the window and is subject to LBT [4].

In the case of multiple switching points within a shared gNB COT, the probability that the CSI-RS is not transmitted is high. As such, to allow for more flexibility in scheduling, NR-U should consider a mode that configures a CSI-RS resource window for periodic or semi-persistent CSI-RS resources. Note that a CSI-Report window may be configured independently of the CSI-RS resource for periodic or semi-persistent CSI-reporting. This is illustrated in Figure 3.

Figure 3: Periodic/SP CSI-RS and Periodic/SP CSI-reporting

Proposal 4: For aperiodic CSI-RS, NR-U should consider enabling multiple CSI-RS resource transmission opportunities in the case that LBT is needed before the CSI-RS transmission.

Proposal 5: For periodic and semi-persistent CSI-RS, NR-U should consider configuring a CSI-RS resource window that is transmitted in a periodically occurring time window that has a fixed duration, a configurable period and is subject to LBT. The periodic or SPS CSI-RS can be sent as part of an NR-U-DRS window.

# DL Control (PDCCH) and Data Channels (PDSCH)

Rel-15 specifications modifications w.r.t. CORESET configuration and DL control structures:  In LAA, the start of the transmission following a successful LBT is limited to the beginning of the next slot (i.e., Symbol 0 or 7 within a subframe). This approach often results in inefficient resource utilization in scenarios that the LBT procedure is successfully completed at the middle of the slot. In NR-U, to maximize the resource usage efficiency, the transmission should be allowed at any time following a successful LBT procedure. This may require modifications w.r.t. the CORESET configuration and DL control structures. For example, the UE can be scheduled with a mini-slot granularity during the random transmission stage and later be scheduled at the slot-level granularity during the deterministic transmission stage. An example of NR-U frame structure with LBT procedure and the associated PDCCH monitoring is shown in Figure 1.

Figure 4: An example of NR-U frame structure with LBT procedure

Proposal 6: Consider PDCCH monitoring, where monitoring on a mini-slot granularity is used at the beginning of the COT and slot-based monitoring is used at the beginning of the first full slot of the COT

Enhancements to PDSCH mapping type B: In NR-U, there may be some uncertainty in the transmission since there are multiple LBT requirements based on the gaps between the switching points in a shared gNB COT.

For single and multiple DL-to-UL or UL-to-DL switching within a shared gNB COT, and with a gap of less than 16 us, no-LBT can be used and there is no need for any changes to the NR Release 15 DL control and DL data channels for use in NR-U.

However, when the gap exceeds 16 us, a form of LBT is needed, e.g., one-shot LBT. In this case, there is a possibility that there may be an LBT failure and no guarantee of availabile resources, and scheduling cannot be done in a deterministic manner. As such, multiple LBT attempts may be made and the UE should expect to receive or transmit the signal or channel in multiple transmission opportunities.

In current agreements, for uplink transmission, scheduling multiple TTIs for PUSCH transmission using a single UL grant or separate UL grants in the same PDCCH monitoring occasion have been identified as beneficial.

Similarly, for DL transmissions, in the case of multiple switching points within the shared gNB COT, where the gap between the first and second DL transmission exceeds 16 us the gNB may have to perfom an LBT before transmission. The PDCCH, combined with higher layer signaling, may configure multiple transmission opportunities for data or signal transmission. If transmission in one resource is successful, the additional transmission opportunities can be reallocated by the gNB and ignored by the UE. If the transmission in one resource fails, the additional transmission opportunities may be used by the gNB without the need for additional signaling and the UE is going to monitor PDSCH on those additional resources. This may be important for a DL data channel in which a DCI with slot aggregation is used to schedule data transmission over multiple downlink slots that span transmission over single or multiple switching points within a shared gNB COT as illustrated in Figure 5 as it reduces the need for additional signaling to allocate resources in the middle of the aggregation.

Figure 5: Multiple Transmission Opportunities for DL data channel

Proposal 7: Scheduling multiple transmission opportunities for PDSCH over multiple TTIs each using a separate DL grant in the same PDCCH monitoring occasion is beneficial and should be supported in NR-U.

Proposal 8: Scheduling multiple TTIs for PDSCH transmission opportunities using a single DL grant is beneficial and should be supported in NR-U.

# Summary

This contribution discussed some details regarding the DL channels for NR-Unlicensed. In the following, the discussed proposals are listed:

Proposal 1: Multiplexing Pattern 1 should consider the gap between the SS/PBCH block and the CORESET/RMSI to take into account the need for LBT.

Proposal 2: Multiplexing Pattern 2/3 may be considered for 30kHz SCS and below in the case of 20MHz minimum bandwidth for NR-U.

Proposal 3: The design of the NR-U-DRS window should take into consideration the multiple SS/PBCH blocks transmitted and the fact that NR-U may be a standalone system.

Proposal 4: For aperiodic CSI-RS, NR-U should consider enabling multiple CSI-RS resource transmission opportunities in the case that LBT is needed before the CSI-RS transmission.

Proposal 5: For periodic and semi-persistent CSI-RS, NR-U should consider configuring a CSI-RS resource window that is transmitted in a periodically occurring time window that has a fixed duration, a configurable period and is subject to LBT. The periodic or SPS CSI-RS can be sent as part of an NR-U-DRS window.

Proposal 6: Consider PDCCH monitoring, where monitoring on a mini-slot granularity is used at the beginning of the COT and slot-based monitoring is used at the beginning of the first full slot of the COT

Proposal 7: Scheduling multiple transmission opportunities for PDSCH over multiple TTIs each using a separate DL grant in the same PDCCH monitoring occasion is beneficial and should be supported in NR-U.

Proposal 8: Scheduling multiple TTIs for PDSCH transmission opportunities using a single DL grant is beneficial and should be supported in NR-U.

# References

RAN1#92bis Chairman’s Notes, Sanya, China, April 2018.

RAN1#93 Chairman’s Notes, Busan, Korea, May 2018.

3GPP TS 36.213 V14.5.0, E UTRA Physical layer procedures

RAN1#94 Chairman’s Notes, Gothenburg, Sweden, Aug 2018.

R1-1809822, Feature lead summary for NR-U DL Signals and Channels, Motorola Mobility, Lenovo

R1-1811205 Initial Access and Mobility Procedures in NR-U, InterDigital