3GPP TSG RAN WG1 Meeting #95 R1-18xxxxx Spokane, USA, November 12th – 16th, 2018 CR-Form-v11.2. CHANGE REQUEST. 38.212; CR; DRAFT; rev; -; Current version:; 15.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on UCI on PUSCH transmission. Source to WG:; Fujitsu. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2018-11-2. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In TS 38.212 subclause 6.2.7 when HARQ-ACK is transmitted on PUSCH, if the number of HARQ-ACK bits is 2 or less, then resources are reserved on PUSCH for 2 bits. However, accorduing to the current description this same resources are reserved for the case of 0 bits HARQ-ACK. This implies that the resources would always be reserved, even if there are no HARQ-ACK bits to be transmitted, including cases where it is not possible for there to be any HARQ-ACK bits. But this may not be the common understanding in RAN1 or the original intention, so clarification is needed. The solution proposed here is to remove the case of “0 bits” HARQ-ACK and rely on specification TS 38.213 to correctly determine the number of HARQ-ACK bits to be transmitted (even some of them have dummy or default values), and hence the resources which would need to be reserved on PUSCH. If this proposal does not lead to the required behaviour, then an alternative would be to delete the case of 0 bits, but add the clarification “or there are no HARQ-ACK bits to be multiplexed on PUSCH”.. Summary of change:; Remove the case of 0 HARQ-ACK information bits in determining resources reserved on PUSCH for HARQ-ACK. Consequences if not approved:; Ambiguous specifications. Clauses affected:; 6.2.7. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

### 6.2.7	Data and control multiplexing

< …..Unchanged parts omitted……>

The multiplexed data and control coded bit sequence  is obtained according to the following:

Step 1:

Set  for ;

Set  for ;

Set  for ;

Set  for ;

if the number of HARQ-ACK information bits to be transmitted on PUSCH is 1 or 2 bits

the number of reserved resource elements for potential HARQ-ACK transmission is calculated according to Subclause 6.3.2.4.1.1, by setting ;

denote  as the number of coded bits for potential HARQ-ACK transmission using the reserved resource elements;

if frequency hopping is configured for the PUSCH, let  and ;

if frequency hopping is not configured for the PUSCH, let ;

denote  as the set of reserved resource elements for potential HARQ-ACK transmission, in OFDM symbol , for ;

Set ;

Set ;

for ;

for  to

;

while

if

if

;

;

end if

if

;

;

end if

for  to

;

end for

end if

;

end while

end for

else

for ;

end if

Denote  as the number of elements in .