3GPP TSG RAN WG1 Meeting #95 R1-18xxxxx Spokane, USA, November 12th – 16th, 2018 CR-Form-v11.2. CHANGE REQUEST. 38.213; CR; DRAFT; rev; -; Current version:; 15.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on Maintenance of HARQ-ACK transmission. Source to WG:; Fujitsu. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2018-11-2. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In deternining the semi-static HARQ-ACK codebook the UE determines a set of occasions for candidate PDSCH reception which also includes occasions for SPS PDSCH release. In several places in 9.1.2 and 9.1.2.1 the « occasions for candidate PDSCH reception » are mentioned but inclusion of SPS PDSCH release is missing in some cases. This should be corrected for consistency and to avoid possible misunderstanding. In 9.1.2, 9.1.2.1 and 9.1.2.2 both « occasions for candidate PDSCH receptions » and « occasions for candidate PDSCH reception » appear. The latter formulation is preferable, since the former is not good use of English. In 9.1.2 UE behaviour has been recently clarified for the following special case of HARQ-ACK on PUCCH only for:- - a SPS PDSCH release indicated by DCI format 1_0 with counter DAI field value of 1 on the PCell, or - a PDSCH reception scheduled by DCI format 1_0 with counter DAI field value of 1 on the PCell, or - SPS PDSCH reception(s) This change is not properly reflected in 9.1.2.2 for HARQ-ACK on PUSCH. Summary of change:; - Consistent wording added for « occasions for candidate PDSCH reception and SPS PDSCH release » - Consistent wording introduced for « occasions for candidate PDSCH reception » - Description in 9.1.2.2 aligned with recent updates to 9.1.2 - Some editorial modifications. Consequences if not approved:; Incomplete and incorrect specifications. Clauses affected:; 9.1.2, 9.1.2.1, 9.1.2.2. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

### 9.1.2	Type-1 HARQ-ACK codebook determination

This subclause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static.

A UE reports HARQ-ACK information for a corresponding PDSCH reception or SPS PDSCH release only in a HARQ-ACK codebook that the UE transmits in a slot indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format 1_0 or DCI format 1_1. The UE reports NACK value(s) for HARQ-ACK information bit(s) in a HARQ-ACK codebook that the UE transmits in a slot not indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format 1_0 or DCI format 1_1.

If the UE is provided with pdsch-AggregationFactor,  is the value of pdsch-AggregationFactor; otherwise, . The UE reports HARQ-ACK information for a PDSCH reception from slot  to slot  only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot , where  is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format. If the UE reports HARQ-ACK information for the PDSCH reception in a slot other than slot , the UE sets a value for each corresponding HARQ-ACK information bit to NACK.

If a UE reports HARQ-ACK information in a PUCCH only for

-	a SPS PDSCH release indicated by DCI format 1_0 with counter DAI field value of 1 on the PCell, or

-	a PDSCH reception scheduled by DCI format 1_0 with counter DAI field value of 1 on the PCell, or

-	SPS PDSCH reception(s)

within the occasions for candidate PDSCH reception or SPS PDSCH release as determined in Subclause 9.1.2.1, the UE determines a HARQ-ACK codebook only for the SPS PDSCH release or only for the PDSCH reception or only for the SPS PDSCH reception(s) according to the corresponding  occasion(s) on respective serving cell(s); otherwise, the procedures in Subclause 9.1.2.1 and Subclause 9.1.2.2 for a HARQ-ACK codebook determination apply.

#### 9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a serving cell , an active DL BWP, and an active UL BWP, as described in Subclause 12, the UE determines a set of  occasions for candidate PDSCH reception or SPS PDSCH release for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot . If serving cell  is deactivated, the UE uses as the active DL BWP for determining the set of  occasions for candidate PDSCH reception a DL BWP provided by firstActiveDownlinkBWP. The determination is based:

a)	on a set of slot timing values  associated with the active UL BWP

a)	If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for DCI format 1_1 on serving cell ,  is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} for DCI format 1_0

b)	If the UE is configured to monitor PDCCH for DCI format 1_1 for serving cell ,  is provided by dl-DataToUL-ACK for DCI format 1_1

b)	on a set of row indexes  of a table that is provided either by a first set of row indexes of a table that is provided by PDSCH-TimeDomainResourceAllocationList in PDSCH-ConfigCommon or by Default PDSCH time domain resource allocation A [6, TS 38.214], or by the union of the first set of row indexes and a second set of row indexes, if provided by PDSCH-TimeDomainResourceAllocationList in PDSCH-Config, associated with the active DL BWP and defining respective sets of slot offsets , start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214]

c)  on the ratio  between the downlink SCS configuration  and the uplink SCS configuration  provided by subcarrierSpacing in BWP-Downlink and BWP-Uplink for the active DL BWP and the active UL BWP, respectively

d)	if provided, on TDD-UL-DL-ConfigurationCommon and TDD-UL-DL-ConfigDedicated as described in Subclause 11.1.

For the set of slot timing values , the UE determines a set of  occasions for candidate PDSCH reception or SPS PDSCH release according to the following pseudo-code. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to a SPS PDSCH release is same as for a corresponding SPS PDSCH reception in case the UE did not detect the SPS PDSCH release.

Set  - index of occasion for candidate PDSCH reception or SPS PDSCH release

Set

Set

Set  to the cardinality of set

Set k =0 – index of slot timing values , in descending order of the slot timing values, in set  for serving cell

while

if

Set – index of a DL slot within an UL slot

while

Set  to the set of rows

Set  to the cardinality of

Set  – index of row in set

if slot  is same as or after a slot for an active DL BWP change on serving cell  or an active UL BWP change on the PCell and slot  is before the slot for the active DL BWP change on serving cell  or the active UL BWP change on the PCell

continue;

else

while

if the UE is provided TDD-UL-DL-ConfigurationCommon  or TDD-UL-DL-ConfigDedicated and, for each slot from slot  to slot , at least one symbol of the PDSCH time resource derived by row  is configured as UL where  is the k-th slot timing value in set ,

;

end if

;

end while

if the UE does not indicate a capability to receive more than one unicast PDSCH per slot and ,

The UE does not expect to receive SPS PDSCH release and unicast PDSCH in a same slot;

else

Set  to the cardinality of

Set  to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of

while

Set

while

if  for start OFDM symbol index  for row

; - index of occasion for candidate PDSCH reception or SPS PDSCH release associated with row

;

end if

;

end while

;

Set  to the smallest last OFDM symbol index among all rows of ;

end while

end if

end if

;

end while

end if

;

end while

For occasions of candidate PDSCH reception corresponding to rows of associated with a same value of , where , the UE does not expect to receive more than one PDSCH in a same slot.

If a UE receives a SPS PDSCH, or a SPS PDSCH release, or a PDSCH that is scheduled by a DCI format 1_0 and if

-	the UE is configured with one serving cell, and

-	, and

-	higher layer parameter PDSCH-CodeBlockGroupTransmission is not provided to the UE

the UE generates HARQ-ACK information only for the transport block in the PDSCH or only for the SPS PDSCH release.

If a UE receives a SPS PDSCH, or a SPS PDSCH release, or a PDSCH that is scheduled by a DCI format 1_0 and if

-	the UE is configured with more than one serving cell, or

-	, and

-	higher layer parameter PDSCH-CodeBlockGroupTransmission is provided to the UE

the UE repeats  times the HARQ-ACK information for the transport block in the PDSCH or for the SPS PDSCH release.

A UE does not expect to detect a DCI format switching a DL BWP within  symbols prior to a first symbol of a PUCCH transmission where the UE multiplexes HARQ-ACK information, where  is defined in Subclause 9.2.3.

If a UE is provided with dl-DataToUL-ACK, the UE does not expect to be indicated by DCI format 1_0 a slot timing value for transmission of HARQ-ACK information that does not belong to the intersection of the set of slot timing values {1, 2, 3, 4, 5, 6, 7, 8} and the set of slot timing values provided by dl-DataToUL-ACK for the active DL BWP of the corresponding serving cell.

If an occasion for a candidate PDSCH reception can be in response to a PDCCH with DCI format 1_1 and if maxNrofCodeWordsScheduledByDCI indicates reception of two transport blocks, when the UE receives a PDSCH with one transport block, the HARQ-ACK information is associated with the first transport block and the UE generates a NACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is not provided and generates HARQ-ACK information with value of ACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is provided.

A UE determines  HARQ-ACK information bits, for a total number of  HARQ-ACK information bits, of a HARQ-ACK codebook for transmission in a PUCCH according to the following pseudo-code. In the following pseudo-code, if the UE does not receive a transport block or a CBG, due to the UE not detecting a corresponding DCI format 1_0 or DCI format 1_1, the UE generates a NACK value for the transport block or the CBG. The cardinality of the set  defines a total number  of occasions for PDSCH reception or SPS PDSCH release for serving cell  corresponding to the HARQ-ACK information bits.

Set  – serving cell index: a lower index corresponds toa lower RRC index for the corresponding cell

Set - HARQ-ACK information bit index

Set  to the number of serving cells configured by higher layers for the UE

while

Set  – index of occasion for candidate PDSCH reception or SPS PDSCH release

while

if harq-ACK-SpatialBundlingPUCCH is not provided, PDSCH-CodeBlockGroupTransmission is not provided, and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell ,

= HARQ-ACK information bit corresponding to a first transport block of this cell;

;

= HARQ-ACK information bit corresponding to a second transport block of this cell;

;

elseif harq-ACK-SpatialBundlingPUCCH is provided, and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell ,

= binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks of this cell - if the UE receives one transport block, the UE assumes ACK for the second transport block;

;

elseif PDSCH-CodeBlockGroupTransmission is provided, and  CBGs are indicated by maxCodeBlockGroupsPerTransportBlock for serving cell ,

Set - CBG index

while

= HARQ-ACK information bit corresponding to CBG  of the first transport block;

if the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell

= HARQ-ACK information bit corresponding to CBG  of the second transport block;

end if

;

end while

, where  is the value of maxNrofCodeWordsScheduledByDCI for the active DL BWP of serving cell ;

else

= HARQ-ACK information bit of serving cell ;

;

end if

;

end while

;

end while

If , the UE determines a number of HARQ-ACK information bits  for obtaining a transmission power for a PUCCH, as described in Subclause 7.2.1, as  where

-	 is the number of transport blocks the UE receives in PDSCH reception occasion  for serving cell  if harq-ACK-SpatialBundlingPUCCH and PDSCH-CodeBlockGroupTransmission are not provided, or the number of transport blocks the UE receives in PDSCH reception occasion  for serving cell  if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format 1_0, or the number of PDSCH receptions if harq-ACK-SpatialBundlingPUCCH is provided or SPS PDSCH release in PDSCH reception occasion  for serving cell  and the UE reports corresponding HARQ-ACK information in the PUCCH.

-	 is the number of CBGs the UE receives in a PDSCH reception occasion  for serving cell  if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format 1_1 and the UE reports corresponding HARQ-ACK information in the PUCCH.

#### 9.1.2.2	Type-1 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by DCI format 0_0, then

-	if the UE has not received any PDSCH or SPS PDSCH release that the UE transmits corresponding HARQ-ACK information in the PUSCH, based on a value of a respective PDSCH-to-HARQ feedback timing field in a DCI format scheduling the PDSCH reception or the SPS PDSCH release or on the value of the dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, in any of the  occasions for candidate PDSCH reception by DCI format 1_0 or DCI format 1_1 or SPS PDSCH on any serving cell , as described in Subclause 9.1.2.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission;

-	else the UE generates the HARQ-ACK codebook as described in Subclause 9.1.2.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH, unless the UE receives only a SPS PDSCH release indicated by DCI format 1_0 with counter DAI field value of 1, or only SPS PDSCH reception(s), or only a PDSCH that is scheduled by DCI format 1_0 with a counter DAI field value of 1 on the PCell in the  occasions for candidate PDSCH reception in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the SPS PDSCH reception(s) or only for the PDSCH reception as described in Subclause 9.1.2.

A UE sets to NACK value in the HARQ-ACK codebook any HARQ-ACK information corresponding to PDSCH reception or SPS PDSCH release that the UE detects in a PDCCH monitoring occasion that starts after a PDCCH monitoring occasion where the UE detects a DCI format 0_0 or a DCI format 0_1 scheduling the PUSCH transmission.

A UE does not expect to detect a DCI format switching a DL BWP within  symbols prior to a first symbol of a PUSCH transmission where the UE multiplexes HARQ-ACK information, where  is defined in [6, TS 38.214].

If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by DCI format 0_1, the UE generates the HARQ-ACK codebook as described in Subclause 9.1.2.1 when a value of the DAI field in DCI format 0_1 is  except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH. The UE does not generate a HARQ-ACK codebook for multiplexing in the PUSCH transmission when  unless the UE receives only a SPS PDSCH release indicated by DCI format 1_0 with counter DAI field value of 1, or only SPS PDSCH reception(s), or only a PDSCH that is scheduled by DCI format 1_0 with a counter DAI field value of 1 on the PCell in the  occasions for candidate PDSCH reception in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the SPS PDSCH reception(s) or only for the PDSCH reception as described in Subclause 9.1.2.