3GPP TSG RAN WG1 #99 Meeting	    						R1-1913337

Reno, USA, November 18th – 22nd, 2019

Agenda item:	7.2.4.1

Source: 	Samsung

Title: 	Feature lead summary#2 for agenda item 7.2.4.1 Physical layer structure for sidelink

Document for:	Discussion and Decision

Introduction

This document is for the summary and discussion based on the contributions submitted to RAN1#98bis AI 7.2.4.1.

Considering priority and the number of discussion paper, the following issues will be discussed first. Except the following topics, CSI-RS design, PSSCH DMRS design, and PSFCH design will be further discussed in RAN1#99.

Agreements:

- Regarding the previous agreement that RE mapping of the 2nd stage SCI, frequency-first mapping within the PSSCH is used:

  - The REs for the 2nd SCI are not interlaced with (localized in) PSSCH data RE.

  - The 2nd stage SCI is mapped in frequency first with RB granularity, and then mapped in the next symbol(s).

    - The mapping is done by mapping to all RBs in the all sub-channels for the scheduled PSSCH in one symbol first before moving on the next symbol, except possibly the following:

      - FFS whether to allow FDM between the 2nd stage SCI and PSCCH or not

      - FFS whether to allow FDM between the 2nd stage SCI and PSSCH DM-RS REs

- For modulation order of the 2nd stage SCI, QPSK is used.

- FFS the case of standalone SCI (if supported)

- Proposal (offline consensus)

  - For RE mapping of the 2nd stage SCI, FDM between 2nd stage SCI and PSSCH DMRS in the same symbol is allowed.

Issue 7-A.

- Proposal (offline consensus)

  - Repetition of PSFCH format 0 (one-symbol PSFCH format agreed in RAN1#97) to two consecutive symbols is used.

    - This implies that, two consecutive symbols are always used for transmission of PSFCH format 0,

    - Note: The first symbol can be used for AGC training.

- Proposal (not consensus)

  - Down-select one of the following alternatives.

    - Alt 1.

      - One symbol is used for gap right after PSSCH transmission.

      - One symbol is used for gap right after PSFCH transmission.

    - Alt 2.

      - For 15/30 kHz SCS,

        - Half symbol is used for gap right after PSSCH transmission.

        - Half symbol is used for gap right after PSFCH transmission.

        - FFS on the starting timing of PSFCH

      - For 60/120 kHz SCS,

        - One symbol is used for gap right after PSSCH transmission.

        - One symbol is used for gap right after PSFCH transmission.

    - FFS more gap symbols are defined in order to handle timing advance for share carrier.

- Proposal (offline consensus)

  - (working assumption) PSSCH DMRS can be FDMed with PSCCH when the number of DMRS symbols is larger than 1.

    - In a symbol containing PSSCH DMRS, every RB used for PSSCH contains PSSCH DMRS.

  - FFS whether one symbol DMRS is supported

Proposal (offline consensus)

  - L1 source ID is carried in 2nd SCI.

Proposal (Not offline consensus)

For L1 destination ID,

  - Alt 1. L1 destination ID is carried in 1st SCI.

    - Objected by Futurewei, QC, LGE, Nokia, NSB, Huawei, HiSi, NEC, OPPO

    - Alt 1’. Part of L1 destination ID (e.g. 4 bits) is carried in 1st SCI.

    - Objected by Futurewei, QC, LGE, Huawei, HiSi, NEC, OPPO, Levono, Mot, SS

  - Alt 2. L1 destination ID is not carried in 1st SCI.

    - Objected by CATT, vivo, IDC, Levono, Mot, Kyocera, Intel, MediaTek, SS, Fraunhofer, ATT, Panasonic

    - Alt 2’. L1 destination ID is not carried in 1st SCI, and reserve bits are in 1st SCI

      - Objected by CATT, vivo, IDC, Levono, Mot, Kyocera, Intel, MediaTek, SS, Fraunhofer, ATT

    - Alt 2-1. L1 destination ID is carried in 2nd SCI.

    - Alt 2-2. L1 destination ID is scrambled in CRC of 2nd SCI.

Coverage-BLER performance,

power consumption-decoding burden,

Forward compatibility

PSSCH length

- Proposal

  - For Rel-16 V2X, it is supported that PSSCH has equal to or larger than X symbols except AGC symbol.

    - X = 7

DMRS patterns

- Proposal

  - For PSSCH DMRS positions, the following table is supported, where l0 indicates the first symbol of the last PSCCH symbol except AGC symbol (i.e., l0 indicates the 2nd SL symbol in a slot).

| PSSCH length (except AGC symbol) | dmrs-AdditionalPosition |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 2 | - | - | - | - |
| 3 | l0 | l0 | l0 | l0 |
| 4 | l0 | l0 | l0 | l0 |
| 5 | l0 | l0 | l0 | l0 |
| 6 | l0 | l0 | l0 | l0 |
| 7 | 4 | 4 | 4 | l0 |
| 8 | l0 | l0, 8 | l0, 8 | l0, 8 |
| 9 | l0 | l0, 8 | l0, 8 | l0, 8 |
| 10 | l0 | l0, 10 | l0, 8 | l0, 8 |
| 11 | l0 | l0, 10 | l0, 7, 10 | l0, 7, 10 |
| 12 | l0 | l0, 10 | l0, 7, 10 | l0, 6, 9, 12 |
| 13 | l0 | l0, 12 | l0, 8, 12 | l0, 6, 9, 12 |
| 14 | l0 | l0, 12 | l0, 8, 12 | l0, 6, 9, 12 |

DMRS

Issue 6-C

- Proposal

  - NR PUSCH DMRS for CP-OFDM sequence is the baseline for PSSCH DMRS sequence at least with the following modification.

    - n_SCID is

      - Alt 1. fixed as 0.

      - Alt 2. determined by CRC of the corresponding 1st SCI.

      - Alt 3. determined by destination ID

    - n_ID^{N_SCID} is

      - Alt 1. (pre-)configured.

      - Alt 2. determined by CRC of the corresponding 1st SCI.

      - Alt 3. determined by destination ID

  - For PSSCH DMRS sequence, Pseudo-random sequence in NR Uu is reused.

CSI-RS

Issue 8-A

- Proposal

  - Resource mapping patterns of Rel-15 NR Uu CSI-RS with 1 or 2 antenna port(s) with density 1 are used for SL CSI-RS.

    - This means that Rows 2 and 3 of Table 7.4.1.5.3-1 with density 1 are used for SL CSI-RS.

- Proposal

  - NR CSI-RS sequence is the baseline for SL CSI-RS sequence at least with the following modification.

    - n_ID is determined by

      - a (pre-)configured value.

      - a value configured by PC5 RRC

      - CRC of the 1st SCI

Issue 4-B

PSCCH DMRS

- Proposal

  - NR PDCCH DMRS sequence is the baseline for PSCCH DMRS sequence at least with the following modification.

    - n_ID is determined by

      - a (pre-)configured value.

      - a value configured by PC5 RRC

      - CRC of the 1st SCI

PTRS

Issue 8-B

- Proposal

  - RB offset is down-selected in RAN1#99.

      - Alt 1. Fixed as 0.

      - Alt 2. Determined based on L1 destination ID.

      - Alt 3. Determined based on L1 source ID.

      - Alt 4. Determined based on CRC of PSCCH.

  - Association with one or two of the DMRS port(s) is used, down select in RAN1#99 among:

      - Alt 1. The number of PT-RS antenna ports is the same as the number of DMRS antenna ports.

      - Alt 2. The number of PT-RS antenna ports and the association between DM-RS port and PT-RS port are (pre)configured.

      - Alt 3. One PT-RS port is supported and the PT-RS port is associated with the lowest DMRS port index.

Issue 4-A

- Proposal

  - A single number of PRBs for mapping PSCCH is (pre-)configured per resource pool.

  - Candidate numbers of PRBs for PSCCH are

    - For 2-symbol PSCCH,{15, …}

    - For 3-symbol PSCCH,{10, …}

- Proposal

  - Candidates of PSSCH DMRS time-domain patterns in a slot are specified regardless of subcarrier spacing.

    - (working assumption) DMRS patterns for PUSCH mapping type B are baseline for PSSCH DMRS patterns when two or more symbols are used for PSSCH DMRS.

      - l is defined relative to the start of the PSCCH scheduling PSSCH resources except AGC symbol.

      - FFS: when only one symbol DMRS is used.

- Proposal (to be discussed) whether to allow FDM between the 2nd stage SCI and PSCCH or not

  - For RE mapping of the 2nd stage SCI, FDM between 2nd stage SCI and PSCCH in the same symbol is [not] allowed.

    - Alt 1. Allowed:

    - Alt 2. Not allowed:

- Proposal

  - For Mode 1, Tx UE chooses PSSCH DMRS time-domain pattern, if multiple patterns can be used.

  - RAN1 further discusses whether the available PSCCH DMRS pattern(s) is/are based on SCS, UE speed, MCS, traffic type, or feedback information.

Issue 4-C.

- Proposal for 1st SCI

  - 1st SCI includes at least

    - Priority (QoS value),

    - PSSCH resource assignment (frequency/time resource for PSSCH),

    - PSSCH DMRS pattern in time-domain,

    - Resource reservation,

    - 2nd SCI format (if multiple formats are supported),

    - 2nd SCI code rate (or aggregation level)

    - MCS

    - Destination ID,

    - FFS on antenna ports, TX power indication

Issue 9.

- Proposal

For AGC purposes, 1st SL symbol in a slot is a copy of

- Alt 1. 2nd symbol of the slot

- Alt 2. PSSCH DMRS symbol

Issue 5-A. 2nd SCI contents

  - Source ID

    - 1st SCI

    - 2nd SCI

    - Scrambled in CRC of 2nd SCI:

  - Destination ID

    - 1st SCI

    - 2nd SCI

    - Scrambled in CRC of 2nd SCI

Issue 6-H.

  - Whether to support subband precoding or not

Issue 6

  - Indication of MCS table

  - TBS determination

  - Whether to apply LBRM

Scrambling sequence determination

- PSCCH scrambling

- PSSCH scrambling

- Sequence PSFCH format 0

Issues for discussion

- Issue 1. Resource Pool/BW

- Time-domain resource pool configuration

Agreements (RAN1#98bis): - A slot is the time-domain granularity for resource pool configuration. - To down-select: - Alt 1. Slots for a resource pool is (pre-)configured with bitmap, which is applied with periodicity - Alt 2. Slots for a resource pool is (pre-)configured, where the slots are applied with periodicity. - FFS: signaling details - FFS: how to apply the above bitmap signaling, e.g., to all slots or only to a set of slots - FFS: symbols for sidelink in the slot, how to indicate for the case when not all symbols are for SL

- Resource pool configuration

  - Alt 1: [Nokia, NSB], [vivo], [CATT], [Intel], [TCL], [Fraunhofer], [Sony], [ZTE, Sanechips], [Spreadtrum], [Ericsson], [NEC], [InterDigital], [Panasonic], [Apple], [Qualcomm]

  - Alt 2: [Huawei, HiSilicon]

- Observation: This issue was discussed in RAN1#98bis. The companies have the same view as that meeting.

- Proposal for agreement

  - Slots for a resource pool is (pre-)configured with bitmap, which is applied with periodicity.

- Subchannel size

Agreements (RAN1#98bis): - Support {10, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. - FFS other values (e.g., 4, 5, 6, etc.) - One value of the above set is (pre)configured for the sub-channel size for the resource pool. - Size of PSCCH: X - X N, where N is the number of PRBs of the subchannel - X is (pre)-configurable with values FFS, X Note: Huawei considers the above agreements are a mistake

- Additional subchannel size

  - 5 PRBs: [Huawei, HiSilicon], [LGE]

  - 6 PRBs: [Intel]

- Proposal for agreement

  - RAN1 further discuss whether/which additional subchannel size(s) is/are used.

- Simultaneous transmission in SL BWP and UL BWP

There are only few papers discussing on whether simultaneous transmission in SL BWP and UL BW is allowed or not. RAN1 needs discuss further this issue.

- Proposal

  - RAN1 further discuss on whether simultaneous transmission in SL BWP and UL BW is allowed.

- Other topics

There are papers discussing others related to resource pool and SL BWP as below. RAN1 needs to discuss the following topics.

  - Supported SL bandwidth in Rel-16

    - Support NR Uu bandwidth [Samsung]

  - Common reference point for SL BWP

    - HW: point A and CRB per SCS

  - PRB index

    - Defined in SL BWP [Huawei, HiSilicon]

- Issue 3. PSCCH/PSSCH/PSFCH multiplexing within a slot

- Multiplexing of PSCCH and PSSCH

There was a working assumption on Option 3 for multiplexing of PSCCH and PSSCH. There was no concrete conclusion for Option 1A/1B/2.

- Observation: Majority companies support Option 3 multiplexing while Options 1B/2 needs more discussion. Other options seem not to get many supports.

To decide further support Option 1B/2, RAN1 may need the concrete design of PSCCH and PSSCH first.

- Proposal for agreement

  - Confirm the working assumption that, for multiplexing of PSCCH and PSSCH in a slot, Option 3 is supported.

    - FFS: whether/when PSCCH/PSSCH FDMed is allowed

- PSFCH-only transmission within a slot

- Observation: Most of discussion papers seem to discuss PSFCH structure with the assumption of PSFCH-only transmission within a slot. Even though RAN1 did not discuss this topic explicitly, but it seems to be agreeable.

- Proposal for agreement

  - A UE may transmit only PSFCH in a slot.

- Multiplexing between PSCCH/PSSCH and PSFCH

There are the following alternatives for each issue as below.

- For transmission perspective from a UE

    - Alt 1. Allow FDM between PSCCH/PSSCH and PSFCH

    - Alt 2. Not allow FDM between PSCCH/PSSCH and PSFCH

  - For system/resource pool perspective,

    - Alt 1. Allow FDM between PSCCH/PSSCH and PSFCH

    - Alt 2. Not allow FDM between PSCCH/PSSCH and PSFCH

- Observation: For UE transmission perspective, many of proponents for allowing FDM between PSCCH/PSSCH and PSFCH seems also to propose long PSFCH format. It seems long PSFCH format discussion should be done first.

- Proposal for agreement

  - RAN1 needs to discuss PSFCH formats first.

- Issue 4. PSCCH design

Agreements: - Rel-15 NR PDCCH DMRS pattern is reused for PSCCH DMRS pattern. - For frequency-domain pattern for PSCCH DMRS, reuse Rel-15 NR PDCCH DMRS, i.e., comb-4 fixed RE mapping for PSCCH DMRS. - (Working assumption) For time-domain pattern for PSCCH DMRS, every symbol of PSCCH has PSCCH DMRS REs. - FFS: how to initialize DMRS sequence generator Agreements: - PSCCH for 1st stage SCI with 2 and 3 symbols is supported in Rel-16. - FFS: other length(s) of symbols (e.g., all symbols) - The number of symbols above excludes AGC symbols if any - The number of PSCCH symbols is explicitly (pre-)configured per Tx/Rx resource pool Agreements: - For the number of bits of L1 IDs, - Layer-1 destination ID: 16 bits - Layer-1 source ID: 8 bits Agreements: - Support 1st stage SCI in PSCCH in one subchannel only. - Within one subchannel, there is at most one 1st stage SCI, except for spatial re-use Agreements [98b-NR-10] - For the starting symbol of PSCCH in a slot, 2nd SL symbol in the slot is used. - FFS: which signal/channel(s) is mapped in the 1st SL symbol in the slot. It is not precluded to map certain portion of PSCCH to the 1st SL symbol in a slot. - FFS: whether/how to support that PSSCH DMRS and PSCCH are mapped in the same OFDM symbol - If RAN1 decides to support mapping PSSCH DMRS and PSCCH in the same OFDM symbol, then this mapping within a single sub-channel is only supported for sub-channel sizes >= 20 PRBs. - Note: This might not have specification impact, pending the outcome of other discussions in RAN1#99. - Note: This does not imply that PSSCH DMRS and PSCCH are mapped in the same OFDM symbol within the same sub-channel for other cases and within the different sub-channels.

- Number of PRBs for a PSCCH

How many symbols/PRBs are used for PSCCH should be discussed.

  - Symbols:

    - All symbols in a slot: [Huawei, HiSilicon], [CATT], [MediaTek]

  - PRBs

    - [Huawei, HiSilicon]: [12, 15, 18] PRBs for 2-symbol PSCCH, [8, 10, 12] PRBs for 3-symbol PSCCH.

    - [Intel]:15 PRBs for 2-symbol PSCCH, 10 PRBs for 3-symbol PSCCH.

    - [ZTE, Sanechips]: 10 or 15 PRBs, which is (pre-)configured.

    - [Samsung]: [3, 6, 12, 24, 48] PRBs for 2-symbol PSCCH and [2, 4, 8, 12, 32] PRBs for 3-symbol PSCCH.

    - [LGE], [Ericsson]: 5, 10, 15, 20, 25, 50

    - [Qualcomm]: 10, 15, 20, 30

    - 1 subchannel [CATT], [Futurewei], [Ericsson], [MediaTek]

- Observation: It seems there are not much discussion on the number of PSCCH symbols/PRBs. Among companies providing views, majority is to support up to 3 symbols for PSCCH. For longer PSCCH, further discussion is needed.

- Proposal

  - A single number of PRBs for mapping PSCCH is (pre-)configured per resource pool.

  - RAN1 discusses the candidate PRB numbers for PSCCH to be supported.

- DMRS for PSCCH

The remaining issue is how to initialize DMRS sequence generator.

- Parameter (on how to replace N_ID)

  - Fixed: [Nokia], [NSB], [CATT],

  - Randomly selected: [vivo], [Intel], [LGE]

  - (pre-)configured: [Ericsson]

- Proposal

  - RAN1 further discusses how to initialize DMRS sequence generator.

- Contents in 1st SCI

RAN1 agreed to support two-stage SCI including the following conclusion.

- If two-stage SCI is supported, the following details are used. - Information related to channel sensing is carried on 1st-stage. - 2nd-stage is decoded by using PSSCH DMRS. - Polar coding used for PDCCH is applied to 2nd-stage - Payload size for 1st-stage in two-stage SCI case is the same for unicast, groupcast, and broadcast in a resource pool. - After decoding the 1st-stage, the receiver does not need to perform blind decoding of 2nd-stage. - FFS other details

Some papers discuss the details of 1st SCI including contents. The views can be summarized as follows.

- Candidate list of contents in 1st SCI

  - Priority

    - Supported by [Huawei, HiSilicon], [vivo], [CATT], [Lenovo, Motorola], [OPPO], [Samsung], [Intel], [Futurewei], [LGE], [Apple], [Kyocera, Continental], [Qualcomm], [Fraunhofer], [Sequans]

  - PSSCH resource assignment

    - Supported by [Huawei, HiSilicon], [vivo], [CATT], [OPPO], [Samsung], [Intel], [Fraunhofer], [Futurewei], [ZTE, Sanechip], [LGE], [Apple], [NTT DOCOMO], [Kyocera, Continental], [Qualcomm], [Sequans]

  - MCS

    - Supported by [Huawei, HiSilicon], [MediaTek], [CATT], [Mitsubishi], [ZTE, Sanechip], [NTT DOCOMO], [ASUSTek], [Kyocera, Continental], [Qualcomm], [Sequans]

  - Resource reservation

    - Supported by [Huawei, HiSilicon], [vivo], [CATT], [OPPO], [Samsung], [Intel], [Fraunhofer], [Futurewei], [LGE], [Apple], [NTT DOCOMO], [Kyocera, Continental], [Qualcomm], [Sequans]

  - retransmission index

    - Supported by [Huawei, HiSilicon], [CATT], [OPPO],

  - DMRS pattern

    - Supported by [Huawei, HiSilicon], [CATT], [OPPO], [Intel], [Samsung], [ZTE, Sanechip], [LGE], [Apple], [Kyocera, Continental], [Qualcomm], [Sequans]

  - AL for 2nd SCI (or code rate, or beta offset like UCI mapping)

    - Supported by [Huawei, HiSilicon], [Nokia, NSB], [vivo], [CATT], [Lenovo, Motorola], [Sony], [Futurewei], [LGE], [NTT DOCOMO], [Kyocera, Continental], [Qualcomm], [Panasonic], [Sequans], [Apple]

  - 2nd SCI format,

    - Supported by [Huawei, HiSilicon], [ZTE, Sanechips], [OPPO], [Samsung], [Intel], [Fraunhofer], [Futurewei], [ZTE, Sanechip], [LGE], [NTT DOCOMO], [Kyocera, Continental], [Qualcomm], [Fraunhofer], [Sequans], [Apple]

  - antenna ports

    - Supported by [Huawei, HiSilicon], [OPPO], [LGE], [CATT], [Qualcomm]

  - TX power indication

    - Supported by [OPPO]

  - destination ID

    - Supported by [vivo], [CATT], [Intel] (partially), [Fraunhofer], [Apple], [NTT DOCOMO], [Kyocera, Continental], [Fraunhofer], [Sequans]

    - No ID in 1st SCI: [Huawei, HiSilicon], [LGE]

  - Source ID

    - Supported by [vivo], [Xiaomi]

  - 1 bit QCL

    - Supported by [MediaTek]

- Proposal for 1st SCI

  - 1st SCI includes at least

    - Priority (QoS value),

    - PSSCH resource assignment (frequency/time resource for PSSCH),

    - PSSCH DMRS pattern in time-domain,

    - Resource reservation,

    - 2nd SCI format (if multiple formats are supported),

    - 2nd SCI code rate (or aggregation level)

    - MCS

    - Destination ID

    - FFS on, antenna ports, TX power indication

- Location of PSCCH

- Support that the lowest PRB of a PSCCH is the same as lowest PRB of the corresponding PSSCH. - FFS: Also support that the highest PRB of a PSCCH is the same as highest PRB of the corresponding PSSCH

  - Support also that the highest PRB of a PSCCH is the same as highest PRB of the corresponding PSSCH

    - Yes:

    - No: [Nokia, NSB]

  - Random subchannel offset between a PSCCH and the associated PSSCH: [MediaTek]

- Proposal

  - RAN1 further discusses this issue.

- Others topics

  - Confirm the working assumption that, for time-domain pattern for PSCCH DMRS, every symbol of PSCCH has PSCCH DMRS REs.

    - [Huawei, HiSilicon], [Nokia, NSB], [vivo], [Spreadtrum], [LGE], [Ericsson], [NTT DOCOMO]

  - Scrambling sequence initialization

    - preconfigured: [LGE], [Ericsson]

  - Other length of PSCCH

    - 1 symbol: [Ericsson]

    - All available SL symbols in a slot: [Huawei, HiSilicon]

  - Single-port transmission of PSCCH [Intel], [TCL], [Panasonic], [Qualcomm], [CATT]

  - Handling L1 ID

RAN1#AH1901 Agreements: - Layer-1 destination ID can be explicitly included in SCI - FFS how to determine Layer-1 destination ID - FFS size of Layer-1 destination ID - The following additional information can be included in SCI - Layer-1 source ID - FFS how to determine Layer-1 source ID - FFS size of Layer-1 source ID - HARQ process ID - NDI - RV - FFS whether some of the above information may not be present etc. in some operations (e.g., depending on whether they are used for unicast, groupcast, broadcast)

    - Source ID

      - 1st SCI: [vivo]

      - 2nd SCI: [Intel], [LGE]

      - Scrambled in CRC of 2nd SCI:

    - Destination ID

      - 1st SCI: [vivo], [Intel]

      - 2nd SCI: [LGE]

      - Scrambled in CRC of 2nd SCI: [Huawei, HiSilicon], [ASUSTeK]

- Proposal for PSCCH

  - Confirm the working assumption that, for time-domain pattern for PSCCH DMRS, every symbol of PSCCH has PSCCH DMRS REs.

  - Single antenna port is used for PSCCH transmission.

## Issue 5. 2nd SCI design

Agreements: - The 2nd stage SCI is carried within the resource of the corresponding PSSCH. - Scrambling operation for the 2nd stage SCI is applied separately with PSSCH Agreements: - Support 1st stage SCI in PSCCH in one subchannel only. - Within one subchannel, there is at most one 1st stage SCI, except for spatial re-use - For RE mapping of the 2nd stage SCI, frequency-first mapping within the PSSCH is used. To down-select: - Alt 1. The REs for the 2nd SCI are not interlaced with (localized in) PSSCH data RE. - Alt 1-1. only RBs in the subchannel having the corresponding 1st stage SCI can be possibly used for mapping the 2nd stage SCI - Alt 1-2. only RBs in the all sub-channels for the scheduled PSSCH can be possibly used for mapping the 2nd stage SCI. - Alt 2. The REs for the 2nd stage SCI can be interlaced with (distributed in) PSSCH data RE. - Whether to allow mapping with the same symbol of PSSCH DMRS - For modulation order of the 2nd stage SCI, to down-select: - Alt 1. Fixed as QPSK - Alt 2. Same as PSSCH - The same PSSCH DM-RS port(s) is used for transmitting the 2nd stage SCI. - When PSSCH is 2-layer, FFS how to map the 2nd stage SCI modulation symbols to the two layers, to down-select: - Alt 1: when PSSCH is 2-layer, the same modulation symbol of the 2nd stage SCI is mapped to the two layers - Alt 2: when PSSCH is 2-layer, different modulation symbols of the 2nd stage SCI are mapped to the two layers - A combination thereof

- Mapping of 2nd SCI

After discussion in RAN1#98bis, there are three alternatives for mapping of 2nd SCI in PSSCH as illustrated below.

  - Subchannel/localization

    - Alt 1-1: [Fraunhofer], [Futurewei], [Ericsson], [Kyocera, Continental]

    - Alt 1-2: [Huawei, HiSilicon], [TCL], [Lenovo, Motorola], [Qualcomm]

    - Alt 2: [Huawei, HiSilicon], [Nokia, NSB], [vivo], [MediaTek], [CATT], [Intel], [Samsung], [ZTE, Sanechip], [Spreadtrum], [LGE], [NEC], [InterDigital], [OPPO], [Apple], [NTT DOCOMO], [ASUSTeK], [Xiaomi]

  - Modulation order

    - Alt 1. Fixed as QPSK: [Nokia, NSB], [MediaTek], [Intel], [TCL], [Fraunhofer], [Lenovo, Motorola], [Sony], [Futurewei], [ZTE, Sanechip], [Ericsson], [NEC], [InterDigital], [OPPO], [Qualcomm], [Kyocera, Continental]

    - Alt 2. Same as PSSCH: [Huawei, HiSilicon], [Samsung], [Spreadtrum], [LGE], [Apple], [NTT DOCOMO]

  - When PSSCH is 2-layer,

    - Alt 1. same modulation symbol: [MediaTek], [Lenovo, Motorola], [ZTE, Sanechip], [NEC], [InterDigital], [OPPO]

    - Alt 2. different modulation symbols: [Huawei, HiSilicon], [Nokia, NSB], [Lenovo, Motorola], [Futurewei], [Samsung], [Apple], [NTT DOCOMO]

    - Configurable: [Ericsson]

  - allow mapping with the same symbol of PSSCH DMRS

    - Alt 1. Allow: [Huawei, HiSilicon], [MediaTek], [TCL], [ZTE, Sanechips]

    - Alt 2. Not allow: [vivo], [Intel], [Spreadtrum]

- Proposal for agreement

  - The REs for the 2nd stage SCI can be interlaced with (distributed in) PSSCH data RE.

  - QPSK is used for modulation order of the 2nd stage SCI.

  - When PSSCH is 2-layer, different modulation symbols of the 2nd stage SCI are mapped to the two layers.

  - For RE mapping of the 2nd stage SCI, FDM between 2nd stage SCI and PSSCH DMRS in the same symbol is allowed.

- Contents in 2nd SCI

This issue depends on the contents of 1st SCI. Therefore, RAN1 discusses this issue after design of 1st SCI.

- Proposal

  - RAN1 discusses the contents of 2nd SCI.

- CRC scrambling for 2nd SCI

  - L1 destination ID [Huawei, HiSilicon], [ASUSTek]

  - No scrambling: [vivo]

- Proposal

  - RAN1 discusses how to scramble the CRC of the 2nd SCI.

- Transmission of 2nd SCI for broadcast

  - No: 2nd SCI transmission for broadcast transmission [Huawei, HiSilicon], [InterDigital]

  - Yes: [NTT DOCOMO]

- Proposal

  - RAN1 discusses whether the 2nd SCI is required to decode for broadcast data transmission.

## Issue 6. PSSCH design

- Frequency-domain DMRS pattern for PSSCH

Regarding the frequency-domain DMRS pattern, the following options are proposed.

- Opt 1. Use Rel-15 PDSCH DMRS type 1 as a baseline for PSSCH DMRS: [vivo], [MediaTek], [Intel], [TCL], [Sony], [Spreadtrum], [ZTE, Sanechips], [Ericsson], [OPPO], [Panasonic], [Qualcomm], [NTT DOCOMO], [InterDigital]

- Opt 2. Use Rel-15 PDSCH DMRS type 2 as a baseline for PSSCH DMRS: [Samsung], [NEC]

- Opt 3. Use Rel-15 PDSCH DMRS type 1 and type 2 as baselines for PSSCH DMRS: [Huawei, HiSilicon], [Nokia, NSB], [CATT], [Mitsubishi], [Futurewei], [LGE], [Sequans]

Also, whether to support FDM between PSSCH and its DMRS in the same symbol is to be discussed.

  - Support FDM

    - Configured per resource pool: [Nokia, NSB]

  - Not support

- Observation: 13 contributions propose to use type 1 DMRS while 2 contributions proposes to use type 2. Also, 6 contributions propose to configure one of them.

- Proposal for Agreement

  - For PSSCH DMRS pattern in frequency domain, use both type 1 and type 2 of Rel-15 PDSCH DMRS.

  - The type is (pre-)configurable per resource pool.

- Time-domain DMRS pattern for PSSCH

There are remaining issues on Time-domain DMRS pattern for PSSCH including

  - Who decides the exact pattern for Mode 1

  - How to decide the pattern (e.g. subset restrictions)

  - The exact symbol position of the pattern

  - Multiplexing with PSCCH

The contributions can be summarized as below.

For the exact location of DMRS symbols,

  - Alt 1. Use NR Uu DMRS positions as baseline [NEC], [InterDigital]

    - Alt 1-1. Use NR PDSCH DMRS as baseline: [Samsung], [Apple]

    - Alt 1-2. Use NR PUSCH DMRS with some modifications: [LGE]

  - Alt 2. New pattern(s):

    - [Huawei, HiSilicon]

      - Front loaded: {1, 4}, {1, 6}, {1, 9}, {1, 4, 7}, {1, 6, 11}, {1, 4, 7, 10}, {1, 4, 7, 10, 13}

      - Non-front loaded: patterns with 2,3,4 symbols

    - [Nokia, NSB]

      - {3, 9}, {3, 6, 9}, {3, 6, 9, 12} or {4, 9}, {4, 6, 9}, {4, 6, 9, 12} depending on PRBs having or not having PSCCH

    - [vivo]

      - For 2-symbol PSCCH, 15 kHz: {3}, {3, 7, 11}, {3, 5, 8, 11}, {3, 5, 7, 9, 11} , 30 kHz: {3}, {3, 7, 11}, {3, 5, 8, 11}, 60 kHz: {3}, {3, 11}

      - For 3-symbol PSCCH, 15 kHz: {4}, {3, 7, 11}, {3, 5, 8, 11}, {3, 5, 7, 9, 11} , 30 kHz: {4}, {4, 7, 11}, {3, 5, 8, 11}, 60 kHz: {4}, {4, 11}

    - [CATT]

      - {6}, {4, 10}, {4,7,10}, {3,6,9,12} for slots having PSFCH

      - {4}, {4,8} for slots not having PSFCH

    - [Apple]

      - Conditionally {[l_0, 11], [l_0, 7, 11] or [l_0, 6, 9, 11]} or {[l_0, 9] or [l_0, 6, 9]} or {[l_0, 11] or [l_0, 6, 11]} or {[1, l_0, 7, 11]} or {[1,11], [1, 6, 11] or [1, l_0, 7, 11]} or {[1,9] or [1, 6, 9]} depending on subchannel size, existence of PSFCH, number of PSCCH symbols

For the determination of the exact pattern for Mode 1,

  - Alt 1. By gNB

    - Supported by [Huawei, HiSilicon], [Lenovo, Motorola], [Sony]

  - Alt 2. By transmitter UE

    - Supported by [CATT], [TCL], [Mitsubishi], [Sony], [OPPO], [Samsung], [Spreadtrum], [Ericsson], [Apple], [Sequans]

Regarding the patterns, the available DMRS pattern depends on

  - Alt 1. By subcarrier spacing: [vivo], [CATT], [Sony], [Qualcomm], [Sequans]

  - Alt 2. By UE speed: [vivo], [CATT], [OPPO], [Sony], [Qualcomm], [Sequans]

  - Alt 3. By MCS: [Mitsubishi], [OPPO], [Qualcomm], [Sequans]

  - Alt 4. Traffic type: [Sony]

  - Alt 5. Feedback information from the receiver: [Samsung]

For multiplexing with PSCCH in the same symbol,

  - Alt 1. Allow to have PSCCH and DMRS of PSSCH in the same symbol: [InterDigital]

    - With the restriction of sub-channel >= 20 PRBs; [Nokia, NSB], [NTT DOCOMO]

  - Alt 2. Not allow: [MediaTek]\

- Proposal

  - RAN1 first discusses whether FDM between PSCCH and DMRS of PSSCH is supported.

    - Alt 1. Allow FDM between PSCCH and DMRS of PSSCH in the same symbol

    - Alt 2. Not Allow FDM between PSCCH and DMRS of PSSCH in the same symbol

  - RAN1 further discusses the exact DMRS positions in a slot.

- Proposal

  - For Mode 1, Tx UE chooses PSSCH DMRS time-domain pattern, if multiple patterns can be used.

  - RAN1 further discusses whether the available PSCCH DMRS pattern(s) is/are based on SCS, UE speed, MCS, traffic type, or feedback information.

Also, following the principle of Rel-15 NR Uu,

  - Candidates of PSSCH DMRS time-domain patterns in a slot are specified regardless of subcarrier spacing.

- Proposal

| D | dmrs-AdditionalPosition |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 2 | - | - | - | - |
| 3 | l0 | l0 | l0 | l0 |
| 4 | l0 | l0 | l0 | l0 |
| 5 | l0 | l0 | l0 | l0 |
| 6 | l0 | l0 | l0 | l0 |
| 7 | l0 | l0 | l0 | l0 |
| 8 | l0 | l0, 8 | l0, 8 | l0, 8 |
| 9 | l0 | l0, 8 | l0, 8 | l0, 8 |
| 10 | l0 | l0, 10 | l0, 8 | l0, 8 |
| 11 | l0 | l0, 10 | l0, 7, 10 | l0, 7, 10 |
| 12 | l0 | l0, 10 | l0, 7, 10 | l0, 6, 9, 12 |
| 13 | l0 | l0, 12 | l0, 8, 12 | l0, 6, 9, 12 |
| 14 | l0 | l0, 12 | l0, 8, 12 | l0, 6, 9, 12 |

- Sequence of DMRS

  - Pseudo-random sequence in NR Uu is reused. [Huawei, HiSilicon], [Ericsson]

  - sequence generation (on how to replace N_ID^{N_SCID} of NR PDSCH DMRS)

    - CRC of the corresponding SCI: [Huawei, HiSilicon], [Intel] (CRC of 1st SCI), [LGE], [Ericsson]

    - Destination ID: [vivo] for uni/groupcast, [NTT DOCOMO]

    - Source ID: [vivo] for broadcast

  - how to replace N_SCID

    - fixed as 0

- Proposal

  - Pseudo-random sequence in NR Uu is reused for PSSCH DMRS.

  - RAN1 further discusses sequence generation of PSSCH DMRS.

- Number of TB per PSSCH

In RAN1#96bis, the working assumption was made only to use 1 TB per PSSCH. It was considered that only up to 2 layers can be used for PSSCH.

  - Confirm WA: [Spreadtrum], [Mitsubishi], [vivo], [OPPO], [TCL], [Samsung], [LGE], [MediaTek], [CATT]

  - Also, support 2 TBs per PSSCH: [Huawei, HiSilicon]

- Observation: There is one paper proposing two TBs per PSSCH while 8 papers propose to confirm the WA.

- Proposal

  - Conform the working assumption that 1 TB is supported per PSSCH.

- Maximum modulation order (multiple MCS Tables) supported

Agreements: - 256QAM is supported for SL. - Support of 256QAM by a UE is FFS between mandatory vs. based on UE capability from the Rx perspective - Support of 256QAM is based on UE capability from the Tx perspective - 64QAM is mandatory Agreements: - Three MCS tables supported in Rel-15 NR Uu CP-OFDM are also used for SL. - Support of the the low-spectral efficiency 64QAM MCS table is an optional UE feature in SL as in the Uu link - For each resource pool, at least one MCS table is (pre)-configured - FFS whether or not to introduce a case where the MCS table can be overwritten by PC5 RRC or indicated in SCI - Each resource pool is only configured with one 1st stage SCI PSCCH format

  - Support of 256QAM

    - Mandatory: [Ericsson], [NEC]

    - UE capability from the RX perspective: [Huawei, HiSilicon], [Apple], [NTT DOCOMO], [Qualcomm]

  - MCS table overwritten by PC5 RRC

    - [Nokia, NSB], [Samsung], [OPPO], [Apple], [Qualcomm]

    - No: [Ericsson], [NEC]

  - MCS table indicated by SCI

    - [MediaTek], [LGE], [Ericsson], [InterDigital]

- Proposal

  - Support of 256QAM is based on UE capability from the Rx perspective.

  - For unicast transmission, PC5 RRC reconfigures which MCS table is used.

  - RAN1 further discusses whether/which additional MCS table(s) is (pre-)configured.

- TBS determination

- Following can be considered for TBS determination.

  - Available symbols for PSSCH transmission per slot: [vivo]

  - PSCCH resource: [vivo], [LGE], [Apple]

  - Overhead of CSI-RS and PTRS: [vivo], [Intel], [Apple]

  - AGC symbol and GP symbol: [vivo], [Intel], [Apple]

  - overhead of DMRS: [vivo], [Apple], [NTT DOCOMO]

  - overhead of 2nd stage SCI: [vivo], [Intel], [LGE], [Apple]

  - existence of PSFCH: [Intel], [Apple]

- Proposal

  - For TBS determination for NR sidelink, TBS determination of NR Uu link is a starting point

    - FFS: how to obtain the number of RE (N_RE) for TBS determination

- Length indication of PSSCH

- No partial scheduling of PSSCH among available SL symbols in a slot: [TCL]

- Others

- Tx diversity scheme

- PSSCH length (related to LS from RAN2, R1-1907999)

- Question 1: During online discussion, RAN2 has considered whether to have the SL LCP restriction for PSSCH duration. As this may be dependent on whether flexible PSSCH length for NR V2X SL communication is supported or not, RAN2 has not made any agreements yet. Therefore, RAN2 would like to ask RAN1: - Whether flexible PSSCH length would be supported for NR V2X PC5 communication? - If flexible PSSCH length is to be supported, how can this be configured?

  - Alt 1. Starting symbol and length of PSSCH are not indicated by SCI 1/2.

  - Alt 2. Starting symbol and length of PSSCH are indicated by SCI 1/2.

- Scrambling sequence for PSSCH

  - Depend on (pre)configured value: [LGE]

  - Depend on CRC of PSCCH: [LGE]

  - Depend on CRC of 2nd SCI: [Apple]

- Relationship of frequency locations of PSCCH and PSSCH

- PRB bundling

  - [Huawei, HiSilicon]: 2, 4 PRBs

- Using LBRM Limited Buffer Rate Matching (LBRM) for Rel-15 NR PDSCH is the baseline for LBRM of PSSCH

  - Supported by [Ericsson]

- Issue 7. PSFCH design

Agreements [98b-NR-09] - For the agreed sequence-based PSFCH format with one symbol (not including AGC training period), - 1-PRB is used - Only 1 bit can be carried for the case of N = 1, where N denotes the period of slot having PSFCH resource in a resource pool - FFS: for the case of N = 2, 4 - Note: Each company is encouraged to discuss on how to handle AGC issue for the agreed sequence-based PSFCH format with one symbol (not including AGC training period) to decide whether/how to support 2-symbol PSFCH format.

- Time-domain repetition of PSFCH format 0 (agreed in RAN1#97)

Some papers discussed with the following alternatives.

- Alt 1: Support repetition to two symbols

  - Supported by [Huawei, HiSilicon], [Nokia, NSB], [CATT], [ZTE, Sanechip], [Spreadtrum], [TCL], [Samsung], [Intel], [ITL]. [Ericsson], [NEC], [NTT DOCOMO], [Qualcomm], [Xiaomi], [Apple]

  - Supported with one-symbol PSFCH as well by [vivo]

- Alt 2. No support of repetition

  - Supported by [TCL], [Sony], [Panasonic], [OPPO], [Xiaomi]

- Proposal

  - Repetition of PSFCH format 0 to two symbols is used.

- Number of bits carried by PSFCH format 0 (agreed in RAN1#97) for N=2,4

Some papers discussed with the following alternatives.

- Alt 1. 1 bit

  - Supported by [Huawei, HiSilicon], [Nokia, NSB], [vivo], [MediaTek], [CATT] for N=2, [ZTE, Sanechips], [Qualcomm], [Ericsson], [OPPO], [NTT DOCOMO], [Xiaomi]

- Alt 2. Up to 2 bits

  - Supported by [CATT] for N=4, [TCL], [Sony], [Samsung], [LGE], [Apple], [NEC], [InterDigital]

- Alt 3. Up to 4 bits

  - Supported by [Intel], [OPPO]

- Proposal

  - [1 or 2] bits are supported for PSFCH format 0.

- Parameter of PSFCH format 0 (agreed in RAN1#97)

m_o

  - (Pre-)configured per resource pool: [vivo], [Samsung], [Ericsson]

  - L1 ID: [Sony], [OPPO]

  - Depend on PSFCH resource: [LGE]

Based on PUCCH format 0, sequence group u and the sequence number v

  - Determined as “Enabled”: [LGE]

  - Determined as “Disabled”: [Ericsson]

- Proposal

  - RAN1 further discusses the design parameters for PSFCH format 0.

- Other PSFCH format(s)

Other than PSFCH format, some papers discuss to support other PSFCH format as below.

- PSFCH format based on PUCCH format 2

  - [Spreadtrum]

- Sequence-based long PSFCH format (spanning all SL symbols in a slot)

  - [Huawei, HiSilicon], [Nokia, NSB]

- Proposal

    - RAN1 further discuss on other additional PSFCH format(s).

- Others

  - PSFCH hopping: [Intel]

- Issue 8. Other Reference Signals

- Sidelink CSI-RS

Agreements: - Resource mapping of SL CSI-RS is performed by using one SL CSI-RS pattern in an RB, where the possible patterns in an RB are a subset of NR Uu CSI-RS time-frequency/CDM resource mapping patterns in an RB - The subset is to be pre-defined by spec - FFS how the one pattern is determined (but not part of SCI) - FFS which subset Agreements: - SL CSI-RS is transmitted by a UE only if: - when the corresponding PSSCH is transmitted (as agreed before) by the UE, and, - when SL CQI/RI reporting is enabled by higher layer signaling, and - when enabled, if the corresponding SCI by the UE triggers the SL CQI/RI reporting

- Observation: Some papers discuss the details of CSI-RS, which can be summarized as follows.

  - SL CSI-RS and PSSCH are multiplexed in the same symbol: [Futurewei], [Samsung],

  - [Spreadtrum]: density 3 for 1 port, density 1 for 2 ports

  - Subset:

    - Up to 4th rows of Uu CSI-RS: [Intel]

    - 2nd/3rd rows of Uu CSI-RS: [vivo], [Ericsson], [Qualcomm]

    - Following patters are supported [Huawei, HiSilicon]

| - Row | Ports X | Density ρ | CDM type | k,l | CDM group index j | k' |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | No CDM | k0,lEnd | 0 | 0 |
| 2 | 2 | 1 | FD-CDM2 | k0,lEnd | 0 | 0, 1 |

where k0∈0,1,2,…,11 for the 1st row, k0∈0,2,4,6,8,10 for the 2nd row, lEnd is the OFDM symbol index corresponding to the last symbol of PSSCH within a slot.

    - Reuse Uu pattern with the same number of port(s): [Futurewei], [InterDigital] (with density 1)

  - Signaling

    - [Huawei, HiSilicon]: (pre-)configured to Tx UE, PC5-RRC to Rx UE

    - [Vivo], [NTT DOCOMO]: PC5-RRC from Tx UE to Rx UE

    - [LGE]: pre-configured set and SCI indication

  - No overlapping of SL CSI-RS with SCI and PSSCH DMRS: [Futurewei]

  - No FDM of SL CSI-RS with PSSCH DMRS: [Qualcomm]

  - Sequence

    - Sequence generation initialization (on how to replace N_ID)

      - Depend on PSCCH CRC [LGE]

      - (Pre-)configured: [InterDigital]

      - By PC5 RRC: [NTT DOCOMO]

- Proposal

  - Resource mapping patterns of Rel-15 NR Uu CSI-RS with 1 or 2 antenna port(s) with density 1 are used for SL CSI-RS.

    - This means that Rows 2 and 3 of Table 7.4.1.5.3-1 with density 1 are used for SL CSI-RS.

- Sidelink PT-RS

For FR2, - Sidelink PT-RS RE patterns are the same as Rel-15 NR Uu - Support multiple densities in time and frequency domains, as Uu - The equivalent of PTRS-UplinkConfig giving the bandwidth and MCS thresholds for setting the densities is (pre-)configured per resource pool - RE offset is determined based on a (pre-)configured resourceElementOffset value - RB offset is down-selected in RAN1#99. - Alt 1. Fixed as 0. - Alt 2. Determined based on L1 destination ID. - Alt 3. Determined based on L1 source ID. - Alt 4. Determined based on CRC of PSCCH. - Association with one or two of the DMRS port(s) is used, down select in RAN1#99 among: - Alt 1. The number of PT-RS antenna ports is the same as the number of DMRS antenna ports. - Alt 2. The number of PT-RS antenna ports and the association between DM-RS port and PT-RS port are (pre)configured. - Alt 3. One PT-RS port is supported and the PT-RS port is associated with the lowest DMRS port index. - Sidelink PT-RS are not mapped to 1st stage SCI REs and SL CSI-RS REs. - Sidelink PTRS symbols are not mapped to PSSCH symbols carrying PSSCH DMRS. - The Rx UE does not perform blind de-rate matching of 2nd stage SCI REs - FFS Details

- Observation:

  - RB offset

    - Alt 1. Fixed as 0.

      - [Samsung], [ZTE, Sanechip], [Spreadtrum],

    - Alt 2. Determined based on L1 destination ID

      - [InterDigital]

    - Alt 3. Determined based on L1 source ID.

      - [vivo], [InterDigital]

    - Alt 4. Determined based on CRC of PSCCH.

      - [Huawei, HiSilicon], [CATT], [LGE]

  - Association with DMRS ports

    - Alt 1: [Huawei, HiSilicon], [vivo], [Qualcomm]

      - Fixed association: [Huawei, HiSilicon]

    - Alt 2. [CATT], [LGE]

    - Alt 3. [Samsung], [ZTE, Sanechip], [Spreadtrum], [InterDigital]

- Proposal

  - RAN1 further discusses which alternative is used for each issue for PTRS design.

- Issue 9. Handling AGC and Tx/Rx switching gap

- Whether to define preamble/RS/signal for AGC training

There are the following options being considered for handling AGC.

- Opt 1. Define AGC training preamble/RS/signal [CATT]

- Opt 2. Use comb-type mapping [LGE], [Ericsson], [OPPO]

- Opt 3. Using mapping of PSCCH and PSSCH [LGE], [Qualcomm]

- Opt 4. Use time-first RE mapping [Intel], [LGE],

- Opt 5. Leave up to UE implementation

- Design of 1st symbol

  - Mapping of PSSCH

    - [NTT DOCOMO]

  - Copy of the first symbol having PSSCH

    - [Nokia, NSB]

  - Copy of the first symbol of PSCCH

    - [Intel], [ZTE, Sanechips], [NEC], [Xiaomi]

  - Copy of PSSCH DMRS

    - [Samsung], [OPPO]

- Proposal

For AGC purposes, 1st SL symbol in a slot is a copy of

- Alt 1. 2nd symbol of the slot

- Alt 2. PSSCH DMRS symbol

- Issue 9. Other aspects

Reference

- R1-1911882	Sidelink physical layer structure for NR V2X	Huawei, HiSilicon

- R1-1911952	Discussion of Physical layer structure for sidelink	Nokia, Nokia Shanghai Bell

- R1-1912020	Physical layer structure for NR sidelink	vivo

- R1-1912077	Control Channel Design and its Efficient Transmission in NR-V2X	Fujitsu

- R1-1912104	Discussion on sidelink physical layer structure	MediaTek Inc.

- R1-1912153	Physical layer structure for NR sidelink	CATT

- R1-1912203	Physical structure details for NR V2X sidelink communication	Intel Corporation

- R1-1912239	Physical layer structure for sidelink	TCL Communication Ltd.

- R1-1912283	RS design for NR V2X sidelink	Mitsubishi Electric RCE

- R1-1912287	Design of NR V2X Physical Layer Structures	Fraunhofer HHI, Fraunhofer IIS

- R1-1912322	Discussion on physical layer structure for NR sidelink	Lenovo, Motorola Mobility

- R1-1912343	Physical layer structure for NR sidelink	Sony

- R1-1912370	Physical layer structure for NR V2X	ITL

- R1-1912428	Remaining details on physical layer structure for the sidelink	Futurewei

- R1-1912458	On Physical Layer Structures for NR V2X	Samsung

- R1-1912514	NR sidelink physical layer structure	ZTE, Sanechips

- R1-1912575	Discussion on physical layer structure for sidelink	Spreadtrum Communications

- R1-1912586	Discussion on physical layer structure for NR sidelink	LG Electronics

- R1-1912597	PHY layer structure for NR sidelink	Ericsson

- R1-1912615	Physical layer structure for NR sidelink	NEC

- R1-1912738	Physical Layer Structure for NR V2X Sidelink	InterDigital, Inc.

- R1-1912752	Discussion on physical layer structure for sidelink in NR V2X	Panasonic Corporation

- R1-1912757	Physical layer structure for NR sidelink	Sharp

- R1-1912789	Physical layer structure for NR-V2X	OPPO

- R1-1912810	On NR V2X Physical Layer Structure	Apple Inc.

- R1-1912839	Physical layer structure design for sidelink	AT&T

- R1-1912880	Sidelink physical layer structure for NR V2X	NTT DOCOMO, INC.

- R1-1912944	Considerations on Physical Layer aspects of NR V2X	Qualcomm Incorporated

- R1-1913010	On NR sidelink physical layer structure	Sequans Communications

- R1-1913020	Discussion on sidelink structure in NR V2X	ASUSTEK COMPUTER (SHANGHAI)

- R1-1913074	Discussion on physical layer structure for 5G V2X	Xiaomi Communications

- R1-1913156	Sidelink Physical Layer Structure	Kyocera Corporation, Continental Automotive GmbH

Appendix: Previous agreements

Agreements in RAN1#94

Agreements:

- At least PSCCH and PSSCH are defined for NR V2X. PSCCH at least carries information necessary to decode PSSCH.

  - Note: PSBCH will be discussed in the synchronization agenda.

- RAN1 continues study on the necessity of other channels.

- Further study on

  - Whether/which sidelink feedback information is carried by PSCCH or by another channel/signal.

  - Whether/which information to assist resource allocation and/or schedule UE’s transmission resource(s) is carried by PSCCH or by another channel/signal.

  - PSCCH format(s) and content(s) for unicast, groupcast, and broadcast

Agreements:

- RAN1 to continue study on the physical channel considering at least the following aspects:

- Waveform

  - Candidates: CP-OFDM, DFT-s-OFDM

  - Proposals from companies:

    - CP-OFDM only

    - Support both

  - Consideration points:

    - Different channel can have different waveform?

    - Benefit and impact of supporting only one waveform and supporting both waveforms

- Subcarrier spacing

  - Candidates for further study are:

    - FR1: 15 kHz, 30 kHz, 60 kHz, 120 kHz

    - FR2: 30 kHz, 60 kHz, 120 kHz, 240 kHz

  - Companies are encouraged to consider the potential issues and benefit of introducing new subcarrier spacing.

- CP length

- RS design

  - Candidates are:

    - DM-RS

      - DM-RS defined in Rel-15 NR Uu is the starting point.

    - PT-RS

    - CSI-RS

    - SRS

    - AGC training signal

- Channel coding

  - For data, channel coding defined for data in Rel-15 NR Uu is the starting point.

  - For control, channel coding defined for control in Rel-15 NR Uu is the starting point.

- Modulation

- RE mapping and rate-matching

- Scrambling

Agreements:

- RAN1 continues study on the necessity, benefits and relationship between bandwidth part and resource pool.

Agreements:

Agree the following assumptions as tentative assumptions for the simulation at least till RAN1#94bis

- AGC

  - Up to [15] us in FR1. Up to [10] us in FR2.

- TX/RX switching time

  - [13] us in FR1 and [7] us in FR2

- Time error

  - Up to [0.4] us between a UE and its synchronization reference

- Frequency error

  - Up to [0.1] PPM between a UE and its synchronization reference

Agreements:

RAN1 to continue study on multiplexing physical channels considering at least the above aspects:

- Multiplexing of PSCCH and the associated PSSCH (here, the “associated” means that the PSCCH at least carries information necessary to decode the PSSCH).

  - Study further the following options:

    - Option 1: PSCCH and the associated PSSCH are transmitted using non-overlapping time resources.

      - Option 1A: The frequency resources used by the two channels are the same.

      - Option 1B: The frequency resources used by the two channels can be different.

    - Option 2: PSCCH and the associated PSSCH are transmitted using non-overlapping frequency resources in the all the time resources used for transmission. The time resources used by the two channels are the same.

    - Option 3: A part of PSCCH and the associated PSSCH are transmitted using overlapping time resources in non-overlapping frequency resources, but another part of the associated PSSCH and/or another part of the PSCCH are transmitted using non-overlapping time resources.

Illustration of the above options:

Agreements in RAN1#94bis

Agreements:

- NR sidelink supports the SCSs supported by Uu in a given frequency range, i.e., {15, 30, 60 kHz} in FR1 and {60, 120 kHz} in FR2.

  - FFS the supported CP length

  - Baseline is that a UE is not required to receive sidelink transmissions using different SCSs simultaneously in a given carrier.

    - FFS if this applies to sidelink synchronization signals/channels

  - Baseline is that a UE is not required to transmit sidelink transmissions using different SCSs simultaneously in a given carrier.

    - FFS if this applies to sidelink synchronization signals/channels

Continue discussion on the waveform till next meeting – companies are encouraged to perform more analysis/evaluations.

Agreements:

For PSCCH and associated PSSCH multiplexing

- At least one of Option 1A, 1B, and 3 is supported.

  - FFS whether some options require transient period between PSCCH and PSSCH.

- FFS whether to support Option 2

R1-1812017

Agreements:

Sidelink control information (SCI) is defined.

SCI is transmitted in PSCCH.

SCI includes at least one SCI format which includes the information necessary to decode the corresponding PSSCH.

NDI, if defined, is a part of SCI.

Sidelink feedback control information (SFCI) is defined.

SFCI includes at least one SFCI format which includes HARQ-ACK for the corresponding PSSCH.

FFS whether a solution will use only one of “ACK,” “NACK,” “DTX,” or use a combination of them.

FFS how to include other feedback information (if supported) in SFCI.

FFS how to convey SFCI on sidelink in PSCCH, and/or PSSCH, and/or a new physical sidelink channel

FFS in the context of Mode 1:

whether/how to convey information for SCI on downlink

whether/how to convey information of SFCI on uplink

Agreements:

At least resource pool is supported for NR sidelink

Resource pool is a set of time and frequency resources that can be used for sidelink transmission and/or reception.

FFS whether a resource pool consists of contiguous resources in time and/or frequency.

A resource pool is inside the RF bandwidth of the UE.

FFS how gNB and other UEs know the RF bandwidth of the UE

FFS if BWP (if defined) can be used to in defining at least part of resource pool

FFS if the numerology of a resource pool is indicated as a part of (pre-)configuration for resource pool, carrier, band, or BWP (if defined)

UE assumes a single numerology in using a resource pool.

Multiple resource pools can be configured to a single UE in a given carrier.

FFS how to use multiple resource pools when (pre-)configured.

FFS BWP is supported for NR sidelink

FFS whether RAN1 can assume that at most one BWP is configured in a carrier from the system perspective.

It is RAN1 understanding that, in some cases, the entire system bandwidth is covered by a single BWP.

FFS the details of BWP configurations, including the possibility of restricting the number of BWPs

FFS whether BWP for TX and RX is separated or a common BWP applied to both TX and RX

There is at most one activated sidelink BWP for a UE in a given carrier as in the Uu case

Further study the feasibility, benefit, and impact of sidelink BWP switching

Aim to conclude in RAN1#95

Companies are encouraged to provide more analysis, including checking current Rel-15 specification regarding BWP related text

Agreements in RAN1#95

Agreements:

- At least CP-OFDM is supported.

- Continue study on whether to support DFT-S-OFDM including the potential issues and the following potential benefit:

  - Synchronization coverage enhancement

  - PSCCH coverage enhancement, e.g., with Option 2 of PSCCH/PSSCH multiplexing with the restriction that PSCCH and PSSCH use adjacent frequency resources

  - Feedback channel coverage enhancement

- A single waveform is used in all the sidelink channels in a carrier.

  - Note: A sequence based channel can be supported in any waveform.

  - (Pre-)configuration will be used to determine the used waveform if the specification supports multiple waveforms.

Agreements:

- For PSCCH/PSSCH in FR1, NR V2X supports normal CP for 15kHz, 30kHz, 60kHz, and extended CP for 60kHz.

  - FFS extended CP for 30 kHz in FR1.

- FFS CP for PSCCH/PSSCH in FR2

  - E.g., NR V2X supports normal CP for 60kHz and 120kHz, and extended CP for 60kHz

    - FFS extended CP for 120 kHz in FR2.

- Only one combination of CP length and SCS is used in a carrier at a given time for NR V2X UEs communicating with each other using SL

Agreements:

- BWP is defined for NR sidelink.

  - In a licensed carrier, SL BWP is defined separately from BWP for Uu from the specification perspective.

    - FFS the relation with Uu BWP.

  - The same SL BWP is used for both Tx and Rx.

  - Each resource pool is (pre)configured within a SL BWP.

  - Only one SL BWP is (pre)configured for RRC idle or out of coverage NR V2X UEs in a carrier.

  - For RRC connected UEs, only one SL BWP is active in a carrier. No signalling is exchanged in sidelink for activation and deactivation of SL BWP.

    - Working assumption: only one SL BWP is configured in a carrier for a NR V2X UE

      - Revisit in the next meeting if significant issues are found

  - Numerology is a part of SL BWP configuration.

Note: This does not intend to make restriction in designing the sidelink aspects related to SL BWP.

Note: This does not preclude the possibility where a NR V2X UE uses a Tx RF bandwidth the same as or different than the SL BWP.

Working assumption:

- Regarding PSCCH / PSSCH multiplexing, at least option 3 is supported for CP-OFDM.

  - RAN1 assumes that transient period is not needed between symbols containing PSCCH and symbols not containing PSCCH in the supported design of option 3.

  - FFS how to determine the starting symbol of PSCCH and the associated PSSCH

  - FFS for other options. e.g. whether some of them are supported to increase PSCCH coverage.

Send an LS to RAN4 to ask the following for options 1A/1B/3 (adding details of 1A/1B/3 in the LS) – R1-1814089, which is endorsed with the following updates:

- Fixing email address

- “identified” to “are studying”

Final LS in R1-1814165

Working assumption:

- For RAN1 evaluation purpose only, until RAN4 response on AGC and switching time, it is assumed that one symbol is used for AGC and another one symbol is used for TX/RX switching.

Note: TX/RX switching includes transition in the power amplifier.

Agreements in RAN1AH-1901

Conclusion:

  - No extended CP is supported for 30 kHz in FR1 in Rel-16

  - No extended CP is supported for 120 kHz in FR2 in Rel-16

Agreements:

- Confirm the working assumption

  - Working assumption: only one SL BWP is configured in a carrier for a NR V2X UE

Agreements:

- Configuration for SL BWP is separated from Uu BWP configuration signalling.

  - UE is not expected to use different numerology in the configured SL BWP and active UL BWP in the same carrier at a given time.

    - FFS the time scale

    - FFS relation to DL BWP including initial Uu BWP

    - FFS relation in terms of frequency location and bandwidth

Agreements:

- For time domain resources of a resource pool for PSSCH,

  - Support the case where the resource pool consists of non-contiguous time resources

    - FFS details including granularity

- For frequency domain resources of a resource pool for PSSCH,

  - Down select following options:

    - Option 1: The resource pool always consists of contiguous PRBs

    - Option 2: The resource pool can consist of non-contiguous PRBs

Agreements:

- Multiple DMRS patterns in time domain are supported for PSSCH

  - FFS: Whether a DMRS pattern is selected based on the subcarrier spacing

  - FFS: Single or multiple DMRS pattern(s) per a resource pool

  - FFS: How TX UE and RX UE can be aligned in terms of the DMRS pattern used for PSSCH

  - FFS: RE mapping, sequence generation

- Continue to study DMRS pattern in frequency domain for PSSCH

  - E.g. Whether multiple patterns are supported, whether PDSCH/PUSCH DMRS configuration 1 or 2 is reused.

Agreements:

- Support PT-RS for PSSCH for FR2

Conclusion:

RAN1 to conclude on the need of physical channel for discovery in RAN1#96.

Agreements in RAN1#96

Agreements:

- Rel-16 NR sidelink supports CP-OFDM only.

Agreements:

- For PSCCH/PSSCH in FR2, NR V2X supports normal CP for 60 kHz, 120 kHz, and extended CP for 60 kHz.

- Note: it is understood that PSFCH follows the same CP as PSCCH/PSSCH

Agreements:

- For the operation regarding PSSCH, a UE performs either transmission or reception in a slot on a carrier.

- NR sidelink supports for a UE:

  - A case where all the symbols in a slot are available for sidelink.

  - Another case where only a subset of consecutive symbols in a slot is available for sidelink

    - Note: this case is not intended to be used for the ITS spectra, if there is no forward-compatibility issue. Finalize in the WI phase whether there is such an issue or not

    - The subset is NOT dynamically indicated to the UE

    - FFS the supported slot configuration(s)

    - FFS whether/how to operate it in partial coverage scenarios

Agreements:

- At least for sidelink HARQ feedback, NR sidelink supports at least a PSFCH format which uses last symbol(s) available for sidelink in a slot.

Agreements:

- RAN1 concludes that no additional physical channel needs to be defined for the purpose of discovery in Rel-16.

Agreements:

- (Pre-)configuration indicates the time gap between PSFCH and the associated PSSCH for Mode 1 and Mode 2.

Agreements:

- In mode 1 for unicast and groupcast, it is supported for the transmitter UE via Uu link to report an indication to gNB to indicate the need for retransmission of a TB transmitted by the transmitter UE.

  - FFS the format of the indication, e.g., in the form of HARQ ACK/NACK, or in the form of SR/BSR, etc.

- RAN1 continues discussion on whether to support report from the receiver UE

  - No inter-BS communication will be considered.

To discuss aspects related to 1st sub-bullet & 2nd bullet during this week -revisit later

Agreements:

- Sidelink HARQ ACK/NACK report from UE to gNB is not supported in Rel-16.

Agreements:

- For unicast RX UEs, SL-RSRP is reported to TX UE

- For sidelink open loop power control for unicast for the TX UE, TX UE derives pathloss estimation

  - Revisit during the WI phase w.r.t. whether or not there is a need regarding how to handle pathloss estimation for OLPC before SL-RSRP is available for a RX UE

Agreements:

- TPC commands for SL PC are not supported

R1-1903597

Agreements:

- For sidelink groupcast, it is supported to use TX-RX distance and/or RSRP in deciding whether to send HARQ feedback.

  - Details to be discussed during WI phase, including whether the information on TX-RX distance is explicitly signaled or implicitly derived, whether/how this operation is related to resource allocation, accuracy of distance and/or RSRP, the aspects related to “and/or”, etc.

  - This feature can be disabled/enabled

Working assumption:

- For unicast, the following CSI reporting is supported based on non-subband-based aperiodic CSI reporting mechanism assuming no more than 4-port:

  - CQI

  - RI

  - PMI

- CSI reporting can be enabled and disabled by configuration.

  - It is supported to configure a subset of the above metric for CSI reporting.

- There is no standalone RS transmission dedicated to CSI reporting in Rel-16

- NR sidelink CSI strives to reuse the CSI framework for NR Uu.

  - Discuss details during WI phase

Agreements:

- RAN1 concludes the following regarding beam management:

  - Beam management is beneficial

  - RAN1 has conducted limited study on the beam management.

  - In FR1, it is feasible to support V2X use cases without beam management.

  - In FR2, it is feasible to support some V2X use cases without beam management in some scenarios.

    - Panel selection is necessary to improve the communication range in FR2.

Conclusion:

- There is no consensus in supporting beam management for normative work for NR V2X in Rel-16.

Agreements in RAN1#96bis

Agreements:

- Polar code adopted for Rel-15 NR DCI is applied to PSCCH.

- LDPC codes used for Rel-15 NR DL-SCH is applied to a transport block delivered by PSSCH.

Agreements:

- The starting symbol and the number of symbols for a PSCCH are assumed to be known to the receiving UE before decoding the PSCCH.

Agreements:

- For the purpose of evaluation of PSCCH design, RAN1 assumes 60 bits, 90 bits, 120 bits as the total SCI sizes including 24 bits CRC.

  - Other sizes are not precluded.

Agreements:

  - QPSK is used for PSCCH.

Working assumption:

  - Transmission of 1 TB with up to 2 layers in a PSSCH is supported.

Agreements:

- At least for transmission perspective of a UE in a carrier, at least TDM between PSCCH/PSSCH and PSFCH is allowed for a PSFCH format for sidelink in a slot.

  - FFS the details of the corresponding PSFCH format

  - FFS whether it is also applicable from system/resource pool perspective or not

  - i.e., in this case, there is no simultaneous transmission of PSCCH and PSFCH and there is no simultaneous transmission of PSSCH and PSFCH.

  - FFS FDM between PSCCH/PSSCH and a PSFCH format which uses last symbol(s) available for sidelink in a slot

  - FFS TDM/FDM between PSCCH/PSSCH and other PSFCH format(s), if supported, which is/are different from the PSFCH format which uses last symbol(s) available for sidelink in a slot

Agreements in RAN1#97

Agreements:

- A sequence-based PSFCH format with one symbol (not including AGC training period) is supported.

  - This is applicable for unicast and groupcast including options 1/2.

  - Sequence of PUCCH format 0 is the starting point.

  - FFS: 1 PRB or multiple PRBs is/are used for this PSFCH format

  - FFS: feasible number of HARQ-ACK bits, mapping of HARQ-ACK bit

- FFS whether to support the following formats

  - X-symbol PSFCH format with a repetition of the one-symbol PSFCH format (not including AGC training period).

    - E.g. X=2

  - A PSFCH format based on PUCCH format 2

  - A PSFCH format spanning all available symbols for sidelink in a slot

Agreements:

- Transmission of PSSCH is mapped onto contiguous PRBs only

Agreements:

- Sub-channel size is (pre)configurable.

  - FFS details (e.g., possible sizes, a minimum size etc.)

Conclusion:

- If two-stage SCI is supported, the following details are used.

  - Information related to channel sensing is carried on 1st-stage.

  - 2nd-stage is decoded by using PSSCH DMRS.

  - Polar coding used for PDCCH is applied to 2nd-stage

  - Payload size for 1st-stage in two-stage SCI case is the same for unicast, groupcast, and broadcast in a resource pool.

  - After decoding the 1st-stage, the receiver does not need to perform blind decoding of 2nd-stage.

  - FFS other details

- Companies are encouraged to perform analysis (e.g., flexibility, complexity, forward compatibility, overhead, spec impact, latency, robustness, reliability, etc.)/evaluations with details of the SCI contents comparing single-stage vs. two-stage SCI. Aim to conclude in RAN1#98

Working assumption:

- Rel-15 PDSCH DMRS Configuration type 1 and/or type 2 are reused for frequency-domain pattern of PSSCH DMRS.

  - FFS whether to support either one or both types

  - FFS details on multiplexing of different ports for PSSCH DMRS

Agreements in RAN1#98

Agreements:

- In physical layer perspective, a (pre-)configured resource pool can be used for all of unicast, groupcast, and broadcast for a given UE.

  - There is no (pre-)configuration to inform which cast types are used for the resource pool.

Agreements:

- (Pre-)configuration of one or more PSSCH DMRS pattern(s) in time domain per a resource pool is supported.

- Exact DMRS pattern is indicated by SCI

  - FFS details, including whether or not to have the indication bit in case of one (pre)configured DMRS pattern

- For Mode 2, DMRS pattern is chosen by the transmitter UE from the (pre)configured patterns for the resource pool.

  - FFS: case for Mode 1

  - FFS: whether/how to use restrictions for choice of DMRS pattern

- FFS on details on time-domain pattern

- FFS the number of possible DMRS patterns

- Note: it is not intended to specify DM-RS based resource pool selection

Agreements:

- Support 2-stage SCI

  - 1st SCI is carried in PSCCH.

  - FFS: other details

Agreements in RAN1#98bis

Agreements:

- A slot is the time-domain granularity for resource pool configuration.

  - To down-select:

    - Alt 1. Slots for a resource pool is (pre-)configured with bitmap, which is applied with periodicity

    - Alt 2. Slots for a resource pool is (pre-)configured, where the slots are applied with periodicity.

  - FFS: signaling details

  - FFS: how to apply the above bitmap signaling, e.g., to all slots or only to a set of slots

  - FFS: symbols for sidelink in the slot, how to indicate for the case when not all symbols are for SL

Agreements:

- Support (pre-)configuration of a resource pool consisting of contiguous PRBs only

Agreements:

- For the number of bits of L1 IDs,

  - Layer-1 destination ID: 16 bits

  - Layer-1 source ID: 8 bits

Agreements:

- 256QAM is supported for SL.

  - Support of 256QAM by a UE is FFS between mandatory vs. based on UE capability from the Rx perspective

  - Support of 256QAM is based on UE capability from the Tx perspective

  - 64QAM is mandatory

Agreements:

- Three MCS tables supported in Rel-15 NR Uu CP-OFDM are also used for SL.

  - Support of the the low-spectral efficiency 64QAM MCS table is an optional UE feature in SL as in the Uu link

- For each resource pool, at least one MCS table is (pre)-configured

  - FFS whether or not to introduce a case where the MCS table can be overwritten by PC5 RRC or indicated in SCI

- Each resource pool is only configured with one 1st stage SCI PSCCH format

Agreements:

- Rel-15 NR PDCCH DMRS pattern is reused for PSCCH DMRS pattern.

  - For frequency-domain pattern for PSCCH DMRS, reuse Rel-15 NR PDCCH DMRS, i.e., comb-4 fixed RE mapping for PSCCH DMRS.

  - (Working assumption) For time-domain pattern for PSCCH DMRS, every symbol of PSCCH has PSCCH DMRS REs.

  - FFS: how to initialize DMRS sequence generator

Agreements:

- PSCCH for 1st stage SCI with 2 and 3 symbols is supported in Rel-16.

  - FFS: other length(s) of symbols (e.g., all symbols)

  - The number of symbols above excludes AGC symbols if any

- The number of PSCCH symbols is explicitly (pre-)configured per Tx/Rx resource pool

Agreements:

- Resource mapping of SL CSI-RS is performed by using one SL CSI-RS pattern in an RB, where the possible patterns in an RB are a subset of NR Uu CSI-RS time-frequency/CDM resource mapping patterns in an RB

  - The subset is to be pre-defined by spec

  - FFS how the one pattern is determined (but not part of SCI)

  - FFS which subset

Agreements:

- SL CSI-RS is transmitted by a UE only if:

  - when the corresponding PSSCH is transmitted (as agreed before) by the UE, and,

  - when SL CQI/RI reporting is enabled by higher layer signaling, and

  - when enabled, if the corresponding SCI by the UE triggers the SL CQI/RI reporting

Agreements:

- The 2nd stage SCI is carried within the resource of the corresponding PSSCH.

- Scrambling operation for the 2nd stage SCI is applied separately with PSSCH

Agreements:

- Support 1st stage SCI in PSCCH in one subchannel only.

  - Within one subchannel, there is at most one 1st stage SCI, except for spatial re-use

- For RE mapping of the 2nd stage SCI, frequency-first mapping within the PSSCH is used. To down-select:

  - Alt 1. The REs for the 2nd SCI are not interlaced with (localized in) PSSCH data RE.

    - Alt 1-1. only RBs in the subchannel having the corresponding 1st stage SCI can be possibly used for mapping the 2nd stage SCI

    - Alt 1-2. only RBs in the all sub-channels for the scheduled PSSCH can be possibly used for mapping the 2nd stage SCI.

  - Alt 2. The REs for the 2nd stage SCI can be interlaced with (distributed in) PSSCH data RE.

  - Whether to allow mapping with the same symbol of PSSCH DMRS

- For modulation order of the 2nd stage SCI, to down-select:

  - Alt 1. Fixed as QPSK

  - Alt 2. Same as PSSCH

- The same PSSCH DM-RS port(s) is used for transmitting the 2nd stage SCI.

  - When PSSCH is 2-layer, FFS how to map the 2nd stage SCI modulation symbols to the two layers, to down-select:

    - Alt 1: when PSSCH is 2-layer, the same modulation symbol of the 2nd stage SCI is mapped to the two layers

    - Alt 2: when PSSCH is 2-layer, different modulation symbols of the 2nd stage SCI are mapped to the two layers

    - A combination thereof

Agreements:

- Support {10, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size.

  - FFS other values (e.g., 4, 5, 6, etc.)

- One value of the above set is (pre)configured for the sub-channel size for the resource pool.

- Size of PSCCH: X

  - X  N, where N is the number of PRBs of the subchannel

  - X is (pre)-configurable with values FFS, X

Note: Huawei considers the above agreements are a mistake

Agreements [98b-NR-09]

- For the agreed sequence-based PSFCH format with one symbol (not including AGC training period),

  - 1-PRB is used

  - Only 1 bit can be carried for the case of N = 1, where N denotes the period of slot having PSFCH resource in a resource pool

  - FFS: for the case of N = 2, 4

- Note: Each company is encouraged to discuss on how to handle AGC issue for the agreed sequence-based PSFCH format with one symbol (not including AGC training period) to decide whether/how to support 2-symbol PSFCH format.

Agreements [98b-NR-10]

- Support that the lowest PRB of a PSCCH is the same as lowest PRB of the corresponding PSSCH.

  - FFS: Also support that the highest PRB of a PSCCH is the same as highest PRB of the corresponding PSSCH

- For the starting symbol of PSCCH in a slot, 2nd SL symbol in the slot is used.

  - FFS: which signal/channel(s) is mapped in the 1st SL symbol in the slot. It is not precluded to map certain portion of PSCCH to the 1st SL symbol in a slot.

- FFS: whether/how to support that PSSCH DMRS and PSCCH are mapped in the same OFDM symbol

  - If RAN1 decides to support mapping PSSCH DMRS and PSCCH in the same OFDM symbol, then this mapping within a single sub-channel is only supported for sub-channel sizes >= 20 PRBs.

    - Note: This might not have specification impact, pending the outcome of other discussions in RAN1#99.

    - Note: This does not imply that PSSCH DMRS and PSCCH are mapped in the same OFDM symbol within the same sub-channel for other cases and within the different sub-channels.

Agreements [98b-NR-11]

For FR2,

- Sidelink PT-RS RE patterns are the same as Rel-15 NR Uu

- Support multiple densities in time and frequency domains, as Uu

  - The equivalent of PTRS-UplinkConfig giving the bandwidth and MCS thresholds for setting the densities is (pre-)configured per resource pool

- RE offset is determined based on a (pre-)configured resourceElementOffset value

- RB offset is down-selected in RAN1#99.

  - Alt 1. Fixed as 0.

  - Alt 2. Determined based on L1 destination ID.

  - Alt 3. Determined based on L1 source ID.

  - Alt 4. Determined based on CRC of PSCCH.

- Association with one or two of the DMRS port(s) is used, down select in RAN1#99 among:

  - Alt 1. The number of PT-RS antenna ports is the same as the number of DMRS antenna ports.

  - Alt 2. The number of PT-RS antenna ports and the association between DM-RS port and PT-RS port are (pre)configured.

  - Alt 3. One PT-RS port is supported and the PT-RS port is associated with the lowest DMRS port index.

- Sidelink PT-RS are not mapped to 1st stage SCI REs and SL CSI-RS REs.

- Sidelink PTRS symbols are not mapped to PSSCH symbols carrying PSSCH DMRS.

- The Rx UE does not perform blind de-rate matching of 2nd stage SCI REs

  - FFS Details