TSG-RAN Working Group 2 Meeting #9	 TSGR2#9(99)J69

Sophia Antipolis, November 29th to December 3 th 1999

Source: 	InterDigital

Title: 			Non-octet aligned transport block sizes in TDD mode

Agenda Item:	18

Document for:	Decision

Proposed Change Request to 25.331:

| 3GPP/SMG Meeting #? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  | ???99??? |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location, Country, DD-DD MMM YYYY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | TS 25.331 |  |  |  |  |  |  |  | CR |  |  | 116 |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 3.0.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  | RAN#6 |  |  |  |  |  | for approval |  |  |  |  |  |  | X |  |  | Strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  |  | For information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  | X | UTRAN / Radio |  |  |  |  |  |  |  |  |  | X | Core Network |  |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | InterDigital |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | 01/12/99 |  |  |  |  |  |
| Subject: |  |  | Non-octet aligned transport block sizes in TDD mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| Shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| With an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | X |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | In RLC UM and AM modes RLC PDU’s are octet aligned. Since in TDD mode MAC headers ate not octet aligned the resulting transport block sizes will NOT be octet aligned. TB size = MAC header + RLC PDU Size Therefore in TDD mode either a bit specific Transport Block Size is needed or a octet aligned RLC PDU size must be specified in the TFS. Since the bit specific parameter creates considerable additional signaling load the RLC PDU method is preferred. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 10.2.5.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a C

#### 10.2.5.4 Transport Format Set (TFS)

| Information Element/Group name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Transport channel type |  |  |  |  |
| Dedicated transport channels |  |  |  |  |
| Dynamic Transport Format Information |  | 1 to maxTFcount |  | The first instance of the parameter Dynamic transport format information correspond to Transport format 0 for this transport channel, the second to transport format 1 and so on. |
| Number of Transport blocks | M |  | Integer(0..4095) |  |
| CHOICE RLC mode | C-Blocks |  |  |  |
| CHOICE Transparent mode RLC PDU size |  |  |  |  |
| Size type 1 |  |  | Enumerated(1..128) | 1 bit granularity |
| Size type 2 |  |  | Enumerated(136, 144..256) | 8 bit granularity |
| Size type 3 |  |  | Enumerated(272, 288..1024) | 16 bit granularity |
| Size type 4 |  |  | Enumerated(1088, 1152..4992) | 64 bit granularity |
| CHOICE Acknowledged mode RLC PDU size |  |  |  |  |
| Size type 1 |  |  | Enumerated(24,32..272) | 8 bit granularity |
| Size type 2 |  |  | Enumerated(304, 336..1040) | 32 bit granularity |
| Size type 3 |  |  | Enumerated(1104, 1168..4944) | 64 bit granularity |
| CHOICE Unacknowledged mode RLC PDU size |  |  |  |  |
| Size type 1 |  |  | Enumerated(16,24..264) | 8 bit granularity |
| Size type 2 |  |  | Enumerated(296,328..1032) | 32 bit granularity |
| Size type 3 |  |  | Enumerated(1096,1160..5000) | 64 bit granularity |
| Semi-static Transport Format Information |  |  |  |  |
| Transmission time interval | M |  | Enumerated(10, 20, 40, 80) |  |
| Type of channel coding | M |  | Enumerated(No coding, Convolutional, Turbo) |  |
| Coding Rate | C-Coding |  | Enumerated(1/2, 1/3) |  |
| Rate matching attribute |  |  | Integer(1..maxRM) |  |
| CRC size | M |  | Enumerated(0, 8, 16, 24) |  |

| Common transport channels |  |  |  |  |
| --- | --- | --- | --- | --- |
| Dynamic Transport Format Information |  | 1 to maxTFcount |  | The first instance of the parameter Dynamic transport format information correspond to Transport format 0 for this transport channel, the second to transport format 1 and so on. |
| Number of Transport blocks | M |  | Integer(0..4095) |  |
| CHOICE mode |  |  |  |  |
| FDD |  |  |  |  |
| CHOICE Transport block size | C-Blocks |  |  |  |
| Size type 1 |  |  | Enumerated(48,56..296) | 8 bit granularity |
| Size type 2 |  |  | Enumerated(312, 328..1320) | 16 bit granularity |
| Size type 3 |  |  | Enumerated(1384, 1448..4968) | 64 bit granularity |
| TDD |  |  |  |  |
| CHOICE RLC mode | C-Blocks |  |  |  |
| CHOICE Transparent mode RLC PDU size |  |  |  |  |
| Size type 1 |  |  | Enumerated(1..128) | 1 bit granularity |
| Size type 2 |  |  | Enumerated(136, 144..256) | 8 bit granularity |
| Size type 3 |  |  | Enumerated(272, 288..1024) | 16 bit granularity |
| Size type 4 |  |  | Enumerated(1088, 1152..4992) | 64 bit granularity |
| MAC Header Type | O |  | Boolean | Default is non-dedicated logical channel MAC header type |
| CHOICE Acknowledged mode RLC PDU size |  |  |  |  |
| Size type 1 |  |  | Enumerated(24,32..272) | 8 bit granularity |
| Size type 2 |  |  | Enumerated(304, 336..1040) | 32 bit granularity |
| Size type 3 |  |  | Enumerated(1104, 1168..4944) | 64 bit granularity |
| MAC Header Type | O |  | Boolean | Default is non-dedicated logical channel MAC header type |
| CHOICE Unacknowledged mode RLC PDU size |  |  |  |  |
| Size type 1 |  |  | Enumerated(16,24..264) | 8 bit granularity |
| Size type 2 |  |  | Enumerated(296,328..1032) | 32 bit granularity |
| Size type 3 |  |  | Enumerated(1096,1160..5000) | 64 bit granularity |
| MAC Header Type | O |  | Boolean | Default is non-dedicated logical channel MAC header type |
| Semi-static Transport Format Information |  |  |  |  |
| Transmission time interval | M |  | Enumerated(10, 20, 40, 80) |  |
| Type of channel coding | M |  | Enumerated(No coding, Convolutional, Turbo) |  |
| Coding Rate | C-Coding |  | Enumerated(1/2, 1/3) |  |
| Rate matching attribute | M |  | Integer(1..maxRM) |  |
| CRC size | M |  | Enumerated(0, 8, 12, 16, 24) |  |

| - Condition | - Explanation |
| --- | --- |
| - Blocks | - This IE is only present if IE “Number of Transport Blocks” is greater than 0. |
| - Coding | - This IE is only present if IE “Type of channel coding” is “Convolutional” |

| - Range Bound | - Explanation |
| --- | --- |
| - MaxTFcount | - Maximum number of different transport formats that can be included in the Transport format set for one transport channel is 32. |
| - MaxRM | - Maximum number that could be set as rate matching attribute for a transport channel. |

| - CHOICE RLC mode | - Condition under which the given RLC mode is chosen |
| --- | --- |
| - Transparent mode RLC PDU size | - The RLC entity mapped to this transport channels is using Transparent mode RLC |
| - Acknowledged mode RLC PDU size | - The RLC entity mapped to this transport channels is using Acknowledged mode RLC |
| - Unacknowledged mode RLC PDU size | - The RLC entity mapped to this transport channels is using Unacknowledged mode RLC |

| - CHOICE Transport channel type | - Condition under which the given Transport channel type is chosen |
| --- | --- |
| - Dedicated transport channels | - The transport channel that is configured with this TFS is of type DCH |
| - Common transport channels | - The transport channel that is configured with this TFS is of a type not equal to DCH |

- <Note: The parameter “rate matching attribute” is in line with the RAN WG1 specifications. However, it is not currently in line with the description in 25.302.>