3GPP TSG-RAN2 Meeting #103	R2-1811445

# Gothenburg, Sweden, 20th Aug 2018 - 24th Aug 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0019 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on RLC parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Unlike in LTE, both short SN and long SN can be configured for SRB. In the current TS38.306, UE reports am-WithShortSN to indicates whether it supports short SN for RLC AM which seems to cover both SRB and DRB. On the other hand, TS38.331 specifies short SN as the default length of RLC SN for SRB which means that UE shall support short SN for SRB regardless of am-WithShortSN. Thus, according to the current definition of am-WithShortSN, gNB does not understand whether default value for SRB is supported by UE until UE capability is obtained. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that am-WithShortSN addresses whether UE supports short RLC SN for AM DRB and also similar changes were done for um-WIthLongSN and um-WithShortSN for alignment. This CR is for both EN-DC and SA. Impact analysis: Impacted functionality: Bearer configuration Inter-operability: If the proposed change is implemented by the UE and not by the gNB, there is no interoperability issue. If the proposed change is implemented by the eNB and not by the UE, there may be an interoperability issue that UE may not support . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NW may not be able to use default configuration for SRB until gNB obtains the UE capability information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 4.2.5 |  |  |  |  |  |  |  |  |

Start of change

4.2.5	RLC parameters

| Definitions for parameters | Per | M | FDD-TDD diff |
| --- | --- | --- | --- |
| am-WithShortSN Indicates whether the UE supports AM DRB with 12 bit length of RLC sequence number. | UE | Yes | No |
| um-WIthLongSN Indicates whether the UE supports UM DRB with 12 bit length of RLC sequence number. | UE | Yes | No |
| um-WithShortSN Indicates whether the UE supports UM DRB with 6 bit length of RLC sequence number. | UE | Yes | No |

End of change