3GPP TSG-RAN WG2 #104	 TDoc R2-1817925

# Spokane, WA, USA, 12th – 16th November 2018

Agenda Item:	11.4.5

Source:	Ericsson

Title:	On NR sidelink admission control

Document for:	Discussion, Decision

# Introduction

Paper [1] explains the motivation to evaluate and enhance QoS framework to support V2X services. As proposed in the same paper, current 5GS QoS framework for Uu interface is capable for supporting V2X services, while it’s suggested that NR Sidelink (SL) can inherit similar QoS framework as in Uu interface, i.e. based on QoS flow and radio bearer. This paper further studies the admission control in SL, which helps to manage QoS flows and radio bearers especially when the channel is congested.

# Discussion

## Admission control in SL

In LTE SL V2X, there is no explicit admission control mechanism on the traffic flow associated to a given V2X service. This implies that even when the channel load is high, any V2X service, even with low priority, can be admitted into the system and be processed at AS layer. As such, the MAC entity needs to find available SL resources to accommodate packets of this service, even when the channel congestion would not allow to reach satisfactory QoS performances. As a consequence, if channel congestion is high and the UE keeps injecting packets in the system, that will further congest the channel, thereby affecting the performances of other UEs as well.

In LTE SL V2X, there is no admission control on the traffic flow associated to a V2X service. The consequence is that packets may be injected into the AS layers, even if the channel congestion and resource availability does not allow to reach the desired QoS performances of the incoming traffic flow.

Another advantage of having per flow/bearer level admission control is that if the service required QoS cannot be fulfilled due to not enough resources, the corresponding flow/bearer will not be established, and the service will not be activated.

Therefore, in our understanding, a more concrete admission control on the QoS flow and radio bearer is beneficial in SL. In this way, the UE will have the possibility to perform admission control directly on the QoS flow associated to a given V2X service, which maps to one radio bearer. The admission control decision could be made based on the required QoS characteristics described above of an incoming flow, e.g. data rate, PDB, reliability, ARP, etc, and the channel congestion. Admission control should be applicable both for UE-autonomous resource allocation and gNB-scheduled resource allocation.

RAN2 studies NR SL admission control both for UE-autonomous resource allocation and gNB-scheduled resource allocation.

NR SL admission control may take into account both QoS properties (e.g. 5QI, ARP, etc) of an incoming QoS flow and sidelink channel status.

The admission control may be handled differently depending on whether gNB-scheduled resource allocation is performed or UE-autonomous resource allocation.

Admission control in Uu interface is based on explicit bearer request and configuration signaling. Similar approach can be used for SL when in gNB-scheduled mode, i.e. mode 1. NR SL UE may notify the gNB about the presence of a new incoming QoS flow, and the gNB may accept/reject the UE request, or even preempt other ongoing flows/bearers. On the other hand, for UE autonomous resource allocation, (pre)configuration may provide different congestion thresholds for the different QoS properties, so that the transmitting UE can perform admission control on the basis of (pre)configured rules.

In gNB-scheduled resource allocation, explicit admission control signaling can be used to request/configure/reject/preempt a SL QoS flow between NR SL UE and gNB.

In autonomous mode, the admission control on an incoming SL QoS flow is performed by a transmitting UE following (pre)configured criteria, e.g. congestion/interference levels provided via RRC/SIB.

When in UE-autonomous mode, RAN1/RAN2 may also study if any form of interaction between UEs is needed. In some cases, it might be beneficial if a single packet transmission of low priority is pre-empted by the packet transmission of higher priority of another UE. In LTE SL, high priority transmissions are allowed to use the same resources as on-going low priority transmissions, which may cause severe interference. Therefore, allowing pre-emption of already booked resources might be beneficial.

RAN1/RAN2 study how to enable effective packet pre-emption in autonomous mode, e.g. via SL pre-emption signalling between different UEs.

Agree the TP in Annex

# Conclusions

In section 2 we made the following observations:

Observation 1	In LTE SL V2X, there is no admission control on the traffic flow associated to a V2X service. The consequence is that packets may be injected into the AS layers, even if the channel congestion and resource availability does not allow to reach the desired QoS performances of the incoming traffic flow.

Observation 2	Another advantage of having per flow/bearer level admission control is that if the service required QoS cannot be fulfilled due to not enough resources, the corresponding flow/bearer will not be established, and the service will not be activated.

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 studies NR SL admission control both for UE-autonomous resource allocation and gNB-scheduled resource allocation.

Proposal 2	NR SL admission control may take into account both QoS properties (e.g. 5QI, ARP, etc) of an incoming QoS flow and sidelink channel status.

Proposal 3	In gNB-scheduled resource allocation, explicit admission control signaling can be used to request/configure/reject/preempt a SL QoS flow between NR SL UE and gNB.

Proposal 4	In autonomous mode, the admission control on an incoming SL QoS flow is performed by a transmitting UE following (pre)configured criteria, e.g. congestion/interference levels provided via RRC/SIB.

Proposal 5	RAN1/RAN2 study how to enable effective packet pre-emption in autonomous mode, e.g. via SL pre-emption signalling between different UEs.

Proposal 6	Agree the TP in Annex

# References

- R2-1817924, QoS principles, Ericsson, RAN2#104, Spokane, USA, Nov. 2018

# Annex	Text Proposal for TR 38.885

5.2	QoS management

Admission control is needed to support QoS management in NR SL. Admission control is studied taking into account both mode-1 and mode-2 operations. For example:

- For mode-1, explicit admission control signaling can be used to request/configure/reject/preempt a SL QoS flow between NR SL UE and gNB.

- For mode-2, the admission control on an incoming SL QoS flow is performed by a transmitting UE following (pre)configured criteria, e.g. provided via RRC/SIB.

In autonomous mode, packet level pre-emption among UEs is also needed and is FFS.