3GPP TSG RAN WG2 Meeting #105		                                           R2-1900145

Athens, Greece, 25th February - 1st March 2019

Source:               ZTE

Title:                    Security issue on local LMF

Agenda item:      11.8.2

Document for:    Discussion and Decision

Introduction

In RAN#81, a new SID was approved to study NR positioning support. The following objectives to enhance the architecture was included in the scope of the study [1].

“Study of positioning architecture for location services, functional interfaces, protocol, and procedures for supporting NR dependent positioning technologies (if needed; otherwise, need to be confirmed) ”

A  new agreement was made during RAN2#104 meeting to study the impact of supporting  LM functionality in RAN:

“RAN2 will study the RAN2 protocol impacts associated with supporting location management functionality in RAN”

while most companies agreed that a local location management function will reduce the positioning delay, the information safety problem was not fully discussed, which will become more sensitive when associated with client’s accurate position.  In this paper,  we will illustrate the security issue of the local LMF.

Discussion

Targets UE’s IMSI/SUPI exposure to location server

In LTE, the international mobile subscriber identity(IMSI) is used to identify the user of a cellular network and is a unique identification associated with all cellular networks. In 5G, a somewhat more generic term is now used, the Subscription Permanent Identifier, or SUPI for short. Basically this is the most private information of a certain client since it has one to one correspondence to the target UE.

In LTE, the location server was a node of core network so generally the IMSI can be provided to the LCS server according to the stage 2 description [2]:

The following attributes are identified for Location Service Request information flow:

- Target UE identity, (either one or both of MSISDN and IMSI, or SIP-URI, or pseudonym);

More specifically the MME can transfer the IMSI to the E-SMLC in the SLs interface according to table 7.3.1-1 in [3]:

# Table 7.3.1-1: Location Request message contents

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.4.2 |  | YES | reject |
| Correlation ID | M |  | 7.4.28 |  | YES | reject |
| Location Type | M |  | 7.4.3 |  | YES | reject |
| E-UTRAN Cell Identifier | M |  | E-CGI / 7.4.4 |  | YES | ignore |
| LCS Client Type | O |  | 7.4.5 |  | YES | reject |
| LCS Priority | O |  | 7.4.6 |  | YES | reject |
| LCS QoS | O |  | 7.4.7 |  | YES | reject |
| LCS Service Type ID | O |  | 7.4.30 |  | YES | ignore |
| UE Positioning Capability | O |  | 7.4.8 |  | YES | reject |
| Include Velocity | O |  | 7.4.9 |  | YES | reject |
| IMSI | O |  | 7.4.10 |  | YES | ignore |
| IMEI | O |  | 7.4.11 |  | YES | ignore |
| NOTE: The IMSI should be sent preferably if known. The IMEI may be sent if the IMSI is not known, or in addition to the IMSI for the purpose of allowing correlation between the two identities. |  |  |  |  |  |  |

where IMEI(International Mobile Equipment Identity) is as same private as IMSI.

Observation 1: In LTE the target UE’s IMSI will be provided from MME to the E-SMLC.

Currently the 5GC-MT-LR procedure’s stage 2 description for both regulatory and commercial use cases are still following the framework of  EPC-MT-LR’s [4].

And the stage 3 technical specification interprets it more explicitly. The LMF can be the consumer of several services from AMF according to the table 5.1.1 in [5]:

Table 5.1-1 List of AMF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Namf_Communication | N1MessageNotify | Subscribe/Notify | Peer AMF, LMF, PCF |
|  | N2InfoNotify |  | LMF, AMF |
|  | N1N2MessageSubscribe |  | PCF |
|  | N1N2MessageUnSubscribe |  | PCF |
|  | N1N2MessageTransfer | Request/Response | Peer AMF, SMF, SMSF, LMF, PCF |
|  | N1N2TransferFailureNotification | Subscribe/Notify | SMF, SMSF, LMF |
|  | NonUeN2MessageTransfer | Request/Response | Peer AMF, LMF, CBCF, PWS-IWF |
|  | NonUeN2InfoSubscribe | Subscribe/Notify | CBCF, PWS-IWF |
|  | NonUeN2InfoUnSubscribe |  | CBCF, PWS-IWF |
|  | NonUeN2InfoNotify |  | LMF, CBCF, PWS-IWF |

and the N1N2MessageTransfer service operation can be used in positioning related procedures:

-     UE assisted and UE based positioning procedure

-	Network assisted positioning procedure

The NF Service Consumer of the services may include the following information in the HTTP Request message body:

-	SUPI

-	PDU Session ID or LCS Correlation ID depending on the N1/N2 message class to be transferred

-	N2 SM Information (PDU Session ID, QoS profile, CN N3 Tunnel Info, S-NSSAI)

-	N1 Message Container (e.g. LPP message, SMS, UPDP message)

-	N2 Information Container (e.g. NRPPa message)

According to the description above, the target UE’s SUPI may still be provided from AMF to LMF during a positioning session.

Observation 2: In NR the target UE’s SUPI will be transformed from AMF to the LMF.

Risk of IMSI/SUPI exposure to RAN

The worry of exposing IMSI/SUPI to some illegal IMSI catcher has been a consideration of wireless communication system since its birth for many years. An international mobile subscriber identity-catcher, or IMSI-catcher, is a telephone eavesdropping device used for intercepting mobile phone traffic and tracking location data of mobile phone users. Essentially a "fake" mobile tower acting between the target mobile phone and the service provider's real towers, it is an example of a man-in-the-middle (MITM) attack. 3GPP has already done a lot of work to avoid this problem whereas sophisticated attacks may be able to downgrade 3G and LTE to non-LTE network services which do not require mutual authentication [6].

To mitigate the risk of IMSI/SUPI exposure to a fake/pseudo base station, 5G has introduced SUCI（SUbscription Concealed Identifier). And the IMSI paging procedure was removed with only temporary identifier 5G-GUTI paging left because recent publications cite paging the UE with its permanent identifier as a security threat of tracking the UE and compromising subscriber privacy [7]. But to add a local LMF in RAN, if the origin interface is reused, will somehow make the SUPI available by RAN side again and increase the security risk. We can not see how this risk can be avoided naturally in the solution 26 and 28 in [8].

The existing solutions above may be good reference for us, similar approach may be needed to replace target UE’s SUPI with a temporary or concealed identifier when positioning related messages were transformed to the local LMF. A quite new interface may even be needed measuring against the Nls, which would require more discussion even beyond RAN2 scope. In conclusion, we give the following proposal:

Proposal 1:While agree that the local LMF can reduce the positioning latency, the security problem should be fully considered with LS to other possibly involved groups if necessary.

Conclusion

Based on the discussion above, we have the following observations and proposal:

Observation 1: In LTE the target UE’s IMSI will be provided from MME to the E-SMLC.

Observation 2: In NR the target UE’s SUPI will be transformed from AMF to the LMF.

Proposal 1:While agree that the local LMF can reduce the positioning latency, the security problem should be fully considered with LS to other possibly involved groups if necessary.

References

- [1]	3GPP RP-182155, “New SID: Study on NR positioning support,” Gold Coast, Australia, 2018.

- [2]	3GPP TS 23.271 “Functional stage 2 description of Location Services (LCS)”

- 3GPP TS 29.171 “Location Services (LCS); LCS Application Protocol (LCS-AP) between the Mobile Management Entity (MME) and Evolved Serving Mobile Location Centre (E-SMLC); SLs interface”

- 3GPP TS 23.273 “5G System (5GS) Location Services (LCS); Stage 2”

- 3GPP TS 29.518 “5G System; Access and Mobility Management Services; Stage 3”

- Practical attacks against privacy and availability in 4G/LTE mobile communication systems" (PDF). Retrieved 2017-04-30.”

- 3GPP C1-183064 “Reply LS to LS on paging with IMMI/SUCI in 5GS”

- 3GPP TR 23.731”Study on enhancement to the 5GC LoCation Services (LCS)”