3GPP TSG-RAN WG2 Meeting #105	R2-1900343

# Athens, Greece, 25 Feb - 01 Mar 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1674 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to eLCID support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | INOBEARRAN-Core |  |  |  |  |  |  | Date: |  | 2019-02-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | LTE Rel-15 specified the support for extended number of DRBs (i.e. 15 DRBs), which also necessitated introduction of extended LCID (eLCID) values in MAC. Then the HRLLC WID also utilized the eLCIDs in PDCP duplication, and the NR WID also utilizes LTE LCIDs. For 15 DRBs to PDCP duplication, it is mandated that they need to support the eLCIDs but this is not made clear in the specifications. However, it is not clear when UE supports the extended LCIDs, especially since there is no explicit capability bit for this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add clarification the UE supporting extended number of DRBs or PDCP duplication must also support eLCIDs. - Add a signalled capability indicating whether UE supports eLCIDs (in general). Impact analysis Impacted functionality: Extended LCID support. Inter-operability: - If the network is implemented according to the CR and the UE is not, the UE will not indicate the support for eLCIDs, so there are no inter-operability problems as network assumes UE doesn’t support eLCIDs. - If the UE is implemented according to the CR and the network is not, the network assumes UE doesn’t support eLCID and doesn’t use the eLCIDs, so there are no inter-operability problems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE requirements to support eLCID functionality in Rel-15 are unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.8.9, 4.3.19.20, 4.3.19.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS36.331 CR3835 |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Next Modified Subclause

### 4.3.8	General parameters

<UNNECESSARY PARTS OMITTED>

#### 4.3.8.9	extendedNumberOfDRBs-r15

This field defines whether the UE supports up to 15 DRBs. The UE shall support any combination of RLC AM and RLC UM entities for the configured DRBs. A UE that supports extendedNumberOfDRBs-r15 shall also support the extended number of LCIDs as specified in TS 36.321 [4].

Next Modified Subclause

### 4.3.19	MAC parameters

<UNNECESSARY PARTS OMITTED>

#### 4.3.19.20	extendedLCID-Duplication-r15

This field indicates whether the UE supports use of extended LCIDs 11-17 for PDCP duplication. A UE that supports extendedLCID-Duplication-r15 shall also support the extended number of LCIDs as specified in TS 36.321 [4].

#### 4.3.19.X	extendedLCID-r15

This field indicates whether the UE supports the use extended LCIDs as specified in TS 36.321 [4].