3GPP TSG-RAN WG2 Meeting #105	R2-1900354

Athens, Greece, 25 Feb - 01 Mar 2019

Agenda item:	11.10.5

Source:	Nokia, Nokia Shanghai Bell

Title:	MCG failure from PDU received over SCG

WID/SID:	LTE_NR_DC_CA_enh-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

This contribution discusses the handling of integrity-check failure for a PDCP PDU received by the UE over SCG on a split SRB.

# 2	Discussion

By the current LTE and NR RRC specifications, the UE shall initiate RRC Connection Re-establishment upon the following condition:

1>	upon integrity check failure indication from lower layers concerning SRB1 or SRB2;

SRB1, SRB2 can be configured as split bearer with an SCG RLC. The integrity-check failure on a split SRB can result from a PDCP PDU received over the SCG. Still, RRC re-establishment is triggered regardless of the cell group over which the PDCP PDU failing the integrity check was received.

Observation 1:	On a split SRB, also integrity-check failure of a PDCP PDU received over SCG triggers RRC Re-establishment.

The current specification seems to reflect an assumption that on a split SRB, PDUs received over SCG are always from the same origin as those received over MCG – which is of course the case when everything is in order. However, it is also possible that after a PDU received over SCG fails integrity check, PDUs received over MCG would still keep passing the check. This would be the case if the failing PDU was inserted by an attacker using the SCG.

Observation 2:	Integrity-check failure of a PDU received over SCG may be indication of a problem (such as security attack) related to SCG but unrelated to MCG.

In such a case, RRC re-establishment seems avoidable, and treating the case as SCG failure could be sufficient – if not, a PDU received over MCG will eventually fail integrity check, leading to the RRC re-establishment when necessary.

Proposal:		When a PDCP PDU received over the SCG leg of a split SRB fails integrity check, instead of RRC 	connection re-establishment UE shall trigger SCG failure.

# 3	Conclusion

We discussed the handling of integrity-check failure for a PDCP PDU received by the UE over SCG on a split SRB, making the following observations.

Observation 1:	On a split SRB, also integrity-check failure of a PDCP PDU received over SCG triggers RRC Re-establishment.

Observation 2:	Integrity-check failure of a PDU received over SCG may be indication of a problem (such as security attack) related to SCG but unrelated to MCG.

We propose the following.

Proposal:		When a PDCP PDU received over the SCG leg of a split SRB fails integrity check, instead of RRC 	connection re-establishment UE shall trigger SCG failure.