3GPP TSG-RAN2 Meeting #105	R2-1900639

# Athens, Greece, 25 Feb – 1 Mar 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 3859 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to inter-RAT handover description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current specifications for failure handling in mobility from E-UTRA is incorrect. If the UE performs an inter-system intra-RAT handover (between E-UTRA/EPC and E-UTRA/5GC), it will receive a MobilityFromEUTRACommand from the source node, containing a RRCConnectionReconfiguration message from the target node. If the UE is unable to comply with the RRCConnectionReconfiguration message, it will trigger the failure in section 5.4.2.4 (Handover to E-UTRA reconfiguration) and if the timer T304 included in the RRCConnectionReconfiguration message expires it will trigger the failure in section 5.4.2.5 (Handover to E-UTRA failure). In either cases, the UE will trigger a re-establishment. In the current specification, if the UE fails due to protocol error in the inter-RAT information included in the MobilityFromEUTRACommand, the UE would again initate a connection re-establishment, i.e. perform two concecutive re-establishments. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed procedure text in Mobility from E-UTRA failure to handle the inter-system intra-RAT handover failure case separately Impact analysis Impacted functionality: - Mobility from E-UTRA failure - No impact on EN-DC or NR SA Inter-operability: If the network is implemented according to this CR while the UE is not, the network may try to configure the UE with the first RRCConnectionReconfiguration message after re-establishment, when the UE triggers a new re-establishment. If the UE is implemented according to this CR while the network is not, the UE will trigger two connection re-establishments if it fails an inter-system intra-RAT handover. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE will trigger two connection re-establishments if it fails an inter-system intra-RAT handover. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.4.3.5	Mobility from E-UTRA failure

The UE shall:

1>	if a reconfiguration failure has been triggered by sub-clause 5.4.2.4 (Handover to E-UTRA Reconfiguration failure); or

1>	if the timer T304 included in the RRCConnectionReconfiguration message in the MobilityFromEUTRACommand message expires according to sub-clause 5.4.2.5 (Handover to E-UTRA failure):

2>	the procedure ends;

1>	else:

2>	if T304 configured in the MobilityFromEUTRACommand message expires (mobility from E-UTRA failure); or

2>	if the UE does not succeed in establishing the connection to the target radio access technology; or

2>	if the UE is unable to comply with (part of) the configuration included in the MobilityFromEUTRACommand message:

3>	stop T304, if running;

3>	if the cs-FallbackIndicator in the MobilityFromEUTRACommand message was set to TRUE or e-CSFB was present:

4>	indicate to upper layers that the CS fallback procedure has failed;

3>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, mac-MainConfig and sps-Config;

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

NOTE:	For enhanced CS fallback to CDMA2000 1xRTT, the above UE behavior applies only when the UE is attempting the enhanced 1xRTT CS fallback and connection to the target radio access technology fails or if the UE is attempting enhanced 1xRTT CS fallback along with concurrent mobility to CDMA2000 HRPD and connection to both the target radio access technologies fails.

END OF CHANGES