3GPP TSG-RAN WG2#105	R2-1900694

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	11.10.3

Source:	Ericsson

Title:	Measurements for early setup of CA or DC

Document for:	Discussion, Decision

# Introduction

The Work Item Description for DC and CA enhancements in [1] was approved at the RAN#81 meeting. One of the objectives concerns how to reduce the delay until the setup of DC and/or CA through early measurements:

Early Measurement reporting: Early and fast reporting of measurements information availability from neighbor and serving cells to reduce delay setting up MR-DC and/or CA. [RAN2, RAN4]

  - This objective applies to MR-DC, NR-NR DC and CA

  - The objective should consider measurements in IDLE, INACTIVE mode and CONNECTED mode

  - The impacts on UE power consumption should be minimized

  - The LTE Rel-15 euCA work should be utilized, when applicable

In this document, we discuss what measurements should be performed and provided in order to achieve a fast setup of DC or CA.

# Discussion

## Idle mode measurements in LTE Rel-15

As indicated in the WID ([1]), the LTE Rel-15 euCA work should be utilized, when applicable. The measurements defined in that work item were for setup of CA in LTE and thus included measurements on E-UTRA carriers. The following idle mode measurement configurations were then defined e.g. in the RRCConnectionRelease message:

RRCConnectionRelease-v1530-IEs ::=	SEQUENCE {

drb-ContinueROHC-r15				ENUMERATED {true}			OPTIONAL,	-- Cond UP-EDT

nextHopChainingCount-r15			NextHopChainingCount		OPTIONAL,	-- Cond UP-EDT

measIdleConfig-r15					MeasIdleConfigDedicated-r15	OPTIONAL,	-- Need ON

rrc-InactiveConfig-r15				RRC-InactiveConfig-r15		OPTIONAL,	-- Need OR

cn-Type-r15							ENUMERATED {epc,fivegc}		OPTIONAL,	-- Need OR

nonCriticalExtension				RRCConnectionRelease-v1540-IEs			OPTIONAL

}

MeasIdleConfigDedicated-r15 ::= SEQUENCE {

measIdleCarrierListEUTRA-r15	EUTRA-CarrierList-r15				OPTIONAL,	-- Need OR

measIdleDuration-r15		ENUMERATED {sec10, sec30, sec60, sec120,

sec180, sec240, sec300, spare},

...

}

EUTRA-CarrierList-r15 ::= SEQUENCE (SIZE (1..maxFreqIdle-r15)) OF MeasIdleCarrierEUTRA-r15

MeasIdleCarrierEUTRA-r15::=			SEQUENCE {

carrierFreq-r15						ARFCN-ValueEUTRA-r9,

allowedMeasBandwidth-r15			AllowedMeasBandwidth,

validityArea-r15					CellList-r15					OPTIONAL,	-- Need OR

measCellList-r15					CellList-r15					OPTIONAL,	-- Need OR

reportQuantities					ENUMERATED {rsrp, rsrq, both},

qualityThreshold-r15				SEQUENCE {

idleRSRP-Threshold-r15				RSRP-Range					OPTIONAL,	-- Need OR

idleRSRQ-Threshold-r15				RSRQ-Range-r13				OPTIONAL	-- Need OR

}																	OPTIONAL, 	-- Need OP

...

}

CellList-r15 ::=		SEQUENCE (SIZE (1.. maxCellMeasIdle-r15)) OF PhysCellIdRange

As can be seen, the measurements for fast CA setup in LTE can be configured either on E-UTRA carrier or cell level. For each E-UTRA carrier, the measurements can also be restricted to a validity area indicated as a list of cells, meaning that if the UE reselects to another cell on that carrier frequency it is allowed to stop the measurements. There is a limitation on the duration of the configured idle mode measurements, as indicated through measIdleDuration-r15 in a range from 10 to 300s.

The measurements quantities that can be configured in LTE Rel-15 are RSRP, RSRQ or both.

- The measurements for fast CA setup in LTE Rel-15 are limited to E-UTRA carriers or E-UTRA cells.

- The idle mode measurements in LTE Rel-15 can be restricted to a validity area.

- The idle mode measurements in LTE Rel-15 are configured with a limited duration in the range from 10 to 300s.

- The idle mode measurements in LTE Rel-15 support measurement quantities RSRP, RSRQ or both.

## Early measurements on NR carriers

### Measurement levels

In LTE Rel-15, for setup of LTE CA, the idle mode measurements (and thus reporting) can be configured per carrier and (optionally) per cell.

At setup of CA or DC in NR (including EN-DC), in order to configure the UE directly to the correct beam in the target cell, the network also needs information about the beam that the UE considers as the best. The information about the best beam(s) would also enable configuration of CFRA resources (dedicated preambles) in a limited number of beams, which include the one where the UE will access the target cell. It should therefore be possible for the network to configure measurement reporting on beam level in the early measurements in RRC_IDLE and RRC_INACTIVE.

Network may configure beam level reporting for IDLE/INACTIVE measurements on NR carriers.

### Measurement quantities

The idle mode measurements defined for LTE Rel-15 euCA can be configured with the measurement quantities RSRP and/or RSRQ, where RSRP provides information on received power level and RSRQ is computed taking also the total received power into account.

Measurements of radio quality, i.e. taking the interference situation into account, are useful at certain scenarios. Since the RSRQ measurements are based on measurements on symbols that may include data transmissions, they may however not always be reliable, especially if beamforming is used at such data transmissions. RAN2 should therefore also consider inclusion of SINR as measurement quantity for the early measurements. Such SINR measurements would then be defined for the early measurements (performed during a limited time) and thus not for cell (re)selection. It can then be discussed if they should be supported in both RRC_IDLE and RRC_INACTIVE. The use of SINR measurements in RRC_IDLE and/or RRC_INACTIVE should however be consulted with RAN1 and RAN4.

Network may configure the UE to use the RSRP, RSRQ, and SINR as measurement quantity for IDLE/INACTIVE measurements.

### Measurements out of the sync raster

Cells that are configured as SCells may be cells that do not have any SSB in the sync raster, i.e. the cells do not support UEs to camp there and they are only used for UEs in RRC_CONNECTED. Such a cell could then instead have an SSB that is located out of the sync raster. In order to support e.g. fast setup of CA including such SCells, it should therefore be possible to configure the UE to perform early measurements also out of the sync raster. The corresponding measurements then also need to be included in the early measurements reported to the network for fast setup of CA/DC. It is therefore proposed that early measurements (in RRC_IDLE and RRC_INACTIVE) on SSBs that are out of the sync raster should be supported.

Early measurements (in RRC_IDLE and RRC_INACTIVE) on SSBs that are out of the sync raster should be supported.

### Measurements for fast setup of EN-DC

As part of the LTE Rel-15 euCA work item, early measurements in RRC_IDLE were defined for fast setup of CA in LTE. In order to reduce the delay for setup of EN-DC, the idle mode measurements defined in LTE Rel-15 should be extended to support similar measurements of NR carriers and/or cells. Measurement reporting on beam level should then also be supported.

The measurement configuration for the NR carriers/cells may also need to be extended to include e.g. SMTC, CQD parameters and SSB properties per carrier etc.

Idle mode measurements of NR carriers/cells, for fast setup of EN-DC, should be added for RRC_IDLE in LTE.

### Cell quality derivation parameters for early measurements

In LTE Rel-15 solution, the UE may be configured with a list of carriers in suspend (possibly including cell lists). The UE does not require additional information to perform cell quality measurements as it simply uses cell-specific reference signals.

One issue in NR is that both in connected and inactive/Idle the UE requires CQD parameters to calculate cell quality such as number of beams (e.g. SSBs) to be averaged, consolidation threshold (i.e. only beams above threshold are averaged, etc.). For idle/inactive UEs this is provided per carrier in system information. However, for early measurements, the source may, in suspend, configure NR carriers whose CQD configuration is not being broadcasted in SIBs.

Hence, for NR early measurements, CQD parameters need to be provided as part of the dedicated measurement configuration upon suspend.

For early measurements in NR, CQD parameters are provided in suspend per carrier.

# Conclusion

In the previous sections we have made the following observations:

Observation 1	The measurements for fast CA setup in LTE Rel-15 are limited to E-UTRA carriers or E-UTRA cells.

Observation 2	The idle mode measurements in LTE Rel-15 can be restricted to a validity area.

Observation 3	The idle mode measurements in LTE Rel-15 are configured with a limited duration in the range from 10 to 300s.

Observation 4	The idle mode measurements in LTE Rel-15 support measurement quantities RSRP, RSRQ or both.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Network may configure beam level reporting for IDLE/INACTIVE measurements on NR carriers.

Proposal 2	Network may configure the UE to use the RSRP, RSRQ, and SINR as measurement quantity for IDLE/INACTIVE measurements.

Proposal 3	Early measurements (in RRC_IDLE and RRC_INACTIVE) on SSBs that are out of the sync raster should be supported.

Proposal 4	Idle mode measurements of NR carriers/cells, for fast setup of EN-DC, should be added for RRC_IDLE in LTE.

Proposal 5	For early measurements in NR, CQD parameters are provided in suspend per carrier.

# References

RP-182076, WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE_NR_DC_CA_enh-Core), RAN#81