3GPP TSG-RAN WG2 #105	Tdoc R2-1900737

Athens, Greece, February 25th – March 1st 2019

Agenda Item:	12.2.4

Source:	Ericsson

Title:	PUR TA validation & Release

Document for:	Discussion, Decision

# 1	Introduction

The Rel-16 Work Item Descriptions for LTE-M [1] and NB-IoT [2] contain a common objective on improving the uplink transmission efficiency and/or UE power consumption by means of transmission in preconfigured resources:

Improved UL transmission efficiency and/or UE power consumption:

- Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance[RAN1, RAN2, RAN4]

  - Both shared resources and dedicated resources can be discussed

  - Note: This is limited to orthogonal (multi) access schemes

In RAN1 the following agreements have been reached for LTE-M and NB-IoT which are relevant for TA validation (‘LTE-M’ and ‘NB-IoT’ in brackets when not applicable to both):

Agreement In idle mode, the UE will at least consider one or more of the following attributes when validating TA (combination of multiple attributes is allowed): - Serving cell changes (serving cell refers the cell that the UE is camping on) - Time Alignment Timer for idle mode - Serving cell RSRP changes (serving cell refers the cell that the UE is camping on) - FFS Other attributes: - Neighbour cell RSRP change - TDOA of >=2 eNBs - TA History - Subscription based UE differentiation - Others not precluded (for example, attributes that need to be considered for high mobility UEs) Note that UE power consumption should be taken into account for the FFS attributes Agreement For UL transmission in preconfigured resource, fallback mechanism to RACH/EDT procedures is supported. Agreement For transmission in preconfigured UL resources, an RRC idle UE may use the latest TA that passed the validation criteria Agreement In idle mode, at least the following TA validation attributes are supported: - Serving cell changes (serving cell refers the cell that the UE is camping on) - Time Alignment Timer for idle mode - Serving cell (N)RSRP changes (serving cell refers the cell that the UE is camping on) - Based on (N)RSRP measurement definition in existing Rel-15 TS36.214 Agreement The UE can be configured to use at least these TA validation attributes: - Time Alignment Timer for idle mode - Serving cell RSRP changes - Note: the configuration shall support disabling of the TA validation attributes Agreement Include in LS to RAN2, RAN4: RAN1 assumes that a UE transitioning from EDT/connected to idle mode can use the valid TA that was used while in EDT/connected mode.

In RAN2the following agreements have been reached which are relevant for TA validation [4]:

Transmission in dedicated preconfigured uplink resources in IDLE mode is supported for UEs with a valid timing advance. Initially we will focus on dedicated preconfigured uplink resources in idle mode - Shared resources can also be discussed The eNB configures the dedicated preconfigured uplink resources via RRC dedicated signaling. Release of the dedicated preconfigured resources are supported, details for NW triggered and UE triggered are FFS.

In this contribution we continue to discuss some issues on PUR release and failed TA validation which are not covered in the PUR email discussion 104#43.

# 2	Discussion

In the following, dedicated PUR in Idle mode is discussed unless mentioned otherwise.

# 2.1	Timing Advance validation

RAN1 has agreed that a PUR UE “may use the latest TA that passed the validation criteria”, where the validation criteria are 1) Serving cell changes, 2) Idle Time Alignment Timer, and 3) Serving cell RSRP changes (other criteria are still FFS). Since according to the WI-objective “UEs with a valid timing advance” can use PUR, a UE is allowed to use PUR for transmission if it can fulfill the above criteria. More specifically, the RAN1 agreement states that “one or more of the following attributes” and we propose to make this configurable by the network. Since the PUR configuration is only valid in a certain cell (TA is not valid in other cells), this could be configured as part of the PUR configuration not to increase the SI overhead.

The network via RRC signaling configures which or which ones of the supported TA validation mechanisms are to be used for PUR transmissions.

One associated open issue is the behavior when the TA is invalidated. We see two possible solutions for this:

- The PUR configuration is kept and there is signaling to update the TA.

- The PUR configuration is released and the UE falls back to legacy RRC connection establishment or EDT.

First, the signaling required in A is essentially Msg1 and Msg2 of the RA-procedure (possibly aborted after Msg2). Second, the TA is inherently connected to the PUR configuration and if it is no longer valid it should be re-evaluated if the UE should be configured with PUR. I.e. PUR is intended for stationary UEs, and a changing TA is an indication of a moving UE.

Upon invalidation of TA, the PUR configuration is released and the UE falls back to any legacy connection procedure.

That TA is not valid anymore and that the PUR configuration is released would be known to the UE. However, it is an open issue how the eNB should become aware of that the TA evaluation failed, that the UE moved to another cell, or that PUR should be released. The ‘implicit release after m omitted PUR transmissions’ could implicitly achieve the release on the eNB side, but if not configured or set to a high value it would cause some resource waste. We therefore propose that the UE can include an indication of this in the subsequent connection setup.

Implicit PUR release due to invalidation of TA or cell change could be indicated by the UE in the subsequent uplink transmission via dedicated RRC signaling.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The network via RRC signaling configures which or which ones of the supported TA validation mechanisms are to be used for PUR transmissions.

Proposal 2	Upon invalidation of TA, the PUR configuration is released and the UE falls back to any legacy connection procedure.

Proposal 3	Implicit PUR release due to invalidation of TA or cell change could be indicated by the UE in the subsequent uplink transmission via dedicated RRC signaling.

# References

RP-182891, “Revised WID: Additional MTC enhancements for LTE”, Qualcomm, RAN#82, Sorrento, Italy, Dec 2018.

RP-182902, “WID revision: Additional enhancements for NB-IoT”, Huawei, RAN#82, Sorrento, Italy, Dec 2018.

RAN1 Chairman’s Notes, RAN1#94bis, Chengdu, China, October 8th – 12th, 2018.

R2-1818633, “RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC”, Blackberry, RAN2#104, Spokane, USA, November 2018.