3GPP TSG-RAN WG2 Meeting #105 	R2-1900762

Athens, Greece, 25 February – 1 March 2019

Agenda item:		10.4.1.3.11

Source:	Intel Corporation

Title:	RRC processing delay in NR

Document for:		Discussion and Decision

# Introduction

In RAN2#104 meeting, the main aspects for RRC processing delay were agreed in CR R2-1819104. There are following open issues:

FFS	RRC connection re-establishment. (RRCReestablishment) (Reason for FFS is relate to separate discussion on the re-establishment procedure)

3: Optimization on using small processing delay for RRCResume procedure with simple (and typical) configuration could be discussed later based on contributions.

4: Define 16 ms RRC processing delay for the following procedures:

•	…

•	RRC connection Resume (RRCResume) with SCell addition/release or SCG establishment/modification/release (Note that whether SCG configuration will be included in RRCResume is still under discussion)

6: FFS: the value of RRC processing delay for UE capability transfer.

7: FFS: the RRC processing delay requirement when SecurityModeCommand and RRCReconfiguration are transmitted in the same TTI.

In this contribution, we discuss open issues for RRC processing delay.

# Discussion

## Necessity of defining smaller RRC processing delay for RRCResume

One of the open issues regarding RRC processing delay is “Optimization on using small processing delay for RRCResume procedure with simple (and typical) configuration could be discussed later based on contributions.” The agreed RRC processing delay for RRC connection Resume (RRCResume) without SCell addition/release and SCG establishment / modification / release is 10 ms. The natural question is for which requirement to define shorter processing delay? From recent RAN2 discussions, the only motivation is to satisfy IMT-2020 CP latency requirement. Although RAN2#103bis agreed that "The focus should not be on IMT2020 requirements at this stage", we provide our analysis on whether to define smaller RRC processing delay for RRC connection Resume.

It is useful to refer to what has been defined in Rel-15 LTE. In RAN2#102 meeting, it was agreed that if UE supports reduced LTE CP latency to meet 20 ms IMT-2020 requirement, LTE RRC procedure delay is reduced to 3 ms. Following is captured in 36.331 CR R2-1809184.

N = 3 applies for the case of reception of RRCConnectionResume if reducedCP-LatencyEnabled is configured, the UE supports reduced CP latency, and the RRC message only includes MAC and PHY (re-)configurations and does not include (re-)configurations of DRX, SPS, SCells, and MIMO. Further, the UL grant is sent using PDCCH DCI format 0 in common search space. In this scenario, the RRC procedure delay can extend beyond the reception of the UL grant, up to 7 ms.

It should be noted that with 3 ms RRC processing time for RRCConnectionResume, LTE just meets the 20 ms IMT-2020 CP latency requirement.

In IMT-2020 evaluation for NR CP latency, component 9 (RRC procedure delay and the procedure of UL grant) of message RRCResume is assumed to be 7 ms (aligned with the LTE CP latency CR R2-1809184), which was from the agreed value during LTE CP latency discussion. In TR 37.910 [1] Table 5.7.2.1-1, the note for component 9 is as follows:

For step 9 for UL data transfer, the processing delay in the UE (L2 and RRC) is considered, i.e., from reception of RRC Connection Resume to the reception of UL grant. The transmission of UL grant by gNB and processing delay in the UE (processing of UL grant and preparing for UL tx) are also considered. The RRCConnectionResume message only includes MAC and PHY configuration. No DRX, SPS, CA, or MIMO re-configuration will be triggered by this message. Further, the UL grant for transmission of RRC Connection Resume Complete and the data is transmitted over common search space with DCI format 0.

Component 9 includes both the RRC procedure delay and the L1 processing time of UL grant (PUSCH preparation time), which are defined in Table 6.4-1 and 6.4-2 of TS 38.214 [2]). For example, for UE capability 1, PUSCH preparation time is 10 and 12 OFDM symbols for 15 and 30 kHz SCS, respectively. RRC processing time for IMT-2020 evaluation can be calculated by subtracting component 9 with L1 processing time. For example, RRC processing time for 15 kHz SCS and UE capability 1 in IMT-2020 evaluation can be considered as 7 – 10/14 ≈ 6.3 ms. There are different evaluation tables in TR 37.910 [1] for NR CP latency evaluation, and two tables are shown below. Given that it is agreed that RRC processing delay for RRC connection Resume (RRCResume) without SCell addition/release and SCG establishment / modification / release is 10 ms, calculation is done to show whether 20 ms CP latency can be met with the 10 ms RRC processing delay. Given that PUSCH preparation time defined in TS 38.214 [2] is less than 1 ms, RRC processing time in IMT-2020 evaluation is at least 6 ms. Therefore compared with 10 ms RRC processing delay requirement, there is at most 10 – 6 = 4 ms increase of RRC processing delay. In summary, all the cases with less than 16 ms CP latency can still meet the 20 ms IMT-2020 CP latency requirement. These cases are marked with green. For 15 kHz SCS, the increase of RRC processing time from 6.3 ms to 10 ms is 10 – 6.3 = 3.7 ms. Therefore for the case of 16.1 ms CP latency, the CP latency corresponding to 10 ms RRC processing delay is 16.1 + 3.7 = 19.8 ms < 20 ms, therefore still satisfying IMT-2020 requirements. It should be also noted that PDSCH Type B mapping is mandatory, according to TS 38.306. Given that 20 ms IMT-2020 requirements can be satisfied for all the cases of Type B mapping below, we can conclude that 10 ms RRC processing time for RRCConnectionResume is sufficient to meet IMT-2020 CP latency requirement of 20 ms.

Table 5.7.2.1-2 Control plane latency for NR FDD (ms)

(a) PRACH length = 2 OFDM symbols

| Resource mapping type | Non-slot duration | UE capability 1 |  |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15kHz SCS | 30kHz SCS | 60kHz SCS | 120kHz SCS | 15kHz SCS | 30kHz SCS | 60kHz SCS |
| Type A | M =4 | 15.4 | 13.1 | 12.3 | 11.7 | 15.0 | 12.8 | 12.1 |
|  | (4OS non-slot) |  |  |  |  |  |  |  |
|  | M =7 | 15.6 | 13.4 | 12.4 | 11.7 | 15.2 | 13.2 | 12.2 |
|  | (7OS non-slot) |  |  |  |  |  |  |  |
| Type B | M=2 | 13.3 | 12.0 | 11.8 | 11.3 | 13.0 | 11.9 | 11.6 |
|  | (2OS non-slot) |  |  |  |  |  |  |  |
|  | M =4 | 13.8 | 12.3 | 12.0 | 11.5 | 13.4 | 12.1 | 11.7 |
|  | (4OS non-slot) |  |  |  |  |  |  |  |
|  | M =7 | 14.7 | 12.8 | 12.2 | 11.6 | 14.3 | 12.6 | 12.0 |
|  | (7OS non-slot) |  |  |  |  |  |  |  |

Table 5.7.2.1-5 Control plane latency for NR TDD (ms)
(Frame  structure: DUDU, without GP)

(a) PRACH length = 2 OFDM symbols

| Resource mapping type | Non-slot duration | UE capability 1 |  |  |  | UE capability 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15kHz SCS | 30kHz SCS | 60kHz SCS | 120kHz SCS | 15kHz SCS | 30kHz SCS | 60kHz SCS |
| Type A | M =4 (4OS non-slot) | 17.0 | 14.1 | 12.7 | 11.7 | 16.1 | 13.6 | 12.5 |
|  | M =7 (7OS non-slot) | 17.2 | 14.3 | 12.8 | 11.7 | 16.4 | 13.7 | 12.6 |
| Type B | M=2 (2OS non-slot) | 13.9 | 12.3 | 12.2 | 11.6 | 13.9 | 12.4 | 11.8 |
|  | M =4 (4OS non-slot) | 14.2 | 12.6 | 12.2 | 11.6 | 14.1 | 12.4 | 11.9 |
|  | M =7 (7OS non-slot) | 15.2 | 13.3 | 12.8 | 11.7 | 14.6 | 12.8 | 12.4 |

Observation 1: 10 ms RRC processing time for RRCResume is sufficient to meet IMT-2020 CP latency requirement of 20 ms.

It is therefore proposed to not define smaller processing delay (less than 10 ms) for RRCResume procedure.

Proposal 1: Smaller RRC processing delay (less than 10 ms) for RRCResume procedure is not defined.

## RRC processing delay for UE capability transfer

In TS 36.331, RRC processing delay for UE capability transfer is 15 ms if UE has to report EN-DC band combinations, and 10 ms otherwise.

In TS 38.331 v15.4.0, UE can be requested to report UE capability for NR, LTE, or EN-DC, as below:

The UE shall set the contents of UECapabilityInformation message as follows: 1> if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to nr: 2> include in the ue-CapabilityRAT-ContainerList a UE-CapabilityRAT-Container of the type UE-NR-Capability and with the rat-Type set to nr; 2> include the supportedBandCombinationList, featureSets and featureSetCombinations as specified in clause 5.6.1.4; 1> if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to eutra-nr: 2> if the UE supports EN-DC: 3> include in the ue-CapabilityRAT-ContainerList a UE-CapabilityRAT-Container of the type UE-MRDC-Capability and with the rat-Type set to eutra-nr; 3> include the supportedBandCombinationList and featureSetCombinations as specified in clause 5.6.1.4; 1> if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to eutra: 2> if the UE supports E-UTRA: 3> include in the ue-CapabilityRAT-ContainerList a ue-CapabilityRAT-Container of the type UE-EUTRA-Capability and with the rat-Type set to eutra as specified in TS 36.331 [10], clause 5.6.3.3;

In TS 36.331, UE can be also requested to report UE capabilities for NR, LTE, or EN-DC, as well as capabilities for 2G/3G RATs. Given that UE can be requested to report UE capabilities for NR, LTE, or EN-DC by both eNB and gNB, it is natural to apply the RRC processing time defined in LTE to NR regarding UE capability transfer. In addition, there seems no strong motivation to reduce RRC processing time for UE capability transfer.

Regarding the RRC processing time to report NR only UE capability information, RAN2#101bis meeting has the following agreement:

Agreements for EN-DC and NR SA:

…

5:	LTE delay requirement for the UE capability message should be extended for the case that compressed capabilities are provided. For NR the compression needs to be considered when the delay requirement is determined.

Compression in UE capability means composing feature sets with some of the features sets used in multiple band combinations. Compression is needed to report UE capability for EN-DC as well as NR band combinations. Therefore it is proposed that RRC processing delay for UE capability transfer is 15 ms if UE has to report EN-DC band combinations or NR band combinations; otherwise 10 ms. A change to TS 36.331 is needed as above agreement is not reflected, and corresponding 36.331 CR is in R2-1901735 [10].

Proposal 2: For TS 38.331, RRC processing delay for UE capability transfer is 15 ms if UE has to report EN-DC band combinations or NR band combinations, and 10 ms otherwise.

Proposal 3: For TS 36.331, RRC processing delay for UE capability transfer to report NR band combinations is corrected to 15 ms.

## RRC processing delay when SecurityModeCommand and RRCReconfiguration are transmitted in the same TTI

In LTE, when SecurityModeCommand and RRCConnectionReconfiguration are transmitted in the same TTI, the RRC processing delay is 20 ms, which is the largest value (same as the RRCConnectionReconfiguration involving SCell addition/release and SCG establishment/release), and is smaller compared with the sum of RRC processing delay for SecurityModeCommand (10 ms) and RRCConnectionReconrfiguration (15 ms).

The motivation of specifying the requirements for the combined procedure of SecurityModeCommand and RRCConnectionReconfiguration is to satisfy the LTE control plan latency requirement (from IDLE to CONNECTED), as in email discussion summary [5]. Some discussion history in LTE: in RAN2 LTE RRC AH meeting, after discussing R2-082944 [6], RAN2 agreed to Add row with “SMC + Reconfiguration at connection establishment”. In contribution R2-086299 [7], it was argued that in terms of UE processing, the combined processing for SMC and RRC Connection Reconfiguration is by no means faster than the processing of two separate messages. The RRC processing time requirements for LTE were based on R2-090678 [8] and was finally agreed in R2-090774 [9].

In NR, as was agreed in RAN2#104 meeting, RRC processing time for SecurityModeCommand and RRCReconfiguration is 5 and 10 ms, respectively. It should be noted that for NR, defining the RRC processing delay for the combined procedure of SecurityModeCommand and RRCConnectionReconfiguration is not much motivated given that in NR, CP latency is evaluated for RRC_INACTIVE to RRC_CONNECTED, while SecurityModeCommand and RRCConnectionReconfiguration are for the transition from RRC_IDLE to RRC_CONNECTED. In NR, the processing delay for SecurityModeCommand is already very short (5 ms), and there is not much room for further reduction. Therefore if it is necessary to define requirement for this case, it is propose to consider to define the requirement as the sum of the individual RRC processing time of SecurityModeCommand and RRCReconfiguration, i.e. 15 ms.

Proposal 4: There is no strong motivation to define RRC processing delay for the combined procedure of SecurityModeCommand and RRCConnectionReconfiguration. If the requirement is kept for this case, it is proposed to define the requirement as 15 ms.

## RRC processing delay for RRC re-establishment

In RAN2#104 meeting, RRC re-establishment was discussed, and below is the related Chair notes:

R2-1819051	[offline discussion 078] Re-establishment handling	Intel Corporation 	report

-	Ericsson would prefer to avoid the double RACHs. Maybe the UE processing could be relaxed for this case to avoid the problem.

Agreements

1:	No change is made to the specifications.  RACH, if needed, is considered acceptable for the re-establishment Complete.

2: 	No change is made to the specifications.  No additional L2 parameter updates will be supported with first reconfiguration after re-establishment.

FFS Can be discussed whether the re-establishment processing time could be relaxed so that a UE implementation could avoid the RACH

Given the above agreement, there are two cases to consider for the UE processing time:

- RRCReestablishment and RRCReconfiguration are not transmitted in the same MAC PDU (note that we use the terminology of “same MAC PDU” compared to “same TTI” in LTE, since there is no TTI concept in NR, and DL PDSCH duration is flexible). In this case, RRC processing time for RRCReestablishment can be defined without consideration of the processing time for RRCReconfiguration

- RRCReestablishment and RRCReconfiguration are transmitted in the same MAC PDU. In this case, RRC processing time is defined for the combined procedure of reestablishment+reconfiguration (similar to what is done for SMC+reconfiguration as discussed in previous section).

In LTE, RRC processing time for RRC connection reestablishment and reconfiguration (not involving SCG/SCell) are 15 ms. In NR, RRC processing time for RRC reconfiguration (not involving SCG/SCell) is 10 ms. Therefore it is proposed that RRC processing time for RRC reestablishment in NR as a separate procedure is also 10 ms. For the combined procedure, it is proposed to take the sum (20 ms) as the RRC processing delay requirement.

Proposal 5: RRC processing delay for RRCReestablishment is 10 ms if RRCReconfiguration is not transmitted in the same MAC PDU. RRC processing delay for RRCReestablishment + RRCReconfiguration is 20 ms if the two DL messages are transmitted in the same MAC PDU.

## RRC processing delay regarding SCG configuration in RRCResume

In CR R2-1817089 [4], it is proposed to remove the FFS regarding SCG configuration in RRCResume since it is not supported in ASN1. The changes proposed are included in the rapporteur CR. CR R2-1819104 for RRC processing time captures requirements for RRC resume (scell addition/release) without including SCG configuration, therefore is aligned with the agreement on not including SCG configuration in RRCResume. So no change is needed.

Observation 2: No need to define RRC processing delay requirements regarding SCG configuration in s. There is no specification impact.

# Conclusion

In this contribution, we discuss RRC procedure delay in NR. We have the following observations:

Observation 1: 10 ms RRC processing time for RRCResume is sufficient to meet IMT-2020 CP latency requirement of 20 ms.

Observation 2: No need to define RRC processing delay requirements regarding SCG configuration in s. There is no specification impact.

We propose the following:

Proposal 1: Smaller RRC processing delay (less than 10 ms) for RRCResume procedure is not defined.

Proposal 2: For TS 38.331, RRC processing delay for UE capability transfer is 15 ms if UE has to report EN-DC band combinations or NR band combinations, and 10 ms otherwise.

Proposal 3: For TS 36.331, RRC processing delay for UE capability transfer to report NR band combinations is corrected to 15 ms.

Proposal 4: There is no strong motivation to define RRC processing delay for the combined procedure of SecurityModeCommand and RRCConnectionReconfiguration. If the requirement is kept for this case, it is proposed to define the requirement as 15 ms.

Proposal 5: RRC processing delay for RRCReestablishment is 10 ms if RRCReconfiguration is not transmitted in the same MAC PDU. RRC processing delay for RRCReestablishment + RRCReconfiguration is 20 ms if the two DL messages are transmitted in the same MAC PDU.

Corresponding CR to TS 38.331 is in R2-1900763 [3].

# References

[1] 3GPP TR 37.910 v1.0.0, "Study on Self Evaluation towards IMT-2020 Submission"

[2] 3GPP TS 38.214: “NR; Physical layer procedures for data”

[3] R2-1900763, Intel, "Correction on RRC processing delay"

[4] R2-1817089, Nokia et al, "SCG and resume"

[5] R2-090070, Qualcomm, "Email discussion summary on [64_LTE_13] RRC processing delay"

[6] R2-082944, Panasonic, "Definition of RRC procedure delay"

[7] R2-086299, Qualcomm, "RRC processing delay"

[8] R2-090678, Panasonic et al, "Value “N” in the RRC processing delay requirement"

[9] R2-090774, Panasonic, "Value “N” of RRC processing delay requirements"

[10] R2-1901735, Intel, "Correction on RRC processing delay on UE capability reporting"