3GPP TSG-RAN WG2 Meeting #105	R2-1900819

Athens, Greece, 25 February-1 March 2019

Agenda Item:	11.8.2

Source: 	MediaTek Inc.

Title:  	Positioning server functionality in NG-RAN

Document for:	Discussion and decision

# Introduction

During discussions at RAN2#104, it was agreed that RAN2 can further consider the possibility of decentralising some positioning server functionality into the gNB.  SA2 have agreed that “NG-RAN node enhanced with the support of location management functionality may be selected to perform UE positioning locally in the RAN under the condition that RAN WG(s) concludes that NG-RAN supports location management functionality” ([1], section 8.8).  During email discussion [104#35] (summarised in [2]), there was majority support for allowing location management functionality in NG-RAN.  This document examines the protocol impact of such an approach.

# Discussion

Considering the SA2 status and the need to support very low latency in some commercial use cases, having positioning functionality at the edge of the network is attractive.  During a typical positioning operation, communicating with the NG-RAN instead of the CN would save several RAN-CN round trips (e.g., considering request/response pairs for retrieving the UE capability, delivery of assistance data, and finally the delivery of actual location measurements—and further, the possibility of multiple positioning operations in succession towards the same UE, such as performing E-CID before OTDOA).

As noted in discussion at RAN2#104, there are additional reasons to consider locating server functionality at the gNB, such as scalability and, for some use cases (e.g. IIoT), the ability to limit the distribution of positioning related information through the network.  Thus on balance, we consider that the “server at the edge” architecture is a beneficial option to have available.

Proposal 1: From RAN2 perspective, it is beneficial to allow positioning functionality at the gNB.

The potential latency benefits depend on reducing the number of roundtrips between the UE and the CN, thus any “local server” functionality should be designed to support as much of the UE-facing positioning functionality as possible.  For example, it is desirable in terms of reduced latency if not only the request for measurements (LPP Request Location Information) comes from the local server, but also the assistance data (LPP Provide Assistance Data) and the request for UE capabilities (LPP Request Capabilities).  This does not necessarily mean that all the same methods would be supported by a local server and a centralised one (for instance, UTDOA might not be a good match for a local server).

Proposal 2: If the gNB supports location management functionality, it should support the full suite of LPP procedures.

Support of LPP at the gNB requires some form of transport, e.g. containerising LPP in an RRC message.  The DLInformationTransfer and ULInformationTransfer messages could be noncritically extended to have an LPP container as an alternative to the existing NAS container, or a pair of new messages covering the DL and UL directions (LPPDLInformationTransfer/LPPULInformationTransfer?) could be defined specifically for this purpose.

Proposal 3: Transport of LPP messages between the gNB and the UE can be supported by the DLInformationTransfer and ULInformationTransfer messages or by a new pair of RRC messages.  A decision on which messages are used can be taken in the normative phase.

Transporting LPP via RRC would create the somewhat peculiar situation that an LPP session could involve either NAS or RRC transport, and the UE needs to be aware of which protocol is in use.  The LPP requirements for message transmission indicate “deliver the message to lower layers for transmission”, which is still valid, but the definition of “lower layers” would be flexible and the UE implementation would need to be sensitive to whether the active session used NAS or RRC.

In most cases, it seems obvious that the UE would follow the protocol established by the selected server in the first message.  However, in the case of an MO-LR (not identified for support in the RAN study item, but considered by SA2 in their related study), the UE would need to take this decision itself, as the first LPP message can come from the UE in this scenario.  In general, the UE would not know at the time of an MO-LR initiation if the serving gNB had positioning server functionality; thus it would need to contact the AMF as usual, which means the initial LPP message would go through NAS.  For subsequent messages, it would be possible to have the constraint that MO-LR operations always use the centralised LMF, or to allow “redirection” of a positioning session so that later messages can use RRC as transport.

Proposal 4: Consider whether any steps should be taken in Rel-16 to ensure compatibility of the positioning server in RAN with the MO-LR functionality.

Regarding the delivery of assistance data, as noted above, it is obviously helpful from the latency standpoint if it can be sent from the local server in the gNB rather than a centralised LMF, saving a CN roundtrip.  However, for the gNB to collect and maintain OTDOA assistance data, it would need to receive it either from other gNBs (similar to the NRPPa OTDOA information exchange) or from a centralised LMF that has already collected it from the gNBs.

If the gNB maintains assistance data itself based on configuration from neighbouring gNBs, the natural approach is to support transporting NRPPa over Xn, so that the OTDOA information exchange can take place with the existing NRPPa protocol (Figure 1).

Figure 1: Assistance data delivery obtained from neighbouring gNB

On the other hand, if assistance data are delivered from a centralised LMF to the gNB’s local server function, NRPPa would need to be enhanced to include assistance data delivery (as a non-UE-associated procedure).

Another alternative as suggested by some comments in [2] would be to define a new “NRPPb” protocol specifically for use over Xn; we do not see a major advantage of defining a new protocol but this could be discussed by RAN3 in the normative phase.

Proposal 5: From RAN2 perspective, delivery of assistance data to the local server appears feasible from neighbouring gNBs via NRPPa over Xn, via a new “NRPPb” protocol, or from the centralised LMF via enhancements to NRPPa.  RAN3 would need to confirm this and take a decision on the approach during normative work.

# Conclusion

Proposal 1: From RAN2 perspective, it is beneficial to allow positioning functionality at the gNB.

Proposal 2: If the gNB supports location management functionality, it should support the full suite of LPP procedures.

Proposal 3: Transport of LPP messages between the gNB and the UE can be supported by the DLInformationTransfer and ULInformationTransfer messages or by a new pair of RRC messages.  A decision on which messages are used can be taken in the normative phase.

Proposal 4: Consider whether any steps should be taken in Rel-16 to ensure compatibility of the positioning server in RAN with the MO-LR functionality.

Proposal 5: From RAN2 perspective, delivery of assistance data to the local server appears feasible from neighbouring gNBs via NRPPa over Xn, via a new “NRPPb” protocol, or from the centralised LMF via enhancements to NRPPa.  RAN3 would need to confirm this and take a decision on the approach during normative work.

# References

- TR 23.731, “Study on Enhancement to the 5GC LoCation Services”

- R2-1900753, “Report on [104#35][NR] Positioning SI (Intel)”, Intel, RAN2#105