3GPP TSG RAN WG2 Meeting #105                                            	R2-1900866

Athens, Greece, 25th February - 1st March, 2019

Source:	OPPO

Title:	Discussion on Scenarios of NR Mobility Enhancements

Agenda Item:	11.9.4

Document for:	Discussion and Decision

Introduction

In RAN #80, a new Work Item on NR mobility enhancements was approved, and the two main objectives are to study solutions to reduce interruption time during HO/SCG change and to improve HO/SCG change reliability and robustness [1]. In this contribution, we would like to clarify the scope of this WI and provide our initial thoughts to some solution directions.

Discussion

Regarding deployment scenarios to be discussed in this WI, following note was included in the WID:

Note: The following aspects should be considered in above objectives.

- Inter and intra frequency handover/SCG change

- Inter-CU, intra-CU/inter-DU and intra-DU handover/SCG change

- Synchronous and asynchronous deployments as assumed in Rel-15 NR

- UE capability on the number of Tx/Rx chains

- Low and high velocity

- FR1 and FR2 frequencies

It is noted that RAN#80 also approved another WI on even further mobility enhancement for LTE [2] and RAN2 already spent two meetings on the discussion. The two WIs share very similar objectives and the scope remains yet to be clarified. We think that the first priority for the two WIs should be intra-system mobility, i.e. intra-LTE mobility and intra-NR mobility, while inter-RAT mobility enhancement between LTE and NR should not be in the scope of Rel-16 WIs. More specifically, intra-NR mobility here means source/target MNs for handover case, and source/target SNs for SCG change are all gNBs.

Proposal 1: RAN2 only consider intra-NR mobility scenarios for the WI discussion.

In the WI discussion of LTE_feMob, two main solutions has been proposed in the context of meeting objective of reducing interruption time, which are eMBB-based solution and DC-based solution. We think in NR study phase, these two solutions can be baseline to be considered for meeting the same objective. Compared to eMBB-based solution, DC-based solution may need some further change (e.g. the new role change procedure) to NR-NR DC procedures. However, NR-NR DC is still subject to the completion of Rel-15 late drop. To avoid using wrong working assumption, the safest way would be to wait until late drop is ready before discussing the details of DC-based on solution. Another way can be to work on both solutions in parallel and then reflect the potential change (if any) of late drop to the DC-based solution. We slightly prefer the later way as we expect that NR-NR DC will not bring major changes.

Proposal 2: When discussing DC-based solution, RAN2 should take the progress of late drop into account.

Conclusions

In this contribution, we discussed the target scenario of NR mobility enhancement and potential solutions, and proposed following proposals:

Proposal 1: RAN2 only consider intra-NR mobility scenarios for the WI discussion.

Proposal 2: When discussing DC-based solution, RAN2 should take the progress of late drop into account.

References

- RP-181433, New WID: NR mobility enhancements

- RP-181475, New Work Item on even further Mobility enhancement in E-UTRAN