3GPP TSG-RAN WG2 Meeting #105	R2-1901282

Athens, Greece, 25 Feb - 1 Mar 2019

Agenda item:	11.8.2

Source: 	Huawei, HiSilicon

Title: 	Considerations on UTDOA in NR

Document for:	Discussion and Decision

Introduction

In the last RAN1 meeting, there were the following agreements on UL only based positioning

Agreement: - NR UL SRS is used as a starting point for design and analysis of UL PRS - Further study if and which enhancements are needed - FFS: NR UL PRS relationship with UL BWP and component carrier Agreement: FFS: At least the following aspects for NR UL PRS design - Use of UL beam sweeping at FR2 - Beam sweeping includes possibility of quasi-omni transmission - Use of UL beam alignment at FR2 through DL reception and beam correspondence - UL Power control aspects - UL timing advance aspects Agreement: The following measurements for NR UL PRS at serving and neighbor gNBs should be supported: - UL RTOA measurements - UL Angle Of Arrival (AOA) measurements (including Azimuth and Zenith Angles) - UL RSRP (reference signal received power) measurements - gNB RX-TX time difference measurements

According to this agreement, UTDOA will be supported in NR. In this contribution, we provide our consideration on UTDOA in NR from RAN2 perspective in the stage-2 aspects and RRC configuration.

Discussion

According to [1], the uplink positioning include two procedures: uplink information request and delivery procedure and uplink positioning information update procedure. The Information Request procedure for Uplink positioning, which is shown in Fig. 1, is used by the E-SMLC to obtain measurement results from the LMU which it uses to calculate a position for the UE. To do this, it needs to first indicate to the serving eNB the need to configure the UE to transmit SRS (see 5.2.2) signals and retrieve target UE configuration data from the eNodeB.

Figure 1 Uplink information request procedure

Uplink Positioning Information Update Procedure is used to notify the E-SMLC of a change to the UE SRS transmission configuration for one or more cells with respect to a previously sent UTDOA INFORMATION RESPONSE message. If the configured SRS transmission for the target UE has been modified, or if a handover of the target UE has been triggered, the eNB uses the Uplink Positioning Information Update procedure to inform the E-SMLC.

Figure 2 Uplink Positioning Information Update Procedure

According to the agreement in the last RAN1 meeting, both use of UL beam sweeping at FR2 and use of UL beam alignment at FR2 through DL reception and beam correspondence are supported. For beam sweeping based UL PRS design, the UTDOA procedure is similar to LTE and the current mechanism can work because beam sweeping based NR SRS configuration has been supported in NR specs.

Observation 1: For beam sweeping based UL PRS design, the current UTDOA procedure could work.

So, the current UTDOA procedure could be as a baseline of UTDOA in NR.

Proposal 1: UTDOA procedure in LTE could be used as the baseline of UTDOA in NR．

To achieve beam alignment at FR2 through DL reception and beam correspondence, LUM should be capable of transmitting DL signal and then UE can decide UL PRS beam based its receiving DL beam. Moreover, UL PRS configuration should support associating UL PRS beam to DL signal beam of non-serving gNB, but this is not supported in current NR specs. Hence, it is proposed to support the configuration of associating UE’s UL PRS (i.e. SRS) beam to DL signal beam of non-serving gNB in NR.

Proposal 2: To achieve beam alignment at FR2 through DL reception and beam correspondence, NR should support the configuration of associating UE’s UL PRS (i.e. SRS) beam to DL signal beam of non-serving gNB.

In SRS configuration, pathlossReferenceRS that includes a ssb-Index or a csi-RS-Index is optionally configured per SRS-ResourceSet. The UE calculates DL pathloss from the indicated SSB or CSI-RS and uses it to determine the SRS transmit power towards a serving cell. Unlike the currently supported SRS usages in NR specs, a SRS resource for positioning usage may need to be received at a non-serving gNB. In such a case, the SRS transmit power should be determined based on the pathloss of a DL RS that is transmitted from the said non-serving gNB. As such, NR should support the configuration of DL pathloss RS of non-serving gNBs.

Proposal 3: To determine the required UL PRS (i.e. SRS) transmit power for non-serving gNBs, NR should support the configuration of DL pathloss RS of non-serving gNBs.

Conclusion

In this contribution, we provide our consideration on UTDOA in NR from RAN2 perspective, and we get the following observation and proposals:

Observation 1: For beam sweeping based UL PRS design, the current UTDOA procedure could work.

Proposal 1: UTDOA procedure in LTE could be used as the baseline of UTDOA in NR．

Proposal 2: To achieve beam alignment at FR2 through DL reception and beam correspondence, NR should support the configuration of associating UE’s UL PRS (i.e. SRS) beam to DL signal beam of non-serving gNB.

Proposal 3: To determine the required UL PRS (i.e. SRS) transmit power for non-serving gNBs, NR should support the configuration of DL pathloss RS of non-serving gNBs.

References

[1] 3GPP TR 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN ".