3GPP TSG-RAN2 Meeting #105	R2-1901315

# Athens, Greece, 25 Feb – 1 Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 3893 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FailureInformation for Scell failure for MR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The use of FailureInformation message in case of radio link failure on a SCell has been agreed so fare only for the EN-DC option. However, this can be easily extended (and used) also for NE-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The use of FailureInformation message is extended to cover NR-DC. - 5.6.21.3 Action related to transmission of FailureInformation message - Added procedural text for sending the FailureInformation message in case of NE-DC Impact analysys If the network implements the CR and the UE does not, the network will not be informed that a failure has been detected on a SCell thus leading to wrong action. If the UE implements the CR and the UE does not, upon detecting an RLF on a SCell there will be a wrong network behaviour or eventually a RRC re-establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Upon detecting an RLF on a SCell (i.e., in case we have CA duplication), this will trigger RRC re-establishment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.5.3 Action related to transmission of FailureInformation message |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.331 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.6.21.3	Actions related to transmission of FailureInformation message

When initiating the procedure according to 5.6.21.2, the UE shall:

1>	if the procedure is initiated to report RLC failure, set the contents of the FailureInformation message as follows:

2>	set logicalChannelIdentity to the logical channel identity of the RLC entity;

2>	set cellGroupIndication to the cell group where the RLC entity is located;

2>	set failureType to the type of failure that has been detected;

1>	if used to inform the network about a failure for an SCG RLC bearer:

2>	if UE is in NE-DC;

3>	submit the FailureInformation message via the NR MCG embedded in failureInformationSCG as specified in TS 38.331, clause 5.5.5.1 [82].

END OF CHANGES