3GPP TSG-RAN WG2 Meeting #105	R2-1901408

Athens, Greece, 25 Feb - 01 Mar 2019	R2-18xxxxx

Agenda item:	10.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Possibly contradicting note in TS 37.340 Annex A

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In RAN2 #103-bis, in the contribution [1] had illustrated all the possible cases where one RRC message include both RB removal and RB addition with the same drb-Identity and RAN2 agreed the restriction on RB removal and addition with the same drb-identity in a LTE RRC message is only applicable for the case where a single radioResourceConfigDedicated IE includes both of them in Rel-15.

Figure 1-1: Possible cases where one RRC message contains both RB removal and RB addition [1]

The picture above seems to assume that the bearer type change is only done from MCG bearer to/from SCG bearer. However, it does not seem to discuss how split bearer, where the DRB ID of a bearer will be present in both NR RadioResourceConfig (for PDCP config) and LTE RadioResourceConfigDedicated (for RLC&MAC config).

This contribution discusses the above issue.

# 2	Discussion

The question to be clarified is how the LCID change “via RLC bearer release and add (including RLC re-establishment)” in Stage 2 of TS 37.340 is achieved in Stage 3 of TS 36.331 LTE RRC, e.g. for MCG RLC leg handling in case of bearer type change from MN terminated split bearer to SN terminated split bearer.

As per Stage 2 annex of TS 37.340:

NOTE 1: For EN-DC and NGEN-DC MCG, NE-DC SCG: the MAC/RLC behaviour depends on the solution selected by the network. It can be PCell handover (for EN-DC and NGEN-DC) or PSCell change (for NE-DC), which triggers MAC reset and RLC re-establishment. Alternatively, the logical channel identity can be changed, either via RLC bearer release and add (including RLC re-establishment), or via reconfiguration of the RLC bearer with RLC-re-establishment. For EN-DC and NGEN-DC SCG, NE-DC MCG, NR-DC MCG and SCG: the MAC/RLC behaviour depends on the solution selected by the network. It can be reconfiguration with sync, with MAC reset and RLC re-establishment. Alternatively, the logical channel identity can be changed via RLC bearer release and add. NOTE 2: Void NOTE 3: For EN-DC and NGEN-DC: Re-establishment and release. For NE-DC and NR-DC: Release. NOTE 4: For NE-DC: Re-establishment and release. For EN-DC, NGEN-DC and NR-DC: Release.

As per Stage 3 of TS 36.331:

#### 5.3.10.3 DRB addition/ modification The UE shall: 1> for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment including the case when full configuration option is used): 2> if the concerned entry of drb-ToAddModList includes the drb-TypeLWA set to TRUE (i.e. add LWA DRB): 3> perform the LWA specific DRB addition or reconfiguration as specified in 5.3.10.3a2; 2> if the concerned entry of drb-ToAddModList includes the drb-TypeLWIP (i.e. add LWIP DRB): 3> perform LWIP specific DRB addition or reconfiguration as specified in 5.3.10.3a3; 2> else if drb-ToAddModListSCG is not received or does not include the drb-Identity value (i.e. add MCG DRB or MCG RLC bearer for EN-DC): 3> if pdcp-Config is received, establish a PDCP entity and configure it with the current MCG security configuration and in accordance with the received pdcp-Config; 3> if rlc-Config is received, establish an MCG RLC entity or entities in accordance with the received rlc-Config; 3> if logicalChannelIdentity and logicalChannelConfig are received, establish an MCG DTCH logical channel in accordance with the received logicalChannelIdentity and the received logicalChannelConfig; 3> if pdcp-Config is not received, after processing nr-RadioBearerConfig1 and nr-RadioBearerConfig2 if present in the RRCConnectionReconfiguration message which triggered the execution of the DRB addition/modification procedure, associate MCG RLC bearer with the NR PDCP entity associated with the same value of drb-Identity in the current UE configuration as specified in TS 38.331 [82]; 2> if a DRB was configured with the same eps-BearerIdentity (fullConfig or change to E-UTRA PDCP): 3> associate the established DRB with corresponding included eps-BearerIdentity; 2> else if the entry of drb-ToAddModList includes pdcp-config (establishment of bearer with E-UTRA PDCP): 3> indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers; 1> for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration (DRB reconfiguration): 2> if the DRB indicated by drb-Identity is an LWA DRB (i.e. LWA to LTE only or reconfigure LWA DRB): 3> perform the LWA specific DRB reconfiguration as specified in 5.3.10.3a2; 2> else if the concerned entry of drb-ToAddModList includes the drb-TypeLWA set to TRUE (i.e. LTE only to LWA DRB): 3> perform the LWA specific DRB reconfiguration as specified in 5.3.10.3a2; 2> if the concerned entry of drb-ToAddModList includes the drb-TypeLWIP (i.e. add or reconfigure LWIP DRB): 3> perform LWIP specific DRB addition or reconfiguration as specified in 5.3.10.3a3; 2> if drb-ToAddModListSCG is not received or does not include the drb-Identity value: 3> if the DRB indicated by drb-Identity is an MCG DRB or configured with MCG RLC bearer in EN-DC (reconfigure MCG RLC bearer for EN-DC or reconfigure MCG DRB): 4> if the pdcp-Config is included: 5> reconfigure the PDCP entity in accordance with the received pdcp-Config; 4> if the rlc-Config is included: 5> if reestablishRLC is received, re-establish the RLC entity of this DRB; 5> reconfigure the RLC entity or entities in accordance with the received rlc-Config; 4> if the logicalChannelConfig is included: 5> reconfigure the DTCH logical channel in accordance with the received logicalChannelConfig; NOTE: Removal and addition of the same drb-Identity in a single radioResourceConfigDedicated is not supported. In case drb-Identity is removed and added due to handover or re-establishment with the full configuration option, the eNB can use the same value of drb-Identity.

Observation 1: As per RAN2 discussion, the Stage 2 objective to achieve the LCID change should be via TS36.331::radioResourceConfigDedicated:: drb-ToReleaseList(for RLC bearer release)  and drb-ToAddModList (for RLC bearer add) with same DRB ID.

Observation 2: At the same time, TS36.331 5.3.10.3 has the limitation that “Removal and addition of the same drb-Identity in a single radioResourceConfigDedicated is not supported”.

Proposal 1: RAN2 to address how to resolve the contradiction between Stage 2 of TS 37.340 and Stage 3 of TS 36.331.

# 4	Summary

In this contribution, we have discussed a possible issue how LCID change “via RLC bearer release and add (including RLC re-establishment)” in Stage 2 of TS 37.340 is achieved in Stage 3 of TS 36.331 LTE RRC, e.g. for MCG RLC leg handling in case of bearer type change from MN terminated split bearer to SN terminated split bearer. We think there is a possible contradiction that disallows Stage 2 objective from being implemented by Stage 3.

Observation 1: As per RAN2 discussion, the Stage 2 objective to achieve the LCID change should be via TS36.331::radioResourceConfigDedicated:: drb-ToReleaseList(for RLC bearer release)  and drb-ToAddModList (for RLC bearer add) with same DRB ID.

Observation 2: At the same time, TS36.331 5.3.10.3 has the limitation that “Removal and addition of the same drb-Identity in a single radioResourceConfigDedicated is not supported”.

Proposal 1: RAN2 to address how to resolve the contradiction between Stage 2 of TS 37.340 and Stage 3 of TS 36.331.