3GPP TSG-RAN2 105                                           R2-1902123

Athens, Greece, 25th February- 1st March 2019

Agenda item:	11.4.2.1

Source:	Samsung

Title:	RLM based AS level link management for unicast in NR-V2X

Document for:	Discussion & Decision

# Introduction

In this contribution, we discuss the RLM based AS level link management for unicast in NR-V2X based on the the report of the email discussion [104#56] in [1].

# Discussion

According to the outcome of the email discussion [104#56] in [1], the following proposals on RLM/RLF were made:

Proposals:

1:	SL RLM/RLF based AS level link management is supported.

2:	Uu RLM model is considered as baseline for SL RLM.

3:	As a baseline, after AS level link failure is declared, the SL AS level link is kept and recovered by AS layer.

4:	The AS level link status (e.g., failure and recovery) should be informed to upper layer. The detailed information exchanged between layers should be decided together with SA2.

5:	If SL RLC AM is supported for unicast, RLF declaration could be triggered by indication from RLC that the maximum number of retransmissions has been reached.

It needs to be discussed how the SL AS level link is recovered by AS layer. One method discussed in [1] is to adopt the RRC connection re-establishment procedure for SL as Uu. The main intention of the RRC connection re-establishment procedure is to find available radio resources i.e. cell. If this principle is applied to SL, SL AS recovery is to find available SL resources for unicast. So, the PC5-RRC AS-layer configuration including SL radio resource allocation/configuration can be considered for SL AS link recovery.

Proposal 1: RAN2 to study at least PC5-RRC AS-layer configuration for SL AS link recovery. Further details can be discussed in WI stage.

# Conclusion

Based on the above, RAN2 is requested to discuss and if possible agree on the following proposals:

Proposal 1: RAN2 to study at least PC5-RRC AS-layer configuration for SL AS link recovery. Further details can be discussed in WI stage.

# References

[1] R2-190xxxx, “Summary of 104#56NRV2X AS level link management for unicast (Apple).”