3GPP TSG-RAN2 105                                           R2-1902124

Athens, Greece, 25th February- 1st March 2019

Agenda item:	11.4.2.2

Source:	Samsung

Title:	On Supporting Inter-RAT Resource Allocation and Configuration for V2X Sidelink

Document for:	Discussion & Decision

# Introduction

In this contribution, we first provide our understanding of the following proposal made at the outcome of the email discussion [104#59] in [1].

Proposal 1-15: It is suggested to discuss whether RAN2 can agree that UE configured with MR-DC receives V2X specific SIB only at PCell from MN RAT to acquire NR/LTE sidelink specific configuration e.g. resource pool configuration.

Then, we discuss how to support inter-RAT resource allocation for V2X sidelink.

# Discussion

It is necessary to clarify that the UE is in RRC_CONNECTED when it is configured with MR-DC.

Observation 1: When the UE is configured with MR_DC, the UE is in RRC_CONNECTED not either RRC_IDLE nor RRC_INACTIVE.

For a UE in RRC_CONNECTED regardless of whether it is configured with MR-DC, the network can provide resource allocation and configuration via RRC-dedicated signalling. As discussed in our companion contribution [2], for a UE configured with MR-DC, it seems more appropriate to provide resource allocation and configurations for multi-RAT sidelink specific configuration if NR/LTE cells are connected to 5GC. So, it does not need to acquire NR/LTE sidelink specific configuration e.g. resource pool configuration for a UE configured with MR-DC via V2X specific SIB.

Proposal 1: When a UE is configured with MR-DC, the UE does not need to receive V2X specific SIB to acquire NR/LTE sidelink specific configuration.

For a UE in either RRC_IDLE or RRC_INACTIVE, the LTE/NR cell connected to 5GC should provide inter-RAT sidelink resource configuration via V2X-specific SIB(s).

Proposal 2: Both LTE and NR cells connected to 5GC should provide both LTE and NR V2X SL configurations via V2X-specific SIB(s) for UEs in RRC_IDLE and RRC_INACTIVE.

# Conclusion

Based on the above, RAN2 is requested to discuss and if possible agree on the following proposals:

Observation 1: When the UE is configured with MR_DC, the UE is in RRC_CONNECTED not RRC_IDLE and RRC_INACTIVE.

Proposal 1: When a UE is configured with MR-DC, the UE does not need to receive V2X specific SIB to acquire NR/LTE sidelink specific configuration.

Proposal 2: Both LTE and NR cells connected to 5GC should provide both LTE and NR V2X SL configurations via V2X-specific SIB(s) for a UE in RRC_IDLE and RRC_INACTIVE.

# References

[1] R2-19xxxxx, Report of [104#59][NR/V2X] Resource allocation (LG)

[2] R2-1901783, RRC-dedicated signalling based resource allocation/configuration in MR-DC, Samsung