# 3GPP TSG-RAN WG2 Meeting #104 	R2-1902148

# Athens, Greece, 25 Feb - 1 Mar 2019	Revision of R2-1817910

Agenda item:	12.1.2, 12.2.2

Source: 	Qualcomm Incorporated

Title: 	Further discussion on MT-initiated DL EDT

Document for:     Discussion/Decision

# Introduction

The WI description for additional MTC enhancements for LTE is given in [1]. The WI description for additional NB-IoT enhancements is given in [2]. One of the common objectives of both of the WIs is:

Improved DL transmission efficiency and/or UE power consumption: - Specify support for mobile-terminated (MT) early data transmission (EDT) [RAN2, RAN3]

In this contribution, we discuss some aspects of the MT EDT. This document is revision of R2-1817910 considering the discussion and agreements in RAN2#104 [3] and further explaining some aspects of our input to the email discussion [4].

# Background

Rel-15 enhancements support MO-EDT for both CP and UP CIoT EPS optimizations (also known as CP-EDT and UP-EDT, respectively). Release 15 already support DL EDT (i.e., EDT for MT data) if the UL EDT has been initiated. That is, DL data in msg4 is already possible. This is supported for both CP-EDT and UP-EDT. However, what is not supported is MT-initiated EDT when UL EDT is not initiated by the UE. Therefore, RAN2#103bis agreed on the following [3]:

=> RAN2 intends to support MT-initiated EDT for both CP and UP solutions. => The intention to use MT-EDT is for user data, i.e. not for NAS signalling.

Further, in the last RAN2#104 meeting, several MT-EDT solution options were proposed and following was agreed:

- MT EDT are evaluated at least based on battery life, network resource efficiency, security, reliability and potential impact on core network. - MT-EDT is intended for DL data which can be transmitted in one transport block. - Use cases that require DL data transmission with or without UL data transmission as a response should be supported for MT-EDT.

# Solution options for MT-initiated EDT

In a MT call for an IDLE UE, the first downlink message is the paging message from the network. Once the UE receives paging message (i.e., when a paging record corresponding to UE’s NAS identity is present), it initiates random access procedure. Therefore, the MT-initiated early data transmission in DL can be achieved by various options.

The following four options were captured in chair’s notes from RAN2#103bis [3], which were also part of email discussion [104#49] [4] with slightly different names:

- MT data in paging message (i.e., Paging-A in email discussion)

- MT data scheduled in paging message (i.e., Paging-B in email discussion)

- MT data after paging message and PRACH preamble transmission (i.e., “DL data after preamble” in email discussion [4])

- MT data in Msg4 (i.e., Msg4-A in email discussion)

In the following, we discuss potential solutions assuming CP-EDT as baseline (i.e., MT data encapsulated in NAS PDU). Specific differences between CP and UP cases may be discussed once a general framework is first agreed.

For reference, existing MT data transport using NAS in CP CIoT EPS Optimization is shown in the Figure 1 below (from TS 23.401). In the call flow, step 5 can potentially include all Msg1-Msg5 from legacy RACH procedure. Our objective here is to optimize steps 5 + 14 as well as move step 4 & part of 5 to later and skip steps 6 to 11 in current procedure. The proposed (simplified) call flows for different options are discussed below.

Figure 1 MT Data transport in NAS PDUs (Figure 5.3.4B.3-1 in TS 23.401).

Note that in Figure 1, the steps with dashed-lines starting at step 16 represent the procedure assuming UL data exists at the UE. In case of no UL data, all steps with dashed-lines after step 15 could be skipped.

Note also that, at the end of this procedure, UE would be in RRC_CONNECTED state, and further steps are necessary for RRC connection release which includes multiple signaling steps not shown here before the UE can move to IDLE.

## Paging-A: DL data in paging message

It should be possible to include additional payload in paging record for the UE. The payload would be included as a NAS message. The advantage is that there is no need to have a full random access procedure. Only Msg1 transmission may be needed which can be used as an ACK for MT data.

Furthermore, RAN1 impact may be minimal with such a solution. However, the challenge is that the paging load may significantly increase since each paging record could be large. The proposed call flow with small DL data in paging is shown below in Figure 2.

Figure 2 DL data in paging message

The steps are briefly described below:

1) S-GW receives MT data. It needs to know that the UE for which it is directed supports MT data over paging.

2) The S-GW sends a “MT-EDT request” to the MME along with the data to transmit. The MME buffers this data.

Note, if the “data” is not included in step 2, it may be sent after 3a separately (which corresponds to step 11 in existing procedure).

3a) In case of MME overloading, it can reject the request. Otherwise it confirms it.

A) The MME encapsulates the data in a NAS PDU and prepares a paging message.

Note, this corresponds to step 12 in current procedure.

3b) MME sends the paging msg (with data) to the eNBs in the tracking area.

Note, this can be viewed as steps 3 and 13 combined in the current procedure. Also note, paging record includes ue-Identity which can be s-TMSI or IMSI. Additionally, since the paging record may be sent prior to AS security activation, the NAS PDU needs to be protected by NAS security.

B)  Upon reception of the paging message, the eNB determines a PRACH resource for contention-free random access and include it in the paging message.

4)  The UE monitors for paging. If the UE decodes paging containing MT data, the UE delivers the PDU to NAS.

Note, this corresponds to existing steps 4 and 14 combined.

C) The eNB monitors for PRACH transmission from the UE.

5) If the MT data reception was successful in step 4, the UE transmits the dedicated PRACH preamble which indicates ACK for the MT data.

Note that steps B, (part of) 4, C and 5 together correspond to step 14 in current procedure. In addition, step 5 in proposed procedure also corresponds to part of step 5 in current procedure.

6) eNB ACKs the MME about successful transfer of DL data, which corresponds to step 15 in existing flow.

Note that in the proposed method, the UE starts from IDLE state, and at the end of the procedure continues to be in RRC_IDLE, thereby avoiding the costly procedure of RRC connection release.

## Paging-B: RNTI in paging message

Including the actual payload in paging message may increase the size of paging message and hence the paging load significantly. This means the UEs for which the paging is not intended may also need to decode the large paging messages. Therefore, it may be reasonable to include a RNTI information in the paging record instead of the actual payload. After receiving such information, the UE monitors MPDCCH scrambled with a given RNTI. The PDSCH contains a NAS PDU. This is illustrated in the following figure.

The paging message may include contention free PRACH which can be used as ACK for the DL as an option to PUCCH based ACK.

Figure 3 RNTI in paging message

## DL data after preamble

The main issue with the above solutions is that all the eNBs where the UE will be paged will need to allocate large PDSCH (either to send paging message with payload or to send payload in a separate PDSCH assignment). This can lead to significant resource wastage at the eNB as well as high backhaul load between MME and eNBs (since the payload needs to be sent to all eNBs in the paging area).

To overcome this, MME can indicate to the eNBs in the paging area that MT-EDT is required for a certain UE. Paging record for the UE can indicate that MT-EDT is intended along with dedicated PRACH resource (preamble) and RNTI. The UE after receiving the page uses the PRACH resource to transmit the dedicated preamble.

In DL data after preamble, dedicated PRACH preamble is used to avoid preamble collision.

Only the eNB that receives the PRACH preamble needs to retrieve the NAS PDU from MME and respond with downlink transmission using the RNTI. This way while PRACH preamble and RNTI would be reserved in all the eNBs that page the UE, only one eNB will need downlink PDSCH resources for the actual payload.

In DL data after preamble, only one eNB need to allocate PDSCH resources for the actual payload.

This is illustrated in the following figure.

Figure 4 DL data after preamble transmission

## Msg4-A: DL data in Msg4 based on MO-EDT

In this option, the paging message should indicate that paging is for MT EDT (i.e., MT-initiated DL EDT), otherwise it is not clear whether UE should use EDT or non-EDT procedure.

Other steps are similar to Rel-15 MO EDT, as depicted in figure below.

After UE uses EDT PRACH in step 5, network provides and EDT grant which is generally larger than the UE may need. Therefore, unnecessary padding is required in Msg3 UL. Note, the RRC message and contents of Msg3 depend on CP or UP solution; however padding will be required in both cases.

Figure 5 DL data in Msg4 based on MO-EDT

# Comparison of different solution options

Among the options above, the first option may have high paging overhead, unless the payload is very small. The second option may need many eNBs to allocate PDSCH resources for a single UE in IDLE mode as the network would not exactly know where the UE is camped. First and second options can be suitable for stationary/static UEs.

DL data in paging message may increase paging overhead unless the payload is very small.

DL data in paging message or RNTI in paging message may be suitable for stationary/static UEs.

The fourth option does not provide any gain in terms of amount of signalling steps compared to Rel-15 DL EDT.

The main strength of option 4 is that at the first look it may look as the simplest solution with the least specification impact as most of the steps are similar to Rel-15 EDT; however, after careful analysis, that may not be the case. For example, at least the MT-EDT indication needs to be added in the paging message as described above, and solution to avoid overscheduling would need to be specified as described next.

One of the disadvantages of option 4 is that UE would be provided with unnecessarily large EDT grant, hence padding is required to be transmitted in the UL, resulting in unnecessary power consumption at UE side and resource wastage at network side.

DL data in Msg4 causes overscheduling for Msg3 requiring UL padding which results in UE power consumption and resource wastage.

It may be argued that MT-EDT specific TBS can solve this issue. But that is not entirely correct because at this step of the procedure eNB does not know whether the UE is intending MO-EDT or is responding to MT-EDT.

MT-EDT specific TBS does not solve the problem of unnecessary Msg3 padding in option 4.

Additionally, based on Rel-15 experience, this involves several LS exchanges with RAN1 as it introduces further work in RAN1 side, which nullifies the main strength of option 4 (spec impact).

It is unnecessary and undesirable to introduce MT-EDT specific TBS for Msg3.

It may also be argued that Msg3 overscheduling can be solved by using legacy (non-EDT) Msg1 and Msg2 for MT-EDT, e.g., see [5]. However, the problem with such proposal is that network does not know whether the UE is ready to receive DL data in msg4. Note that according to the procedure for legacy RRC suspend/resume (without Rel-15 EDT), the UE cannot receive DL data in msg4 (in RRCConnectionRelease or RRCConnectionResume), because the radio bearers are not resumed yet. So, the UE must follow Rel-15 EDT procedure for being able to receive DL data in msg4. For this, UE needs to be aware that MT-EDT is being performed.

For DL in Msg4 based on MT-EDT to work, the UE should be aware that MT-EDT is being initiated by the network.

Legacy Msg1 and Msg2 cannot be used for MT EDT.

As summarized in Table 1 below, DL data after preamble transmission is the most efficient one considering the trade-off between UE power consumption (number of messages involved for small data transmission) and resource efficiency from network perspective. In addition, above methods can be achieved with enhancements mostly handled by RAN2 keeping the impact minimal to other WGs.

DL data after preamble transmission is a reasonable trade-off for MT EDT.

Table 1 Comparison of different MT EDT solution options

| Option | Pros | Cons |
| --- | --- | --- |
| 1. Paging-A | - Most optimal solution from power consumption point of view without considering impact to other UEs. - Best suited for use cases with very small payload and stationary/static UE | - High downlink radio resource usage, especially where eNB does not know current coverage level. - Multiple eNBs need to transmit larger paging message in multiple cells if the UE is not known to be stationary/static - Moderate specification impact - Increased power consumption for all other UEs, i.e., network-wide average UE power consumption is higher, not only in the cell where UE is camped, but potentially in nearby cells as well |
| 2. Paging-B | - Significantly lower power consumption as number of signalling steps are greatly reduced - Best suited for stationary/static UE | - Multiple eNBs need to allocate DL resources in multiple cells if the UE is not known to be stationary/static - Moderate specification impact |
| 3. DL data after preamble | - eNB allocates DL resources only in the cell the UE is camping on. - Avoid broadcasting heavy payload in multiple cells - Improved power consumption as number of signalling steps are reduced - Improved power consumption as PRACH preamble collision is avoided as dedicated preamble is used | - Moderate specification impact - Need to reserve dedicated PRACH resources |
| 4. Msg4-A | - Minimum spec changes anticipated among the solution options. | - Potentially wasted PDSCH resources due to overscheduling of Msg3. - Potentially increased UE power consumption due to need of padding tx in Msg3 - Delay and power consumption to deliver MT data no better than Rel-15 MO EDT. |

One commonality of all the above options is that the MT-initiated DL EDT relies on MT-EDT indication in paging message. Therefore, we propose:

MT-EDT indication is included in paging message.

Based on the above discussion and comparison of different solution options on the table, we propose RAN2 to agree that:

DL data after preamble transmission is baseline for MT EDT.

Further details can be discussed regarding CP-EDT and UP-EDT cases once the general option is agreed upon.

# Conclusion

In this contribution, we discussed some aspects of support for MT EDT with paging enhancements. Based on the above discussion, we made the following observations:

Observation 1.	In DL data after preamble, dedicated PRACH preamble is used to avoid preamble collision.

Observation 2.	In DL data after preamble, only one eNB need to allocate PDSCH resources for the actual payload.

Observation 3.	DL data in paging message may increase paging overhead unless the payload is very small.

Observation 4.	DL data in paging message or RNTI in paging message may be suitable for stationary/static UEs.

Observation 5.	DL data in Msg4 causes overscheduling for Msg3 requiring UL padding which results in UE power consumption and resource wastage.

Observation 6.	MT-EDT specific TBS does not solve the problem of unnecessary Msg3 padding in option 4.

Observation 7.	It is unnecessary and undesirable to introduce MT-EDT specific TBS for Msg3.

Observation 8.	For DL in Msg4 based on MT-EDT to work, the UE should be aware that MT-EDT is being initiated by the network.

Observation 9.	Legacy Msg1 and Msg2 cannot be used for MT EDT.

Observation 10.DL data after preamble transmission is a reasonable trade-off for MT EDT.

Based on the discussion and observations, we propose:

Proposal 1.	MT-EDT indication is included in paging message.

Proposal 2.	DL data after preamble transmission is baseline for MT EDT.

# References

- RP-181878, Revised WID: Additional MTC enhancements for LTE, Ericsson

- RP-181674, Revised WID: Additional enhancements for NB-IoT, Huawei

- R2-1818524, Report from Break-Out Session, Session Chair (Ericsson)

- R2-1901143, Email discussion report for [104#49] MT EDT, Huawei

- R2-1816634, Mobile-terminated early data transmission, Ericsson