3GPP TSG-RAN WG2 Meeting #105	R2-1902232

Athens, Greece, 25th February - 1st March 2019

Source: 	R2 Vice Chairman (Mediatek)

Title:	Report from session on NR UP, IAB WI, NR-U WI, NR IIoT SI

## 10.3	Stage 3 user plane

Documents in this agenda item will be handled in the NR user plane break out session

Essential functional corrections will be prioritized. For clarity and consistency enhancements, please pre-discuss and seek support with the TS rapporteur.

### 10.3.1	MAC

#### 10.3.1.1	MAC general aspects

Corrections related to BWP and SUL and general issues

R2-1901009	Miscellaneous corrections	Samsung	CR	Rel-15	38.321	15.4.0	0603	-	F	NR_newRAT-Core

- 	There are comments on PHR, P̃CMAX,f,c should be removed from the TS

- 	Samsung clarifies that no functional change is intended

Revised in R2-1902350

Comeback (101)

R2-1902350	Miscellaneous corrections	Samsung	CR	Rel-15	38.321	15.4.0	0603	1	F	NR_newRAT-Core

agreed

R2-1901010	How to handle DL/UL resource activated/deactivated by MAC CEs upon BWP/SCell deactivation	Samsung	discussion	Rel-15	NR_newRAT-Core

P1:

- 	ZTE think that for BWP deactivation the resource state should be suspended. Same thing for P4 and 5.

Noted

R2-1901012	Clarification on DL/UL resource activated/deactivated by MAC CEs upon BWP/SCell deactivation	Samsung	CR	Rel-15	38.321	15.4.0	0605	-	F	NR_newRAT-Core

- 	Nokia wonders then what happens as activation of Scell. Samsung think explicit activation is needed.

- 	Nokia would like some time to check. Huawei also would like to check.

Time to check. Comeback.

- 	Samsung reports after checking that we should leave this to R1

Not pursued

R2-1901011	Clarification on PUSCH resource for semi-persistent CSI resource upon deactivation of BWP	Samsung	CR	Rel-15	38.321	15.4.0	0604	-	F	NR_newRAT-Core

- 	LG think this can be reactivated acc to R1 specification. Samsung think that “clear” means that RRC configuration is still kept, and reactivation is based on RRC config. ZTE think “clear” is correct. Huawei think this is already clear in R1 TS and not needed in R2 TS. Samsung think this could be regarded as editorial. Ericsson think this is not needed.

- 	Ericsson think that the word “cleared” is different than R1 terminology “suspend” and we then need to explain this somehow fi we have this.

- 	LG also think this is confusing and think for MAC-only cleared, we need to change other terminology.

Can check offline if ok to merge into rapporteur CR, and whether a terminology can be agreed.

- 	After offline Samsung reports that it would be ok to only rely on R1 text.

Not pursued

R2-1902134	Correction on the time of activation for BWP	Huawei, HiSilicon	CR	Rel-15	38.321	15.4.0	0635	-	F	NR_newRAT-Core

- 	ZTE think the first part is not needed. LG agrees, QC too

- 	LG think the intention of the second change is ok but would like to change the wording. QC think the second change is not needed either, timing is not in the scope of the MAC. Lenovo agrees. Ericsson too.

Not pursued

Timer Start

R2-1901816	CR on 38.321 for Clarification for starting point of MAC timers	LG Electronics Inc.	CR	Rel-15	38.321	15.4.0	0628	-	F	NR_newRAT-Core

Not pursued

R2-1901918	Discussion on ambiguity in starting instance of MAC timer	Huawei, HiSilicon	discussion	Rel-15	NR_newRAT-Core	R2-1818146

Noted

R2-1901919	Clarification on the starting instance of the MAC timers	Huawei, HiSilicon,	CR	Rel-15	38.321	15.4.0	0631	-	F	NR_newRAT-Core

Not pursued

R2-1901777	CR of 38.321 on the MAC timer	vivo	CR	Rel-15	38.321	15.4.0	0438	2	F	NR_newRAT-Core	R2-1818171

Tdoc above moved from 10.3.1.6

Not pursued

DISCUSSION on the above documents

- 	Several companies think the current text is clear as it refers to transmissions etc that obvisouly take place on a BWP UL or DL. QC would prefer a note.

- 	Chair wonders about drx-HARQ-RTT-TimerDL.

- 	Huawei think there are also other timers, e.g. BWP and Scell inactivity timers, and it should al be aligned.

- 	LG think that the applicability becomes too wide if we have a general note acc to Huawei CR.

- 	Vivo agrees with LG.

- 	Ericsson do not support a note.

- 	Huawei wonders about the inactivity timers.

- 	Ericsson think that for drx-HARQ-RTT-TimerDL we can clarify. Nokia think that for this case it is clarified in RRC field description. QC are not sure. Ericsson think it refers to DL.

- 	Chair: it seems that for the timers discussed, the intended behaivour is aligned with the proposals, e.g. the Huawei note. But there is no willingness to clarify in the TS, as it should be obvious.

Noted, Not pursued

MTTD

R2-1902199	Handling of exceeding MTTD	Google Inc.	discussion	Rel-15

- 	Vivo think R4 do not need to fix this.

- 	Nokia think this can be brought to R4 directly. Lenovo agrees.

- 	Google think that for EN-DC R4 decided to not stop the transmission.

- 	Ericsson think that R4 may need to change something but not R2

Noted

Chair: it seems there might be an inconsistency, e.g. MAC referring to a case that doesn’t exist. Suggestion to bring this up in R4 for clarification there.

R2-1900434	Clarification on handling of exceeding MTTD	Google Inc.	CR	Rel-15	38.321	15.4.0	0597	-	F	NR_newRAT-Core

- 	Chair wonders if there really is anything in R4 TS then, given previous discussion.

Postpone

R2-1902136	Correction on Time Alignment	Huawei, HiSilicon	CR	Rel-15	38.321	15.4.0	0637	-	F	NR_newRAT-Core

Moved from 10.3.1.8

Not treated

Text enhancement

R2-1901243	Clarification on MAC timer running and stopping	Huawei, HiSilicon	CR	Rel-15	38.321	15.4.0	0611	-	F	NR_newRAT-Core

- 	LG think this is not needed.

Not pursued

#### 10.3.1.2		Random access

Corrections related to random access procedure, except multi-beam aspects

RSRP

R2-1900127	Measurement to select UL Carrier for Random Access_Option 1	Samsung Electronics Co., Ltd	CR	Rel-15	38.321	15.4.0	0529	1	F	NR_newRAT-Core	R2-1816309

- 	QC and LG think that the current text leaves this for UE implementation. Huawei agrees. Lenovo think that pathloss reference is used for power control and should be clear in R1 TS. Vivo think this is not clear in R1 TS for Idle UEs.

- 	Samsung think the network need to know the measurement.

- 	Ericsson wonders about backwards compatibility.

Not pursued

R2-1900128	Measurement to select UL Carrier for Random Access_Option 2	Samsung Electronics Co., Ltd	CR	Rel-15	38.321	15.4.0	0530	1	F	NR_newRAT-Core	R2-1816310

Not treated

R2-1901246	Corrections on RSRP threshold in RACH	Huawei, HiSilicon	CR	Rel-15	38.321	15.4.0	0613	-	F	NR_newRAT-Core

R2-1902655	Corrections on RSRP threshold in RACH	Huawei, HiSilicon	CR	Rel-15	38.321	15.4.0	0613	-	F	NR_newRAT-Core

R2-1901736	On configuration of rsrp-ThrehsoldSSB	Intel Corporation	CR	Rel-16	38.331	15.4.0	0937	-	F	NR_newRAT-Core

R2-1902226	On configuration of rsrp-ThrehsoldSSB	Intel Corporation	CR	Rel-16	38.331	15.4.0	0937	1	F	NR_newRAT-Core

4 CRs above Not Pursued

DISCUSSION

- 	Samsung think that Intel CR implements the general assumption. Intel think there may be another threshold with similar problem.

- 	Huawei think that the network maybe doesn't want to provide this always.

- 	Nokia are ok with the Intel approach. LG as well.

- 	Ericsson think we should change in MAC, if we need to change at all. Most likely nothing is needed.

- 	LG think the current TS is according to the Huawei intention. QC agree that we should not do anything, and in particular not in RRC.

Offline (100), clarify what is the assumption, and whether something need to be clarified (Huawei).

- 	Ericsson think we don’t need to have “if configured” everywhere in MAC. Nokia agrees. LG think we need this, as we have behaviour in MAC.

- 	Intel are ok to not change network behaviour.

- 	CATT think we can capture something as a general note in the MAC TS.

- 	ZTE think the network will always configure this anyway

R2 don’t make the change to mandate the network to configure rsrp-ThresholdSSB

R2 don’t change MAC now, but assumes that in this case the condition is false when rsrp-ThresholdSSB is not configured.

Misc

R2-1901028	Remaining issue in random access procedure	ASUSTeK	discussion	Rel-15	38.321	NR_newRAT-Core

- 	LG think that T300 or RetxBSR timer will trigger recovery of this situation so it is not critical.

- 	Vivo think UE implementation can fix this.

- 	Ericsson wonders if the UE doesn’t need to SI in any case in a new cell, so wouldn’t the Si be requested first. ASUS think there is nothing specified about this.

- 	QC think the ASUS problem can only happen with a bad UE impl. Lenovo agrees. LG as well.

Noted

Not pursued, not essential.

R2-1901029	Remaining issue in random access procedure	ASUSTeK	CR	Rel-15	38.321	15.4.0	0607	-	F	NR_newRAT-Core

Not pursued

R2-1901248	Corrections on SpCell only parameters	Huawei, HiSilicon, Samsung (Rapporteur)	CR	Rel-15	38.321	15.4.0	0615	-	F	NR_newRAT-Core

- 	LG think that the condition should be for “if groupBconfigured is..” instead.

- 	Ericsson think this is not necessary.

- 	Rapporteur indicate that the word within brackets two places can be removed, to avoid inconsistency.

Not pursued

Remove the word within brackets, two places, in the rapporteur CR.

R2-1901253	Discussion on reserved values and bits with backoff	Nokia, Nokia Shanghai Bell	discussion	Rel-15	NR_newRAT

- 	LG think the reserved values should be interpreted as the highest value.

- 	Samsung think we can follow the LTE principle.

- 	QC think a reserved value means ignore, i.e. no backoff. Mediatek agrees, and nothing ned to changed in the TS.

- 	Ericsson think that longer values might be introduced, and then legacy UEs would apply zero.

R2 confirms that a UE that receives a reserved BI value will interpret this as “no backoff”

R2-1901254	Correction to reserved backoff parameter values	Nokia, Nokia Shanghai Bell	CR	Rel-15	38.321	15.4.0	0620	-	F	NR_newRAT

Not treated

R2-1902133	CR to 38.321 on RA-RNTI calculation	Huawei, HiSilicon, Qualcomm, Intel	CR	Rel-15	38.321	15.4.0	0634	-	F	NR_newRAT-Core

- 	Docomo think that the UE can get this from the RRC configuration. Huawei think that it is not specified that the UE shall apply the prach SCS and there could be a mistake that someone could use the BWP SCS. CATT wold be ok to clarify. ZTE think this is needed.

- 	Ericsson think this is not necessary. LG agrees and think that as t_id cannot exceed the physical limit it is obvious that this cannot go wrong.

- 	Ericsson would be ok to just refer to “clause 5.3.2 in TS 38.211 [8]”. Lenovo think we refer to this parameter already in the MAC TS so it is not new.

Revised in R2-1902368 (rev 1), to add arch option applicability in the coversheet.

R2-1902368 Agreed unseen

R2-1902135	Correction on RACH resources for CFRA on SCell	Huawei, HiSilicon	CR	Rel-15	38.321	15.4.0	0636	-	F	NR_newRAT-Core

- 	LG think this is not correct.

- 	Samsung think the proposed CR is correct.

- 	Nokia are ok to remove “for SpCell only” the rest is obvious. Ericsson agrees.

The change to remove “for SpCell only” is agreed.

Merged with the rapporteur CR.

Optimizations

R2-1901245	Corrections on the prioritized RACH	Huawei, HiSilicon	CR	Rel-15	38.321	15.4.0	0612	-	F	NR_newRAT-Core

- 	Samsung proposes to wait for a stringent definition of handover from the CP session, expected next meeting, and can consider to clarify then.

Not Pursued

R2-1900577	Corrections to RACH procedure for SI request	ZTE Corporation, Sanechips	CR	Rel-15	38.321	15.4.0	0598	-	F	NR_newRAT-Core

=> Revised in R2-1902287

R2-1902287	Corrections to RACH procedure for SI request	ZTE Corporation, Sanechips	CR	Rel-15	38.321	15.4.0	0598	1	F	NR_newRAT-Core

R2-1901247	Corrections on the RACH resource of SI request	Huawei, HiSilicon	CR	Rel-15	38.321	15.4.0	0614	-	F	NR_newRAT-Core

R2-1901244	Discussion on the handover and reconfiguration with sync	Huawei, HiSilicon	discussion	Rel-15	NR_newRAT-Core

4 tdocs not treated

TEI16 – Postpone August

R2-1901723	PRACH Prioritization Scope and Parameters for MPS	Perspecta Labs, OEC, AT&T, Sprint	discussion	Rel-16

Not Treated

#### 10.3.1.3		Multi-beam operation

Corrections related to multi-beam operation, beam failure detection, beam failure recovery.

R2-1900582	CR on reconfiguration of beamfailurerecovery during on-going RACH procedure	ZTE Corporation, Sanechips	CR	Rel-15	38.321	15.4.0	0600	-	F	NR_newRAT-Core

=> Revised in R2-1902289

R2-1902289	CR on reconfiguration of beamfailurerecovery during on-going RACH procedure	ZTE Corporation, Sanechips	CR	Rel-15	38.321	15.4.0	0600	1	F	NR_newRAT-Core

-  	LG think this is not needed, even if the UE do this the result is just some additional re-initializations. Panasonic agrees and think this is a corner case.

Not pursued

R2-1901249	Corrections on HARQ for Msg2 of CFRA BFR	Huawei, HiSilicon	CR	Rel-15	38.321	15.4.0	0616	-	F	NR_newRAT-Core

- 	LG thikn this is not needed, ZTE think the CR is wrong. Samsung clarifies that the MSG2 is PDCCH itself.

- 	Several companies think this is clear already in the TS. The payload part scheduled is not to be regarded to be RAR.

Not pursued

R2-1901250	Corrections on BFD reset	Huawei, HiSilicon	CR	Rel-15	38.321	15.4.0	0617	-	F	NR_newRAT-Core

- 	LG think this is similar to ZTE CR. This is a corner case. Ericsson agrees.

Not pursued

R2-1901255	Discussion on BFR with CBRA	Nokia, Nokia Shanghai Bell	discussion	Rel-15	NR_newRAT

- 	Nokia suggest to not treat this as R1 need to make corrections first.

- 	Vivo think it is difficult to discuss in R1 as L1 is agnostic to how the RACH is used.

Noted

R2-1901256	Correction to SSB selection with CBRA	Nokia, Nokia Shanghai Bell	CR	Rel-15	38.321	15.4.0	0621	-	F	NR_newRAT

Not treated

#### 10.3.1.4	PHR

Corrections related to PHR

R2-1901916	Correction on PH omitting of dynamic power sharing incapable UE	NTT DOCOMO, INC., Qualcomm Incorporated, Nokia, Huawei, MediaTek, CATT, NEC, Fujitsu	CR	Rel-15	38.321	15.4.0	0630	-	F	NR_newRAT-Core

- 	LG think this CR is not needed. Docomo explains that in the MAC CE format there is no possiblity to remove the Pcell.

Revised in R2-1902366 (rev 1), to add arch option applicability in the coversheet.

R2-1902366 Agreed unseen

R2-1901928	Clarification on PH omitting of dynamic power sharing incapable UE	NTT DOCOMO, INC., Qualcomm Incorporated, Nokia, Huawei MediaTek CATT, NEC, Fujitsu	CR	Rel-15	36.321	15.4.0	1433	-	F	NR_newRAT-Core

Revised in R2-1902367 (rev 1), to add arch option applicability in the coversheet.

R2-1902367 Agreed unseen

R2-1900581	CR on PHR for the UE without capability of power sharing	ZTE Corporation, Sanechips	CR	Rel-15	38.321	15.4.0	0599	-	F	NR_newRAT-Core

revised

R2-1902288	CR on PHR for the UE without capability of power sharing	ZTE Corporation, Sanechips	CR	Rel-15	38.321	15.4.0	0599	1	F	NR_newRAT-Core

- 	Docomo think this is not needed. Ericsson agrees. Lenovo too.

Not pursued

R2-1901778	Determination of PH Type 1 or Type 3	vivo	CR	Rel-15	38.321	15.4.0	0626	-	F	NR_newRAT-Core

- 	Nokia think this is already covered by selection virtual / real. Lenovo agrees. LG agrees it is not needed.

- 	Huawei think this is needed, as virtual PHR can be type 1 or type 3. Lenovo think this is clear in Ran1 TS. Vivo point out that R1 TS has different language.

- 	Huawei anyway thikn it matters for the network.

Not pursued

R2-1901779	PH value for SUL	vivo	discussion	Rel-15	38.321	NR_newRAT-Core

Noted

R2-1901780	PH value for SUL	vivo	CR	Rel-15	38.321	15.4.0	0627	-	F	NR_newRAT-Core

- 	Lenovo agrees that UL skip should not impact PH type.

- 	LG think this is clear already

- 	Nokia think that current text don’t refer to the payload transmission at all so the skip or non-skip has no impact.

Not pursued

#### 10.3.1.5 SPS and Configured Grant

Corrections related to Configured grant and SPS

R2-1901542	Correction of Configured Grant collision	Sequans Communications	discussion	NR_newRAT-Core

- 	LG think the current text is clear that DG is always prioritized and the CR is not needed. ZTE agrees. QC agrees.

- 	Huawei also think this was covered in the current TS, and the model is that each repetition in a bundle is a grant.

- 	CATT think the proposal for issues 1 and 2 are relevant.

Not agreed

R2-1901543	Correction of Configured Grant collision_Option1	Sequans Communications	CR	Rel-15	38.321	15.4.0	0623	-	F	NR_newRAT-Core

R2-1901544	Correction of Configured Grant collision_Option2	Sequans Communications	CR	Rel-15	38.321	15.4.0	0624	-	F	NR_newRAT-Core

2 tdocs not treated

#### 10.3.1.6	DRX

Corrections related to DRX

R2-1900852	Extension of DRX ACTIVE time	SHARP	discussion	NR_newRAT-Core	R2-1816879

- 	Vivo think the network knows the DRX state and can ensure that the UE goes to the correct beam. Nokia and QC think nothing is needed.

- 	Sharp think that it is likely that the network cannot send the reconfiguration immediately, as it would wait for a measurement.

- 	LG think that UE is in active time after RAR acc. to current TS. Sharp think there is no RAR, but just a PDCCH.

- 	There seems to be some sympathy there might be some issue, but no willingness to fix this now

Not agreed

R2-1900853	CR to TS 38.321 Extension of DRX ACTIVE time	SHARP	CR	Rel-15	38.321	15.4.0	0539	1	F	NR_newRAT-Core	R2-1816883

Not treated

R2-1901013	Correction on the stop of drx-RetransmissionTimer for the configured assignment/grant	Samsung	CR	Rel-15	38.321	15.4.0	0606	-	F	NR_newRAT-Core

Not pursued

R2-1901101	Clarification to DRX ambiguous period	CATT	CR	Rel-15	38.321	15.4.0	0609	-	F	NR_newRAT-Core

- 	LG think there is no problem with the current text.

Contents agreed, merge with the rapporteur CR

Initial State

R2-1901222	Discussion on the initial sate of DRX	Huawei, HiSilicon	discussion	Rel-15	NR_newRAT-Core

Noted

This issue was handled at a previous meeting. Nothing needed.

R2-1901223	Correction to the initial state of DRX	Huawei, HiSilicon	CR	Rel-15	38.321	15.4.0	0610	-	F	NR_newRAT-Core

R2-1901781	Short or long DRX cycle after RRC configuration	vivo	discussion	Rel-15	38.321	NR_newRAT-Core

2 tdocs not treated

PDCCH monitoring

R2-1901773	CR on 38.321 for Clarification to PDCCH monitoring	LG Electronics Inc., MediaTek, CATT, Qualcomm Inc.	CR	Rel-15	38.321	15.4.0	0549	1	F	NR_newRAT-Core	R2-1817347

Contents agreed, merged with rapporteur CR

R2-1901050	Physical Layer Limitations for PDCCH Monitoring	Nokia, Nokia Shanghai Bell	CR	Rel-15	38.321	15.4.0	0608	-	F	NR_newRAT

Tdoc above moved from 10.3.1.1

Not pursued

DISCUSSION on 2 above CRs

- 	Ericsson prefers 1773.

R2-1901920	Discussion on start of drx-onDurationTimer	Huawei, HiSilicon	discussion	Rel-15	NR_newRAT-Core

Noted

R2-1901921	Correction to start of drx-onDurationTimer	Huawei, HiSilicon	CR	Rel-15	38.321	15.4.0	0632	-	F	NR_newRAT-Core

- 	QC and Intel think this can be handled by network configuration. LG agrees, and the problem is very small.

- 	QC think that this is non-backwards compatible.

- 	LG point out that the “misconfiguration” is allowed in LTE and as long as network and UE has the same understanding there is no problem.

- 	Interpretation: When the TS says “start” a timer while timer is running then nothing will happen. LG thikn this is already captured.

- 	Chair: There is no support to fix.

Not pursued

#### 10.3.1.7	MAC PDU format

Corrections related to MAC PDU and MAC CE formats, if any

R2-1901251	Correction on the Format field in MAC subheader	Huawei, HiSilicon	CR	Rel-15	38.321	15.4.0	0618	-	F	NR_newRAT-Core

- 	LG think this is up to implementation but expects most UEs and gNBs to behave like this.

- 	QC think this is not needed

Not pursued

#### 10.3.1.8	Other

Other corrections on topics not included in the detailed agenda items, e.g. PDCP duplication, LCP, HARQ, SR, BSR.

R2-1900108	Correction to uplink transmission	Google Inc.	CR	Rel-15	38.321	15.4.0	0596	-	F	NR_newRAT-Core

=> Revised in R2-1902241

R2-1902241	Correction to uplink transmission	Google Inc.	CR	Rel-15	38.321	15.4.0	0596	1	F	NR_newRAT-Core

- 	LG are ok, but want to ask rapporteur, as there are other similar cases for Boolean parameters.

Contents agreed, merged with rapporteur CR

R2-1900796	Remaining issues on SCell activation/deactivation	ZTE Corporation, Sanechips	discussion	Rel-15	NR_newRAT-Core

Noted

R2-1901994	CR to the operation of SCell activation/deactivation	ZTE Corporation, Sanechips	CR	Rel-15	38.321	15.4.0	0633	-	F	NR_newRAT-Core

-> Revised in R2-1902290

R2-1902290	CR to the operation of SCell activation/deactivation	ZTE Corporation, Sanechips	CR	Rel-15	38.321	15.4.0	0633	1	F	NR_newRAT-Core

- 	Nokia think the behaviour at activation of an already activated sCell is deliberate and needed.

- 	LG think that the redundancy is not problematic.

Not pursued

R2-1900941	Correction on repetition transmission	Spreadtrum Communications	CR	Rel-15	36.331	15.4.0	3869	-	F	NR_newRAT-Core	Withdrawn

R2-1900953	Correction on repetition transmission	Spreadtrum Communications	CR	Rel-15	38.321	15.4.0	0602	-	F	NR_newRAT-Core

- 	Spreadtrum explains that this fixes an inconsistency between L1 and L2 TS.

Removal of the word “dynamic” is agreed, merged with the rapporteur CR.

R2-1901252	Correction on the BSR triggering SR	Huawei, HiSilicon	CR	Rel-15	38.321	15.4.0	0619	-	F	NR_newRAT-Core

R2-1901454	Corrections on SR Triggering	InterDigital	CR	Rel-15	38.321	15.4.0	0622	-	F	NR_newRAT-Core

DISCUSSION

- 	LG agrees with the intention but do not support.

- 	QC think that no change is needed.

- 	Samsung think that the second change is needed. The first one maybe not.

- 	QC think this is a behaviour change and changes the UE behaviour. Samsung think the intended behaviour is acc to the CR and there should be no change to UE implementations. Huawei agrees. MTK agrees as well.

- 	Nokia think that only for duplication case there would be multiple BSR triggered at the same time but for this case, it should be ok to only send one.

- 	IDT think that with QC interpretation there is more inconsistencies as it would be difficult to choose the SR configuration.

- 	Oppo agrees with Nokia and QC. Ericsson also don’t want to fix this now.

- 	Samsung think there are consequences for mixed traffic, e.g. URLLC + eMBB, and SR may not be properly triggered for such case for URLLC traffic.

- 	Chair: there is significant support to make the change, and the current TS seems somewhat inconsistent. However, the side effect of not fixing seems limited, and there is also significant resistance to clarify.

- 	Chair: There seems to be an inconsistency in the TS in the selection of SR resource but we don’t clarify this in this release.

2 CRs above Not pursued

R2-1901763	Correction to SR procedure under beam failure	Qualcomm Inc	CR	Rel-15	38.321	15.4.0	0625	-	F	NR_newRAT-Core

- 	LG think this is an optimization and not essential.

- 	Nokia prefers to not make this change.

- 	Ericsson wonders how long time the BFR will take, e.g. in relation to SR prohibit timer. QC think the time can be significant when CBRA is used.

- 	Nokia think SR + Grant may still work during BF. LG agrees.

- 	Lenovo has some sympathy.

- 	MTK think this is a corner case and would be ok to not do this.

- 	Chair: some interest but also opposition.

Not pursued

R2-1901886	SR_COUNTER initialization due to RRC reconfiguration	Fujitsu	CR	Rel-15	38.321	15.4.0	0629	-	F	NR_newRAT-Core

- 	LG think that we don’t need this

- 	Fujitsu explains that the conseq of not approved is that the SR_Counter value may be inherited instead of reset at reconfiguration, and may reach the max value faster then intended.

- 	QC don’t think this is needed, and think this would not be a problem for a UE implementation.

- 	Nokia think this is a corner case

Not Pursued.

### 10.3.2	RLC

Corrections related to RLC

R2-1901242	Corrections on RLC state variables	Huawei, HiSilicon	CR	Rel-15	38.322	15.4.0	0029	-	F	NR_newRAT-Core

- 	LG think this is not needed, and some part could possibly be a rapporteur CR.

- 	MTK think there are some very small editorials that can be corrected and the coversheet is not correct.

- 	Huawei think there is no “Side” to a RLC entity so the first correction is correct.

- 	Comments: 1st change is ok, 2nd change not needed, 3rd change ok, 4th change is ok, 5th change ok (the renumbering), last change “of the UMD PDU” is not needed.

- 	Comments: Cat shall be D, Interoperability statement shall indicate “no issue” for both cases. Conseq if not approved need to be changed.

Revision in R2-1902359 (Huawei)

R2-1902359	Corrections on RLC state variables	Huawei, HiSilicon	CR	Rel-15	38.322	15.4.0	0029	1	D	NR_newRAT-Core

agreed

### 10.3.3	PDCP

Corrections related to PDCP

R2-1901893	PDCP corrections	LG Electronics Inc. (Rapporteur), Huawei, HiSilicon	CR	Rel-15	38.323	15.4.0	0028	-	F	NR_newRAT-Core

- 	Samsung think that for PDCP control PDUs they are not stored after transmission and there is no ambiguity.

- 	Ericsson would be ok either way.

- 	MTK supports the change

- 	Samsung think this could lead to confusion for implementers

Not pursued

R2-1901239	Correction on the PDCP re-establishment for AM DRB	Huawei, HiSilicon	CR	Rel-15	38.323	15.4.0	0025	-	F	NR_newRAT-Core

- 	LG think this is a critical problem but the solution here is not ok.

- 	Samsung think there is no problem. QC also think there is no problem.

- 	LG think we already discussed PDU discard, and think this would be a simple solution. Huawei agrees that discard of data would be a good solution and we should discard the data at suspend.

- 	MTK think we need to think more about this. Ericsson would also like to check.

- 	Nokia think that the receive side is cleared at suspend. Possibly transmit side need additional action.

- 	Proposed resolution after some discussion: Discard remaining PDUs and SDUs at TX side at suspend

Comeback (109) after checking, revision in R2-1902360, Huawei

- 	Huawei think the main question is if we change the previous agreements to not discard PDCP SDUs in the TX.

- 	Chair wonders if the problem occurs only if PDCP SDUs are associated with SN.

Two possible ways:

- Discard SDUs in TX at suspend.

- Clarify that the current text on continue of SN do not apply to the RRC resume case.

- 	Ericsson want to follow the earlier agreement and have designed acc to this, i.e. do b). Nokia agrees. Mediatek agrees.

- 	Samsung suggest a), but would be ok with a note. LG also think a is best, and if b is adopted LG would prefer to change in the suspend section.

- 	Chair think that this is not super important, and that also a UE that would not follow this exactly in R15 would still work ok.

Clarify that the current text on continue of SN for SDUs stored at suspend (at TX side) do not apply to the RRC suspend-resume case, i.e. that PDCP essentially treat such SDUs as new SDUs.

[105#xx][NR UP] Correction on PDCP re-establishment for AM DRB (Huawei)

Intended outcome: Agreed CR

Deadline: 1 week

R2-1901241	Correction on PDCP SN length	Huawei, HiSilicon	CR	Rel-15	38.323	15.4.0	0027	-	F	NR_newRAT-Core

- 	LG and Ericsson think that Implementers will have implemented correctly and we shold regard this as a clarification.

Soften the interoperability statement

Revised in R2-1902361, which is agreed unseen

R2-1901240	Correction on the PDCP SR	Huawei, HiSilicon	CR	Rel-15	38.323	15.4.0	0026	-	F	NR_newRAT-Core

- 	Nokia think the network would not consider such PDCP SR.

- 	Samsung think this is an optimization and not needed. Ericsson agrees.

R2 assumes that additional SR at RRC resume is not a problem for the network.

Not pursued

### 10.3.4	SDAP

Corrections related to SDAP

R2-1901915	SDAP end marker clarification for new QoS flow configuration	Qualcomm Incorporated	discussion	Rel-15	37.324	NR_newRAT-Core

- 	ASUStek think the UE should not send the end-marker, but would prefer another change.

- 	LG think this makes sense, but the CR is late and not critical.

- 	QC think that if not clarified then different UEs may behave differently.

- 	MTK think it is better to not do this.

- 	Ericsson think that at the network side there is no problem if there is additional end-marker at configuration.

- 	Huawei think that from SDAP point of view the QoS flow already exist and is mapped to default bearer before the (re)configuration and the end marker should be sent ..

P1:

- 	Huawei clarifies that Option B is the current UE behaviour.

- 	QC think option A is the current UE behaviour.

On P1, Option B is the current UE behaviour

Noted

R2-1901930	Clarification on SDAP end-marker procedure	Qualcomm Incorporated	CR	Rel-15	37.324	15.1.0	0013	-	F	NR_newRAT-Core

## 11.1	Integrated Access and Backhaul for NR

(NR_IAB-Core; leading WG: RAN2; REL-16; started: Dec 18; target; Dec 19; WID: RP-182882)

Time budget: 2 TU

Documents in this agenda item will be handled in a break out session

### 11.1.1	Organisational

Including incoming LSs, draft TS, rapporteur inputs, etc

R2-1900809	IAB WI workplan	Qualcomm Inc	Work Plan	Rel-16

- 	Rapporteur reports that a SI has been started in SA3 but proposes to not wait for SA3 for security discussions.

Noted

R2-1900810	IAB baseline for 38300	Qualcomm Inc	CR	Rel-16	38.300	15.4.0	0141	-	B	NR_IAB-Core

- 	Chair think this should be a draft CR.

- 	Nokia think that the text goes beyond agreements.

- 	Samsung think that the TR also goes beyond agreements and proposes and email discussion.

- 	Huawei wonders if there are parts of the TP that should be done by RAN3.

- 	Chair think that e.g. the protocol pictures contain both R3 and R2 parts.

- 	QC proposes to have a joint email discussion with R3.

- 	Huawei wonder about terminology. We should agree something.

We capture progress in a running stage-2 CR

[105#xx][IAB] IAB running CR (Qualcomm)

Intended outcome: Agreed Running CR, Capture agreements from this meeting,

Deadline: One week

[105#xx][IAB] RAN2 and RAN3: IAB Miscellaneous (Qualcomm)

Intended outcome: Report, identify and capture “easy agreements” from the SI, agree on a name for the new protocol,

Deadline: Next meeting

R2-1900946	A better name for the IAB Adaptation layer	AT&T	discussion

- 	Huawei think that the naming should e.g. contain the word backhaul, e.g. “backhaul adaptation layer”, LG proposes to do this in the email discussion.

- 	QC think we should avoid the word “adaptation”

noted

R2-1901192	On the impact of IAB work on Core Network procedures and signalling	Samsung R&D Institute UK	discussion

noted

R2-1901193	Draft LS on IAB impact to CN	Samsung R&D Institute UK	LS out	To:SA2, SA5	Cc:CT1, SA3

- 	Ericsson think the LS is too early, and the LS is too high level.

- 	AT&T think we should wait until we have more clear progress. LG wonders whether multiple PDU sessions is needed in OAM LS.

- 	Intel wonders if there is a relation to SA3 SI. Intel think not.

Noted

R2-1902645	[Offline discussion 107] IAB email discussions Qualcomm

- 	Huawei think that Adapt entities should be discussed separately.

- 	Chair proposes to remove the part on entities

[105#xx][IAB] Routing (LG)

Intended outcome: Report, identify options, possible easy agreements, see also R2-1902645

Deadline: Next Meeting

What information needs to be held on the adapt header for routing, and how is this information used on the adapt layer entity 1) for the forwarding of PDUs from RX part to TX part, and 2) for performing of routing by the TX part (e.g., routing table with (routing Id, BH link Id)-pairs).

What identifiers would be best used for the table entries (e.g. IAB-node address for routing Id and C-RNTI for BH-link Id).

[105#xx][IAB] Bearer mapping (Ericsson)

Intended outcome: Report, identify options, possible easy agreements, see also R2-1902645

Deadline: Next Meeting

the TR defines the term “bearer mapping” as “UE-bearer to RLC-channel mapping” but this is too restrictive since it does not apply to F1-C. Further, F1-C messages of different type might also have different priorities and therefore use different RLC channels. This raises the following questions:

What are the criteria we want to allow for the mapping of an Adapt PDU to a BH RLC channel? (e.g. QoS, UP vs CP, PDU session, F1 connection, others).

What information must be available at the access-IAB-node to enable this mapping in upstream direction? (e.g. mapping table with (F1-connection Id, BH LCID)-pairs).

Which of these criteria require a re-mapping on intermediate IAB-nodes? (I don’t see any. May be I am wrong.)

What information must be available at the intermediate-IAB-node to enable this re-mapping? (e.g. none if remapping is not required).

What information needs to be carried in the adapt header to enable the remapping on the intermediate-IAB-node?

### 11.1.2	User plane aspects

Including consideration of adaptation layer, multi-hop RLC ARQ, scheduler and QoS impacts

Stage-2

R2-1901836	Multiple connectivity in IAB networks	Huawei, HiSilicon	discussion	Rel-16	NR_IAB-Core

DISCUSSION

P1 P2:

- 	Ericsson wonders whether we can have PDCP on an IAB link, as PDCP is just in the UE.

- 	QC support P1. QC think this is just baseline.

- 	Nokia think P1 is just normal DC.

- 	LG think that P2 is not needed, as duplication is not useful if there is a single path.

- 	Huawei think we can agree for single CU.

- 	Samsung think the discussion for access link and the IAB link is different.

- 	Chair: Companies think P1 and P2 are just as Rel-15 Uu, but it is not clear how this can be particularly useful in the IAB case.

P3:

- 	Ericsson think this could be difficult without PDCP.

- 	Nokia think this could be realized by just having two separate RLC-channels / RLC-bearers,

- 	not clear whether this is DC.

noted

R2-1900628	Route redundancy support with multi-connectivity	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_IAB-Core

DISCUSSION

- 	Nokia think Multi-MT can be done without standards impact.

- 	QC think P2 first part, is exactly supported in the WID, but wonders about the work-split

- 	KDDI think we cannot agree this now.

noted

R2-1901480	IAB protocol stack options	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_IAB-Core

- 	QC think this shows that there is indeed a solution with IPv4 and there is no problem. Option B seems to resolve the issue.

- 	QC think this should be decided by R3.

- 	LG think that R3 should discuss this and would like to not capture anything in R2.

- 	Huawei think to R2 the 1-to-1 bearer mapping need to GTP-U TEID ..

- 	Ericsson think there are other options.

- 	AT&T think IPv6 will be used

- 	Chair Comment (no time to discuss to conclusion): Most companies seems to think that the problem exists, and it seems to be resolved by option B. No consensus that this is the best solution. No consensus at this meeting to agree this in R2. Expect this can be discussed in R3

Noted

R2-1901827	Remaining issues of the UP protocol architecture	Huawei, HiSilicon	discussion	Rel-16	NR_IAB-Core

R2-1901419	IAB user plane aspects	Sony	discussion	Rel-16	NR_IAB-Core

R2-1901405	Issues to be decided in IAB Stage2 design	KDDI Corporation	discussion

L2 - Flow Control

R2-1901387	Flow Control in IAB networks	Ericsson	discussion

R2-1901835	Flow control for IAB networks	Huawei, HiSilicon	discussion	Rel-16	NR_IAB-Core

R2-1901393	Flow control mechanism for DL	Intel Corporation	discussion	Rel-16	NR_IAB-Core

R2-1902016	Downlink flow control mechanism in IAB	LG Electronics Inc.	discussion	Rel-16	NR_IAB-Core

R2-1901065	Overview of flow control solutions	Samsung	discussion	Rel-16	NR_IAB-Core

R2-1900221	Consideration of flow control	CATT	discussion

R2-1901549	Queue Management vs Flow Control for Congestion Handling	Sequans Communications	discussion	Rel-16	NR_IAB-Core	R2-1817337

R2-1902128	Discussion on flow control	ITL	discussion	R2-1815530

L2 - Flow Control UL

R2-1901394	Flow control mechanism for UL	Intel Corporation	discussion	Rel-16	NR_IAB-Core

R2-1902023	Consideration on uplink data congestion handling	LG Electronics Inc.	discussion	Rel-16	NR_IAB-Core	R2-1818059

L2 - QoS

R2-1901330	Time stamping for User Plane Latency Control	Ericsson	discussion	Rel-16	NR_IAB-Core

R2-1901323	QoS Mapping to Backhaul Bearers in IAB Network	Ericsson	discussion	Rel-15	NR_IAB-Core

R2-1902024	QoS and Fairness enforcement in IAB networks	Futurewei Technologies	discussion	Rel-16

L2 - MT-specific aspects

R2-1901828	Support of traffic terminated at MT of IAB node	Huawei, HiSilicon	discussion	Rel-16	NR_IAB-Core

R2-1902019	Consideration on RLC channel type for MT’s own traffic	LG Electronics Inc.	discussion	Rel-16	NR_IAB-Core

Adapt / L2 - Bearer Mapping

R2-1900243	Common design and QoS Handling for IAB	CATT	discussion

R2-1902015	QoS based many-to-one bearer mapping	LG Electronics Inc.	discussion	Rel-16	NR_IAB-Core

R2-1901826	Bearer mapping in IAB network	Huawei, HiSilicon	discussion	Rel-16	NR_IAB-Core

R2-1900977	Discussion on IAB bearer-to-RLC-channel mapping	ITRI	discussion	NR_IAB-Core

R2-1900562	Consideration on user plane one-to-one bearer mapping	ShenZhen Zhongxing Shitong	discussion

R2-1900563	Consideration on user plane many-to-one bearer mapping	ShenZhen Zhongxing Shitong	discussion

Adapt - General

R2-1901829	Adaptation layer modelling	Huawei, HiSilicon	discussion	Rel-16	NR_IAB-Core

Mod Proposal 1: RAN2 confirms that routing and bearer mapping are adaptation layer functions.

DISCUSSION

- 	ZTE think that routing and bearer mapping is in adapt layer but not flow control. Samsung agrees that flow control is not in the adapt layer.

- 	LG support P1. QC suggest to move out the flow control for now.

- 	LG think that there is one adaptation layer entity for “UL” and “DL”. Huawei think that there is one entity on each sides of the BH RLC link.

- 	KDDI wonders how routing would work if we have several adaptation layer

- 	AT&T think that the adaptation layer entities need to be different.

- 	Chair think it is FFS how we model w.r.t protocol entities, and FFS how these are configured, F1-AP or RRC.

Noted

RAN2 confirms that routing and bearer mapping (e.g. mapping of BH RLC channels) are adaptation layer functions

Agreements based on discussion:

R2 assumes that TX part of adaptation layer performs routing and “bearer mapping”, RX part of adaptation layer performs “bearer demapping”

R2 assumes that SDUs are forwarded from RX part of adaptation layer to TX part of adaptation layer (for the next hop) for packets that are relayed by the IAB node.

FFS how we model w.r.t protocol entities, e.g. whether separate for DU and MT or not, and FFS how these are configured, F1-AP or RRC.

R2-1902017	Consideration on the adaptation layer	LG Electronics Inc.	discussion	Rel-16	NR_IAB-Core

R2-1901159	Fundamental issues for adaptation layer design	Samsung R&D Institute UK	discussion

R2-1900811	Functional view of IAB adaptation layer	Qualcomm Inc	discussion	Rel-16

R2-1900626	Adaptation layer contents and configuration	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_IAB-Core

R2-1900561	Discussion on IAB UP protocol stack and adaptation layer	ShenZhen Zhongxing Shitong	discussion

R2-1901831	Header compression in adaptation layer	Huawei, HiSilicon	discussion	Rel-16	NR_IAB-Core

R2-1901830	Adaptation layer header format design	Huawei, HiSilicon	discussion	Rel-16	NR_IAB-Core

R2-1901325	Adaptation Layer Header Content	Ericsson	discussion	Rel-15	NR_IAB-Core

Adapt - Routing

R2-1901389	Adaptation layer for IAB	Intel Corporation	discussion	Rel-16	NR_IAB-Core

R2-1901326	Packets Forwarding in IAB Networks	Ericsson	discussion	Rel-16	NR_IAB-Core

R2-1900944	Route selection in case of route redundancy	AT&T	discussion

R2-1902170	Discussion on route selection in IAB	LG Electronics	discussion	NR_IAB-Core

R2-1900572	Consideration on routing in IAB	ShenZhen Zhongxing Shitong	discussion

R2-1901639	Routing function and configuration	Futurewei Technologies	discussion	Rel-16

R2-1901646	[Draft] LS on destination address for IAB routing	Futurewei Technologies	LS out	Rel-16	To:3GPP TSG RAN WG3

RLC PDCP - Lossless

R2-1900567	Consideration on UL lossless delivery in IAB	ShenZhen Zhongxing Shitong	discussion

R2-1902022	SN length for multi-hop RLC ARQ	LG Electronics Inc.	discussion	Rel-16	NR_IAB-Core	R2-1818062

R2-1902165	E2E reliability in hop-by-hop RLC ARQ	LG Electronics	discussion	NR_IAB-Core

R2-1900918	Consideration of multi-hop RLC ARQ	Kyocera	discussion

R2-1901196	Solutions for reliable IAB end-to-end transmission for the hop-by-hop RLC ARQ case	Samsung R&D Institute UK	discussion

R2-1900978	Function of Adaptation Layer to cope with multi-hop RLC ARQ	ITRI	discussion	NR_IAB-Core

R2-1901395	Need for Loss less delivery with hop-by-hop ARQ	Intel Corporation	discussion	Rel-16	NR_IAB-Core

R2-1901825	Lossless data delivery	Huawei, HiSilicon	discussion	Rel-16	NR_IAB-Core

RLC - Other

R2-1901824	General RLC impacts	Huawei, HiSilicon, CMCC	discussion	Rel-16	NR_IAB-Core

R2-1900979	RLC Enhancement in IAB	ITRI	discussion	NR_IAB-Core

MAC - Scheduling

R2-1900943	Radio-aware scheduling in IAB	AT&T	discussion

R2-1901396	Handling of fairness in IAB	Intel Corporation	discussion	Rel-16	NR_IAB-Core

R2-1902164	LCG based UL grant	LG Electronics Inc.	discussion	Rel-16	NR_IAB-Core

R2-1900954	IAB scheduling	Potevio	discussion	Rel-16

MAC - Scheduling Low Latency

R2-1900925	Consideration of low latency scheduling for multi-hop backhauling	Kyocera	discussion

R2-1901198	On pre-emptive SR and BSR in IAB networks	Samsung R&D Institute UK	discussion

R2-1902167	Enhancement for low latency uplink scheduling	LG Electronics	discussion	NR_IAB-Core

R2-1901328	Uplink scheduling in IAB networks: BSR, SR, and LCP	Ericsson	discussion	Rel-16	NR_IAB-Core

R2-1901834	Uplink scheduling enhancement	Huawei, HiSilicon	discussion	Rel-16	NR_IAB-Core

R2-1900310	Triggering condition of Pre-BSR	vivo	discussion

R2-1900568	Discussion on low latency scheduling in IAB	ShenZhen Zhongxing Shitong	discussion

R2-1901030	Discussion on SR/BSR triggering in multi-hop IAB network	ASUSTeK	discussion	Rel-16	NR_IAB-Core

R2-1901391	Uplink latency reduction	Intel Corporation	discussion	Rel-16	NR_IAB-Core

MAC - Other

R2-1901329	LCID and LCG extension for IAB nodes	Ericsson	discussion	Rel-16	NR_IAB-Core

R2-1901187	Some considerations on LCID space extension	Samsung R&D Institute UK	discussion

R2-1901832	LCID extension for IAB backhaul link	Huawei, HiSilicon	discussion	Rel-16	NR_IAB-Core

R2-1902021	Consideration on LCID and LCG space extension	LG Electronics Inc.	discussion	Rel-16	NR_IAB-Core

R2-1901191	Further details on the LCID space extension	Samsung R&D Institute UK	discussion

R2-1901833	LCG space extension for IAB backhaul link	Huawei, HiSilicon	discussion	Rel-16	NR_IAB-Core

R2-1901258	Discussion on MAC subheader structure for IAB	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_IAB-Core

### 11.1.3	Control plane aspects

Including consideration of control plane protocol stack and control plane procedures (e.g. topology management, route management, etc)

Misc

R2-1901771	Congestion reporting and handling for IAB networks	Futurewei Technologies	discussion	Rel-16

R2-1901382	Support for LTE deployment at IAB node sites	Ericsson	discussion	Rel-16	NR_IAB-Core

R2-1901636	Supporting of using SA IAB in NSA network	Futurewei Technologies	discussion	Rel-16

R2-1901638	Discussion on IAB NSA procedure and SRB transmission	Futurewei Technologies	discussion	Rel-16

L2 - Configuration

R2-1901324	Backhaul Channel Setup and Modification Procedure for IAB Networks	Ericsson	discussion	Rel-15	NR_IAB-Core

- 	The proposal is essentially that Donor CU is controlling the setup and modification of all backhaul channels. ZTE agrees with this, but think a lot of details is missing.

- 	Samsung think some modification are needed to the figure. Ericsson clarifies that the RRC message is carried in the F1 message.

- 	Huawei think this is RAN3 details.

- 	LG are ok with the proposals.

- 	Samsung wonders who controls the QoS. Ericsson think we just reuse the CUDU arch. Samsung think this is problematic as there is no PDCP on the backhaul channels. Ericsson think that the QoS is provided by Donor CU, by F1 config, as in current CU-DU arch. KDDI think this part is ok.

R2 assumes that Donor CU is controlling the setup and modification of all backhaul channels in the IAB network below the Donor.

R2-1900565	Discussion on BH RLC channel configuration in IAB network	ShenZhen Zhongxing Shitong	discussion

Proposal 1: For UE DRB with one-to-one bearer mapping, separate BH RLC channel should be setup for it.

Modified Proposal 2: For UE DRB with many-to-one bearer mapping, the RLC channel associated with IAB node existing BH RLC channel might be reused as BH RLC channel to forward traffic of UE DRB if they have similar QoS profile.

- 	LG think P1 is reasonable. Ericsson agrees and think it is obvious.

- 	Ericsson think it is complicated to discuss MT traffic in this.

- 	Samsung think that P1 and the modified P2 are the definition of 1-to-1 and 1-to-N

- 	AT&T think that the original proposal with MT bearer could be useful.

-	CATT wonder about the MT bearers.

- 	intel think that we need to define BH RLC channel

R2 assumes that a separate BH RLC channel should be setup for each UE DRB with one-to-one bearer mapping.

R2 assumes that For UE DRB with many-to-one bearer mapping, a BH RLC channel associated with IAB node existing BH RLC channel might be reused as BH RLC channel to forward traffic of UE DRB (e.g. if the BH RLC Channel supports the required UE DRB QoS).

R2-1900945	Control plane signaling to support IAB routing	AT&T	discussion

- 	Chair think that this assumes that Donor CU also controls the adaptation layer.

- 	Sony wonders if the route update would be very frequent. AT&T think it may happen at radio changes, and in such case, it could potentially be frequent.

- 	Nokia wonder if the assumption for p5 is that routes are configured. AT&T confirms that the assumption is that there are a set of configured routes that can be used by the UE.

- 	LG wonders what is assumed to be decided locally. AT&T think this is the next level of detail.

- 	Huawei think it is confusing that the word route update is used for two cases.

- 	Lenovo wonders if route switch involves connection setup.

R2 assumes that Donor CU configures the Adaptation layer, and R2 assumes that the routing is a function of the Adaptation layer. FFS the detail routing functionality, e.g. what is configured vs. what is decided locally.

R2-1901390	Configuration of Bearer Mapping for IAB	Intel Corporation	discussion	Rel-16	NR_IAB-Core

- 	P2: Samsung wonders if this is just for the Access DU, and is wondering what the bearer mapping of an MT is about. Ericsson think this is for every IAB node. ZTE agrees this is for every IAB node, and wonders if we really need both F1 signalling and RRC signalling to configure the adaptation layer. ZTE and LG think that adaptation layer only need to be configured by either F1AP or RRC. Huawei think there are two adaptation layers, one associated with DU and one associated with MT.

- 	Huawei think we should not configure the same thing by two different protocols. QC think we have two different directions, which could be different and think that the DU adaptation layer is for downstream, and the MT adaptation layer is for upstream. Nokia agrees with this.

- 	Samsung think we should do configurations in one shot. AT&T think “one procedure” might be a new thing. Intel hopes we don’t optimize too much.

- 	Ericsson want to clarify that even if we use the word “bearer mapping” we are actually discussing mapping of BH RLC channels as well, not just UE bearers. QC think this is ok.

noted

R2-1901406	Consideration on routing management procedure	KDDI Corporation	discussion

R2-1901420	IAB control Plane aspects	Sony	discussion	Rel-16	NR_IAB-Core

R2-1902130	IAB bearer mapping decision and configuration	Futurewei Technologies	discussion	Rel-16

R2-1901327	IAB Node Integration Procedure	Ericsson	discussion	Rel-16	NR_IAB-Core

Start

R2-1900903	Overview on IAB node setup (SA)	Samsung	discussion	Rel-16

R2-1900904	Overview on IAB node setup (NSA)	Samsung	discussion	Rel-16

R2-1901725	Topology in IAB system	Lenovo, Motorola Mobility	discussion	Rel-16	NR_IAB-Core	R2-1817520

RRC misc

R2-1901059	IAB RRC state machine	Samsung	discussion	Rel-16	NR_IAB-Core

R2-1901421	IAB System information handling	Sony	discussion	Rel-16	NR_IAB-Core

R2-1901497	IAB node in EN-DC	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_IAB-Core

R2-1901647	IAB RRC Connection Establishment	Futurewei Technologies	discussion	Rel-16

R2-1902065	RRC signaling structure for IAB	Futurewei Technologies	discussion	Rel-16

RLF handling

R2-1900242	Route Adaptation upon Backhaul Failure	CATT	discussion

DISCUSSION

- 	QC think what we agreed in the SI is a DL RLF notification, to allow child nodes to take recovery actions.

- 	Intel think we need to focus on the procedure to recover connection rather than recovery of data. CATT agrees.

- 	QC think we should split into RLF notification and RLF recovery.

- 	Huawei wonders how the upstream node can detect RLF. Intel think there is no RLF detection in upstream node now.

- 	Chair wonders what is done by the upstream node. CATT think the routing tables can be updated. LG think that the main purpose is to recover DL data that was buffered.

Baseline:

R2 assumes there is a RLF notification at BH Link RLF, at least to downstream node(s)

R2-1902189	Detection of BH Failure	LG Electronics France	discussion	NR_IAB-Core

DISCUSSION

- 	KDDI think there may also be some link problem detection in F1 interface, so this might be possible today.

R2-1902063	RLF handling in intermediate IAB nodes	Samsung R&D Institute UK	discussion

DISCUSSION

- 	Chair think this paper raises that Dual Connectivity or other supported Alternate Routes could be utilised for recovery at a failure of a BH link.

- 	ZTE supports P3

- 	Ericsson agrees that if DC is used we can use all the recovery modes supported there.

- 	LG and Nokia think that we will not enhance DC RLF behaviour in this WI.

- 	Ericsson think we should first address the single link problem. CATT agrees.

Alternate Routes and/or Dual Connectivity (if agreed) could be utilised at recovery at a failure of a BH link.

Current UE RLF detection and recovery is reused as baseline

R2-1901386	Handling of link failures in IAB networks	Ericsson	discussion	Rel-16	NR_IAB-Core

- 	LG support proposal 1, and think that the intention is to minimize interruption.

- 	AT&T wonders what the actions should be for 1 and 2.

- 	Intel agrees with the proposals.

FFS whether other indications are needed, e.g. when link has recovered, or when recovery is in progress

R2-1901068	RLF in backhaul link	Lenovo, Motorola Mobility	discussion	Rel-16	NR_IAB-Core

R2-1901717	Downstream notification of BH RLF in architecture 1a	SHARP Corporation	discussion

R2-1902018	BH RLF reporting to IAB donor node	LG Electronics Inc.	discussion	Rel-16	NR_IAB-Core

R2-1900812	Backhaul RLF notification for IAB	Qualcomm Inc	discussion

R2-1900627	Backhaul link RLF handling	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_IAB-Core

R2-1901392	Backhaul RLF handling	Intel Corporation	discussion	Rel-16	NR_IAB-Core

R2-1901634	Backhaul RLF Recovery	Futurewei Technologies	discussion	Rel-16

R2-1900571	Discussion on IAB BH RLF handling	ShenZhen Zhongxing Shitong	discussion

R2-1902188	Backhaul Link Failure Recovery	LG Electronics France	discussion	Rel-16	NR_IAB-Core

R2-1902192	Tansport of BH RLF notification	LG Electronics France	discussion	NR_IAB-Core

R2-1900919	Consideration of topology adaptation upon BH RLF	Kyocera	discussion

R2-1901023	Consideration on RLF handling in IAB	Hisense	discussion

R2-1901031	On topology adaptation upon backhaul-link-failure recovery	ASUSTeK	discussion	Rel-16	NR_IAB-Core

CP transport

R2-1900569	Discussion on IAB CP protocol stack	ShenZhen Zhongxing Shitong	discussion

R2-1900570	Discussion on control plane bearer mapping	ShenZhen Zhongxing Shitong	discussion

R2-1902184	Overview on control signaling transmission in IAB	Samsung	discussion	Rel-16	NR_IAB	Late

R2-1901909	Bearer mapping for control plane signalling	Futurewei Technologies	discussion	Rel-16

R2-1901996	Down selection on CP protocol options	Futurewei Technologies	discussion	Rel-16

Access Control

R2-1901718	Access restrictions (barring) in IAB	SHARP Corporation	discussion

R2-1902154	Access Control for IAB node	LG Electronics Inc	discussion	Rel-16	NR_IAB-Core

Other

R2-1901719	Cell Selection and Cell Reselection procedures of IAB	Futurewei Technologies	discussion	Rel-16

R2-1901728	IP address management for IAB nodes	Futurewei Technologies	discussion	Rel-16

R2-1901383	IP address assignment for IAB nodes	Ericsson	discussion	Rel-16	NR_IAB-Core

R2-1901384	Minimizing CN functionalities for IAB networks	Ericsson	discussion	Rel-15	NR_IAB-Core

Withdrawn

R2-1902020	LCG based UL grant	LG Electronics Inc.	discussion	Rel-16	NR_IAB-Core	Withdrawn

### 11.1.4	Other

Security

R2-1901418	Security aspects and considerations for IAB	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_IAB-Core

DISCUSSION

- 	QC think that P1 is not consistent with Arch 1a, and think that SA3 will address p2 on their own. AT&T agrees. Samsung agrees that SA3 will look at p2, no R2 action is needed.

- 	QC think that we should not do arch 1b, with PDCP security.

- 	On P2, Huawei point out that we haven’t specified the adaptation layer header yet and we may need to communicate later.

- 	Ericsson think P2 violates the WID where is says explicitly that adaptation layer carries IP.

- 	Nokia think that with the current baseline there are problems with bearer mapping, as pointed out in another document.

Noted

R2-1901067	Security framework for the NR integrated access backhaul	Samsung	discussion	Rel-16	NR_IAB-Core

- 	TCL support to do no action.

Noted

R2-1901801	Security protection of F1 over wireless backhaul	Futurewei Technologies	discussion	Rel-16

Moved from 11.1.2

Noted

R2-1901385	IAB node authentication and authorization	Ericsson	discussion	Rel-15	NR_IAB-Core

- 	Chair think this is not a R2 matter

- 	Samsung think this is a CN matter

Noted

Other

R2-1901582	Overview of RAN1 impacts	Futurewei Technologies	discussion	Rel-16

## 11.2	NR-based Access to Unlicensed Spectrum

(NR_unlic-Core; leading WG: RAN1; REL-16; started: Dec 18; target; Dec 19; WID: RP-182878)

Time budget: 1 TU

Documents in this agenda item will be handled in a break out session

Running CR

- 	Rapporteur explains that there will be a running CR to capture agreements

[105#xx][NR-U] NR-U agreements (Qualcomm)

Intended outcome: Agreed Running CR (draft CR), capture agreements

Deadline: 1 week

LS in

R2-1900025	Reply to reply LS on wideband carrier operation for NR-U (R1-1901460; contact: LGE)	RAN1	LS in	Rel-16	NR_unlic-Core	To:RAN4, RAN2

noted

R2-1900044	Reply LS on wideband carrier operation for NR-U (R4-1816658; contact: Ericsson)	RAN4	LS in	Rel-16	FS_NR_unlic	To:RAN1	Cc:RAN2

noted

### 11.2.1	User plane

#### 11.2.1.2		MAC

MAC impacts other than RACH

LBT - General

R2-1901332	Modelling of LBT in MAC layer	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

R2-1900676	MAC aspects of LBT for NR-U	ZTE Corporation, Sanechips	discussion

BSR/PHR

R2-1901213	Impact of LBT on power headroom reporting functionality	Lenovo, Motorola Mobility	discussion	Rel-16	NR_unlic-Core

R2-1901669	BSR/PHR report for NR-U	Ericsson	discussion	NR_unlic-Core

UL LBT Failures

R2-1901674	Handling UL LBT failures	Ericsson	discussion	NR_unlic-Core

R2-1901094	Detecting and handling systematic LBT failures in MAC	MediaTek Inc.	discussion	Rel-16	NR_unlic-Core

Proposal 1: Systematic LBT failures are handled independently for each procedure.

2 tdocs noted

DISCUSSION on the two documents above

- 	Oppo wonders what is the systematic LBT failure.

- 	MTK think systematic LBT failure is when the system doesn’t work due to extensive LBT failures. Ericsson think the outcome need to be discussed, and think it would typically be RLF recovery.

- 	Lenovo wonders about the MTK procedures. MTK think that not all procedures will trigger RLF, and think different procedures may have different performance due to LBT setting.

- 	QC agrees that only some procedures should escalate to RLF.

- 	LG don’t want to have a new procedure or timers, and think the existing procedures can be used.

- 	ZTE think that one difference to normal RLM is that this is only triggered by transmissions, and agrees that it depends on the procedures.

- 	Ericsson think that consistent LBT failure could be caused by a hidden node and the some action need to be taken. Ericsson think it doesn’t matter what is the procedure, and think per-procedure handling is more complex.

- 	Convida think we should mimic as much as possible the licensed operation, at least for short bursts of LBT failures. This would be the simplest.

- 	Intel wonders if UL triggered new behaviour is needed at all. Is this just to handle hidden nodes. ZTE think problems occur also for high load.

- 	Huawei are not sure it is needed to introduce anything new. LBT failure can lead to increment of the current counters and can trigger the current actions. Vivo agrees with this.

- 	Ericsson think that LBT outcome can be dependent on CAPC.

- 	Google think we need some mechanism, otherwise the UE will be hanging.

- 	vivo wonders if R1 need to be involved. Chair think this might be later.

Consistent LBT failures can lead to RLF, at least for UL transmissions, for which consistent failures can currently eventually lead to RLF

R2-1901181	Impact of UL LBT failure	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_unlic-Core

R2-1901189	Introducing LBT failure counter in MAC layer	OPPO	discussion	Rel-16

R2-1901537	NR-U LBT Impact on Preamble Transmission Counting	Convida Wireless	discussion	Rel-16	FS_NR_unlic	R2-1816769

CG

R2-1901334	Transmission with configured grant for NR-U	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

R2-1901675	On Autonomous UL Transmissions for NR-U	Ericsson	discussion	NR_unlic-Core

R2-1901810	Consideration on configured uplink grant	LG Electronics Inc.	discussion	Rel-16	NR_unlic-Core	R2-1818232

R2-1900719	Configured UL grant enhancement for NR Unlicense	Intel Corporation	discussion	Rel-16	NR_unlic-Core

R2-1900235	Discussion on the Configurations of Configured Grant in NR-U	vivo	discussion

R2-1900939	Autonomous Uplink Transmission in NR-U	Spreadtrum Communications	discussion	Rel-16

R2-1900250	Enhancements of configured grant in NR-U	OPPO	discussion

CG collision

R2-1901335	Coexistence between configured grant and dynamic grant in NR-U	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

R2-1901670	Coexistence Between Configured and Dynamically Scheduled UL Grants	Ericsson	discussion	NR_unlic-Core

R2-1901812	Consideration on collision avoidance between uplink grants	LG Electronics Inc.	discussion	Rel-16	NR_unlic-Core	R2-1818223

SR

R2-1901797	Consideration on SR counter and SR prohibit timer	LG Electronics Inc.	discussion	Rel-16	NR_unlic-Core	R2-1818216

R2-1901457	Scheduling request in NR-U	InterDigital	discussion	Rel-16	NR_unlic-Core

R2-1901676	Scheduling request for NR-U	Ericsson	discussion	NR_unlic-Core

R2-1901910	Consideration on SR transmission	Xiaomi Communications	discussion	Rel-16

R2-1901333	Discussion on SR for NR-U	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

R2-1900717	Scheduling Request enhancement for NR Unlicensed	Intel Corporation	discussion	Rel-16	NR_unlic-Core

R2-1901080	LBT impact on SR transmissions in MAC	MediaTek Inc.	discussion	Rel-16	NR_unlic-Core

R2-1900938	LBT Impact on RACH and SR	Spreadtrum Communications	discussion	Rel-16

R2-1901160	Discussion on SR transmission for NR-U	Google Inc.	discussion	Rel-16	FS_NR_unlic	R2-1817095

R2-1900234	Discussion on SR Procedure for NR-U	vivo	discussion

R2-1900252	SR transmission and procedure for NR-U	OPPO	discussion

R2-1900585	SR Procedure for NR-U	PANASONIC R&D Center Germany	discussion

DRX

R2-1902150	DRX for NR-U	Qualcomm Incorporated 	discussion

R2-1901671	DRX enhancement for NR-U	Ericsson	discussion	NR_unlic-Core

R2-1901180	Discussion on DRX for NR-U	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_unlic-Core

R2-1900584	DRX Procedure for NR-U	PANASONIC R&D Center Germany	discussion

4 tdocs noted

DISCUSSION on the documents above

- 	QC think it could make sense if some aspects of DRX configurations oculd be different for lic and unlic for a UE.

- 	MTK think having multiple DRX configurations for a UE bring no power consumption benefit. Intel also don't see any need. For Dual connectivity this is anyway possible. ZTE also think this is not needed

- 	Oppo think this is also discussed in the power saving SI, we can just reuse what they do.

- 	ZTE recalls similar discussions for LAA.

- 	Lenovo think the main point is to discuss whether we have common active time or not.

- 	LG think unlicensed for NR-U is different, for NR-U there would be less scheduling opportunities than for LTE LAA, and think now multiple configs can be useful.

- 	Nokia think the main question is whether this is essential or not for NR-U and consider this is not essential. Ericsson agrees. IDT agrees and think the most important point is reachbility which is no problem where there is also a LIC link.

- 	Oppo wonders if the active time extension is due to extend if the gNb acquires the channel late in the active time. Panasonic agrees.

- 	Nokia think that for active time extension there could be aspects that R2 can discuss without R1. Nokia think that

- 	Oppo think that DRX active time could be different even though the DRX configuration.

In NR-U, DRX On-duration starts as in Rel-15 NR (except for potentially have a new switch trigger to go to short DRX).

One DRX configuration for one MAC entity (no change)

FFS if DRX active time somehow be extended, or go to short DRX, by a non-data DL transmission (not WUS)

R2-1901912	DRX enhancement for NR-U	Xiaomi Communications	discussion	Rel-16

R2-1901337	Discussion on DRX for NR-U	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

R2-1901943	DRX enhancements for NR-U	CMCC	discussion

R2-1900237	Discussion on Single or Multiple DRX Configurations in NR-U	vivo	discussion

R2-1900251	DRX operation for NR-U	OPPO	discussion

R2-1900678	DRX operation for NR-U	ZTE Corporation, Sanechips	discussion

R2-1900718	DRX enhancement for NR-u	Intel Corporation	discussion	Rel-16	NR_unlic-Core

R2-1902012	DRX for unlicensed operation	LG Electronics Inc.	discussion	NR_unlic-Core

Beam Failure

R2-1901971	Discussion on BFR procedure in NR-U	CMCC	discussion	Rel-16

R2-1901758	Enhanced beam failure detection operation for NR-U	LG Electronics Inc.	discussion	NR_unlic-Core	R2-1818157

Bandwidth

R2-1901336	Discussion on wideband operation for NR-U	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

R2-1901760	MAC impacts of multiple CCAs in wide band operation	LG Electronics Inc.	discussion	NR_unlic-Core

R2-1900249	BWP operation impacts for NR-U	OPPO	discussion

R2-1901716	Wideband operation in NR-U	InterDigital	discussion	Rel-16	NR_unlic-Core

R2-1900875	Wideband operation for NR-unlicensed	Intel Corporation	discussion	Rel-16	NR_unlic-Core

Other

R2-1901167	Discussion on padding BSR for NR-U SA	Google Inc.	discussion	NR_unlic-Core

#### 11.2.1.1		RACH

Aspects of 4 step RACH specific to unlicensed operation. Generic discussion of 2 step RACH will take place under the 2 step RACH WI which is due to start from April and discussion of aspects of 2 step RACH specific to unlicensed will be deferred until that WI has made some progress.

Resource Selection

R2-1902132	Configuration and Selection for RACH Resources	Qualcomm Incorporated	discussion

R2-1900231	Discussion on Multiple Transmission Opportunities for Msg3	vivo	discussion

R2-1900139	Random Access Resource Selection in NR-U	Samsung Electronics Co., Ltd	discussion	Rel-16	NR_unlic-Core

R2-1900232	Enhance RACH with Additional Transmission Opportunities in Frequency Domain	vivo	discussion

R2-1901169	Discussion on additional transmission opportunities for msg1 and msg3	Google Inc.	discussion	NR_unlic-Core

R2-1901212	Diversity in RACH transmissions	Lenovo, Motorola Mobility	discussion	Rel-16	NR_unlic-Core

R2-1901259	Increasing Tx opportunities for RA messages	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_unlic-Core

R2-1901567	Additional RACH Opportunities for NR-U	Convida Wireless	discussion	Rel-16	NR_unlic-Core

R2-1901944	Considerations on BWP switching and multi-activation for NR-U	CMCC	discussion	R2-1818129

Measurement

R2-1901756	Enhanced RACH procedure based on channel busy level in NR-U	LG Electronics Inc.	discussion	NR_unlic-Core	R2-1818142

R2-1901678	Discussions on RACH enhancements for NR-U	Ericsson	discussion	NR_unlic-Core

RAR window

R2-1901093	Extended ra-ResponseWindow and RA-RNTI calculation	MediaTek Inc.	discussion	Rel-16	NR_unlic-Core

R2-1900136	Random Access Response Reception in NR-U	Samsung Electronics Co., Ltd	discussion	Rel-16	NR_unlic-Core

R2-1900137	MAC TP_Supporting RAR Window Size larger than 10ms in NR-U	Samsung Electronics Co., Ltd	draftCR	Rel-16	38.321	15.4.0	NR_unlic-Core

R2-1900138	RRC TP_Supporting RAR Window Size larger than 10ms in NR-U	Samsung Electronics Co., Ltd	draftCR	Rel-16	38.331	15.4.0	NR_unlic-Core

R2-1901339	Remaining issues on RAR windows for RACH in NR-U	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

R2-1901677	RA-RNTI to handle longer RAR window for NR-U	Ericsson	discussion	NR_unlic-Core

R2-1901907	Consideration on extending RAR window size	Xiaomi Communications	discussion	Rel-16

General

R2-1900248	Enhancements of 4-steps RACH in NR-U	OPPO	discussion

R2-1902200	4-step RACH procedures for NR-U	Qualcomm Incorporated	discussion

R2-1900679	4-step RACH procedure for NR-U	ZTE Corporation, Sanechips	discussion

R2-1900714	Random access procedure for NR-u	Intel Corporation	discussion	Rel-16	NR_unlic-Core

R2-1901079	LBT impact on Random Access procedure in MAC	MediaTek Inc.	discussion	Rel-16	NR_unlic-Core

R2-1900233	Considerations on RACH Counter and Timers in NR-U	vivo	discussion

R2-1901888	UE behaviour when the preamble is not transmitted due to LBT failure	Fujitsu	discussion	Rel-16	NR_unlic-Core

R2-1901211	Modifications to RACH procedure due to LBT	Lenovo, Motorola Mobility	discussion	Rel-16	NR_unlic-Core	R2-1816435

R2-1901338	RACH procedure for NR-U	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

R2-1901432	Considerations BWP for initial access for NR unlicensed operations	Sony	discussion	Rel-16	NR_unlic-Core

R2-1901456	Random access in NR-U	InterDigital	discussion	Rel-16	NR_unlic-Core

R2-1901754	4-step RACH procedure for NR-U	LG Electronics Inc.	discussion	NR_unlic-Core	R2-1818100

Counters and timers

R2-1901673	Handling of RA counters and timers in NR-U	Ericsson	discussion	NR_unlic-Core

R2-1901679	LS on Using Notification of Suspending Power Ramping Counter in Case of LBT Failures	TSG RAN WG2	LS out	FS_NR_unlic	To:TSG RAN WG1

R2-1901785	Remaining considerations on preamble transmission counter and power ramping counter	LG Electronics Inc.	discussion	Rel-16	NR_unlic-Core	R2-1818216

2-Step RACH

R2-1901752	2-step Random Access for NR-U	Ericsson	discussion	Rel-16

R2-1901972	Discussion of Contention-based 2-step RACH procedure	CMCC	discussion	Rel-16

R2-1901973	Discussion of transmission of MsgA	CMCC	discussion	Rel-16

Other

R2-1901906	RACH Enhancements in NR-U	Apple	discussion	Rel-16	NR_unlic-Core

R2-1901887	CFRA enhancement for NR-U	Fujitsu	discussion	Rel-16	NR_unlic-Core	R2-1817276

#### 11.2.1.3		Other

User plane impacts other than MAC

### 11.2.2	Control plane

#### 11.2.2.1		Inactive and Idle mode

Impacts to 38.304: mobility, paging in idle and inactive modes, system information

Paging

R2-1900238	Paging Enhancements for NR-U	vivo	discussion

R2-1901455	Paging procedure in NR-U	InterDigital	discussion	Rel-16	NR_unlic-Core

Questions for Clarification

- 	LG wonders if the UE measure something, or if the network monitors and somehow indicates. IDT has assumed that the UE measures. Nokia wonders if this means that there need to be reference symbols at each paging occasion. IDT think not. MTK wonders if this means that the UE and Network may have different opinion on the channel state.

R2-1902216	Paging and SI broadcasting for NR-U	Qualcomm Incorporated	discussion

3 tdocs noted

DISCUSSION on the 3 tdocs above

- 	Nokia agrees that only initial BWP or active BWP shall be considered/

- 	Nokia think that also non-consecutive POs can be considered for the “extension”. LG agrees.

- 	LG also agrees with QC:P2

- 	Samsung think we just use the current formula to define the PO and maybe only the number of monitoring occasions need to be extended.

- 	Oppo wonders if the UE need to keep monitoring paging to the end always.

- 	xiaomi wonders it the proposed agreement precludes multiple POs. Chair think this is included

- 	Nokia think that in any case LBT increases the power consumption. Nokia think dynamic termination is more important than dynamic extension.

- 	LG wonders how dynamic extension would work. LG are not sure UE behaviour for dynamic termination need to be specified.

- 	ZTE think that vivo P4 is just current mechanism.

A UE can be configured for an additional number of monitoring occasions at or after or before (FFS) its calculated PO (when the paging message can be transmitted)

FFS dynamic extension

FFS dynamic termination

R2-1900522	Paging in NR-U	Nokia, Nokia Shanghai Bell	discussion	Rel-15	NR_unlic-Core

R2-1901522	Paging for NR-U	Convida Wireless	discussion	Rel-16	FS_NR_unlic

R2-1901750	Additional paging transmission opportunities in NR-U	Ericsson	discussion	Rel-16

R2-1901905	Differentiation of Paging Messages for NR-U UE	Apple	discussion	Rel-16	NR_unlic-Core

R2-1900135	Multiple POs for Paging in NR-U	Samsung Electronics Co., Ltd	discussion	Rel-16	NR_unlic-Core

R2-1900720	Paging enhancement for NR-U	Intel Corporation	discussion	Rel-16	NR_unlic-Core

R2-1900131	Extended PO for Paging in NR-U	Samsung Electronics Co., Ltd	discussion	Rel-16	NR_unlic-Core

R2-1900133	38.304 TP for supporting Extended PO for Paging in NR-U	Samsung Electronics Co., Ltd	draftCR	Rel-16	38.304	15.2.0	NR_unlic-Core

R2-1900134	RRC TP for supporting extended PO for paging in NR-U	Samsung Electronics Co., Ltd	draftCR	Rel-16	38.304	15.2.0	NR_unlic-Core

R2-1900253	Paging enhancements in NR-U	OPPO	discussion

R2-1900673	Increasing time-domain paging occasions for NR-U	ZTE Corporation, Sanechips	discussion

R2-1900976	Discussion on the additional paging locations for the NR-U	ITRI	discussion	NR_unlic-Core

R2-1901342	Consideration on paging for NR-U	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

System Information

R2-1901942	Issues and solutions for SI message scheduling in NR-U	CMCC	discussion

R2-1900523	System information in NR-U	Nokia, Nokia Shanghai Bell	discussion	Rel-15	NR_unlic-Core

R2-1900140	SI Message Transmission in NR-U	Samsung Electronics Co., Ltd	discussion	Rel-16	NR_unlic-Core

R2-1900240	Enhancements of System Information in NR-U	vivo	discussion

R2-1900255	System information enhancements for NR-U	OPPO	discussion

R2-1900367	SI scheduling enhancements for NR-U	MediaTek Inc.	discussion	Rel-16	NR_unlic-Core

R2-1900587	Additional SI transmission opportunities in NR-U	PANASONIC R&D Center Germany	discussion

R2-1901341	Consideration on SI for NR-U	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

R2-1901747	Extended, overlapping SI-windows in NR-U	Ericsson	discussion	Rel-16

R2-1901748	Dedicated RRC signaling for request and delivery of on-demand SI in NR-U	Ericsson	discussion	Rel-16

R2-1900675	Extending SI-window for NR-U	ZTE Corporation, Sanechips	discussion

R2-1901745	Indication of duration of broadcast of requested on-demand SI in NR-U	Ericsson	discussion	Rel-16

R2-1901746	How to handle failure to transmit SI update notifications in NR-U	Ericsson	discussion	Rel-16

Mobility

R2-1900239	Cell Reselection for NR-U	vivo	discussion

Questions for Clarifications

- 	LG wonders if this info can only be provided for RPLMN or also for other PLMN. Vivo think all neighbours can be provided.

R2-1901818	Criteria for camping on non-best cell	LG Electronics Inc.	discussion	Rel-16	NR_unlic-Core

2 tdocs noted

DISCUSSION on the 2 tdocs above

- 	Nokia think this topic can be low priority as the system would work anyway. QC think we should help the UE.

- 	Intel think that the minimum we need to do is to allow cell reselection to non-best cells, i.e. avoid barring the frequency.

- 	Comcast think that the latency is a serious problem, and it is more than just power consumption.

R2-1900583	Cell Reselection Procedure for NR-U	PANASONIC R&D Center Germany	discussion

R2-1900927	Reselection considerations for NR-U	Kyocera	discussion

R2-1900245	Idle/Inactive Mode Measurements in NR-U		MediaTek Inc.	discussion

R2-1902208	Idle/Inactive procedures for NR-U	Qualcomm Incorporated	discussion

R2-1901343	Discussion on mobility in idle and inactive mode for NR-U	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

R2-1900713	Enabling the UE to camp on non-best cell	Intel Corporation	discussion	Rel-16	NR_unlic-Core

R2-1900524	Cell selection and reselection in NR-U	Nokia, Nokia Shanghai Bell	discussion	Rel-15	NR_unlic-Core

R2-1900674	Considerations on cell selection/reselection for NR-U	ZTE Corporation, Sanechips	discussion

R2-1901000	Introducing issues for NR-U Idle mode operation	Samsung	discussion	NR_unlic-Core

R2-1900141	SI Acquisition for Camping on a Non-best Cell in NR-U	Samsung Electronics Co., Ltd	discussion	Rel-16	NR_unlic-Core

R2-1901819	PLMN check without reading SIB1	LG Electronics Inc.	discussion	Rel-16	NR_unlic-Core

R2-1901965	Discussion on NR-U cell selection/reselection	CMCC	discussion	NR_unlic

R2-1900244	Cell Selection and Reselection in NR-U	MediaTek Inc.	discussion	R2-1818291

R2-1900142	Cell Barring in NR-U	Samsung Electronics Co., Ltd	discussion	Rel-16	NR_unlic-Core

Other

R2-1901749	Managing PCI collisions in NR-U	Ericsson	discussion	Rel-16

#### 11.2.2.2		Connected mode and RRC

General Mobility Aspects: How to find and identify NR-U target cell(s).

Impact to 38.331: RLM/RLF, mobility in connected mode (note that mobility solutions to be covered by the NR Mobility Enh WI are not to be discussed).

RLM RLF

R2-1900937	Consideration of LBT failure in NR-U	Spreadtrum Communications	discussion	Rel-16

R2-1901751	Handling DL LBT failures	Ericsson	discussion	Rel-16

R2-1902141	RLM procedures for NR-U	Qualcomm Incorporated	discussion

R2-1900715	Considerations of RLF/RLM for NR-u	Intel Corporation	discussion	Rel-16	NR_unlic-Core

R2-1900439	RLM/RLF measurement on NR-U	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_unlic-Core	R2-1817191

R2-1900672	Discussion on RLM/RLF for NR-U	ZTE Corporation, Sanechips	discussion

R2-1900241	RAN2 Impacts of RLM for NR-U	vivo	discussion

R2-1900254	RLM enhancements for NR-U	OPPO	discussion

R2-1901345	Discussion on RLF for NR-U	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

R2-1901715	RLM and RLF in NR-U	InterDigital	discussion	Rel-16	NR_unlic-Core

R2-1902169	Impact of LBT failure on RLF	LG Electronics Inc.	discussion

Measurements & Mobility

R2-1902138	RRM procedures for NR-U	Qualcomm Incorporated	discussion

R2-1901966	Discussion on NR-U connected mode RRM measurement	CMCC	discussion	NR_unlic

R2-1902025	Measurement for medium contention	LG Electronics Inc.	discussion	NR_newRAT-Core

R2-1900440	Considerations on mobility and measurement gap enhancements in NR-U	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_unlic-Core

R2-1900438	Considerations on RSSI and CO measurements	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_unlic-Core

R2-1900586	Channel Occupancy Report in NR-U	PANASONIC R&D Center Germany	discussion

R2-1900876	RRM measurement and reporting in NR unlicensed	Intel Corporation	discussion	Rel-16	NR_unlic-Core

R2-1900935	NR-U Measurement and Mobility Consideration	Spreadtrum Communications	discussion	Rel-16

R2-1901344	Discussion on measurements for NR-U	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

R2-1901744	Mobility in NR-U	Ericsson	discussion	Rel-16

R2-1901820	Measurement rules in NR-U	LG Electronics Inc.	discussion	Rel-16	NR_unlic-Core

R2-1901821	Utilization of channel occupancy in NR-U	LG Electronics Inc.	discussion	Rel-16	NR_unlic-Core

Other

R2-1900246	Handover Improvement in NR-U	MediaTek Inc.	discussion

R2-1902156	Support of conditional handover for NR-U	LG Electronics Inc	discussion	Rel-16	NR_unlic-Core

LBT Failure Indication

R2-1901001	On indicating LBT failure for NR-U	Samsung	discussion	NR_unlic-Core

#### 11.2.2.3		Other

E.g. system topics for Stand Alone, if any.

### 11.2.3	Other

Including general topics covering both CP and UP, organisational

CAPC

R2-1901340	Consideration on channel access priority class	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

R2-1900441	On Channel Access priority Class selection in NR-U	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_unlic-Core

R2-1900236	Discussion on the Service Based Channel Access Priority	vivo	discussion

R2-1900368	CAPC for SRBs	MediaTek Inc.	discussion	Rel-16	NR_unlic-Core

R2-1901776	MAC impact on channel access priorities	LG Electronics Inc.	discussion	Rel-16	NR_unlic-Core

R2-1900677	UL Data multiplexing and channel access priority	ZTE Corporation, Sanechips	discussion

R2-1900247	CAPC for RACH and PUCCH in NR-U	MediaTek Inc.	discussion

R2-1902155	Access priority for transmissions over control plane in NR-U	LG Electronics Inc	discussion	Rel-16	NR_unlic-Core

R2-1901672	Discussions on channel access priority in NR-U	Ericsson	discussion	NR_unlic-Core

Other

R2-1901032	Discussion on IDC problems in NR-U	ASUSTeK	discussion	Rel-16	NR_unlic-Core	R2-1818190

R2-1901346	IDC for NR-U	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

R2-1901158	Discussion on SUL for NR-U	Google Inc.	discussion	Rel-16	FS_NR_unlic	R2-1817105

R2-1900680	Charging aspects of NR-U	ZTE Corporation, Sanechips	discussion

R2-1900681	Draft LS on charging aspects of NR-U	ZTE Corporation, Sanechips	other

Email discussions, to consolidate/clarify agreements from SI, find additional “easy” agreements, and identify questions to be resolved for discussion next meeting.

[105#xx][NR-U] Connected mode measurements (Qualcomm)

Intended outcome: Report, clarify agreements from SI, identify tentative “easy” agreements, and questions to be resolved

Deadline: Next Meeting

[105#xx][NR-U] LBT impact in MAC (except RACH and SR) (Huawei)

Intended outcome: Report, clarify agreements from SI, identify tentative “easy” agreements, and questions to be resolved

Deadline: Next Meeting

[105#xx][NR-U] RACH 4-step and SR (Oppo)

Intended outcome: Report, clarify agreements from SI, identify tentative “easy” agreements, and questions to be resolved

Deadline: Next Meeting

[105#xx][NR-U] Configured Grants (Ericsson)

Intended outcome: Report, clarify agreements from SI, identify tentative “easy” agreements, and questions to be resolved

Deadline: Next Meeting

## 11.7 Study on NR Industrial Internet of Things (IoT)

(FS_ NR_IIOT; leading WG: RAN2; REL-16; started: Jun 18; target; Mar 19; SID: RP-182090)

Time budget: 2 TU

Documents in this agenda item will be handled in a break out session

### 11.7.1	General

Rapportur input etc.

LS in

R2-1900071	LS on RAN Impact analysis due to TSN (S2-1813392; contact: Nokia)	SA2	LS in	Rel-16	FS_Vertical_LAN	To:RAN1, RAN2	Cc:RAN3, RAN, SA

noted

R2-1900023	Reply LS on RAN Impact analysis due to TSN (R1-1901442; contact: Nokia)	RAN1	LS in	Rel-16	FS_NR_IIOT	To:SA2	Cc:RAN2, RAN3, SA

noted

R2-1900026	Reply LS on TSN requirements evaluation (R1-1901470; contact: Nokia)	RAN1	LS in	Rel-16	FS_NR_IIOT	To:RAN2	Cc:RAN3, SA1, SA2

- 	Nokia indicates that there is a TP in R1

noted

R2-1900033	Reply LS on TSN requirements evaluation (R3-187252; contact: Nokia)	RAN3	LS in	Rel-16	FS_NR_IIOT	To:RAN2	Cc:RAN1, SA1, SA2

- 	Nokia indicates that there is a TP in R3

noted

R2-1900060	Reply LS on TSN requirements evaluation (S1-183707; contact: Qualcomm)	SA1	LS in	Rel-16	FS_NR_IIOT	To:RAN2, SA2	Cc:RAN1, RAN3

- 	QC indicate that reply to Q2 is wrong and corrected in the LS below

noted

R2-1902302	Reply LS on TSN requirements evaluation (S1-190233; contact: Nokia)	SA1	LS in	Rel-15	FS_NR_IIOT	To:RAN2, SA2	Cc:RAN1, RAN3

noted

TR

R2-1900629	TR 38.825 v0.0.2	Nokia, Nokia Shanghai Bell	draft TR	Rel-16	38.825	0.0.2	FS_NR_IIOT

R2-1900630	TR 38.825 v0.1.0	Nokia, Nokia Shanghai Bell	draft TR	Rel-16	38.825	0.1.0	FS_NR_IIOT

- 	630 is a clear version of 629.

endorsed

R2-1900631	TP with missing agreement and feedback from SA1, SA2 and RAN3	Nokia, Nokia Shanghai Bell	pCR	Rel-16	38.825	0.1.0	FS_NR_IIOT

R2-1902545	TP with missing agreement and feedback from SA1, SA2 and RAN3	Nokia, Nokia Shanghai Bell	pCR	Rel-16	38.825	0.1.0	FS_NR_IIOT

agreed

R2-1900836	Update of the use case and scenarios for I-IOT	vivo	discussion	Late

noted

Conclusion section in the TR, in R2-1902357, offline (104)

R2-1902357	TP Conclusion section	Nokia, Nokia Shanghai Bell	pCR	Rel-16	38.825	0.1.0	FS_NR_IIOT

Revised in R2-1902373

R2-1902373	TP Conclusion section	Nokia, Nokia Shanghai Bell	pCR	Rel-16	38.825	0.1.0	FS_NR_IIOT

- 	For Intra-UE prioritization, QC suggest to use the word “address” instead of “specify”, and a L1 impact text is missing, and think the agreement wording shall be used as much as possible. Ericsson proposes to remove “MAC”. LG also like to remove the work “MAC” in the section of intra-UE prioritization. CMCC also agrees.

- 	Huawei think the TP is ok, but wonders if we should assign a scenario number for the collision of multiple CGs.

- 	Apple think “Per-packet selective duplication” shall be added to the list, and would like to know what is meant by the conclusions. Nokia clarifies that the intention is to pick the solutions that shall be worked on in the WI. LG support to add this to the list. Oppo too. Huawei disagrees and think this was not supported by many.,

- 	LG think R2 didn't decide anything on the “UE based criteria ..”. Nokia are ok to make this more vague.

- 	Ericsson think we shall remove “Support of finer granularity of CG periodicities compared with existing ones” CATT are ok to remove. LG want to keep, Intel support to remove. Nokia agrees.

- 	Ericsson think we shall remove ”Support of efficient co-existence of periodic deterministic and non-periodic non-deterministic traffic. Candidate solutions are captured in section 6.5.2. The choice and the details of the solution should be decided in WI phase”. Nokia are ok to remove

- 	QC think we should clarify that Bullet 2 in intra UE prio requires further physical layer evaluation.

For Intra-UE prioritization, Remove “MAC”

Change “UE based criteria ..” to “Other solutions not precluded, e.g. MAC CE based control, UE based criteria .. ”.

Add “Other solutions not precluded, e.g. Per-packet selective duplication”, and that “enhancements for selective discard are not required”

Remove “Support of finer granularity of CG periodicities compared with existing ones”

Remove Support of efficient co-existence of periodic deterministic and non-periodic non-deterministic traffic. Candidate solutions are captured in section 6.5.2. The choice and the details of the solution should be decided in WI phase

clarify that Bullet 2 in intra UE prio requires further physical layer evaluation

last sentence in the HC section should be removed. Align the HC recommendation with agreements

With the changes above, the TP is agreed. Detailed wording can polished in the email discussion on final TR

Closing of the SI

From R2 point of view the SI can be closed

[105#xx][IIOT] Agreed TR 38.825 (Nokia)

Intended outcome: Agreed TR for submission to RP, Incorporating agreed TPs from all groups.

Deadline: 1 week

### 11.7.2	TSN

#### 11.7.2.1	Accurate reference timing

Accurate reference timing: Delivery & related process (e.g. SIB delivery or RRC delivery to UEs, Multiple Transmission points)

Reply to SA2 LS

R2-1901492	Discussion on supporting multiple time domains for TSN	Huawei, HiSilicon	discussion	Rel-16	FS_NR_IIOT

Questions for Clarification

- 	Nokia wonders Why 11#4 would not work. Huawei think it is feasible from RAN perspective but it is an issue for TSN system to follow 3GPP. Ericsson think that aspect is an SA2 aspect.

R2-1901156	RAN impact analysis for TSN	NTT DOCOMO INC.	discussion	Rel-16	38.331	FS_NR_IIOT

Questions for Clarfication:

- 	CMCC wonders if it is a mistake to say that solution 17 has the least impact. Docomo explains that the R1 analysis was not known when this paper was written. Docomo are ok to change view on this.

- 	CATT think that Opt3 is not necessarily a boundary clock.

- 	QC wonders why 11 option3 or solution 28 are preferred with multiple domains. Docomo think that this is difficult for SIB solution and assumes there would be a single clock with this. Intel think there can be multiple clocks.

R2-1900764	RAN impacts foreseen on SA2 solutions for TSN Time synchronization	Intel Corporation	discussion	Rel-16	FS_NR_IIOT

- Ericsson wonders about the PTP support at US side for 11#3

R2-1901296	Technical Overview of Clock Synchronisation Solutions in SA2	Ericsson	discussion	Rel-16	FS_NR_IIOT

- 	QC wonders why scalability is not a problem. Ericsson think the net bitrate for time distribution for all cases is small related to the total capacity.

R2-1901803	Discussion for SA2 LS on RAN Impact analysis due to TSN	Qualcomm Incorporated	discussion

5 tdocs noted

DISCUSSION on 5 tdocs above

- 	Huawei think that even in the case of multiple time domains, they will use the same residence time that can be used, and scalability will be ok.

- 	CMCC has concerns on 11#4

- 	Oppo think solution 19 adds too much complexity in PDCP and think more analysis is needed to conclude whether it is feasible.

- 	Nokia think 19 and 28 are similar. Huawei think 28x2 is different and can be preferred. CATT think that in solution 19 the time-stamping is in every node, while 28x2 the time stamping is in the boundary nodes. Intel think that according to SA2, the time-stamping in 19 and 28x2 takes place in the AS protocol stack while for 11.x is takes place in the translator. Docomo agrees with this.

- 	ZTE also support 28x2 and think it is more similar to 11#3 than 19.

- 	Intel would prefer that the AS protocol stack don’t need to interpret/understand gPTP information.

FEASIBILITY

For solutions 17, R1 expresses feasibility concerns, and R2 concurs. 28x1 seems similar to 17 and the same feasibility concerns applies to it.

All other solutions seems feasible from R2 perspective (although all details are not known).

PREFERENCE

The following Solution seems to have significant impact, i.e. [g]PTP time-stamping done in the AS protocol stack, and is due to this not preferred: 19.

R2 would prefer that the User Plane AS protocol stack don’t need to interpret/understand information inside [g]PTP packets.

Assuming that [g]PTP timestamping is done outside the User Plane AS protocol stack the following solutions can be candidate solutions: 28x2, 11#2, 11#3, 11#4.

Indicative Show of hands (support, multiple choice)

28x2		5

11#2		10

11#3		8

11#4		4

SCALABILITY

- 	Chair wonders if we can adopt the Ericsson proposal and say we haven’t identified any issues? QC wonders why. Ericsson explains that the bitrate should be kbps range also for the worst case, which should be ok.

- 	Nokia think that unicast delivery is an issue.

- 	QC think there may be scalability issues.

- 	Docomo think there may be issues, also with UP data as the update is frequent and priority very high.

- 	QC think that 11#2 and 11#4 has the best scalability characteristics. Ericsson think that broadcast has scalability issues for multiple time domains. Intel agrees with QC. Nokia think that the 32 time domains is a system level requirement, and that a single cell requirement would be lower.

R2 has not fully assessed scalability for the different solutions, there are diverging company opinions.

TP reference timimg, in R2-1902358 (105) Qualcomm, capture at least agreements

R2-1902358	TP reference timimg 	Qualcomm	pCR	Rel-16	38.825	0.1.0	FS_NR_IIOT

In 6.3.3, change to “Protocol aspects of accurate reference time provisioning”

With this change TP is agreed, change taken into account in the next TR version

R2-1900634	Analysis of the synchronization solutions proposed by SA2	Nokia, Nokia Shanghai Bell	pCR	Rel-16	38.825	0.1.0	FS_NR_IIOT	Late

R2-1900149	Views on TSN integration options for time synchronization	CATT	discussion	Rel-16	FS_NR_IIOT

R2-1900554	Discussion on SA2 LS on RAN Impact analysis due to TSN	OPPO	discussion	Rel-16

R2-1901510	RAN2 impacts for TSN Time synchronization solutions	ZTE Corporation, Sanechips	discussion	Rel-16	FS_NR_IIOT

R2-1901476	[DRAFT] Response LS to SA2 on TSN time synchronization	Huawei, HiSilicon	LS out	Rel-16	FS_NR_IIOT	To:SA2	Cc:RAN1, RAN3, RAN, SA

R2-1901162	RAN impact analysis for TSN network	NTT DOCOMO INC.	LS out	Rel-16	FS_NR_IIOT	To:SA2

R2-1902190	[Draft] Reply LS on RAN Impact analysis due to TSN	ZTE Corporation	LS out	Rel-16	FS_NR_IIOT	To:SA2, RAN1	Cc:RAN3

R2-1900555	Draft reply LS on RAN Impact analysis due to TSN	OPPO	discussion	Rel-16

R2-1901297	draft reply LS on RAN Impact analysis due to TSN	Ericsson	LS out	Rel-16	FS_NR_IIOT	To:SA2	Cc:RAN1, RAN3, RAN, SA

Revised in R2-1902352

Comeback (102), ericsson

R2-1902352	draft reply LS on RAN Impact analysis due to TSN	Ericsson	LS out	Rel-16	FS_NR_IIOT	To:SA2	Cc:RAN1, RAN3, RAN, SA

- 	Nokia suggest to add to “RAN2 has not concluded whether there are scalability issues or not.” the continuation “ for solution that requires gPTP time synchronization messages using per-UE unicast transport over the air”.

Change to “RAN2 has not concluded whether there are scalability issues or not for solutions that require transport over the air of gPTP time synchronization messages using per-UE unicast”.

With this change the LS is approved in R2-1902369 (no comeback)

General

R2-1901399	RAN details on the time reference information	Intel Corporation	discussion	Rel-16	FS_NR_IIOT

DISCUSSION

P1

- 	Samsung think that if unicast and broadcast send the same information there is no benefit for unicast. Docomo think there is R3 impact.

- 	Huawei think that security and differentiation of UEs can be done with Unicast.

- 	Nokia support unicast.

- 	LG, Samsung, Oppo think we don’t need unicast.

- 	LG think it would be ok to use unicast in complement to broadcast.

- 	Vivo think we would need to analyse overhead for unicast.

- 	CMCC think we can standardize both, and it is anyway a network choice what to use.

Unicast RRC signaling is also used to distribute timing information.

R2-1900633	Support for Clock Synchronization Service	Nokia, Nokia Shanghai Bell, NTT DOCOMO INC.	pCR	Rel-16	38.825	0.1.0	FS_NR_IIOT

R2-1901295	Reference clock provisioning from gNB to UE	Ericsson	discussion	Rel-16	FS_NR_IIOT

R2-1901976	Accuracy, Unicast signalling and Delivery options of Reference Time	CMCC	discussion

R2-1900150	Time Synchronization in air-interface for IIoT	CATT	discussion	Rel-16	FS_NR_IIOT

R2-1900838	Potential enhancements on the reference time accuracy	vivo	discussion

R2-1900551	Time Synchronization for IIoT	OPPO	discussion	Rel-16

R2-1900837	Discussion on the reference time via dedicate signaling	vivo	discussion

R2-1901401	Remaining issues on time synchronization for IIoT	Huawei, HiSilicon	discussion	Rel-16	FS_NR_IIOT

R2-1901509	Further consideration on accurate reference timing delivery in TSN	ZTE Corporation, Sanechips	discussion	Rel-16	FS_NR_IIOT

R2-1901802	TP for Accurate Reference Timing	Qualcomm Incorporated	discussion

R2-1902178	Reference Timing Delivery for Industrial IOT	Samsung	discussion	Rel-16	FS_NR_IIOT

#### 11.7.2.2	Scheduling Enhancements

Enhancements e.g. for scheduling, to satisfy QoS for wireless Ethernet when using TSN traffic patterns as specified in TR 22.804.

Including output of email discussion [104#36][NR/IIOT] TSN Traffic Patterns (Nokia)

Email Discussion

R2-1900635	Summary of e-mail discussion on TSN traffic patterns (with TP)	Nokia, Nokia Shanghai Bell	pCR	Rel-16	38.825	0.1.0	FS_NR_IIOT

P1

- 	Huawei wonders if we can add the observation that for DL SPS it is observed that the traffic is periodic by gNB.

- 	Chair think we can of course elaborate more in the WI phase. Acc to Ran tradition, the exact resource allocation is anyway always up to gNB.

- 	LG wonders if CN can have traffic pattern information always. Nokia think that even in the distributed model the CN will know better than any other entity.

P3

- 	Ericsson think we cannot agree this now, and it wasn’t part of the email discussion.

- 	CATT would like to keep this.

- 	LG wonders if this really is beneficial.

P2

- 	Vivo would like to add message burst characteristic. Vivo assumes that for each period there are several messages sent in burst.

- 	Oppo wonders if the reference time/offset is one time or two times. Nokia think it is the same thing, and only one is needed. CATT proposes “reference time interval”. QC wonders if the arrival is arrival at RAN or not. CMCC think that in SA2 this time is from the previous node and not at RAN. Chair wonders if this is needed at all. Ericsson think this is very needed, and saves from having to overprovision.

- 	Sequans supports “reference time interval”

- 	Apple wonder if reference time applies also to UL. Nokia think that the transmissions are completely coordinated and also for the UL, the network can configure this properly.

TP is agreed

R2-1901714	[DRAFT] LS on assistance information for TSN traffic flows	Nokia, Nokia Shanghai Bell	LS out	Rel-16	FS_NR_IIOT	To:SA2	Cc:RAN1, RAN3

Change 3rd bullet into “Message arrival time at gNB (DL) and UE (UL)”

Remove survival time

Revised in R2-1902354, which is approved unseen.

General Scheduling CG and SPS

R2-1901974	Outstanding Issues for multiple SPSs/CGs Support	CMCC	discussion

R2-1900636	Further discussion on QoS and scheduling enhancements for TSN	Nokia, Nokia Shanghai Bell	pCR	Rel-16	38.825	0.1.0	FS_NR_IIOT

R2-1900152	Multiple active SPS and Configured Grant Configurations	CATT	discussion	Rel-16	FS_NR_IIOT

R2-1900153	SPS and Configured Grant periodicities	CATT	discussion	Rel-16	FS_NR_IIOT

R2-1900284	Configured grant and SPS for IIOT services	NTT DOCOMO, INC.	discussion	Rel-16	FS_NR_IIOT

R2-1900369	Multiple configured grants for NR IIoT	MediaTek Inc.	discussion	FS_NR_IIOT

R2-1900553	Scheduling enhancements for TSN network	OPPO, vivo	discussion	Rel-16

R2-1900929	Scheduling Enhancements for TSN traffic	Spreadtrum Communications	discussion	Rel-16

R2-1901054	Scheduling enhancement for TSN traffic	Lenovo, Motorola Mobility	discussion	Rel-16	FS_NR_IIOT

R2-1901545	CG enhancements for TSN traffic	Sequans Communications	discussion	Rel-16	FS_NR_IIOT

R2-1901637	Discussion on supporting multiple active configured grants for TSN	Huawei, HiSilicon	discussion	Rel-16	FS_NR_IIOT

R2-1901733	Discussion on multiple active configured grants	LG Electronics Inc.	discussion	Rel-16	FS_NR_IIOT

R2-1901734	Scheduling enhancements for TSN traffic	Intel Corporation	discussion	Rel-16	FS_NR_IIOT

R2-1902010	Potential enhancement on Configured Grant for IIOT	LG Electronics Inc.	discussion	FS_NR_IIOT

R2-1902011	TP on CG enhancement for IIoT	LG Electronics Inc.	discussion	FS_NR_IIOT

R2-1902118	TP on multiple active configured grants	LG Electronics Inc.	discussion	Rel-16	FS_NR_IIOT

DL SPS

R2-1901298	DL SPS enhancements for NR-IIoT	Ericsson	discussion	Rel-16	FS_NR_IIOT

R2-1901368	Discussion on DL SPS enhancement	Huawei, HiSilicon	discussion	Rel-16	FS_NR_IIOT

QoS and traffic pattern information

R2-1901975	Knowledge of TSN traffic information to gNB	CMCC	discussion

R2-1900151	Impact of supporting IEEE 802.1Qbv on 5GS RAN	CATT	discussion	Rel-16	FS_NR_IIOT

R2-1901299	TSN traffic patterns	Ericsson	discussion	Rel-16	FS_NR_IIOT

R2-1901894	Synchronized packet delivery	LG Electronics Inc.	discussion	Rel-16	FS_NR_IIOT

R2-1901895	Draft LS on synchronized packet delivery	LG Electronics Inc.	LS out	Rel-16	FS_NR_IIOT	To:SA2

R2-1902151	Consideration on knowledge of traffic pattern	LG Electronics	discussion	FS_NR_IIOT

R2-1902163	Draft LS to SA2 on knowledge of traffic pattern	LG Electronics	discussion	FS_NR_IIOT

R2-1900839	Scheduling enhancements for periodic deterministic traffic	vivo	discussion	R2-1816940

R2-1900840	Traffic pattern awareness for periodic traffic	vivo	discussion

R2-1901374	Discussion on the scheduling enhancement to support IIoT traffic	Huawei, Hisilicon	discussion	Rel-16	FS_NR_IIOT	R2-1817570

Measurement Gap

R2-1901762	Measurement gap enhancement for TSN traffic	LG Electronics Inc.	discussion	FS_NR_IIOT

R2-1901772	TP on Measurement gap enhancement for TSN traffic	LG Electronics Inc.	discussion	FS_NR_IIOT

R2-1902230	On handling of URLLC traffic during measurement gaps in uplink	Huawei, HiSilicon	discussion	Rel-16	FS_NR_IIOT

Mobility

R2-1900115	Discussion on Scheduling Enhancements for TSN mobility	National Taiwan University	discussion

Mixed traffic

R2-1902006	Scheduling enhancement for TSN traffic	LG Electronics Inc.	discussion	FS_NR_IIOT

R2-1902007	TP on Scheduling enhancement for TSN traffic	LG Electronics Inc.	discussion	FS_NR_IIOT

R2-1902008	BSR enhancement for predictable but varying TSN traffic	LG Electronics Inc.	discussion	FS_NR_IIOT

R2-1902009	TP on BSR enhancement for predictable but varying TSN traffic	LG Electronics Inc.	discussion	FS_NR_IIOT

#### 11.7.2.3	Ethernet Header Compression

Analysis of the benefits and the scenario (e.g. what are the formats and size of Ethernet frame to be considered, are VLAN fields included, protocol termination etc.). Definition of the requirements for a new header compression.

Including output of email discussion [104#37][NR/IIOT] Ethernet Header Compression (Ericsson)

Email Discussion

R2-1901369	[104_37 NR_IIOT] Ethernet HeaderComp_email disc report	Ericsson	report	Rel-16	FS_NR_IIOT

Noted

DISCUSSION

- 	Oppo think that the SNAP field can be considered for compression. Chair think that even if not explicitly in the TR, it can be discussed as additional functionality in the WI phase if applicable then. ZTE agrees and think we need to soften the TP on this part.

- 	Mediatek think that SNAP formats are not used much, and there is no reason to work on it.

- 	Mediatek think we should not rule out padding removal.

- 	LG think padding is not in the header, and it should not be included. MTK thikn padding is easy to detect, remove and add again.

- 	Nokia think the bullet on padding is strange, and need rewording.

- 	QC think we should remove “and are considered for further evaluation”

Include SNAP show of hands

YES		2

NO			7

Ethernet header compression is beneficial in the context of IIOT

In the TP, remove “R2-1816765 indicates additional gains of removing padding when payload size is small”

In the TP remove “and are considered for further evaluation”

In the TP “Additional complexity of removing padding (i.e. Ethernet payload) is worth to be considered in Ethernet header compression must be justified” is changed to “Additional complexity of removing padding in Ethernet header compression must be justified.”

Revision in R2-1902355, which is agreed unseen.

OPTIONS for Ethernet Header Compression

- A new ROHC profile, specified by 3GPP/RAN2

- Ethernet Header Compression specified from scratch by 3GPP/RAN2

- “Uplink Data Compression” aka UDC, with some modification, of the whole UL PDU, based on DEFLATE (the LTE solution)

COMEBACK session

- 	Nokia think that the reliability requirements for UDC is a blocking issue

- 	Nokia think that anyway we should ask IETF. QC support this. LG support as well. Intel think this will delay the work.

Show of hands 1

ALT: a + b, i.e. dedicated HC				11

ALT: c, i.e. general Compression based on UDC		5

Show of hands 2, in case there is preference for a+b

ALT: a		4

ALT: b		11

There is R2 majority support for a dedicated HC scheme

Of the dedicated HC schemes, there is R2 majority support for Ethernet Header Compression specified by 3GPP/RAN2 (no ROHC profile).

R2-1901741	Ethernet header compression	Samsung Research America	discussion	Rel-16	FS_NR_IIOT

- 	QC point out that there is problems e.g. at packet loss. LG think that we must take into account PDU discard even if transmission reliability is high. QC think that due to survival time this can work in the normal case.

- 	MTK think ROHC is more efficient than UDC for typical IIOT packets.

R2-1901417	Further discussion on the Ethernet header compression solutions	Huawei, HiSilicon	discussion	Rel-16	FS_NR_IIOT

R2-1901806	Comparing RoHC and UDC Header Compression for TSN	QUALCOMM Incorporated	discussion

R2-1900637	Ethernet Header Compression options comparison	Nokia, Nokia Shanghai Bell	discussion	Rel-16	FS_NR_IIOT

- 	Nokia think UDC sensitivity to packet loss is a show stopper for IIOT cases.

- 	Nokia also think that dictionary for IIOT payload cannot be defined.

- 	Nokia think that reusing ROHC is easier then specifying a new protocol from scratch.

R2-1900765	Ethernet header compression	Intel Corporation	discussion	Rel-16	FS_NR_IIOT

- 	Nokia think that deflate is an external protocol and wonder if there really is not ext dependency

- 	Huawei think that the efficiency of UDC is the same as the others.

R2-1901815	Comparison of Ethernet header compression schemes	MediaTek Inc.	discussion	Rel-16

- 	MTK explains that the main part to look at compares ROHC and a PDCP RAN2 developed compression scheme.

- 	Nokia think that ROHC can do 1 byte overhead. MTK thikn this is not possible ..

R2-1901896	Considerations on solutions for Ethernet Header Compression	LG Electronics Inc.	discussion	Rel-16	FS_NR_IIOT

- 	MTK think that for the first solution, this is not really a disadvantage.

- 	MTK suggest down-selection between UDC and structure-aware schemes.

- 	Samsung think there is no efficiency problem with UDC, the efficiency can be enhanced by predefined dictionary.

7 tdocs above noted

R2-1900638	[DRAFT] LS on RoHC utilization for Ethernet header compression	Nokia, Nokia Shanghai Bell	LS out	Rel-16	FS_NR_IIOT	To:IETF, IEEE	Cc:TSG RAN

- 	Vivo think we need to ask if IETF would have requirements on 3GPP or on the TS specified by 3GPP.

- 	Huawei wonders when IETF would reply

- 	Ericsson wonders what “requirements” IETF could have.

- 	LG want to ask IEEE if they have concerns with 3GPP specifying a new header compression algorithm.

Change Rel-17 to Rel-16, Change 2018 to 2019

Ask IETF what they think would be needed of 3GPP to specify and maintain a ROHC profile.

Ask IEEE if they have concerns with 3GPP specifying a new Ethernet header compression algorithm.

With these changes, the LS is approved (unseen) in R2-1902372

NOT for INPUT into the LS:

R2 don’t expect a negative reply from IEEE and think thus work could start without awaiting that reply.

R2-1900154	Ethernet Packet Compression	CATT	discussion	Rel-16	FS_NR_IIOT	R2-1816362

R2-1900182	Discussion on Ethernet header compression	OPPO	discussion	Rel-16	FS_NR_IIOT

R2-1900841	Ethernet MAC header compression	vivo	discussion	R2-1816938

R2-1901098	Comparison of Ethernet Header Compression Schemes	MediaTek Inc.	discussion	Rel-16	FS_NR_IIOT	Withdrawn

R2-1901424	Ethernet Header Compression additional requirements	Sony	discussion	Rel-16	FS_NR_IIOT

#### 11.7.2.4	Other

E.g. Performance evaluation of TSN requirements (TR 22.804 clause 8.1).

R2-1900632	[DRAFT] Reply LS on TSN integration in the 5G System	Nokia, Nokia Shanghai Bell	LS out	Rel-16	FS_NR_IIOT	To:SA WG2	Cc:RAN WG1, RAN WG3, SA WG1, TSG RAN

- 	Nokia proposes to agree that that the signalling granularity of reference time information is 10ns. Ericsson support to have finer granularity then current 250 ns.

- 	Vivo think Nokia hasn’t taken into account propagation delay which may be an issue. QC think R1 looked at both small and large cells, and the R1 numbers are after propagation delay compensation.

- 	CATT could be fine with this (change opinion).

- 	Docomo wonders if 100ns network latency is sufficient. Nokia proposes to discuss offline

signalling granularity of reference time information is =<50ns

revised in R2-1902356

Comeback (103), Nokia

R2-1902356	[DRAFT] Reply LS on TSN integration in the 5G System	Nokia, Nokia Shanghai Bell	LS out	Rel-16	FS_NR_IIOT	To:SA WG2	Cc:RAN WG1, RAN WG3, SA WG1, TSG RAN

Change “network interface accuracy” to “network accuracy”

With this change the LS is approved, final version in R2-1902371

R2-1901446	General LCP enhancement for IIoT	Huawei, HiSilicon	discussion	Rel-16	FS_NR_IIOT

R2-1901742	Integrity verification failure handling for IIoT	Samsung Research America	discussion	Rel-16	FS_NR_IIOT

R2-1902186	Integrity verification failure handling for DRB	LG Electronics Inc.	discussion	FS_NR_IIOT

R2-1901465	PDCP discardTimer enhancements	Huawei, HiSilicon	discussion	Rel-16	FS_NR_IIOT	R2-1811839

withdrawn

R2-1902229	Layer 2 parameters enhancements	Huawei, HiSilicon	discussion	Rel-16	FS_NR_IIOT

### 11.7.3	Intra-UE prioritization and multiplexing

Intra-UE prioritization and multiplexing

Including output of email discussion [104#38][NR/IIOT] Intra UE prioritization UL Data Data (Interdigital)

Including output of email discussion [104#39][NR/IIOT] Intra UE prioritization UL Control Data (Huawei)

Rapporteur TP

R2-1901350	TP on Intra-UE Prioritization	Nokia, Nokia Shanghai Bell	pCR	Rel-16	38.825	0.1.0	FS_NR_IIOT

- 	Nokia point out that the change proposed is in 5.4, and this reflects the LS sent to R1.

- 	QC think R1 should handle this. Oppo agrees. Nokia are ok with that.

Not agreed, noted

Data Data Prioritization

R2-1901458	Summary of [104#38][NR/IIOT] Intra UE prioritization UL Data Data (Interdigital)	InterDigital	discussion	Rel-16	FS_NR_IIOT

DISCUSSION

P2

- 	LG has a different view, as the network can prioritize for Dynamic grants.

- 	Ericsson want to change the wording ..

- 	Vivo think there might need to be more interaction between L1 and MAC and MAC might need some L1 information

P3

- 	Lenovo think that the later grant shall override the first one. MTK agrees. LG do not disagree, and think the network would never cause a conflict such that low priority data would be scheduled when high prio transmission has been granted.

- 	Chair think the way the proposed agreement is very strange.

- 	ZTE think that MAC would generate a PDU for each grant in several collision cases.

- 	Spreadtrum wonders if the MCS-CRNTI vs C-RNTI usage

- 	Sony think the UE may not know. Lenovo think that the gNB will know exactly and can decide to schedule based on SR.

- 	LG think that there are case when MAC cannot do the prioritization, e.g. there would not be sufficient time.

- 	QC think that should leave this to Physical layer.

- 	MTK think we need to careful to not generate two PDUs always. Intel agrees. QC agrees.

- 	LG think we should have a common prioritization principle and a common split between MAC and L1 for all cases. Spreadtrum think the case when MAC hasn’t provided the PDU to L1, MAC can prioritize.

- 	Vivo think we don’t need to specify how many PDUs are generated.

- 	QC don’t want the “RAN2 assumes that the later dynamic grant may always be prioritized over and earlier dynamic grant (scenario 3)”

P4

- 	LG think we need to discuss who performs the prioritization.

P5

- 	Nokia think we might not need to agree this. R1 can do this. Lenovo think we don’t need an LS. Intel agrees. Huawei agrees. MTK think we should send an LS saying the opposite.

- 	QC support this.

- 	LG think that R2 should agree on this.

- 	IDT think this would give scheduler flexibility.

- 	QC point out that this would involve interaction between L1 and MAC. MTK think this interaction is there anyway.

- 	Lenovo think this is covered by LCP restrictions. Lenovo don’t see how it helps when prioritizing two dynamic grants that there is a priority indication in the DCI. QC think the difference is related to UCI multiplexing. CATT agrees with MTK and Lenovo and think there is no difference to the timeline. Intel also agrees, and think we cannot omit the MAC LCP. Samsung too.

- 	Ericsson think for now we can resolve all issues, but think also an indication could simplify things. LG think R2 has not decided who does the prioritization yet.

- 	Chair: No agreement on this point. There is some interest to have “priority level indication per grant”, but also quite significant opposition.

P6

- 	Chair wonders if this is for all cases or for a specific case. IDT clarifies that it is applicable to all cases where MAC prioritizes. Lenovo think this is in L1 TS and MAC don’t need to be concerned. CATT agrees that processing times are only defined in R1.

- 	Nokia think that MAC can always prioritize and L1 can always handle conflicts, regardless timing. Huawei think that if MAC can prioritize grant it would be preferable to do this rather than generate another PDU.

- 	LG disagrees to specify a processing time for MAC. ZTE agrees with LG, as processing time would be very dept on PDU size.

- 	Vivo point out that for CG this is up to UE impl.

- 	Chair: No agreement.

RAN2 shall study resource conflicts between multiple active configured grants, in addition to Scenarios 2 and 3, part of UL data-data prioritization.

UE prioritization of a grant when there is at most one dynamic grant in the set of conflicting grants (scenario 2 and CG/CG collision) shall be addressed. MAC specifies currently the UE prioritization of such cases, and modifications to MAC would be required.

RAN2 assumes that the later dynamic grant may always be prioritized over and earlier dynamic grant (scenario 3). One way to realize this is that MAC generate a PDU for each grant and let L1 handle conflicting transmissions. To be confirmed following progress in RAN1. Other solutions are not precluded

For cases when MAC prioritizes a grant, MAC prioritizes the grant on which data of the highest priority can be transmitted according to LCP restrictions and priority configured for each LCH.

R2-1901459	TP on UL Data-Data Intra UE prioritization	InterDigital	pCR	Rel-16	38.825	0.1.0	FS_NR_IIOT

Revised in R2-1902364

Comeback

R2-1902364	TP on UL Data-Data Intra UE prioritization	InterDigital	pCR	Rel-16	38.825	0.1.0	FS_NR_IIOT

- 	Nokia would like to change “To be confirmed following progress in RAN1” to “To be confirmed following evaluation of physical layer aspects”

Change “To be confirmed following progress in RAN1” to “To be confirmed following evaluation of physical layer aspects”

With this change the TP is agreed.

R2-1901348	Intra-UE prioritization for Data-Data Resource Collision	Nokia, Nokia Shanghai Bell	discussion	Rel-16	FS_NR_IIOT

R2-1900579	Consideration on the collision between Multiple Configured Grants	ZTE Corporation, Sanechips	discussion	FS_NR_IIOT

R2-1900580	Consideration on HARQ Conflict Between Configured Grant and Dynamic Grant	ZTE Corporation, Sanechips	discussion	FS_NR_IIOT

R2-1900842	Prioritization between Overlapping Configured Grants	vivo	discussion

R2-1900847	MAC-PHY interaction for overlapping UL grants	III	discussion	Rel-16	FS_NR_IIOT

R2-1901199	Intra-UE prioritization UL Data/Data	Lenovo, Motorola Mobility	discussion	Rel-16	FS_NR_IIOT

R2-1901300	On support of Intra-UE UL Mixed Services	Ericsson	discussion	Rel-16	FS_NR_IIOT

R2-1901039	Intra UE prioritization for UL grants	ASUSTeK	discussion	Rel-16	FS_NR_IIOT

R2-1901422	UL Intra-UE prioritization and multiplexing of Data and Data	Sony	discussion	Rel-16	FS_NR_IIOT

R2-1901441	Collision between Configured and Dynamic grants	Huawei, HiSilicon	discussion	Rel-16	FS_NR_IIOT

R2-1901460	Retransmissions and Resource Conflicts for Intra-UE Prioritization	InterDigital	discussion	Rel-16	FS_NR_IIOT

R2-1901461	Processing Times for Intra-UE Prioritization/Multiplexing for UL Data-Data	InterDigital	discussion	Rel-16	FS_NR_IIOT

R2-1901807	Prioritization between overlapping dynamic grant and configured grant	QUALCOMM Incorporated	discussion

R2-1901967	Consideration of LCP Mapping restriction for URLLC and eMBB service	CMCC	discussion

R2-1900930	Consideration on UL intra-UE prioritization	Spreadtrum Communications	discussion	Rel-16

R2-1901569	Prioritization and Pre-emption of PUSCH Transmissions for I-IoT	Convida Wireless	discussion	Rel-16	FS_NR_IIOT

R2-1901809	DRAFT LS on Dynamic Grant Priority Level Indication	Qualcomm Incorporated	LS out	To:RAN1

R2-1902026	UL data prioritization rule and who prioritize	LG Electronics Inc.	discussion	NR_newRAT-Core

Control Data Prioritization

R2-1901439	Report of 104#39 NR IIOT Intra UE prioritization UL Control Data	Huawei, HiSilicon	discussion	Rel-16	FS_NR_IIOT

Noted

DISCUSSION

P1

- 	LG has concerns on P1. LG think prioritization rule is not needed as BSR will include the information for URLLC as well. Huawei think that a eMBB grant can occupy L1 for long time. Sony agrees with LG. CATT think that if the PUSCH transmission has it cannot include the BSR.

- 	Docomo think we need Dyn grant also for URLLC e,g, for spectrum eff and reliability. Sony think the UE can change the MCS to something more reliable. Lenovo think this doesn’t work.

- 	Oppo also think that if PUCCH and PUSCH are on different cells it still doesn’t work today.

P2:

- 	CATT thikn this is OK as a baseline, but think we should also execture ther proposed rule based on reliability requirement (in addition to priority). Ericsson think that the reliability is related to the priority.

- 	MTK suggests to say that R1 and R2 need to look at this.

- 	ZTE wonders if we could have SRs of different priorities. Spreadtrum wonders what if the PUSCH transmission is short. Lenovo think it is not just about the duration but also about MCS.

- 	Xiaomi would like to extend this to PRACH. Vivo think that if PRACH is for BFR there may be a problem.

- 	ZTE think PRACH is always prioritized, and think this may be a problem sometimes.

- 	Nokia think the PRACH scenario was not included in the scenarios selected.

2a

- 	Chair think this is obvious. LG don’t agree to this.

P4

- 	Ericsson think that there would always be space for a MAC CE in the grant, and maybe the MAC CE has the highest priority. ZTE think that some MAC CEs like periodic BSR/PHR can be predicted by the network. Xiaomi think this case is not so important. Nokia also have some doubts and would be fine to not address this. Oppo also think the issue is not clear.

- 	CATT think that network solutions are complex.

- 	LG think that network impl would involve resource waste, as the network can never fully predict MAC CEs. Lenovo also think that the required over-provisioning is not good. LG and Samsung think this is important. Samsung think 50% of the resource could be wasted.

- 	Xiaomi think resource efficiency is not in the scope.

- 	Chair: Could not agree whether to address the potential issue that the higher priority data packet is segmented and delayed due to the transmission of MAC CE

P5

- 	LG don’t support this, and think it is anyway up to UE implementation. Oppo think that there is an explicit note on the UE implementation. CATT think there would be a network misconfiguration that this case occurs.

Capture into TR 38.825 the issue that the SR triggered by URLLC cannot be sent if there is a UL-SCH resource for eMBB;

Agree and capture into TR 38.825 the solution to address the issue of collision between URLLC SR and eMBB UL-SCH may include: A prioritization rule can be defined to determine whether to transmit SR or PUSCH, e.g. based on the priority of the LCH which triggers the SR and priorities of the data to be transmitted on the PUSCH resource.

Leave to RAN1 to discuss the potential issue related to collision between eMBB PUSCH and HARQ feedback or CSI report for URLLC.

R2-1901440	TP on Intra-UE Prioritization between UL control and data	Huawei, HiSilicon	discussion	Rel-16	FS_NR_IIOT

Revised in R2-1902363

Comeback

R2-1902363	TP on Intra-UE Prioritization between UL control and data	Huawei, HiSilicon	discussion	Rel-16	FS_NR_IIOT

agreed

R2-1900843	SR cancellation for URLLC service	vivo	discussion	R2-1816944

R2-1900846	URLLC transmission within measurement gap	vivo	discussion

R2-1901210	Prioritization between MAC CE and data	Lenovo, Motorola Mobility	discussion	Rel-16	FS_NR_IIOT

R2-1901301	On prioritization between MAC CE and data	Ericsson	discussion	Rel-16	FS_NR_IIOT

R2-1901302	On scheduling request overlapping with UL-SCH	Ericsson	discussion	Rel-16	FS_NR_IIOT

R2-1901349	Intra-UE prioritization for Resource Collision involving Control Channel	Nokia, Nokia Shanghai Bell	discussion	Rel-16	FS_NR_IIOT

R2-1901423	UL Intra-UE prioritization and multiplexing of Control and Data	Sony	discussion	Rel-16	FS_NR_IIOT

R2-1901462	Intra UE prioritization between SR and PUSCH	InterDigital	discussion	Rel-16	FS_NR_IIOT

R2-1901891	Intra-UE handling on URLLC and measurement gap	Fujitsu	discussion	Rel-16	FS_NR_IIOT

R2-1901913	Add an Intra-UE UL Prioritization Scenario:Resource Conflict between PRACH Channel and Control/Data Channel	Xiaomi Communications	discussion	Rel-16

R2-1902171	Prioritization between URLLC SR and eMBB PUSCH	LG Electronics Inc	discussion	FS_NR_IIOT

R2-1902179	Unnecessary Segmentation of URLLC Data due to MAC CE	Samsung	discussion	Rel-16	FS_NR_IIOT

General

R2-1900155	Scenarios Analysis for intra-UE prioritization and multiplexing	CATT	discussion	Rel-16	FS_NR_IIOT

R2-1900156	Solutions for intra-UE prioritization and multiplexing	CATT	discussion	Rel-16	FS_NR_IIOT

R2-1900550	Intra-UE Prioritization for IIoT	OPPO	discussion	Rel-16

R2-1901096	Uplink intra-UE prioritisation	MediaTek Inc.	discussion	Rel-16	FS_NR_IIOT

R2-1901547	Intra-UE Prioritization for IIoT	Sequans Communications	discussion	Rel-16	FS_NR_IIOT

R2-1901890	UL procedure for intra-UE multiplexing	Fujitsu	discussion	Rel-16	FS_NR_IIOT

### 11.7.4	Data duplication and multi-connectivity

Including output of email discussion [104#40][NR/IIOT] PDCP Duplication Enhancement (Ericsson)

Email Discussion

R2-1901367	104_40NR_IIOT_ PDCPduplication_report of email discussion	Ericsson	report	Rel-16	FS_NR_IIOT

Noted

DISCUSSION

P1

- 	Xiaomi think that 4 configurations is ok, but 2 copies is enough. Can we really agree to this? Lenovo think the main question is the diversity gain we get.

- 	AT&T think we can assume this as a baseline, although many cases will not need 4 copies.

P4

- 	Ericsson clarifies that dynamic control means “with MAC CE” or similar. Nokia think there could also be preconfigured criteria for switching. Nokia think we also need more dynamic switching of carrier / cells. Ericsson point out that duplication control is separate from activation etc of Cells/carriers.

- 	MTK wonders if this precludes UE based selection of legs. Ericsson think not. LG think UE based selection is not needed.

- 	LG wonders what would be the benefit of UE based selection.

- 	Lenovo understands that Nokia want flexible LCH restrictions. Huawei think there could be benefits in terms of using lower number of LCH-id.

P5

- 	Ericsson explains that the DC+CA(NR Only) option support up to 4 copies. CATT agrees that we shall restrict this for the other cases. Huawei agrees. Huawei would like to restrict CA to 2 copies.

- 	Oppo think we shouldn’t restrict this now. MTK agrees we shold keep this open. Samsung as well

P6

- 	Fujitsu think that RLC mode is also unclear after the email discussion. LG think that it could make sense to have combinations of RLC modes. Ericsson thikn we don’t need to specify anyting for that. Huawei think we should just agree to support only one mode.

- 	Ericsson anyway think that we should not optimize for this case.

- 	QC thikn that RLC UM only would be used for low latency IIOT cases.

PDCP duplication support a configuration delivering up to 4 copies.

Up to 4 RLC entities/legs per bearer are possible to configure by RRC for PDCP duplication

The NW can dynamically control (MAC CE or similar) how a set or subset of configured RLC entities or legs are used by the UE for PDCP duplication. This does not preclude other methods of leg selection.

The architectural combinations supported for the work on PDCP duplication enhancements are CA, DC(NR only) and DC+CA(NR Only)

R2 assumes that For PDCP duplication, all RLC entities for a RB are configured using the same RLC mode.

Comeback (111) TP updated, revision in R2-1902362 (Ericsson), to align with agreements.

R2-1902362	104_40NR_IIOT_ PDCPduplication_report of email discussion	Ericsson	report	Rel-16	FS_NR_IIOT

TP is agreed

General

R2-1900157	PDCP duplication in IIoT	CATT	discussion	Rel-16	FS_NR_IIOT

R2-1900578	Consideration on the enhancement of PDCP Duplication	ZTE Corporation, Sanechips	discussion	FS_NR_IIOT

R2-1900845	Discussion on duplication configuration	vivo	discussion

R2-1901097	Duplication and multi-link operation	MediaTek Inc.	discussion	Rel-16	FS_NR_IIOT

R2-1901209	Cell restriction for CA duplication	Lenovo, Motorola Mobility	discussion	Rel-16	FS_NR_IIOT

R2-1901352	RLC Entity Configuration and Activation for PDCP Duplication Enhancement	Nokia, Nokia Shanghai Bell	pCR	Rel-16	38.825	0.1.0	FS_NR_IIOT

R2-1901443	PDCP Duplication leftover enhancements	Huawei, HiSilicon	discussion	Rel-16	FS_NR_IIOT

R2-1901804	Discussion on PDCP Duplication Enhancements	III	discussion	Rel-16	FS_NR_IIOT

R2-1901897	TM DRB for IIOT	LG Electronics Inc.	discussion	Rel-16	FS_NR_IIOT	R2-1818108

R2-1902181	Analysis of Duplication with More Than 2 Legs	Samsung	discussion	Rel-16	FS_NR_IIOT	R2-1818466

Leg Selection

R2-1901353	Enhancements for Uplink PDCP Duplication	Nokia, Nokia Shanghai Bell	discussion	Rel-16	FS_NR_IIOT

R2-1901442	Enhancements to PDCP duplication to enable dynamic carrier selection	Huawei, HiSilicon	discussion	Rel-16	FS_NR_IIOT

R2-1901303	Leg selection for UL transmission when multiple legs are configured	Ericsson	discussion	Rel-16	FS_NR_IIOT

R2-1900552	Discussion on data duplication	OPPO	discussion	Rel-16

R2-1902174	Dynamic control of PDCP duplication leg	LG Electronics Inc.	discussion	Rel-16	FS_NR_IIOT

R2-1900928	Enhancements for PDCP duplication	Spreadtrum Communications	discussion	Rel-16

Resource Efficiency

R2-1902183	Resource efficient PDCP duplication operation	Samsung	discussion	Rel-16	FS_NR_IIOT

R2-1901095	Duplicate discard in UM operation	MediaTek Inc.	discussion	Rel-16	FS_NR_IIOT

R2-1901877	PDCP duplication enhancement	Apple Portugal	discussion	Rel-16	FS_NR_IIOT

R2-1901808	PDCP Duplication Enhancements for URLLC traffic	Qualcomm Incorporated	discussion

R2-1901304	Selective duplication	Ericsson	discussion	Rel-16	FS_NR_IIOT

R2-1901351	Resource Efficient PDCP Duplication	Nokia, Nokia Shanghai Bell	discussion	Rel-16	FS_NR_IIOT

R2-1901040	Enhancements for increasing PDCP duplication efficiency	ASUSTeK	discussion	Rel-16	FS_NR_IIOT

R2-1901977	Resource-efficient of multi-leg PDCP duplication	CMCC	discussion

R2-1902070	Selective PDCP duplication	LG Electronics Finland	discussion	Rel-16	FS_NR_IIOT

R2-1900158	Resource efficiency considering PDCP duplication	CATT	discussion	Rel-16	FS_NR_IIOT

Higher Layer multi-connectivity

R2-1901354	TP on Higher-Layer Multi-Connectivity	Nokia, Nokia Shanghai Bell	pCR	Rel-16	38.825	0.1.0	FS_NR_IIOT

Typo “impacta”

TP is agreed, fix typo in the TR

R2-1900160	Discussion on higher layer multi-connectivity	CATT	discussion	Rel-16	FS_NR_IIOT

Other

R2-1900844	Consideration on efficient duplication at SCell failure	vivo	discussion

R2-1901548	BSR operation with CA packet duplication	Sequans Communications	discussion	Rel-16	FS_NR_IIOT	R2-1817317

R2-1900159	Survival time support in 5GS	CATT	discussion	Rel-16	FS_NR_IIOT

Summary

NR UP Rel-15

[105#xx][NR UP] Correction on PDCP re-establishment for AM DRB (Huawei)

Intended outcome: Agreed CR

Deadline: 1 week

IAB

[105#xx][IAB] IAB running CR (Qualcomm)

Intended outcome: Agreed Running CR, Capture agreements from this meeting,

Deadline: One week

[105#xx][IAB] RAN2 and RAN3: IAB Miscellaneous (Qualcomm)

Intended outcome: Report, identify and capture “easy agreements” from the SI, agree on a name for the new protocol.

Deadline: Next meeting

[105#xx][IAB] Routing (LG)

Intended outcome: Report, identify options, tentative easy agreements, see also R2-1902645

Deadline: Next Meeting

[105#xx][IAB] Bearer mapping (Ericsson)

Intended outcome: Report, identify options, tentative easy agreements, see also R2-1902645

Deadline: Next Meeting

NR-U

[105#xx][NR-U] NR-U agreements (Qualcomm)

Intended outcome: Agreed Running CR (draft CR), capture agreements

Deadline: 1 week

[105#xx][NR-U] Connected mode measurements (Qualcomm)

Intended outcome: Report, clarify agreements from SI, identify tentative “easy” agreements, and questions to be resolved

Deadline: Next Meeting

[105#xx][NR-U] LBT impact in MAC (except RACH and SR) (Huawei)

Intended outcome: Report, clarify agreements from SI, identify tentative “easy” agreements, and questions to be resolved

Deadline: Next Meeting

[105#xx][NR-U] RACH 4-step and SR (Oppo)

Intended outcome: Report, clarify agreements from SI, identify tentative “easy” agreements, and questions to be resolved

Deadline: Next Meeting

[105#xx][NR-U] Configured Grants (Ericsson)

Intended outcome: Report, clarify agreements from SI, identify tentative “easy” agreements, and questions to be resolved

Deadline: Next Meeting

IIOT

From R2 point of view the SI can be closed

OPTIONS for Ethernet Header Compression

- A new ROHC profile, specified by 3GPP/RAN2

- Ethernet Header Compression specified from scratch by 3GPP/RAN2

- “Uplink Data Compression” aka UDC, with some modification, of the whole PDU, based on DEFLATE (the LTE solution)

[105#xx][IIOT] Agreed TR 38.825 (Nokia)

Intended outcome: Agreed TR for submission to RP, Incorporating agreed TPs from all groups.

Deadline: 1 week