3GPP TSG RAN2 Meeting #105	R2-1902470

# Athens, Greece, 25th of Feb- 1st of Mar 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.305 | CR | 0080 | rev | 1- | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of additional sensor methods (IMU) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LCS_LTE_acc_enh-Core |  |  |  |  |  |  | Date: |  | 2019-02-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To provide support for a new positioning method based on Motion Sensor |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of motion sensor Impacted functionality: No interoperability issue. Stage 2 Description of Motion Sensor |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between Stage 2 and Stage 3 remains |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3 , 8.X, B1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR 36.355... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Beginning of change

## 4.3	Standard UE Positioning Methods

The standard positioning methods supported for E-UTRAN access are:

-	network-assisted GNSS methods;

-	downlink positioning;

-	enhanced cell ID method;

-	uplink positioning;

-

-	WLAN method;

-	Bluetooth method;

-	Terrestrial Beacon System method;

-	Sensor based methods.

- Barometric Pressure Sensor

- Motion sensors

Hybrid positioning using multiple methods from the list of positioning methods above is also supported.

Standalone mode (e.g. autonomous, without network assistance) using one or more methods from the list of positioning methods above is also supported.

These positioning methods may be supported in UE-based, UE-assisted/E-SMLC-based, eNB-assisted, and LMU-assisted/E-SMLC-based versions. Table 4.3-1 indicates which of these versions are supported in this version of the specification for the standardised positioning methods.

Table 4.3-1: Supported versions of UE positioning methods

| Method | UE-based | UE-assisted, E-SMLC-based | eNB- assisted | LMU-assisted/ E-SMLC-based | SUPL |
| --- | --- | --- | --- | --- | --- |
| - A-GNSS | - Yes | - Yes | - No | - No | - Yes - (UE-based and UE-assisted) |
| - Downlink Note1 | - No | - Yes | - No | - No | - Yes (UE-assisted) |
| - E-CID | - No | - Yes | - Yes | - No | - Yes (UE-assisted) |
| - Uplink | - No | - No | - No | - Yes | - No |
| - Sensors | - Yes | - Yes | - No | - No | - No |
| - WLAN | - Yes | - Yes | - No | - No | - Yes |
| - Bluetooth | - No | - Yes | - No | - No | - No |
| - TBS Note 2 | - Yes | - Yes | - No | - No | - Yes (MBS) |
| - NOTE 1: This includes TBS positioning based on PRS signals. - NOTE 2: In this version of the specification only for TBS positioning based on MBS signals. |  |  |  |  |  |

Barometric pressure sensor, WLAN, Bluetooth, and TBS positioning methods based on MBS signals are also supported in standalone mode, as described in the corresponding sections.

Next Change

#### 4.3..	Motion sensor positioning

The motion sensor method makes use of different sensors such as accelerometers, gyros, magnetometers, to calculate the relative acceleration, velocity and displacement of UE. The Location  server could use the data to  for hybrid positioning.

The operation of the sensor positioning method is described in clause 8.x.

Next Change

## 8.x	Motion sensor positioning method

### 8.x.1	General

Motion sensors can be used to estimate the location of the UE, the UE velocity, altitude, acceleration and displacement, Inertial measurement unit (IMU) which is also referred as inertial navigation system (INS) is based on motion sensors (accelerometers), rotation sensors (gyroscopes), and occasionally magnetic sensors (magnetometers) that continuouslcalculate via dead reckoning the position, orientation, and velocity (direction and speed of movement) of the UE. With the combination of other positioning methods (hybrid) a more  accurate position and final position of the UE can be computed.

The additional sensor positioning method are support in one mode:

UE-Assisted: The UE provides the motion sensors measurements (relative displacement , and reference time stamp to the network side (i.e. E-SMLC) to compute the final position of the UE..

### 8.x.2	Information to be transferred between E-UTRAN Elements

This subclause defines the information (e.g., assistance data, position and/or measurement data) that may be transferred between E-UTRAN elements.

#### 8.x.2.1	Information that may be transferred from the UE to E-SMLC

The information transferred from the UE to the E-SMLC consists of capability information and location measurements or UE position. The supported information elements are given in Table 8.x.2.1-1.

Table 8.x.2.1-1: Sensor Measurement Information that may be transferred from UE to the E-SMLC

| Information | UE-assisted | UE-based/ Standalone |
| --- | --- | --- |
| Displacement Timestamp | Yes | Yes |
| Displacement Information | Yes | Yes |

##### 8.x.2.1.1	UE-assisted mode

In the UE-assisted mode, the UE reports, displacements and reference time stamp.

##### 8.x.2.1.x	UE-based

In UE based mode, the UE reports the displacements and reference time stamp, together with an estimate of the location uncertainty, if available.

The UE should also report an indication of which positioning method(s) have been used to calculate a fix.

##### 8.x.2.1.x	Standalone mode

In UE Standalone mode, the UE reports the displacements and reference time stamp, together with an estimate of the location uncertainty, if available.

The UE should also report an indication of which positioning method(s) have been used to calculate a fix.

##### 8.x.2.1.1.2	UE Displacement and Movement Information

The UE may report movement and displacement information divided into segments, where each segment is for example a piece-wise linear acceleration between two points in time. Given a relative linear acceleration between multiple time points, a UE trajectory can be estimated at the E-SMLC.

#### 8.x.2.2	Information that may be transferred from the E-SMLC to the UE

In this release, no information, e.g. assistance data is transferred to the UE.

### 8.x.3	Motion Sensors Location Information Transfer Procedure

The purpose of this procedure is to enable the E-SMLC to request additional sensor measurements or to enable the UE to provide sensor measurements to the E-SMLC for position calculation.

#### 8.x.3.1	E-SMLC initiated Location Information Transfer Procedure

Figure 8.x.3.1-1 shows the Location Information Transfer operations when the procedure is initiated by the E-SMLC.

Figure 8.x.3.1-1: E-SMLC-initiated Location Information Transfer Procedure

(1)	The E-SMLC sends a LPP Request Location Information message to the UE for invocation of motion sensor positioning. This request includes positioning instructions such as the positioning mode (UE-assisted,), specific requested UE measurements if any, and quality of service parameters (accuracy, response time).

(2)	The UE performs the requested measurements. The UE sends an LPP Provide Location Information message to the E-SMLC before the Response Time provided in step (1) elapsed. If the UE is unable to perform the requested measurements, or if the Response Time provided in step 1 elapsed before any of the requested measurements have been obtained, the UE returns any information that can be provided in an LPP message of type Provide Location Information which includes a cause indication for the not provided location information.

#### 8.x.3.2	UE-initiated Location Information Delivery Procedure

Figure 8.x.3.2-1 shows the Location Information delivery operations for the barometric pressure sensor method when the procedure is initiated by the UE.

Figure 8.x.3.2-1: UE-initiated Location Information Delivery Procedure

(1)	The UE sends an LPP Provide Location Information message to the E-SMLC. The Provide Location Information message may include UE barometric pressure sensor measurements or location estimate already available at the UE.

#### 8.x.3.2	UE-initiated Location Information Delivery Procedure

Figure 8.x.3.2-1 shows the Location Information delivery operations for additional sensor method when the procedure is initiated by the UE.

Figure 8.x.3.2-1: UE-initiated Location Information Delivery Procedure

(1)	The UE sends an LPP Provide Location Information message to the E-SMLC. The Provide Location Information message may include UE sensor measurements or location estimate already available at the UE.

Next Change

## Annex B (informative): Use of LPP with SUPL

The design goal of LPP is to enable it to be used in user plane location solutions such as OMA SUPL ([17], [18]) and this informative annex shows how LPP can be used in SUPL 2.0.

# B.1	SUPL 2.0 Positioning Methods and Positioning Protocols

The following table shows how the 3GPP positioning protocols are supported in SUPL 2.0.

Table B.1-1: SUPL support of positioning methods

| Positioning Protocol: | RRLP (GSM/GPRS/WCDMA/ LTE/WLAN/WiMAX) | RRC (WCDMA) | LPP (LTE) |
| --- | --- | --- | --- |
| Positioning Method: |  |  |  |
| A-GPS (A-GANSS) SET Assisted |  |  |  |
| A-GPS (A-GANSS) SET Based |  |  |  |
| Autonomous GPS/GANSS |  |  |  |
| Enhanced Cell ID |  |  |  |
| Enhanced Observed Time Difference (E-OTD) | (GSM only) | NA | NA |
| Observed Time Difference of Arrival (OTDOA) NOTE 1 | NA |  |  |
| Barometric Pressure Sensors | NA |  |  |
| WLAN | NA |  |  |
| Bluetooth | NA |  |  |
| TBS NOTE 2 | NA |  |  |
| Motion sensors NOTE 3 | NA | NA |  |
| NOTE 1: This includes TBS positioning based on PRS signals, which is only supported in LPP (LTE). NOTE 2: TBS positioning based on MBS signals. NOTE 3: Motion sensors include IMU sensors. |  |  |  |

End of changes