3GPP TSG-RAN WG2 Meeting #105	R2-1902664

Athens, Greece, 25 Feb - 01 Mar 2019

Title:	[DRAFT] LS on UL sharing applicability in different scenarios

Response to:	-

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN WG4, TSG RAN WG1

Cc:

Contact Person:

#### Name:	Tero Henttonen

E-mail Address:	tero.henttonen@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 has been discussing the UE capabilities for UL sharing, and identified that there may be some ambiguities in how they are currently defined in RAN2 specifications:

- It is not clear in the current ul-SharingEUTRA-NR UE capability signalling that this capability indicates UE’s support for UL sharing between E-UTRA band and both for NR SUL and NR FDD bands.

- It is not clear in the UE capabilities that in case of E-UTRA FDD – NR FDD carrier sharing for DL and UL the UE needs to indicate support both for the rateMatchingLTE-CRS and ul-SharingEUTRA-NR UE capabilities.

- RAN2 understood that the RAN4 specifications 38.101-2/3 indicate that the conditions for the UL switching time also apply for certain intra-band contiguous cases, but these are not reflected in the capability description.

RAN2 would like to receive RAN1/4 feedback on these, specifically on how to clarify the capability descriptions:

- Does E-UTRA-NR UL sharing from UE perspective always involve either NR SUL or NR FDD band in the UL of the shared E-UTRA-NR carrier (using TDM only, FDM only, or both TDM and FDM)?

- Does UE supporting UL sharing from UE perspective for a shared E-UTRA-NR carrier (with NR FDD and E-UTRA FDD) also have to support LTE-NR CRS rate matching capability?

- Is the capability for UL switching time used for only UL sharing or also for cases when UE would utilize single UL transmission for intra-band contiguous EN-DC band combination, and whether it is mandatory for UE to report the ul-SwitchingTimeEUTRA-NR

2. Actions:

To RAN1 group.

ACTION: 	RAN2 respectfully asks RAN1 to provide answers to the following questions:

- Does UE supporting UL sharing from UE perspective for a shared E-UTRA-NR carrier (with NR FDD and E-UTRA FDD) also have to support LTE-NR CRS rate matching capability?

To RAN4 group.

ACTION: 	RAN2 respectfully asks RAN4 to provide answers to the following questions:

- Does E-UTRA-NR UL sharing from UE perspective always involve either NR SUL or NR FDD band in the UL of the shared E-UTRA-NR carrier (using TDM only, FDM only, or both TDM and FDM)?

- Does UE supporting UL sharing from UE perspective for a shared E-UTRA-NR carrier (with NR FDD and E-UTRA FDD) also have to support LTE-NR CRS rate matching capability?

- Is the capability for UL switching time used for only UL sharing or also for cases when UE would utilize single UL transmission for intra-band contiguous EN-DC band combination, and whether it is mandatory for UE to report the ul-SwitchingTimeEUTRA-NR

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#105bis		08 - 12 Apr 2018		Xi’an, China

3GPP RAN2#106		13 - 17 May 2018		Reno, USA