3GPP TSG-RAN2#105bis	R2-1905470

Xian, China, April 8-12, 2019

Title:	LS on RACH agreements for NR-U

# Release:	Rel-16

Work Item:	NR_unlic-Core

Attachment: 	None

Source:	RAN2

To:	RAN1

# Contact Person:

# Name:	Ozcan Ozturk

E-mail Address:	oozturk@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1. Overall Description:

RAN2 has discussed 4-step RACH procedures for NR-U and agreed on the following:

- The PREAMBLE_TRANSMISSION_COUNTER is not increased if the preamble is not transmitted due to LBT failure

- As earlier agreed, The POWER_RAMPING_COUNTER is not increased if the preamble is not transmitted due to LBT failure. For this purpose LBT failure indication or equiv. (used for other LBT outcome dependencies) from PHY is used.

- MAC returns to the resource selection step if LBT fails for Msg1 transmission opportunity(ies)

- From MAC perspective, multiple msg1 transmissions are not supported (does not preclude beam sweeping enhancement if decided for NR)

- Actual transmission for MSG1 (LBT success) is used for starting RAR window

- R2 assumes the maximum RAR window size is extended to [20] ms

- We ask R1 regarding the support of multiple MSG3 transmission opportunities

- R2 assumes the range of ra-ContentionResolutionTimer is not extended for NR-U (note this contradicts earlier assumption)

- Either a) the ra-ContentionResolutionTimer is started regardless of the LBT outcome of msg3 transmission or b) ra-ContentionResolutionTimer is started only at successful LBT outcome of msg3 transmission + immediately the UE to restart from RACH resource selection if all MSG3 transmissions fail. FFS

RAN2#105bis has discussed multiple transmission opportunities for msg3 and its impact to msg2. It was assumed that multiple grants can be provided to UE via single or multiple RARs. RAN2 is aware that RAN1 is discussing a proposal for COT sharing between message 2 and 3 and this may be relevant to this discussion.

RAN2 would like to ask RAN1 their feedback on the above RAN2 agreements and considerations on msg3 transmission.

# 2. Actions:

To RAN1

ACTION: 	RAN2 would like to ask RAN1 their feedback on the above RAN2 agreements and considerations on msg3 transmission.

# 3. Date of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting #106			13-17 May 2019	Reno, NV, USA

TSG RAN WG2 Meeting #107	26-30 August 2019	Prague, Czech Republic