3GPP TSG-RAN WG2 #105bis	R2-1905471

Xi’an, China, 8-12 April 2019

Title:	LS on quality report in Msg3 for LTE-M

Response to:

Release:	Rel-16

Work Item:	LTE_eMTC5-Core

Source:	RAN2

To:	RAN1

Cc:	RAN4

Contact Person:

#### Name:	Ritesh Shreevastav

E-mail Address:	ritesh.shreevastav@ericsson.com

1. Overall Description:

For EDT, RAN2 has agreed that a new MAC CE will be defined to carry the Quality Report.

Non-EDT

For the non-EDT case, when eNB provides minimum TBS size of 56 bits in the random access response, RAN2 confirms that two unused bits in MAC subheader can be used to send the Msg3 quality report.

However, if eNB provides the TBS size that is prolonged to the next available TBS size as shown below in Table 1 in the random access response, there is a maximum of 8 bits available for the Msg3 quality report.

Table 1: Possible TBS Sizes for Msg3

|  | Minimum Msg3 TBS [bit] | prolonged Msg3 TBS [bit] |
| --- | --- | --- |
| RRCConnectionRequest | 56 | 72 |
| RRCConnectionResumeRequest (with fullResumeID) | 72 | 88 |
| RRCConnectionResumeRequest (with truncatedResumeID) | 56 | 72 |
| RRCConnectionReestablishmentRequest | 56 | 72 |

EDT

For EDT case, a MAC CE is used and a maximum of 8 bits are available for Mgs3 quality report.

For Msg3 quality report in CONNECTED state, it is possible to use same MAC CE based solution with a maximum of 8 bits available for Msg3 quality report.

2. Actions:

To RAN1, RAN4

ACTION: 	RAN2 respectfully ask RAN1 and RAN4 to consider the above in the discussion on the definition of the Quality Report content and provide feedback on applicability of 2 bits and/or 8 bits for the non-EDT case.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #106	13 - 17 May 2019	Reno, USA

TSG-RAN WG2 Meeting #107	26 - 30 August 2019	Prague, CZ